e-NAV10	Input paper

Agenda item	8

Task Number	6

Author(s)	N.Ward

First draft Performance Standard for Multi-system Receiving Equipment

# 1	SUMMARY

This paper represents the first draft of a proposed Performance Standards for Multi-System Shipborne PNT receiving equipment relevant to e-Navigation.

## 1.1	Purpose of the document

The committee is requested to develop the paper and submit as an input to IMO NAV58.

## 1.2	Related documents

This paper is related to input papers regarding the work item to develop a receiver performance standard (to MSC90), discussion of the Maritime Integrated PNT System, and resulting Gap Analysis.

# 2	BACKGROUND

With PNT defined as one of the core elements of e-Navigation, it is necessary to provide updated and consolidated receiver performance standards to meet these requirements.

# 3	DISCUSSION

The draft document inform discussion at IMO NAV58 and NAV59 meetings, to develop modern PNT receiver performance standards

# 4	REFERENCES

- IMO Resolution A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS).

# 5	ACTION REQUESTED OF THE COMMITTEE

The Committee is requested to:

- Finalise an input to NAV58, suggesting appropriate Performance Standards for a multi-system PNT receiver.

- New Work Item Proposal for IMO NAV58.

SUB-COMMITTEE ON SAFETY OF	NAV 58/xx/xx

NAVIGATION	25 April 2012

58th Session	Original: ENGLISH

Agenda item

Performance Standard for Multi-system Receiving Equipment

Submitted by

| SUMMARY |  |  |
| --- | --- | --- |
| Executive summary: |  | This document sets out a proposed performance standard for ship-borne receiving equipment capable of using signals from multiple systems, including GNSS and its augmentations, as well as terrestrial alternatives. This will avoid the proliferation of standards for individual systems and combinations of systems, reduce delays in introducing new services and enable the full benefits of availability, accuracy, integrity and continuity of multiple systems to be realised. |
| Strategic direction: |  | 5.2 |
| High-level action: |  | 5.2.4 |
| Planned output: |  | 5.2.4.3 |
| Action to be taken: |  | Paragraph 8 |
| Related documents: |  | A.953(23), A.915(22), MSC 86/26 |

Introduction

1.	A generic performance standard for Electronic Position Fixing System (EPFS) receivers is proposed. At present there are separate standards for GPS, GLONASS, Galileo, for differential GPS and GLONASS receivers and for terrestrial systems. Some of these standards are no longer relevant to modern equipment and systems.

2.	As new systems, such as Galileo, Compass and eLoran approach operational status there will be a need to introduce new receiving equipment performance standards for these systems and for each combination of systems (GPS/Compass, GPS/Galileo GPS/eLoran etc) Each standard takes several years to produce, from introducing a new work item through to final approval and because they have been drafted at different times they each represent different stages in the development of the technology.

3.	 MSC 90 has agreed to include the task of developing a multi-system performance standard as a new work programme item for NAV, with a target completion date of 2013 (MSC 90/26).

Requirements

4.	Performance standards should define the requirements, not how they are met. It should be possible to meet those requirements with a variety of equipment and systems, ranging from a standalone, single system hardware receiver to an integrated, multi-system, software defined receiver. It should not be necessary to have individual standards for each system.

5.	Requirements should be set by the users, not determined by what is technically possible, or economically achievable.

6.	Requirements can be set out in terms of four main parameters: availability, continuity, accuracy and integrity.

Further considerations

7.	Introducing a generic performance standard should benefit ship operators, manufacturers and system providers by avoiding costly replacements, simplifying updates, providing market flexibility and making type approval and inspection much easier.

Action requested of the Sub-Committee

8.	The Sub-Committee is invited to note the above observations and to consider the draft Performance Standard annexed to this document.

***

ANNEX

PERFORMANCE STANDARD FOR SHIP-BORNE RECEIVING EQUIPMENT USING MULTIPLE [RADIONAVIGATION] SYSTEMS

1 	INTRODUCTION

  - Global Navigation Satellite Systems such as Galileo, GLONASS, GPS and Compass are space-based positioning, velocity and time systems. Each GNSS space segment will normally be composed of up to 30 satellites, which may be deployed in several orbital planes.  The spacing of satellites in orbit will normally be arranged so that a minimum of four satellites will be in view to users world-wide, with a position dilution of precision (PDOP) <6.

  - Terrestrial Electronic Position Fixing Systems (EPFS) use ground-based transmitting stations. The signals can be processed in a similar way to those from GNSS, using times of arrival from at least three stations to establish a two-dimensional position.

  - An integrated receiver, using a combination of signals from one or more GNSS and terrestrial EPFS, offers improved availability, integrity, accuracy and resistance to interference, increased ease of installation, and the ability to operate in the differential mode (DGNSS, DLoran) when augmentation signals are available.

  - Receiving equipment capable of combining individual measurements from a number of GNSS and terrestrial EPFS to form a single solution can be used for navigational purposes on ships.  Such equipment should, in addition to the general requirements contained in resolution A.694(17), comply with the following minimum performance requirements.

  - These standards cover the basic requirements of position-fixing for navigation purposes only and do not cover other computational facilities which may be in the equipment.

### 2.	COMBINED EPFS RECEIVER EQUIPMENT

2.1	The words “combined EPFS receiver equipment” as used in these performance standards include all the components and units necessary for the system to properly perform its intended functions.  The equipment should include the following minimum facilities;

.1 antennas capable of receiving GNSS and/or terrestrial EPFS signals;

.2 combined EPFS receiver and processor;

.3 means of accessing the computed latitude/longitude position;

.4 data control interface; and

.5 position display or interface to a separate display.

2.2  	The antenna design should be suitable for fitting at a position on the ship which ensures a clear view of the satellite constellations.

### 3.	PERFORMANCE STANDARDS FOR COMBINED EPFS RECEIVER 	EQUIPMENT

The combined EPFS receiver should:

3.1	be capable of receiving and processing the open service signals of one or more GNSS (known as the Standard Positioning Service (SPS) in the case of GPS) and provide position information in latitude and longitude in a suitable worldwide geodetic reference system1 with co-ordinates in degrees, minutes and thousandths of minutes.

3.2	 be provided with means to transform the computed position into data compatible with the datum of the navigational chart in use.  The display and/or any data output should indicate that the co-ordinate conversion is being performed and should identify the co-ordinate system in which the position is expressed;

3.3	operate on at least the L1 frequency signal and range code in the case of Galileo, L1 frequency signal and C/A code for GPS, L1 signal for GLONASS;

3.4	be provided with at least one output from which position information can be supplied to other equipment.  The output of position information should be in accordance with the relevant international standards;

3.5	have static accuracy such that the position of the antenna is determined to within [35]m (95%) in non-differential mode and [10]m (95%) in differential mode with horizontal dilution of precision (HDOP) 4 or position dilution of precision (PDOP) 6;

3.6	have dynamic accuracy such that the position of the ship is determined to within [35]m (95%) in non-differential mode and [10]m (95%) in differential mode with HDOP 4 or PDOP 6 under the conditions of sea states and ship’s motion likely to be experienced at sea;

3.7	be capable of selecting and acquiring automatically the appropriate satellite and/or terrestrially transmitted signals for determination of the ship’s position with the required accuracy and update rate;

3.8	be capable of acquiring position to the required accuracy, within 5 min of powering up;

3.9	be capable of re-acquiring position to the required accuracy, within 2 min, when subjected to a power interruption or loss of signal of 60s;

3.10	generate and output to a display and/or digital interface a new position solution at least once ever 1s;

3.11	provide a minimum resolution of position, i.e. latitude and longitude, of 0.001 minutes;

3.12	generate and output to the digital interface course over the ground (COG), speed over ground (SOG) and universal time co-ordinated (UTC).  Such outputs should have a validity mark aligned with that on the position output.  The accuracy requirement for COG and SOG should not be inferior to the relevant performance standards for heading and SDME;

3.13	have the facilities to process differential correction data fed to it, in accordance with the appropriate ITU-R Recommendations; and

3.14	be capable of operating satisfactorily in typical interference conditions.

### 4.	PROTECTION

4.1	Precautions should be taken to ensure that no permanent damage can result from an accidental short circuit or grounding of the antenna or any of its input or output connections or any of the combined receiver equipment inputs or outputs for a duration of 5 min.

### 5	FAILURE WARNINGS AND STATUS INDICATIONS

5.1	The equipment should provide an indication if the position calculated is likely to be outside the requirements of these performance standards.

5.2	The combined EPFS receiver equipment should provide as a minimum:

.1	an indication within 5s if either:

.1.1 the specified HDOP has been exceeded; or

.1.2 a new position has not been calculated for more than 1s.

Under such conditions, the last known position and the time of the last valid fix, with explicit indication of this state, so that no ambiguity can exist, should be output until normal operation is resumed;

.2	a warning of loss of position.

.3	differential correction status indication of:

.3.1 the receipt of differential correction signals; and

.3.2 whether differential corrections are being applied to the indicated ship’s position;

.4	differential signal integrity status and alarm; and

.5	differential test message display.

Refer to Publication IEC 60945

1 International Terrestrial Reference Frame, or some implementation of it, such as World Geodetic System  (WGS) 84 in the case of GPS, or PZ-90 in the case of GLONASS

2 Publication IEC 61162