e-NAV10	Input paper

Agenda item	8

Task Number	9

Author(s)	Kaisu Heikonen, FTA

Test results on accurate time transfer via DGNSS MF transmissions

# Summary

The attached paper (Annex A) briefly documents results from studies on the time transfer from an accurate time source via communications network to the MF transmitting site, on propagation model for MF transmissions between transmitting site and user equipment and field tests on accurate time transfer via MSK modulated MF DGNSS beacon transmission.

## Purpose of the document

Document is meant to inform WG2 on research done related to MF beacon R-mode and to be considered when updating IALA WWRNP.

## Related documents

Input document “Accurate time transfer to DGPS stations via communications networks” is related to this document.

# Background

# Research on MF beacon R-mode has been done following the recommendations in IALA’s WWRNP. WWRNP encourages Members to contribute to the trials and development of new systems to enhance marine navigation e.g. R-Mode of MF beacons or AIS.

# Action requested of the Committee

The Committee is requested to:

- Note the results from studies and tests made on accurate time transfer over MF DGNSS

- Consider the results from studies and tests when updating WWRNP

Test results on accurate time transfer via DGNSS MF transmissions

This report documents shortly the results from studies and field tests on accurate time transfer via DGNSS MF transmissions commissioned by the Finnish Transport Agency.

The studies were done on the following subjects:

- Time transfer from an accurate time source via communications network to the transmitting station

- Propagation model for MF transmissions between transmitting station and user equipment

The field tests were done on the following subject:

- Accurate time transfer between transmitting station and user equipment via MSK modulated MF DGNSS beacon transmission

Accurate time transfer via communications network

The transition from circuit switched networks to packet switched networks has increased the efficiency of data channels but at the same time complicated time synchronisation and time transfer.

A study by VTT Technical Research Centre of Finland recommends using a closed packet network with a PTP service included if an accuracy of 1 microsecond or less is needed. No field tests were done related to this study but it was estimated based on earlier findings that an accuracy of less than 1µs could be reached (corresponding distance accuracy < 300m). The full report of the study (Accurate time transfer to DGPS stations via communications networks) is available in English language and can be found among the input papers of e-NAV10.

Propagation model for MF transmissions

To be able to compensate the signal delays caused by the MF ground wave propagation path a preliminary propagation model was developed by the VTT Technical Research Centre of Finland. The model included effects from ground surface relief, ground conductivity, frequency and transmitter and receiver heights. It did not include influence from ground surface vegetation and its seasonal variation, atmospheric effects and effects caused by transmitter and receiver equipment.

Results from the model (calculated for southern Finland) showed that:

- Delay caused by the difference between the direct and the propagated path (depending on the surface relief) was around 1-2 ns (0.3-0.6m) over 100km

- Delay on ground wave propagation caused by surface conductivity was around 1.5 µs (450m) over 100 km

Results indicate that it would be essential to create a detailed surface conductivity map for the test area before field tests with mobile unit can be made. The full report of the study is available only in Finnish language.

Accurate time transfer via MSK modulated MF DGNSS Beacon transmission

The test transmissions were carried out using an operational DGNSS Beacon transmitter site. Only modification that was made to the existing equipment was the replacement of the beacon modulator by a test unit.

The test unit was able to convert the RTCM-104 correction information into standard MSK modulated MF signal. In addition it was able to synchronize the transmit frequency (292.5 kHz) as well as the modulation rate and phase into the incoming accurate clock signal.

The accurate time information source used in the field tests was GPS, other options available would have been atomic clock (over network or locally) or MF master-slave synchronization.

The test transmissions were received in a fixed location (40 km from the transmitter) with a measurement receiver. In the receiver the phase of the signal was detected and compared to a reference signal to solve the signal delay. The signal detection and processing was done using standard Matlab program. The sampling time used in the tests was 2-20 sec (which would have been rather long for a mobile receiver).

During the tests time transfer accuracy of 0.8 µs (corresponding to distance of 240m) was reached. It was noted that more advanced signal processing would probably have improved the results. The full measurement report is available only in Finnish language.

Conclusions

A general remark from the research team was that the modulation type used in DGNSS Beacon transmissions is not optimal for time transfer (R-mode). There exist other modulation types like chirp that would be much more suitable for the purpose.

However the field tests indicated that time transfer accuracy of less than 1 µs could be achieved over MF DGNSS Beacon transmissions. Proper modelling of the propagation delay and use of more advanced signal processing on the receiver side would have further improved the results. It seems that the time transfer over network to the transmitting site might become the limiting factor and other options for transmission time synchronisation might have to be used.

There is need for further research on propagation delay modelling as well as for further development of the receiver signal processing. Also more field tests need to be carried out preferably using transmissions from at least two stations simultaneously.

Finnish Transport Agency