### Draft Liaison Note from e-Navigation Committee to ANM Committee

### On Use of AIS for Marking of Man-Made Offshore Structures

The e-Navigation Committee wishes to draw to the attention of the ANM Committee the following points concerning IALA guidance on the marking of man-made offshore structures

- IALA Recommendation O-139 provides comprehensive and clear guidance for the visual marking of Man-Made Offshore Structures (MMOS).

- O-139 appears to be less clear and consistent on the use of other radio AtoNs for additional marking.

  - For MMOS in general, clause 2.1.4 mentions the possible use of

- Maritime radio beacon

- Radar bacon

But there is no mention of AIS

  - For offshore wind farms, clause 2.3.3.1 mentions

- Racons

- Radar reflectors and Radar Target Enhancers

- AIS as an AtoN

  - Clause 2.3.2 concerning marking of individual wind energy structures makes no mention of the additional AtoN noted in clause 2.3.3.1.

- Recommendation A-126 indicates the use of AIS AtoN for marking of MMOS in clauses 4.9 and 4.10

The e-Navigation Committee wishes to suggest to the ANM Committee that: -

- The use of maritime radio beacons for marking of MMOS is no longer appropriate.

- The guidance within O-139 on the use of AIS for marking MMOS could be made more general and more consistent.

Additional information

- An informal poll of a small number of European lighthouses services  showed that presently there are different views on the best use of AIS AtoN for marking of MNOS. There seems to be a  need for further guidance from IALA.