e-NAV10

Agenda item	8 (WG 2)

Task Number	1, 2 and 3

Author(s)	WG 2

Development of Maritime Integrated PNT System

# Summary

The paper focuses on the development of the “Integrated PNT System” in the framework of e-navigation and summarises the discussion results of WG 2 inter-sessional meeting, which was held in Neustrelitz, Germany on 07-09 June 2011.

Chapter 3.1 provides the proposed definition of the “Integrated PNT System” and describes challenges coming from user needs identified within e-navigation and the aimed completion of PNT System under consideration of all service areas and navigation tasks.

Chapter 3.2 contains the specification of primary and secondary PNT output data provided at the ship-side frontend to applications such as AIS and ECDIS.  Aim of the primary PNT output data is the provision of desired position, navigation and time information.  Aim of the secondary output data is the provision of desired assessment results regarding the provided primary PNT data and the PNT system used currently.

Chapter 3.3 proposes the introduction of different integrity levels (scalability of integrity) to take into account their determination and to achieve a harmonised interpretation of assessment results.  The proposed integrity levels could be considered as a first approach to achieve a harmonised indication of reliability of PNT relevant data and used PNT components.

Chapter 3.4 discusses the status of current PNT relevant performance standards and seen challenges coming from definition of “Integrated PNT System” and specification of desired PNT output data. In this frame output data driven performance standards are considered as an efficient alternative for future PS. Seen benefits as well as consequences are outlined.

In Chapter 3.5 the current architecture of the PNT System is shown. By consideration of preliminary identified target functionalities the demand on developments for shore-side and ship-side PNT components is derived.  In the context of shore-side service the extension of services (to support the use of modernised GNSS, to fulfil port requirements, to ensure backup functionality) and the stringent introduction of integrity functionalities are few of the identified challenges.  The “Integrated PNT Module” is responsible as ship-site part of the “integrated PNT System” for the provision of desired PNT data to onboard applications.  The exploitation of ship-side measurement redundancy requires concepts and techniques towards the improvement and indication of reliability of PNT data provision and PNT components in use.

## Action arising from this document

The e-NAV Committee is invited to consider the establishment of an “Integrated PNT System” as a superior objective of e-navigation.  A coordinated development of the “Integrated PNT System” between all involved stakeholders (IMO, IALA, ITU, IEC, RTCM…) should be aimed, promoted and supported by IALA.  This should be communicated by liaisons note(s).

A strong separation between shore-side and ship-side development and standardisation activities is considered critical in relation to harmonisation, interoperability and optimisation of all PNT relevant components, which are aimed for all electronic means by e-navigation.  Contributions of IALA can cover the conceptual and detailed architecture design, the technical gap analysis, the development of the PNT-relevant part of Common Maritime Data & Information Structure and the coordinated enhancement of shore-side Maritime Service Portfolio.  This document can be considered as a first discussion basis for the coordinated development of the Integrated PNT System.  Therefore it is proposed to use this document as IALA input paper to IMO and ITU.

## Related IALA documents

- Related Documents

| Short Name | Title of Document |
| --- | --- |
| [IALA WWRNP] | IALA World Wide Radio Navigation Plan, Edition 1, December 2009 |
| [IALA R-121] | IALA Recommendation R-121: On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz, Edition 1.1, December 2004 |
| [IALA R-129] | IALA Recommendation R-129: On GNSS Vulnerability and Mitigation Measures, Edition 2, December 2008 |
| [IALA R-135] | IALA Recommendation R-135: On The Future of DGNSS, Edition 2, December 2008 |
| [IALA Guidline No. 160] | IALA Guideline No 1060: On Recapitalisation of DGNSS, Edition 1, December 2008 |
| …. |  |

# Background

From references such as [IMO NAV 54/25 Annex 12], [IMO A.915(22)], [IALA R-129] and [IALA WWRNP]:

- Existing and future Global Navigation Satellite Systems (GNSS) like GPS, GLONASS and GALILEO are strategic key elements for resilient PNT data provision.

- Additionally, terrestrial services are used or considered as candidates to improve performance and safety (augmentation services: e.g. IALA Beacon DGNSS, AIS DGNSS, RTK) or to ensure backup functionality (backup services: e.g. eLORAN, R-Mode) respectively to GNSS.

- The interoperability and compatibility of space-based and terrestrial services support their alternatively or complementary application for resilient PNT data provision.

From references such as [IMO MSC.112(73)], [IMO MSC.113(73)], [IMO MSC.114(73)], [IMO MSC.115(73)], and [IMO MSC.233(83)]:

- Several performance standards for ship borne GNSS and DGNSS receivers were developed and approved by IMO in the last decade: GPS/GLONASS, DGPS/DGLONASS, combined GPS/GLONASS, and GALILEO.

- A logical consequence of this sensor related standardisation process is the preparation of PS for multi-system shipborne radio navigation receivers considering the progress in GNSS and equipment.

From references such as [IMO NAV 54/25 Annex 12], [IMO NAV 56/WP5], [IMO A.915(22)],  [IALA R-129] and [IALA WWRNP]:

- An integrated consideration and coordinated application of ship-side and shore-side PNT components should be aimed to:

  - achieve a harmonisation of shared PNT functionalities (ship-side, shore-side);

  - support the optimisation of the  each applicable processing chain for PNT data provision;

  - enable the selection of the best PNT result under consideration of different applicable processing chains in the future.

- Approaches aiming at the fulfilment of the e-navigation user needs “reliability” and “indication of reliability” should be identified and developed.  Their ongoing utilisation within assigned processing chains for PNT data provision requires improved adjustments between single developments.

- An overall specification of accuracy assessment functionalities (algorithm, software) and shared responsibilities (ship-board, shore-side) is necessary towards a harmonised meaning and understanding of integrity monitoring and assessment results.

An overview about components in practice and prospective components for resilient PNT data provision is shown in the following figure. The future application of light grey shown components depends on their technical benefits in relation to technical requirements on the “Integrated PNT System”.  Though the figure should not be understood as a first architecture design of the “Integrated PNT System” it highlights the different working areas for this purpose:

- the completion of Maritime Service Portfolio under consideration of service areas and related performance requirements on PNT data provision;

- the decision about the maritime application and benefit of space based augmentation services;

- the application of modernised GNSS and the utilisation of included redundancy;

- the provision and standardisation of multi-system radio navigation to enable the ship-side utilisation of recognised space-based and shore-based PNT relevant services and signals;

- the use of additional ship-side sensors to improve the resilience of PNT data provision;

- the specification of techniques for value-added data processing to ensure the provision of information describing the PNT data and system integrity (I) as well as alert and status messages (A+S).

- Overview of PNT components (dark grey: components in practice; light grey: additional components)

# Development of “Integrated PNT System”

## Definition of „Integrated PNT System“

Definition:

The “Integrated PNT System” specifies the required overlay of satellite based, ashore and aboard components, whose integrated use ensure the accurate and reliable provision of ships’ position, navigation, and time (PNT) data and assigned integrity data (PNT system and data integrity) to applications during all phases of vessel navigation in a timely, complete and unambiguous manner.

The “Integrated PNT System” monitors the current HW and SW configuration in use as well as the complete PNT output data to generate PNT relevant alerts and to provide PNT status messages (reporting) in a timely, complete and unambiguous manner.

Challenges:

The resilient provision of PNT data should be ensured for all service areas such as ocean, coast, port, and inland waterways. Different service areas result into different performance requirements on PNT data. For the detailed architecture design and gap analysis of the “Integrated PNT System” the necessity is seen to complete and to harmonise the specification of performance requirements in relation to all desired PNT output data.

- Service areas of PNT data provision

The generally admitted processing chain for PNT data provision is composed by GNSS, PNT relevant augmentation and backup services as well as additional ship-side equipment. The application of different processing chains is necessary to ensure the fulfilment of performance requirements in relation to all identified service areas.

- Generally admitted PNT processing chain

In the frame of e-navigation [IMO Nav56] user needs such as

- Identification of reliability by automatically assessment of all navigation relevant data and by graphical indication of assessment results;

- Improvement of reliability by reduction of failures and malfunctions of electronic equipment;

- Assessment and quantification of reliability.

were identified.  These user needs should be taken into account for on-going developments of the “Integrated PNT System” such as detailed architecture design of PNT system, completion of Maritime Service Portfolio (MSP), and specification of all PNT relevant data within Common Maritime Data & Information Structure.

## Primary and secondary PNT output data

In the minimum the “Integrated PNT System” should provide the following primary and secondary output data at the ship-side frontend to applications

- Specification of PNT output data

Primary output data

Aim of the primary PNT output data is the provision of desired position, navigation, and time information:

- Position data describes the latitude, longitude and altitude of ship’s consistent common reference point (CCRP) in a global coordinate system (WGS84);

- Navigation data describes SOG, STW, ROT, Heading and COG of vessel;

- Timing data describes the momentary time and date in a common time system (UTC).

Comment: Especially an extension of desired navigation can be expected in the future.  Yaw, pitch, and roll angles and their rates are suitable indicators for the description of ships’ manoeuvring potential.  Air draught and depth (under keel clearance) are navigation relevant data, which will be derived by application in combination with further externally provided data such tidal and ENC.

Secondary output data:

Aim of the secondary output data is the provision of desired assessment results regarding the provided primary PNT data and the current used PNT system:

- Integrity of position data describing either the estimated accuracy of position data or indicating the fulfilment of momentary accuracy requirements, if they are available;

- Integrity of navigation data describing either the estimated accuracy of navigation data or indicating the fulfilment of momentary accuracy requirements, if they are available;

- Integrity of timing data describing either the estimated accuracy of timing data or indicating the fulfilment of momentary accuracy requirements, if they are available.;

- Integrity of PNT system describing either the usability of the PNT data products in a scalable manner or the fulfilment of all current requirements, if they are available;

- Alerts announcing abnormal situations and conditions of the “Integrated PNT System” requiring attention, decision or action in the frame of ship navigation.  Alerts are divided in three priorities: alarms, warnings and cautions;

- Status messages describing the current HW and SW configuration in use (or their change) for automatic reporting and certification purposes.

## Indication and quantification of reliability

The determination of data and/or system integrity is considered as suitable approach for the indication and quantification of reliability of provided data and used systems. For this purpose the implementation of integrity functionalities is necessary.  Integrity functionalities can be realised by integrity monitoring or integrity modelling or a combination of both.

Data Integrity

Data integrity is additional information per provided primary PNT output data, which is provided in a timely, complete and unambiguous manner to the users to describe, if the specific data should be used or not under consideration of accuracy requirements for each navigation phase.

Data integrity is given, if the specific output data

- is delivered at the expected time interval;

- is provided in the specified formats;

- fulfils the specific accuracy requirements (application specific (dynamic) thresholds).

Optional:

- is delivered in time with the required data attributes (e.g. time stamps).

System Integrity

System integrity (user view on „Integrated PNT System“) is the ability of the „Integrated PNT System“ to provide the user with all required information (data, integrity, status messages) in a timely, complete and unambiguous and accurate manner, and alerts within a specified time when the system should be used with caution or not at all.

System integrity is given, if

- data integrity is fulfilled for all primary PNT output data;

- output data (or attributes) describing the data integrity are provided in a timely, complete and unambiguous and accurate manner (data format and content);

- additional secondary  PNT output data (e.g. alerts, status messages, configuration in use) are provided in a timely, complete and unambiguous and accurate manner (data format and content).

Optional:

- reiteration of received system integrity data from other systems (MSI, ..), if possible;

- all information are delivered in time with the required data attributes (e.g. time stamps).

Quantification of Reliability by Scalability of Integrity

Quantification of reliability requires the specification of reliability levels in form of performance parameters and harmonised instructions for their determination and interpretation.  That requires the completion of accuracy requirements for all desired primary PNT data under consideration of identified service areas and navigation tasks.  Additionally, integrity requirements should be scalable to take into account their determination and to achieve a harmonised interpretation.

Following integrity levels are proposed:

- Low Level: Plausibility checks:

  - based on thresholds or value ranges;

  - requires complete and fixed specification of thresholds and value ranges;

  - enables only the detection of large errors outside the nominal error ranges.

- Medium Level: Consistency check based on comparison of results of different sensors:

  - supports only the assessment of relative accuracy;

  - uncorrelated (avoiding single point data propagation) measurement techniques should be aimed to improve the number of detectable errors in comparison to undetectable errors;

  - an unambiguous result of consistency checks requires specified criteria based on identical determination methods  harmonisation;

  - for unambiguous identification of best PNT data and/or best integrity monitoring functionality the specification of selection method is necessary (indication of reliability).

- High Level: Estimation of accuracy and assessment

  - the estimation techniques must be clarified: monitoring, measurement model, fusion engine to assume multiple hypotheses approach …?

  - different methods can result in different integrity results, therefore the demand on harmonisation of determination methods is seen;

  - the possibility that terrestrial services could support the ship-side integrity monitoring should be taken into account.

The maturity of the chosen approach corresponds to the inverse of the integrity level – the complexity reflects the potential benefit.

## Performance standards and technical requirements

Status

Currently, performance standards (PS) are provided with respect to single sensors or services.  In the normal case the minimum on technical requirements are described. Sensor and service based PS are not necessarily the correct approach for the improved “Integrated PNT System”.  Seen reasons are:

- Due to their historical generation different documents (sensor, service) with inhomogeneous description result into difficult comparability.  Mostly accuracy requirements are given only for specific output data (single P or N or T data).

- The sensor based PS are improper to introduce/specify integrity requirements (only possible, if redundancy is available).

- Improvement of reliability and indication of reliability requires the specification of a common set of requirement types, which should be applied to all navigation relevant sensors and systems: accuracy, precision, integrity, availability and continuity.

- These implicate the demand on introduction of identifiers (parameters) describing the reliability in a common and unambiguous manner.

Challenge

Under consideration of identified user needs in e-navigation the completeness of requirements should be achieved in minimum:

- Introduction of a common performance requirement structure (accuracy, precision, integrity, availability, continuity) supports the completion of requirements and can result in easy comparability;

- For this purpose a common meaning of data and system integrity is necessary;

- A scalability of integrity is necessary with respect to the degree of applied facilities and services (downward and upward compatibility, presence and availability of equipment) – see addendum;

- A common definition of precision is necessary, if it is proposed as an additional requirement.

Approach

Output data driven PS could be considered as an efficient alternative for future PS. They

- support the scalability and completeness of requirements;

- support the establishment of the universal maritime data model;

- allow the application of different sensor and system combinations (modular and open architecture, minimum set of sensors);

- support the scalability with respect to service areas and applications;

- can reduce the number and the realisation process of PS in the frame of e-navigation;

- enable the crossover from current state (minimum) to e-navigation (maximum resp. user needs);

- and support the desired improvement of PNT resilience, whereby fall-back modes (with reduced accuracy) should not be ruled out by this approach.

The introduction of output data driven PS requires rethinking of PS realisation and coordination with other PS activities e.g. INS.  A critical point within determination of overall PNT output data integrity can come from PNT source data, which are influenced by single point failures (e.g. GNSS time).  This should be considered, if integrity assessment techniques are developed and realised.

## Architecture

The detailed architecture design of the “Integrated PNT System” should be driven by the identified target PNT functionalities.

- Target Functionalities of “Integrated PNT System”

Shore-side target functionalities should be:

- provision of GNSS augmentation and correction signals for ships’ navigation in coast, port and for automatic docking;

- provision of GNSS backup services to reduce the vulnerability of GNSS and GNSS based augmentation services;

- provision of PNT relevant MSI such as service level area and tidal information as well as management and assessment of PNT system in use;

- management of PNT system.

Ship-side target functionalities should be:

- provision of primary PNT data;

- provision of secondary PNT data;

- provision of PNT relevant alerts and status messages.

For their realisation following demand on development is seen:

Completion of shore-side part of PNT System: Maritime PNT Service Portfolio

The modernisation of IALA Beacon DGNSS is a running activity und should consider the improvement of integrity functionalities as soon as possible.  This requires:

- the harmonisation of integrity monitoring concept (LIM, FFIM);

- the specification of suitable message types to enable the transmission of improved integrity information;

- the unambiguous specification of assessment methods (data and system integrity per supported service) and applied thresholds;

- and may be the certification of safety-critical services (MSP).

Future maritime augmentation systems should support multi-GNSS approach (redundancy) and performance driven utilisation concept (scalability of accuracy and integrity) based on

- different single-frequency and multi-frequency services per available GNSS;

- different single-frequency and multi-frequency services per applicable GNSS combination;

- monitoring of applicable GNSS and related services to provide information describing data and system integrity.

Future backup systems are necessary to reduce the vulnerability of GNSS based services by provision of suitable backup services.

The introduction of new augmentation and backup services results into additional requirements on shipboard PNT Module and should be considered, if new PS will be developed.

Completion of ship-side part of PNT System: Shipboard PNT Module

The exploitation of GNSS and SBAS modernisation progress should be forced towards resilient PNT data provision.  Currently used PNT core sensors are GNSS and GNSS/DGNSS receivers.  The future use of multi-GNSS navigation receivers should be considered as consequence of on-going modernisation process of GNSS, SBAS and assigned equipment.  This requires:

- that single maritime multi-GNSS receiver should be able to provide the best PVT data (position, velocity, time) under consideration of applicable PVT determination techniques: single-  or multicarrier based signal processing, use of single GNSS or of combined GNSS and use of augmentation services or not;

- for this purpose the multi-GNSS receiver should:

  - include parallel processing chains per applicable technique for PVT determination;

  - utilise integrity information provided by GNSS, DGNSS and SBAS;

  - include assessment functionalities to identify the best PVT result;

  - include integrity monitoring functionalities for the provision of PVT data integrity such as RAIM per applied PVT technique as well as consistency and plausibility test between PVT techniques and used signals.

Due to the vulnerability of GNSS the establishment of a terrestrial backup system is aimed for GNSS independent position determination.  For this purpose the PNT Module would require either a terrestrial radio navigation receiver or multi-radio navigation receiver, which enables the PVT determination with GNSS/DGNSS as well as with the terrestrial backup system (TBS).  This requires:

- that maritime multi-radio navigation receiver should be able to provide the best PVT data (position, velocity, time) under consideration of applicable PVT determination techniques: single-  or multicarrier based signal processing, use of single GNSS or of combined GNSS, use of augmentation services or not, GNSS with TBS, TBS with GNSS, and TBS;

- for this purpose the multi-radio navigation receiver should:

  - include parallel processing chains per applicable technique for PVT determination;

  - utilise integrity information provided by GNSS, DGNSS, SBAS, and TBS;

  - include assessment functionalities to identify the best PVT result;

  - include integrity monitoring functionalities for the provision of PVT data integrity such as RAIM per applied PVT technique as well as consistency and plausibility test between PVT techniques and used signals.

A multi-sensor approach for PNT data provision is necessary, if the complete provision of primary and secondary PNT data is aimed.

- Shipboard PNT Module and its assignment to the Integrated Navigation System and the Integrated PNT System

In this context the shipboard “PNT Module” is considered as the needed overlay of PNT relevant sensors and equipment under consideration of identified performance requirements.  The data-fusion based determination of PNT data and their assessment requires a data processing tool, the so-called “PNT Unit”. Currently, the “PNT Unit” is distributed on the GNSS/DGNSS receiver and on the Integrated Navigation System (INS), whereby GNSS signal relevant tests (RAIM) are realised within the GNSS/DGNSS receiver and PNT data relevant tests (plausibility, consistency, latency, validity,…) are task of the INS ([IMO MSC.252(83)], [IEC 61924]).

Seen disadvantages of this concept are amongst others:

- harmonisation of receiver internal integrity monitoring (multi-GNSS receiver, multi-radio navigation receiver) is difficult because of different receiver firmware and dedicated PVT determination techniques;

- exploitation of modernised navigation services (GNSS, DGNSS, SBAS, TBS) is a receiver internal task and can be utilised only for PVT data provision;

- reduced flexibility regarding implementation of new technologies (sensors, data-fusion based PNT algorithm);

- reduced flexibility regarding indication and improvement of reliability for PNT data provision.

The introduction of a central PNT Unit (may be as part of the INS) would enable that:

- flexible extension of architecture (new sensors such as IMU) is possible;

- introduction of maritime PNT integrity concept is supported;

- introduction of common maritime data model is supported;

- harmonisation of maritime processing techniques (PNT data and integrity determination) can be achieved;

- one PS for P, N and T is necessary in the future;

- INS integrity assessment and alert management (see user need: automatic indication of reliability…) can be improved.

## Role of IALA

The development and establishment of the „Integrated PNT System“ should be considered as a cooperative task of IMO, IALA and further organisations (such as IEC, ITU-R, RTCM).  IALA has the expertise and background to promote the development process.  This should be communicated by suitable liaison note(s) on the one hand.  On the other hand this document should be provided as input paper to IMO to force the coordinated development of the “Integrated PNT System”.

# References

| [IMO A.915(22)] | IMO Resolution A.915 (22): Revised maritime policy and requirements for a future Global Navigation Satellite System, November 2001 |
| --- | --- |
| [IMO MSC.86(70)] | IMO Resolution MSC.86(70): Adoption of new and amended performance standards for navigational equipment. |
| [IMO MSC.112(73)] | IMO Resolution MSC.112 (73): Adoption of the revised performance standard for shipborne Global Positioning System (GPS) receiver equipment. |
| [IMO MSC.113(73)] | IMO Resolution MSC.113 (73): Adoption of the revised performance standard for shipborne GLONASS receiver equipment. |
| [IMO MSC.114(73)] | IMO Resolution MSC.114 (73): Adoption of the revised performance standard for shipborne DGPS and DGLONASS maritime radio beacon receiver equipment. |
| [IMO MSC.115(73)] | IMO Resolution MSC.115 (73): Adoption of the revised performance standards for shipborne combined GPS/GLONASS receiver equipment. |
| [IMO MSC.233(83)] | IMO Resolution MSC.233 (83): Adoption of the revised performance standards for shipborne GALILEO receiver equipment. |
| [IMO MSC.252(83)] | IMO Resolution MSC.252(83), Annex 30: Adoption of the revised performance standards for Integrated Navigation Systems (INS). |
| [IMO NAV 54/25] | IMO MSC Report Annex 12: Draft strategy for the development and implementation of e-navigation. |
| [IMO NAV 56/25] | IMO MSC Report of the Working Group: Development of an e-navigation strategy implementation plan. |
| [IMO NAV 57/WP5] | SUB-COMMITTEE ON SAFETY OF NAVIGATION, 57th session: DEVELOPMENT OF AN E-NAVIGATION STRATEGY IMPLEMENTATION PLAN. Report of the Working Group |
| [IEC 61924] | IEC 61924 (2006-05) Ed. 1.0 – englisch: Maritime navigation and radiocommunication equipment and systems - Integrated navigation systems - Operational and performance requirements, methods of testing and required test results. |

# Action requested of the Committee

The Committee is requested to:

- Consider the establishment of an „Integrated PNT System“ as one of superior objectives of e-navigation.

- Promote the establishment of an „Integrated PNT System“ by IALA contributions to conceptual and architectural design as well as technical gap analysis.

- Initiate or support the preparation of new IMO Performance Standards in the frame of “Integrated PNT System”.

- Support the preparation of new IMO Performance Standards for a multi-system radio navigation receiver in a first step.

- Support the development of the shipboard PNT Module including PNT Unit.

- Promote the development of augmentation services for port and docking and their approval by IMO

- Consider the enhancement of an „Integrated PNT System“ as high-priority task of IALA working program  in cooperation with IMO and related organisations and communicate this by liaison note(s),

- Setup a time plan for IALA contributions which indicates possible inputs regarding the time schedule of other relevant international organisations (such as IMO, ITU-R, IEC, RTCM).