e-NAV10-WG5-WP09; 29 September 2011

Working Paper on shipboard architecture functional requirements (“shipboard architecture shopping list”) as seen from ashore

Motivation for consideration:

There is a requests from the IMO Correspondence Group on e-Navigation for contribution to the ongoing “gap analysis” (e-NAV 10/7/4 refers). In particularly, Question No. 6 refers:

- Question No. 6: “further develop the detailed architecture of both the ship and shore sides, as appropriate, (…) using the overarching e-navigation architecture as a framework”.

The shore side, when providing technical services as part of the future Maritime Service Portfolios (MSPs), needs to rely on a well defined, harmonized shipboard environment which interacts with the shore-provided technical services in a well defined manner on the functional and physical shipboard interface to the communication links to shore (and other ships).

Due to the scalability requirement of the e-Navigation strategy, there is a need to define a minimum shipboard functional technical architecture as well as several degrees of more sophisticated shipboard functional technical architectures to make optimum use of the services provided by the future Maritime Service Portfolios.

Relevant reference documents for consideration:

- IMO NAV57 WP.6 (in part quoted in e-NAV 10/4/3)

- IMO MSC.252(83) (= Revised Performance Standards for Integrated Navigation Systems (INS)) (=MSC83/28/Annex 30)

- IMO NAV54/25/Annex 5 (Draft SN Circular on Guidance for the Application of the Modular Concept to Performance Standards)

- e-NAV 10/7/INF 1 – potential migration path for shipboard architecture developed by Andy Norris

- different working papers of WG5

Initial functional requirements for the (future) shipboard architecture (“shopping list”):

- There should be defined a generic shipboard technical architecture in functional terms as an architectural and design reference.

- This shipboard architecture should include state-of-the-art concepts. This may include hierarchical functional layers combined with a modular device concept as well as flexible interface sentence semantics. (Some details of a future shipboard architecture are provided already in the figures in the attachment, which are pulled together as a collection from various sources).

- It is noted that IMO has created the INS Performance Standards and associated guidelines. That modular Integrated Navigation System (INS) may therefore be considered as the architectural framework for the shipboard technical architecture.

- The concepts of “modules” should be clarified, as there appears to be different understandings regarding the concept of a “module”:

  - “Modules” in the sense of the relevant IMO documentation, namely MSC.252(83) and SN Circular on the Modular Concept to Performance Standards (see above), are: “Integration of Information” (Module A), “Task related requirements for INS” (Module B), “Alert Management” (Module C), “Documentation Requirements” (Module D). These modules could be characterized – from an ontological point of view – as “requirement modules” or “Performance Standards modules”, as they provide a well defined structure of the requirement basis which is the essential content of any Performance Standard.

  - A “module” as understood by e-NAV Committee is a defined set of closely related technical functionalities.

  - It should be noted that both concepts are fully legitimate. However, without reference to the specific domain there may be room for misinterpretation. A harmonization could e. g. be thus achieved by adding a designator such as “PS-Module” for modules in the sense of the present INS-PS and “[Tech-]Module” for the modules in the sense of the functional technical architecture.

  - Such clarification would essentially fulfill the harmonization objective of e-Navigation.

- There should be further defined additional modules for the shipboard INS in the sense of the functional technical architecture (“Tech-Modules”), in particular the PNT- and the AIS/COM-Modules. As far as the PNT module is concerned, the reason is that the shore side, as the recipient of the shipboard electronically determined position of a ship, wants to have assurance, that the relevant data is determined onboard with appropriate quality. As far as the AIS/COM module is concerned, a similar logic applies, namely that the transmission of data ship-shore/shore-ship (including digitally encoded speech) is done with appropriate quality.

- An “e-Navigation INS” should be defined, based on the present INS definitions.

- There should be defined a Capability Negotiation Module (“Tech-Module”) for the shipboard INS. This functional module would interact with the shore side and would provide the shore side with information about the capabilities of the shipboard electronic environment. This in turn would be needed by the shore side to tailor the data exchange with the shipboard electronic environment to the functionalities supported on board.

- A concern is the present technical integrity and quality of shipboard data received ashore (and by other ships). Integrity and quality should be improved, taking into account the data transaction chain(s) from initial shipboard source(s) of the data, through the different stages, to the transmission point(s) ship to shore, as a whole. In particular, reference to the emerging Common Maritime Data Structure (CMDS) by the shipboard electronic environment may be instrumental to achieve that goal.

- Any and all data transmission from ship to shore should include a full set of integrity and quality parameters. This would allow the recipient of any such transmission a better evaluation of the data received.

- The resulting requirements from the shipboard electronic environment to the CMDS, to S-100 (as the underlying methodology standard for CMDS) and to the technical services to be defined in the Maritime Service Portfolios (MSP) should be defined and brought to the attention of the appropriate stakeholders.

- It should be elaborated, whether and which unique technical identifiers are needed to achieve the above integrity goals.

- In order to maintain the integrity and quality, a flexible, safe and secure update management process for shipboard equipment should be established and applied.

Liaison requirements:

It is understood, that to achieve the above features, informal liaison with relevant technical groups would be needed, such as the CIRM e-Navigation group, or the IEC WGs 10A (INS) and 6 (Interfacing).

Attachment: Relevant technical architectural diagrams from different reference document

Figure A-1: Shipboard e-Navigation system architecture with functional building block „Data Evaluation + Data Storage“ further developed in functional modules
(developed 2008 by AIS TWG at their Rotterdam intersessional meeting)

--- Andy Norris diagram

Note: By example, IALA presently is working on future modules of this kind, namely and for example the PNT module, the AIS/COM module. Also, the “Alert Management” module of the INS-PS (“Module C”) could thus be easily translated into an “alert management module” in the sense of the technical functional architecture; also, similarly the human machine interface stipulations of “Module B” of the INS-PS could be translated into an appropriate User Operational Presentation Surface module in the technical functional architecture sense.

By example, there is a precedence for this kind of functionality by the AIS Application Specific Message Function Identifier “Capability Interrogation”, presently defined at ITU-R M.1371.