e-NAV11	Input paper

Agenda item	8.2

Task Number	7

Author(s)	WG2 Correspondence group on PNT

Report of Correspondence Group Meeting

of IALA eNAV working group 2

28th Feb. - 01st  March 2012

# Summary

An inter-sessional correspondence group meeting of e-Navigation Working Group 2 was held in Brest, France, on 28th of February to 01st March 2012. The meeting was supported by CETMEF. The main objective of the meeting was to consolidate the following two papers:

- Draft Performance Standards for Multi-system Radio Navigation Receiver (MSR)

- Resilient PNT concept

The updated versions of the papers were uploaded to the teamsite “Maritime integrated PNT system”. Both papers should become input papers for eNAV11. It is proposed that both papers are combined as a common input paper for NAV58.

# BACKGROUND

### 2.1.1	Opening, general information

Michael Hoppe and Pierre-Yves Martin welcomed the group and gave a brief overview about the proposed agenda. Each participant introduced himself.

2.1.2	Presentations and Discussion on 28th of February

- CETMEF (presentation given by Eric Vassor)

- Short reflection of the tasks of the correspondence group dealing with the concept of Maritime Integrated PNT System(leaded by Michael Hoppe)

    - The task was induced by an action item inside the report of the 10th Meeting of e-Navigation Committee (26th – 30th September 2011) to contribute to the development of an input paper to IMO regarding the PNT system concept  and to prepare an input paper to e-NAV11 regarding Performance Standards for multi system radio navigation receiver (MSR).

    - It was noted that a request for a new work item (MSR Performance Standard) has been submitted to MSC90 and if approved, NAV58 will review the standard.

    - The information was given that Alan Grant from General Lighthouse Authorities of the United Kingdom and Ireland is established as the new chairman of the IALA PNT working group.

- Integrated PNT System and the challenges coming from e-Navigation (presentation given by Evelin Engler)

- GLA view on the need for Multi-System Radio Navigation Receiver (presentation given by Alan Grant)

- LORAN with a focus on eLORAN, DLORAN and experimentations derived in the area of Brest (presentation given by Thierry Desnaes)

- Discussions about different documents with expected influence on the PNT-Unit as modular part of the INS (leaded by Hannu Peiponen)

    - MSC.252(83) Annex 30 (in version of 8th of October 2007) with a specific focus on the differences between alarms, warnings and cautions  concerning the PNT outputs it requires complete new sentences concerning PNT data extended by integrity information to avoid misinterpretations by using older equipment  see IEC61924-2INS.pdf  sentences for advanced alert management

    - MSC.302(87) Annex 21 (in version of 17th of May 2010)

    - Summarizing: the PNT will deliver new data sets to improve the reliability, whereby the interpretation of the content (alarms/warning/caution-level) is furthermore interpreted by the INS itself taking into account different navigational tasks

    - a discussion was started about the best solution concerning the PNT working approach to deliver PNT data as well as integrity data  the parallel processing offers the opportunity to deliver at least one result independent on specific application requirements  it has to be clarified what is the real need related to the user site

The meeting closed at 17:00.

2.2	Follow-up meeting

The WG reconvened on 29th of February and 1st of March.

29th of February 2012

The meeting was opened by Michael Hoppe. It was decided to split the group to work on the both input documents in parallel:

NAV 58_ Multi-system Receiver PS (first comments).doc

 A. Grant

 H. Peiponen

 P.Y. Martin

2012-02-23-DraftIMO-Paper-ResilientPNT.doc

 E. Engler

 K. Heikonen

 M. Hoppe

 T. Noack

In the afternoon session the document NAV 58_ Multi-system Receiver PS (first comments).doc was reviewed by the whole group to get a common understanding of the content. During this session it was addressed that there is a need for defining specific requirements concerning accuracy and integrity for different phases of navigation.

Output 1: 	NAV 58_ Multi-system Receiver PS (first comments) -WG2 draft for discussion - 			tidied output from Brest meeting.docx

The group had a lengthy discussion on the best method of capturing the different user requirements in a suitable, consolidated, manner.  They developed a means of defining different performance requirements in terms of categories CAT I through to CAT IV, with the latter being the most stringent.  On review with the correspondence group, it was felt that the name “CAT” could cause confusion with the aviation community, with the potential to apply the wrong requirement.  It was also felt that this approach could be very powerful going forward, but for the time scale available for the development of the PS, it would be better to restrict the paper to existing IMO requirements.  Therefore the decision was made to link the requirements in the PS to IMO A.915 & A.1046.   It is recommended that the main WG2 committee consider reviewing the categorisation of requirements at a future meeting.

2.4	Follow-up meeting

1st of March 2012

The meeting was opened by Michael Hoppe. The meeting started with a discussion about the discrepancies between the IMO Resolution A.1064 and IMO Resolution A.915(22) concerning the required accuracy in different areas of maritime navigation.

Output 2: 	2012-02-28-DraftIMO-IALA-Paper-PNT-Concept_rev1.doc

The group thanks CETMEF and especially P.Y. Martin for hosting the meeting.

The meeting was closed 12:00 LT.

ANNEX A: Participant List

| Name | First Name | Company |
| --- | --- | --- |
| Engler | Evelin | DLR Institute of Communications and Navigation 17235 Neustrelitz, Kalkhorstweg 53 |
| Grant | Alan | Research and Radionavigation The General Lighthouse Authorities of the United Kingdom and Ireland The Quay, Harwich, Essex. UK. CO12 3JW |
| Heikonen | Keisu | Finnish Transport Agency, Traffic Management P.O.Box 33, FIN-00521 Helsinki |
| Hoppe | Michael | Traffic Technologies Centre GERMAN FEDERAL WATERWAYS AND SHIPPING ADMINISTRATION Am Berg 3, 56070 Koblenz (Germany) |
| Martin | Pierre-Yves | Pôle Radionavigation CETMEF/DT/DSMF/DSMF/PRn 2 rue Pierre Bouguer, France |
| Noack | Thoralf | DLR Institute of Communications and Navigation 17235 Neustrelitz, Kalkhorstweg 53 |
| Peiponen | Hannu | Furuno Finland FURUNO FINLAND OY Niittyrinne 7, P.O.Box 74 FI-02271 Espoo, Finland |

Temporary participants

| Name | First Name | Company |
| --- | --- | --- |
| Denaes | Thierry | DCNS Division Services CS 72837, F 29228 Brest Cedex 2 |
| Manchard | Jacques | Ministry of Ecology, Suitenable Development, Transport and Housing Arche Sud, 92055 La Defense cedex – France |
| Vassor | Eric | CETMEF Head of the Department Maritime and Waterways safety 2 rue Pierre Bouguer, France |

ANNEX B: Document List

Input Documents for Meeting:

| Number | Title |
| --- | --- |
| 01 | Preliminary Agenda |
| 02 | 2012-02-23-DraftIMO-Paper-ResilientPNT.doc |
| 03 | NAV 58_ Multi-system Receiver PS (first comments).doc |
| 04 | Note to IALA e-NAV WG2 on IMO process for PNT |
| 05 | Draft WP - Identified gaps for the future integrated PNT-System. |

Output Documents (Proposed Input to e-NAV11):

| Number | Title |
| --- | --- |
| 01 | Report of the Meeting |
| 02 | Updated version with title “Modular and open concept of Integrated PNT System” |
| 03 | Updated version of “Performance Standard for Multi-system Navigation Receiver Equipment” |