e-NAV11  	Input paper

Agenda item	8.3

Task Number	7

Author(s)	WG2 Correspondence group on PNT

DEVELOPMENT OF AN E-NAVIGATION STRATEGY IMPLEMENTATION PLAN

Modular and open concept of Integrated PNT System

# Summary

This document describes a modular and open concept of Integrated PNT System, which can meet e-navigation user needs such as improvement and indication of reliability. This concept takes into account the enhancement of radio navigation equipment and meets the modular structure of INS.

# Background

A submission to MSC 90 was made by France to put performance standards for a multi-system receiver on the work program for NAV. If the proposal is approved then it is likely that work would begin at NAV 59 in 2013 and if completed at that session, a resolution on the new standards could be approved by MSC 93 (May 2014). If further work is needed at NAV 60, the standards could be approved at MSC 94 (Nov 2014).

If the proposal will not approved at that session, then the next opportunity to get the subject on the work program would be at MSC 91 in November. However, in that case, it might be better to propose the broader topic of resilient PNT for e-Navigation, or Integrated PNT, in order to ensure stronger support.

# Discussion

To further support this initiative it could be helpful for IALA to send an Information Paper to NAV 58, setting out the case for a modular and open Integrated PNT System, showing the architecture envisaged and the role of the multi-system receiver in it.  A revised multi-system receiver standard could be included as an annex, to initiate discussion at NAV 58.

For this purpose the PNT correspondence group has drafted an information paper on a modular and open concept of Integrated PNT System as provided in ANNEX A.

# Action requested of the Committee

The Committee is requested to:

- Consider the draft input paper on a modular and open concept of Integrated PNT System Annex A.

- Further develop the proposed concept at eNAV11.

- Forward the proposed modular and open concept of Integrated PNT System together with the revised performance standard on a multi-system receiver to IMO NAV58 (proposed output of eNAV11).

| - SUBCOMMITTEE ON SAFETY OF NAVIGATION 58th Session Agenda Item … | - NAV58/…… Draft 1 Jan 2012 Original: English |
| --- | --- |

DEVELOPMENT OF AN E-NAVIGATION STRATEGY IMPLEMENTATION PLAN

Modular and open concept of Integrated PNT System

Submitted by IALA

| SUMMARY |  |
| --- | --- |
| Executive summary: | This document describes a modular and open concept of Integrated PNT System, which can meet e-navigation user needs such as improvement and indication of reliability. This concept takes into account the enhancement of radio navigation equipment and meets the modular structure of INS. |
| Strategic Direction: | 5.2 |
| High-level Action: | 5.2.6 |
| Planned output: | 5.2.6.1 |
| Action to be taken: | Paragraph 3 |
| Related documents: | MSC 85/26 (Annex 20 and 21); Res. A.915(22); Res. A.860(22); Res. MSC.112(73); Res. MSC.113(73); Res. MSC.114(73);Res. MSC.115(73); Res. MSC.233(83); Res. MSC.252(83); Res. A.817(19); Res. A.1046; SN.1-Circ.274; NAV 56/WP.5 |

# Introduction

- “E-navigation is the harmonized collection, integration, exchange, presentation and analysis of marine information on board and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment.” (MSC 85/26 Annex 20). Position fixing is one of the 8 identified key elements of e-navigation.

- Primary aim of position fixing is to provide position, velocity, and time data (PVT) for navigators and navigational functions (ECDIS, Track Control Systems, AIS, INS and safety-related functions). In Res. A.915(22) Global Navigation Satellite Systems (GNSS) are recommended for PVT determination.

- A more significant objective is the resilient provision of position, navigation, and timing data (PNT) taking into account that in the future an improvement and indication of reliability (e.g. accuracy and integrity) should be achieved for all navigation relevant data. In this context PNT data encloses PVT data and ship’s parameter describing ship’s current movement and attitude (e.g. heading, rate of turn) (NAV 56/WP.5, see indication of reliability).

- Resilience is the ability of the PNT system to detect and compensate external and internal sources of disturbances, malfunction and breakdowns in parts of the system. This shall be achieved without loss of PNT data provision and preferably without degradation of their performance. The high-level user need “data and system integrity” reflects the need on resilient e-navigation systems (MSC85/26 Annex 20).

- Provision of resilient PNT data relies on the exploitation of modernized and future radio navigation systems, sensors and services. High-level user needs such as “improvement and indication of reliability” should be measurable. Therefore unambiguous specified performance quantities (e.g. accuracy, integrity, continuity, availability) are needed for all PNT data such as given in Res. A.915(22) as requirements on future GNSS for horizontal position and in Res. A 1046 as requirements on a World Wide Radio Navigation System (WWRNS).

- Scalability of requirements is a suitable approach to achieve different PNT solutions. Scalability of requirements means that performance classes should be introduced to manage all requirements, which correspond to the different phases of navigation (ocean, coast, port, docking) and take into account different navigational tasks. This can enable an objective decision finding regarding e-navigation implementations. Different performance classes may require different shore-side services (PNT relevant Maritime Service Portfolio).

- A suitable framework is necessary to compensate gaps (technical, administrative, regulative) towards the stepwise fulfillment of identified user needs (improvement of reliability, indication of reliability, automatic assessment of all navigation relevant data, automatic reporting….). A part of this framework is the integrated PNT concept (NAV 56/WP.5).

- The proposed PNT concept supports the exploitation of modernization processes in radio navigation systems (space-based and terrestrial), ship-side sensors and shore-side services.  The PNT is an open framework supporting the usage of any sensors, services and data sources improving the accuracy or assessing the integrity of provided PNT data and applied components. The concept should be timely considered in maritime standardization and is prepared by the modular concept of performance standards (SN.1-Circ.274).

- The PNT concept takes into account enhancements of PNT sensors and techniques as well as processing facilities:

- The development of a multi-system navigation receiver (MSR) is a necessary completion of WWRNS sensors to enable access on new radio navigation signals (see new performance standard of SHIP-BORNE RECEIVING EQUIPMENT USING MULTIPLE [RADIONAVIGATION] SYSTEMS). The MSR is a ship-borne receiving equipment capable of using signals from multiple systems such as GNSS and shorebased services, which provide augmentations, as well as terrestrial radio navigation signals for PVT data provision.

- Resilient PNT data provision requires integrated PNT data processing, which enables the exploitation of multi-sensor based redundancy to achieve assessment of PNT data reliability (e.g. accuracy, integrity). This may need a future performance standard covering this functionality.

# Concept of maritime Integrated PNT System

- The “Integrated PNT System” is the required overlay of GNSS as part of WWRNS, shore-side PNT services (PNT relevant MSP), shipboard components (PNT Module), and communication links, whose integrated use ensures the accurate and reliable provision of ships’ PNT data to applications during all phases of vessel navigation in a timely, complete and unambiguous manner (Figure 1).

Figure 1	Generic architecture of the maritime Integrated PNT System

- The PNT module represents the set of distributed ship-side HW and SW components. HW components (sensor layer) cover WWRNS sensors and other shipboard navigational sensors including data exchange in-between. SW components (data processing layer) can be either part of single sensors itself (classic approach) or can be realized by the PNT unit as part of INS. The shipboard sensor layer depicts the set of sensors recognized for the provision of PNT data. The shipboard processing layer represents the applied methods for the determination of PNT data. The PNT concept provides a scalable approach for the following solutions:

  - Classic approach operating with stand-alone equipment (Figure 2):

Figure 2	Classic approach of ship-side PNT module

- Stand-alone equipment provides only sensor-specific PNT data e.g. the WWRNS sensors for PVT data and the other shipboard sensors for navigation data. The shipboard processing layer is part of the applied sensors and represents the internal used methods for the provision of respective PNT data. The utilization of PNT relevant augmentation services is organized by WWRNS sensors itself based on included or connected communication equipment. Due to insufficient redundancy within single sensors and unsupported exploitation of multi-sensor based redundancy the classic approach is unable to meet e-navigation user needs such as improvement and indication of reliability based on monitored and assessed data and system integrity.

  - PNT unit approach as part of advanced INS (Error: Reference source not found):

Figure 3	Ship-side PNT module with PNT (data processing) Unit

- The PNT unit is an approach to meet identified user needs such as improvement and indication of data and system integrity. The presented approach enables the use of a variety of PNT relevant sensors and sources. Their combined use exploits the redundancy by processing data on raw data level. The utilization of PNT relevant augmentation coming from different shore site services (MSP), the integration of additional PNT relevant data sources (e.g. ePelorus, Racon), and the application of future PNT relevant MSI (Maritime Safety Information) is supported by this approach. By implementation of integrity monitoring functionalities the PNT unit has the potential to identify and provide the best PNT data and to indicate the current accuracy and integrity. Integrity information taking into account the achieved performance level is the basis for automatic reporting and improved alert management dealing with PNT.

- Today’s INS concept supports the integrated PNT approach.

- “The INS is recognized as aboard system to enhance the safety of navigation by providing integrated and augmented functions to avoid geographic, traffic and environmental hazards” (Res. MSC.252(83)). The PNT (data processing) unit will be part of INS’s Consistent Common Reference System (CCRS) to realize the provision of resilient PNT data related to a single consistent common reference point and to ensure the integrity monitoring of provided PNT data and applied PNT components.

Figure 4	PNT unit as part of INS’s CCRS

# Benefit of Integrated PNT Concept

- The PNT unit as part of the INS data processing layer (CCRS) promotes a harmonized concept related to PNT data provision with respect to:

- standardized techniques for PNT data provision;

- applicable techniques to deliver outputs for different accuracy classes;

- assessment functionalities per required integrity level taking into account the applied components and services;

- unambiguous data content;

- and identification of the achieved performance class.

- The integrated PNT concept

- promotes an open and modular architecture, which enables the use of existing and future PNT relevant sensor and services.

- is open to use different PNT relevant maritime services (part of MSP) towards the improvement of accuracy and integrity of PNT data provision.

# Standardization aspects of integrated PNT system

- SN.1/Circ.274 describes guidelines for application of the modular concept to performance standards. A new performance standard should include the specification of sensor/source module, operational/functional module, interfacing module and system/equipment documentation module.

- By using a modular approach of the PNT concept, a single WWRNS sensor such as MSR can operate in the classic approach or can operate as a sensor for the PNT unit approach. The MSR is a necessary step towards resilient PNT data provision to enable the exploitation of current and future radio navigation systems including assigned augmentation services.

- The PNT Unit is an operational/functional module of the INS and is assigned to the PNT data processing layer. The PNT Unit enables the exploitation of all PNT relevant data sources on raw data level to establish an increased redundancy needed for an automatic accuracy assessment of all provided PNT data.

- The modular PNT concept is sufficient to achieve a „fast track“ change management process for global standardization and type approval of PNT components by applying the modular structure of future performance standards (Nav 54/25 ANNEX 12).

# Actions

- The Sub-Committee is invited to note the Integrated PNT concept provided and to take it into account when further developing the e-navigation strategy implementation plan.

- The Sub-Committee is invited to consider the MSR as an initial step towards resilient PVT data provision.

- The Sub-Committee is invited to consider the PNT Unit as an overarching functional module, which enables accuracy and integrity assessment of all provided PNT data.

As submitted to MSC90 as a new work item