|  | E |
| --- | --- |

| SUB-COMMITTEE ON SAFETY OF NAVIGATION | NAV 58/6 27 April 2012 Original: ENGLISH |
| --- | --- |
| 58th Session |  |
| Agenda item 6 |  |

- DEVELOPMENT OF AN E-NAVIGATION STRATEGY IMPLEMENTATION PLAN

- Submitted by IALA

| SUMMARY |  |
| --- | --- |
| Executive summary: | This document provides some information and comments to NAV 58 on the development of e-navigation. |
| Strategic direction: | 5.2 |
| High-level action: | 5.2.4 |
| Planned output: | 5.2.4.1 |
| Action to be taken: | Paragraph 13 |
| Related documents: |  |

Introduction

- This document aims to provide NAV 58 with some information and comments on the development of e-navigation.

Discussion

- IALA has reviewed the report of the IMO CG on e-Navigation, as submitted to NAV 58.  There is general support amongst the IALA membership for the report and its annexes.  It is noted that Annex 1 (gap analysis) is sufficiently mature to inform the development of proposed solutions (Annex 2).

- Solutions proposed in Annex 2 are aligned with the work of the correspondence group to date and should be prioritised and be the subject of cost benefit and risk analyses.

- A number of issues are addressed below.

Regulatory approach

- The implementation of the IMO e-Navigation strategy is expected to involve the preparation of performance standards, or, as a minimum, harmonisation of equipment. The process for the introduction of new standards can be lengthy.  Adopting an effective approach to this challenge will facilitate the introduction of e-Navigation.  It is also recognised from the gap analysis that legislation and legal issues may have consequences for the development of e-navigation.  The timetable for the actual implementation of e-navigation has not yet been discussed fully, although it is expected that the Strategy Implementation Plan will be completed by 2014.

- New standards typically take 4-5 years to introduce. Also, their full implementation may take 10 years or more. Therefore, if several new items of equipment (or complete systems are involved), the overall process of introducing new standards could become unduly lengthy, thereby negating the expected benefits.  Various possible solutions are envisaged:

- an over-arching performance standard, referencing existing equipment performance standards and test specifications;

- use of a  requirements register and modular structure as an approach to standardisation (e.g. the approach taken in the INS performance standard);

- harmonized provision of shore-based information (and its transmission to ships) as a component of e-Navigation; or

- generic performance standards and test specifications.

There may be other solutions and the best option may be a combination of one or more of the above approaches. It is worth noting that another option of an unregulated, market-driven approach has not received general support.  IALA is ready to assist IMO in this effort, as appropriate.

Usability

- IALA is of the view that the development of a framework for usability for e-navigation solutions is necessary.  However, the choices for an appropriate mechanism to effect this (e.g. ISM code or type approval process) should be explored further.

IHO Registry

- Recognising the need for a Common Maritime Data Structure (CMDS) as an essential component of e-Navigation and noting the consensus on using the IHO Registry as the baseline for the CMDS, IALA has developed procedures, in consultation with IHO, to become a Submitting Organisation and domain owner under the IHO Registry.

- IALA plans to submit applications to IHO for domains covering services within its remit, such as VTS and aids to navigation.

Development of an integrated PNT concept

- IALA is developing the concept of Integrated Positioning, Navigation and Timing (PNT), taking into account the enhancement of radio navigation equipment to meet the modular structure of INS. One element of Integrated PNT would be a multi-system receiver.  Generic performance standards will allow receivers to make use of emerging technology and changes in radio navigation systems.  Such performance standards would address the PNT-related limitations identified in the gap analysis.

- IALA intends to submit an Information Paper on Integrated PNT to IMO NAV.

Portrayal Issues

- IALA is developing recommendations and guidelines related to the presentation/display of e-Navigation information.  How such information will be displayed or presented will depend on the particular task, situation and needs of the user.  Rather than developing prescriptive rules, the focus will be to develop "goal-based" standards, whereby over-arching objectives will be defined, but freedom to innovate will be left to developers and users.  A guiding principle will be that equipment, systems or services provide information content to all users in an effective manner, taking into account the human element.  It is planned that such guidance on the portrayal of e-Navigation information will be available by 2013.

Action requested of the Sub-Committee

- The Sub-Committee is requested to note the above information and decide as appropriate.

**********************