e-NAV11	Input paper

Agenda item	8.2

Task Number	6

Author(s)	N. Ward

Revision of Recommendation R-129

GNSS Vulnerability and Mitigation Measures

# Summary

The paper provides suggested revisions to R-129, to bring it up to date.

## Purpose of the document

The Committee is invited to consider these proposed amendments when preparing a revision of R-129.

## Related documents

Recommendation R-129.

# Background

Recommendation R-129 is important as a basis for IALA strategy on PNT. It was last updated in 2008 and there have been significant changes in GNSS since then.

# Proposed amendments

1. Add reference to RAE Study in Introduction and References: Global Navigation

Space Systems: reliance and vulnerabilities, The Royal Academy of Engineering

ISBN 1-903496-62-4, March 2011.

1.2 Scope should include space weather effects

3.2 Mention jamming by N Korea and spoofing incidents

3.5.1 Needs updating – GLONASS was restored to a constellation of 24 operational satellites in 2010. Dates for Galileo & Compass FOC now stated as 2018 and 2020. Frequency channels should read frequency bands.

3.5.2 eLoran should be noted as potentially a redundant systems. Cellular systems should be shown as an example of Signals of Opportunity (others being TV & Radio broadcasts). Status of Loran in N America needs updating – switched off in 2010, pending a decision on a backup for GPS.

3.5.4 Loss of GNSS does not make ECDIS unusable, it should be possible to input visual or radar bearing.

4. …an action plan is provided…

4.3 delete reference to AIS for collision avoidance

5. 6 eLoran ….coverage and equipment carriage is limited.

Appendix CTI should be 15 mins or N/A