e-NAV 12	Input

Agenda item	9.1

Task Number	12

Author(s)	CDR. Hideki NOGUCHI (JCG)

Report of Channel Management Operation in Tokyo bay

# Summary

This document introduces an incident happened at the Japan Coast Guard (JCG) channel management operation in Tokyo bay.

## Purpose of the document

The purpose of this document is to inform of the incident to the e-NAV members and to suggest the future action of the committee.

# Background

When establishing an AIS base station network in Tokyo bay, JCG decided to use regional AIS channels in order to avoid the VDL overload in the future.  The parameters of the region were as follows:

Frequency

Channel A: 161.575 MHz (2079)

Channel B: 161.875 MHz (2085)

Area

Northeast corner: 35° 50´  N, 140° 15´ E

Southwest corner: 35° 11´  N, 139° 35´ E

The Kannonsaki, Nojimasaki and Izuohshima AIS base station broadcasted Message 22 for the channel management.  Figure 1 shows the area.

These 3 stations broadcast Message 22.

Figure 1: Area of channel management

In 2008, the IMO Maritime Safety Committee (MSC) approved the use of the AIS SART that transmitted AIS channels of AIS 1 and AIS 2 only from January 1, 2010.  Considering that the AIS SART is used in an emergency situation and thus the JCG, as the competent SAR authority, should catch the AIS SART signal when it is transmitted, the JCG reviewed the VDL of Tokyo bay in the future and decided to back the regional AIS channels to the international AIS channels.  This operation was carried out on October 4, 2009 with the advanced notice to mariners and the relevant organizations.

# Operation

An AIS transmitter has 8 memories that save an area of regional AIS channels which is controlled by Message 22.  Therefore the operation needs to clear these memories in addition to stopping the broadcast of Message 22.  According to the ITU-R M.1371 and IEC 61993, there are 4 conditions to clear the memories as follows:

  - The nearest area of the areas which are saved in the memories is more than 500 nautical miles from the present position of the ship.

  - More than five weeks are past since the last Message 22 was received.

  - A newly received area is overlapped partially or totally with the area which is saved in the memory.

  - When all 8 memories saved an area and the ship receives new Message 22, the oldest memory is over written by the Message 22.

Taking these conditions into consideration, the procedure for the operation was decided as follows:

- The Kannonsaki, Nojimasaki and Izuohshima AIS station stop the broadcast of Message 22 that defines the regional AIS channels area.

- The new Message 22 that assigns the international AIS channels to the area is transmitted from kannonsaki, Honmoku and Urayasu AIS station by the regional AIS channels and from Kannonsaki, Honmoku, Urayasu, Nojimasaki and Izuohshima AIS station by the international AIS channels.  It is aimed to invalid the regional AIS channels area by transmitting the new Message 22 for overwriting memories in AIS transponder.

- If the change of the AIS channels is not succeeded by the above mentioned methods,  Message 22 that defines new 8 areas on land where a ship never navigate is transmitted and these new areas overwrite the old areas in the memories.

- In order to cope with a ship in Tokyo bay that switches on the AIS transponder after the AIS channel change and the memories in the AIS still save the area of regional AIS channels, an AIS monitoring in Tokyo bay by the regional AIS channels should continue at least 5 weeks after the change of AIS channels.

On October 4, 2009, after confirming the number of AIS ship stations received at Kannonsaki, Honmoku and Urayasu AIS stations, the procedure a) and b) were conducted.  However, the number of AIS ships that transmitted the regional AIS channels was not reduced as anticipated.   Therefore the procedure c) was conducted as the new 8 areas defined as follow:

Area 1

Northeast corner: 36° 20´  N, 138° 00´ E

Southwest corner: 36° 00´  N, 137° 50´ E

Area 2

Northeast corner: 36° 20´  N, 138° 20´ E

Southwest corner: 36° 00´  N, 138° 10´ E

Area 3

Northeast corner: 36° 20´  N, 138° 40´ E

Southwest corner: 36° 00´  N, 139° 30´ E

Area 4

Northeast corner: 36° 20´  N, 130° 00´ E

Southwest corner: 36° 00´  N, 138° 50´ E

Area 5

Northeast corner: 36° 20´  N, 139° 20´ E

Southwest corner: 36° 00´  N, 139° 10´ E

Area 6

Northeast corner: 36° 20´  N, 139° 40´ E

Southwest corner: 36° 00´  N, 139° 30´ E

Area 7

Northeast corner: 36° 20´  N, 140° 00´ E

Southwest corner: 36° 00´  N, 139° 50´ E

Area 8

Northeast corner: 36° 20´  N, 140° 20´ E

Southwest corner: 36° 00´  N, 140° 10´ E

Figure 2 shows the 8 areas.

Figure 2: 8 Areas defined by Message 22

After the transmission of the Message 22, the total number of AIS shipboard stations transmitting the regional AIS channels was reduced from 224 to 14 and thus it was judged that the operation was succeeded.  However, on October 5, 2009, the number was increased to about 50 and also dozens of AIS ships that entered to Tokyo bay were lost around the entrance from the AIS display of Tokyo-wan VTS center.

Immediately following this incident, the procedure was reviewed and it was found that the length of the longitude side of the areas was less than 20 nautical miles which ITU-R M.1371 defined as the minimum length of the area side.   Therefore the size of the 8 areas was re-defined as follows and the modified Message 22 was transmitted immediately on October 5, 2009:

Area 1

Northeast corner: 36° 25´  N, 137° 30´ E

Southwest corner: 36° 00´  N, 137° 00´ E

Area 2

Northeast corner: 36° 25´  N, 138° 15´ E

Southwest corner: 36° 00´  N, 137° 45´ E

Area 3

Northeast corner: 36° 25´  N, 139° 00´ E

Southwest corner: 36° 00´  N, 138° 30´ E

Area 4

Northeast corner: 36° 25´  N, 139° 45´ E

Southwest corner: 36° 00´  N, 139° 15´ E

Area 5

Northeast corner: 36° 25´  N, 140° 40´ E

Southwest corner: 36° 00´  N, 140° 00´ E

Area 6

Northeast corner: 35° 50´  N, 137° 30´ E

Southwest corner: 35° 25´  N, 137° 00´ E

Area 7

Northeast corner: 35° 50´  N, 138° 15´ E

Southwest corner: 35° 25´  N, 137° 45´ E

Area 8

Northeast corner: 35° 50´  N, 139° 00´ E

Southwest corner: 35° 25´  N, 138° 30´ E

Figure 3 shows the new 8 areas.

Figure 3: New 8 Areas re-defined by Message 22

After the transmission of the modified Message 22, it was confirmed that the number of ships transmitted the regional AIS channels was reduced to 6 out of 250, the total number of AIS ships in Tokyo bay and the phenomena of AIS lost was also solved.  Five weeks after the change of AIS channels in Tokyo bay, it was confirmed that there was no ship transmitted the regional AIS channels from the result of the AIS data analysis and therefore the transmission of the new Message 22 was discontinued on November 9, 2009.  However, the monitor of AIS by the regional AIS channel was continued in order to ensure the operation. On November 27, 2009, after the re-confirmation, the monitor was stopped and the operation was finally completed.

# discussion

Although the incident was caused by a simple mistake, as this report shows, the AIS channel management is not easy task.  There are lessons learned from the JCG experience is that:

- Some AIS shipboard stations do not follow the channel management command.

In the JCG case, if the all AIS shipboard stations followed the ITU-R M.1371, when the procedure b) was carried out, the operation should be completed at that moment.  But not all AIS stations did not follow the command of channel management and thus required more complicated procedure.

- Even though a new standard is established, the old standard equipment is still existed.

If the proposed amendment of the ITU-R M.1371-4 is approved and ITU-R M.1371-5 is issued, the saving period of regional area in the memories will be reduced from 5 weeks to 24 hours.  However, unless all ships change new AIS or update software of AIS, there will be a mixture of new and old standard AIS.

# Action requested of the Committee

The e-NAV Committee is invited to note the report and act as appropriate.