e-NAV12	Input

Agenda item	e-Navigation Architecture

Task Number		Support IMO / IMO NAV e-Nav Correspondence group; 
develop e-Navigation architecture

Author(s)	Jan-Hendrik Oltmann, Chair WG5

Outline / Draft of a future document
‘Structure of the Maritime Service Portfolios (MSPs)’

# Summary

The attached document provides a proposed chapter and content outline of a future document tentatively entitled “Structure of the Maritime Service Portfolios (MSPs)”.  The attached document also contains an initial collection of relevant input to such a document.

This document is intended to serve as a starting point of work in the e-Navigation Committee and IALA at large, with the goal to finalize the description of the “structure of the Maritime Service Portfolios (MSPs)” in due course.

The finalized future document is supposed to serve the following goals:

- Contribution to IMO’s development of the e-Navigation implementation plan:

  - a contribution to the ongoing work of the IMO NAV Correspondence Group e-Navigation, and

  - information or input paper to NAV59 (September 2013);

- Provide structural and architectural guidance for defining the contents of the generic, global Maritime Service Portfolios (MSPs) at IMO and at other relevant international organisations;

- Provide structural and architectural guidance for additional definitions of the Maritime Service Portfolios at regional, national or even local level.

- Provide structural and architectural guidance for the future provision of Maritime Service Portfolios (MSPs) and their associated services with the required service quality levels on a global, regional, national and/or local level.

Except for the purpose of examples, this document is not intended to contain descriptions of specific MSPs and/or specific services, i. e. it does not contain “content”.

# Background

The concept of the Maritime Service Portfolios (MSPs) is an important part of the “e-Navigation Platform Suite” (compare parallel input document).

The IMO NAV Correspondence Group on e-Navigation has now shifted the development of the structure/architecture and content of the Maritime Service Portfolios on a global scale into the focus of attention, and the Chair of the IMO NAV Correspondence Group on e-Navigation, Mr. John Erik Hagen, on the occasion of the IALA VTS2012 Symposiums, 10-14 September 2012, Istanbul, invited IALA specifically to contribute to the further development of the concept of Maritime Service Portfolios.

Also, during the IALA VTS2012 Symposium, it was recognized that the concept of the Maritime Service Portfolios reaches beyond the scope of e-Navigation as such: While e-Navigation deals with supporting aspects and functions of a vessel on its voyage from berth to berth “by electronic means”, the concept of the Maritime Service Portfolios incorporates also some scope of operational services not necessarily supported “by electronic means” (compare VTS2012 Presentation by Capt. Kinam Jeong, entitled “From e-Navigation to Co-Navigation”).

The relationship and interdependency of the MSPs and their associated services (including their  service quality levels) with the IMO proposed future Common Maritime Data Structure (CMDS) should be taken into account when developing the structure of the MSPs.

The relationship and interdependency of the MSPs and their associated services (including their service quality levels) with the future shipboard architecture and shipboard functionality, based on the IMO defined concept of the Integrated Navigation System (INS) and on the future shipboard Integrated Radiocommunication entity (compare NAV58 recognized detailed shipboard architecture) should also be taken into account when developing the structure of the MSPs.

It should be noted, that the concept of MSPs broadly introduces the notion of “Service Level”, which is a well established framework of concepts in the IT domain, into the maritime domain. (It should be noted that there will be in the future most likely “Service Level Stipulations” introduced by the appropriate regulatory instruments of international, regional, national and/or local bodies.)

# Action requested of the Committee

The Committee is requested to:

- Consider the above rationale and recognize the work of the e-NAV Committee on the structure of the Maritime Service Portfolios (MSPs) an integral part of its work and make it a matter of priority;

- Progress the attached draft document as a Working Paper with the goal to submit it as an output document eventually in timely fashion to the IMO NAV Correspondence Group on e-Navigation and as an (information) paper to IMO NAV59 (September 2013). To that end it would be required to prepare an output document from e-NAV13 (March 2013);

- Make “IALA-at-large” (IALA as a whole and its membership) aware, by appropriate means, of the importance of the concept of Maritime Service Portfolios (MSPs) for the work of IALA;

- Develop specific suggestions to foster the work of “IALA-at-large” on the Maritime Service Portfolios (MSPs), their associated services (including their service quality parameters/levels), both in terms of their structure/architecture and content (i.e. individual generic service definitions), “across Committees”.

- Outline / Draft Working Paper ON “The Structure of the Maritime Service Portfolio(S) (MSPs)”

- Introduction

This Chapter introduces the following document in general terms. It provides an overview on the consecutive Chapters.

The Chapters are structured in a way to first provide the “static” architecture, then turning towards their dynamic application to “run-time” and to “migration over time”. Eventually, after all relevant static and dynamic aspects are introduced, an “application guide” is contained.

(Hence, this document has the potential to become – over time – “the MSP Bible”, and thus IALA as the keeper of that document may position itself strategically within the international organisations…)

- Background

This Chapter introduces the background of the structure or architecture of the Maritime Service Portfolios (MSPs). It provides also a rationale.

  - The MSPs and ‘e-Navigation’ – services provided “by electronic means”

This Section addresses the relationship between the MSPs and the IMO defined and IMO governed ‘e-Navigation strategy’.

This Section concludes that the concept of the MSPs is larger than the scope of ‘e-Navigation’ proper, as the MSPs also include services which are not necessarily provided “by electronic means” (while their provision may be supported “by electronic means”).

The definition of “e-Navigation” and the “e-Navigation strategy” are investigated regarding their statements relevant for MSPs, their goals, their services, their service quality parameters/levels and the like.

  - The MSPs and services not provided “by electronic means”

This Section addresses the services (including their associated service quality parameters/levels) which are not provided “by electronic means”, i. e. don’t fall under the scope of e-Navigation, but are still to be recognized as an integral part of the MSPs concept.

- Goals

This Chapter introduces and explains the goals of the Maritime Service Portfolio(s) (MSPs) as derived from the above background consideration.

- Stakeholders at MSPs

This Chapter introduces and explains the stakeholders at MSPs, their fields and their respective contribution. The extensive list of already identified stakeholders (compare Annexes to the IMO e-Navigation strategy document (2008)) is investigated to that end.

- Building Blocks of the MSPs

This Chapter defines the MSPs by introducing their building blocks, namely the “services” (operational, technical), their relevant list of attributes, namely service features, and their appropriate meta-level service quality parameters. Also, the notion of “service fields” (i.e. services of similar kind lumped together) is introduced.

This Chapter draws upon the well-known concept of “Service Level (Agreement)” from the IT domain while adapting it to the specifics of the maritime domain. The well established collection of “best practises” contained in the “IT Infrastructure Library (Version 3) (ITIL V3)” should be considered as a source and should be adapted to the maritime domain, where appropriate.

It should be noted, that “commercial” (as opposed to “free”?!) is just another attribute of a meta-level service quality parameter. (This notion of “commercial” is mentioned at the IMO documentation regarding e-Navigation MSPs.)

- AREA definition framework(s)

This Chapter deals with all aspects of geospatial association of MSPs and their associated services (and their revelant service levels).

  - Introduction to geospatial references of MSPs

This Section introduces all relevant fundamental aspects of the geospatial reference of MSPs and their services. References are introduced to important international standards in this regard.

  - Recognizing and Reconciling different area definitions in the maritime domain

It should be noted, that IMO already has given an area definition regarding MSPs.

It should also be noted, that there are additional area definitions of both IMO, other relevant organisation in the system of the United Nations (such as ITU) and other relevant international organisations (for example Sea Areas A1-A4 in GMDSS). These definitions may be relevant to a certain field within certain MSPs or even for one individual service, only. They are valid definitions, however, which must be taken into account.

This Chapter introduces the different notions of area definitions, identifies – hopefully – many commonalities and tries to reconcile the different area definitions within the overarching structure of the MSP concept.

  - The relationship of area definitions and “coverage” by services

“Coverage” is a very important concept and requirement in particular for technical sensor and communication services (radio determination services, radio location and radio navigation services, radio communication services etc.). Their focus of attention is the feasibility of reliable area coverage, i.e. the area to be covered by service is in their focus, regardless of the user pattern within those areas.

  - Functional Route Topolopy Models (RTMs) and “coverage” by services

The notion of the functional Route Topology Mapping is a very important concept, too, as it approaches the concept of “coverage” from the user pattern point of view: Where users are, there is required “coverage” of services within the MSPs. Since users (in particular vessels) are not regularly distributed evenly over the sea, their “preferred” whereabouts (i.e. their “routes”) can be identified (e.g. based on AIS data analysis) and modelled/mapped into a Route Topology Model (both generically and specifically to a given sea area), using “legs”, “junctions”, and “nodes” (i.e. Ports) as building blocks. Each “leg”, “junction”, and “node” is associated with a set of attributes. “Coverage” is then defined as the MSPs / services provided at certain “legs”, “junctions”, and “nodes”.

It should be noted, that the Route Topology Modelling may be a good tool to serve the reconciliation – on a conceptional level - between vessel traffic needs and (ever more emerging) Maritime Spatial Planning (MSP) concepts.

  - A Composite Area Definition Framework for MSPs

This section draws together the above considerations and attempts to create a composite area definition framework for MSPs.

It is anticipated that there will be, as a result, a layered architecture of area definitions. As long as each layer addresses aspects “orthogonal” to all other layers, there will be no conflict between those layers; in other cases the area definitions must be reconciled in some way.

- Hierarchie of MSPs

It has been recognized that (generic) MSPs themselves can be defined on the global, regional, national and/or local level, with different scope and also area coverage, thus recognizing the IMO stipulated requirement for “scalability” within “e-Navigation”.

Since “e-Navigation” is a global initiative by very definition, it appears as a given, that the global generic MSP definitions would be both the most elaborate, most extensive and most important MSP definitions of all. All regional, national, and local MSP definitions would need to take those global definitions into account when defining their MSPs as augmentations.

Thus, a hierarchy of MSPs emerges, and this Chapter introduces this hierarchy and its features.

- Categorization of MSPs as a Means to Handle Complexity

In this Chapter, based on the above, the notion to categorize both service quality (“service levels”) and the MSPs, is discussed and defined. The categorization serves to handle the complexity inherent with the MSPs, their services and their service levels. Eventually, refencing meaningful categories of MSPs (i.e. well selected bundles of services and their appropriate service quality parameters/levels) would be easier to handle than listing them all at each occurance.

This notion would also draw upon, by analogy in methodology, the well-established example of ILS Categories (Cat I, II, III) in the aviation domain (as suggested by a Danish contribution to the IMO NAV e-Navigation Correspondence group).

- Relationship of MSPs and CMDS as based on IHO Registry

This Chapter introduces all relevant aspects of the relationship between the MSPs and the future Common Maritime Data Structure (CMDS) as based on the IHO Registry.

  - MSP-Almanach in the CMDS

This section maps the above structure of MSPs and their into an appropriate data model of MSPs in the CMDS based on the IHO Registry. Thus, it would be possible to store MSP almanach for sea areas of interest in a vessel’s or VTS’s database,

  - MSP data exchange format(s)

This section deals with exchanging MSP almanachs by means of appropriate data exchange formats.

- Dynamic transition considerations

During its voyage a vessel will most likely transit dynamically between different MSPs while passing through different IMO defined regions: harbour, coastal, open sea, coastal, harbour. Hence, the associated MSPs would be transited at run-time. Should the vessel also use polar routes, then the IMO defined MSPs for those polar areas would be transited, too. The same holds true in principle – but most likely more often in the future with the advent of many off-shore wind farms (refer to Maritime Spatial Planning) – for the IMO defined “off-shore areas” and their associated MSPs.

This Chapter concerns itself with all aspects relevant for safe and reliable transiting through MSPs in different areas, taking into account the above considerations.

- Interaction with Shipboard environment

This Chapter would state requirements from the above in regard to the shipboard environment.

Some requirements have already been collected in some WG5 working paper of the e-Nav Committee.

- Governance of MSPs

IMO has asserted governance of the “e-Navigation” strategy implementation. Beyond that IMO already governs the existing operational services which would be migrated into the global MSPs. Hence, it may be inferred that IMO has asserted governance of the global, generic MSPs at large. This Chapter talks about that governance in relation to governance of appropriate bodies of “their” MSPs on the regional, national and local level.

- Certification of MSPs and Services

This Chapter would address all relevant aspects of future certification of MSPs, associated services and associated service levels in order to provide reliability for shipping when using the MSPs and their associated services.

This Chapter would also discuss the role of the IMO (V)IMSAS scheme when applied to MSPs, associated services and associated service levels.

- Migration and Legacy Aspects

This Chapter introduces migration pathes and suggest legacy policies in regard to the MSPs concepts.

- Application Guidance

This Chapter provides application guidance for the above structure to create MSP definitions on the global, regional, national, and local level.

- References

- Glossary Of Terms

- Appendices