e-NAV12/20

Formerly e-NAV11/WG5/WP1

Working Paper on shipboard architecture functional requirements as seen from ashore

Motivation for consideration:

There is a request from the IMO Correspondence Group on e-Navigation for contribution to the ongoing “development of an e-navigation strategy implementation plan” (MSC 86/23/4 refers). This document seeks to provide input for the further development of the detailed architectures of both ship and shore and also information/data exchange using the overarching e-Navigation architecture as a framework.

The shore side, when providing technical services as part of the future Maritime Service Portfolios (MSPs), needs to rely on a well defined, harmonized shipboard environment which interacts with the shore-provided technical services in a well defined manner on the functional and physical shipboard interface to the communication links to shore (and other ships).

Due to the scalability requirement of the e-Navigation strategy, there is a need to define a minimum shipboard functional architecture as well as several degrees of more sophisticated shipboard functional architectures to make optimum use of the services provided by the future Maritime Service Portfolios. This document presents an intermediate shipboard architecture in functional terms as a postulate that has been used to facilitate the work of IALA on the shore-based e-Navigation architecture. This postulate is offered for consideration knowing that the final ship and shore-based architectures still have to be determined.

Relevant reference documents for consideration:

- IMO MSC 85/25, Annex 20

- IMO MSC 86/23/4, as amended and adopted by MSC 86

- IMO NAV57 WP.6

- IMO MSC.252(83) (= Revised Performance Standards for Integrated Navigation Systems (INS)) (=MSC83/28/Annex 30)

- IMO NAV54/25/Annex 5 (Draft SN Circular on Guidance for the Application of the Modular Concept to Performance Standards)

- Correspondence Group on e-Navigation, Final Draft Report to NAV58, 07 March 2012

- IHO S-100 and S-99 Standards for the IHO Registry

Initial functional requirements for the future shipboard architecture:

- There is defined a generic shipboard technical architecture in functional terms as an architectural and design reference.

- This shipboard architecture includes state-of-the-art concepts. This includes hierarchical functional layers combined with a modular concept as well as flexible interface sentence semantics.

- It is noted that IMO has created the INS Performance Standards and associated guidelines. That modular Integrated Navigation System (INS) is therefore considered as the architectural framework for the shipboard technical architecture. Some details of a future shipboard architecture as built upon the INS are provided in the figure in the attachment.

- There are further defined additional modules for the shipboard functional architecture, in particular regarding the radio communications. In the attached drawing this is reflected as a module tentatively called Integrated Radio Communication.

- A Capability Negotiation functionality is required for the shipboard INS and the Integrated Radio Communication. This functionality interacts with the shore side and provides the shore side with information about the capabilities of the shipboard electronic environment. This in turn is used by the shore side to tailor the data exchange with the shipboard electronic environment to the functionalities supported on board.

- A concern is the present integrity, consistency and quality of information/data received. Integrity, consistency and quality is improved by taking into account the data transaction chain(s) from initial shipboard source(s) of the data, through the different stages, to the transmission point(s) ship to shore /ship to ship, as a whole. This principle applies also to the communications shore to ship, as appropriate. The shipboard e-Navigation environment uses the Common Maritime Data Structure (CMDS), thus fulfilling this requirement.

- The resulting requirements from the shipboard e-Navigation environment to the CMDS, to the IHO Registry based on S-100 and to the technical services contributing to the Maritime Service Portfolios (MSP) are defined and brought to the attention of the appropriate stakeholders.

- Unique technical identifiers to achieve the above integrity goals are defined, elaborated and published.

- In order to maintain the integrity and quality, a flexible, safe and secure update management process for shipboard equipment are established and applied.

Attachment: Relevant technical architectural diagrams, example

PNT Sensors

Depth sensor

Radar Sensor(s)

Sensor/ Source Layer

using CMDS

Consistent Common Reference System

PNT evaluation

System Data

selection

Integrated Radio Communication

VHF

Radio Resource Management

(selection, filtering, routing, signaling and alerting)

Collision avoidance

Route  planning and monitoring

Navigation Control (incl. Track Control)

Alert manag.

Other bridge systems

Integrated Navigation System (INS)

Data Processing Layer

Operational Layer, incl. provision of HMIs

Status and Data Display

Active INS Data base

(Route information, SENC, Hydrographical, Weather,…)

BAM

INS Communication

Safety

…

Interface to  external systems

SAT

Vessel parameters

Static data, pre-packed (e.g.  ENC data)

Dynamic /

real-time data

AIS

…

…

HF

MF

LF

…

By example, there is a precedence for this kind of functionality by the AIS Application Specific Message Function Identifier “Capability Interrogation”, presently defined at ITU-R M.1371.