| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/53 |  |

e-NAV12/46

DRAFT CPM TEXT

AGENDA ITEM 1.15

(Working Group 5B-3)

(WP 5B / SG 4(1), WP 5A, WP 5C, WP 5D, SG 6(1), SG 7(1), (WP 3K), (WP 3M))

1.15	to consider spectrum demands for on-board communication stations in the maritime mobile service in accordance with Resolution 358 [COM6/3] (WRC-12);

Resolution 358 [COM6/3] (WRC-12): Consideration of improvement and expansion of on-board communication stations in the maritime mobile service in the UHF bands

# 3/1.15/1	Executive summary

[Text of the executive summary, not more than half a page of text]

# 3/1.15/2	Background

RR No. 5.287 says that in maritime mobile service 6 frequencies UHF band may be used by on-board communication stations:

5.287	In the maritime mobile service, the frequencies 457.525 MHz, 457.550 MHz, 457.575 MHz, 467.525 MHz, 467.550 MHz and 467.575 MHz may be used by on-board communication stations. Where needed, equipment designed for 12.5 kHz channel spacing using also the additional frequencies 457.5375 MHz, 457.5625 MHz, 467.5375 MHz and 467.5625 MHz may be introduced for on-board communications. The use of these frequencies in territorial waters may be subject to the national regulations of the administration concerned. The characteristics of the equipment used shall conform to those specified in Recommendation ITU-R M.1174-2 (WRC-07).

It should be noted that some administrations use these frequencies for land mobile service. In accordance with RR No. 5.286A the frequency band 450-470 MHz is identified for use by administrations wishing to implement International Mobile Telecommunications (IMT)).

As a consequence during the study period the sharing studies are needed for on-board communications stations and land mobile service stations.

# 3/1.15/3	Summary of technical and operational studies, including a list of relevant ITU-R Recommendations

Recommendation ITU-R M.478, Technical characteristics of equipment and principles governing the allocation of frequency channels between 25 and 3 000 MHz for the FM land mobile service;

Recommendation ITU-R M.1174 contains Technical characteristics of equipment used for on-board vessel communications in the bands between 450 and 470 MHz;

Recommendation ITU-R M.1184 contains the technical characteristics of systems operating in the land mobile-satellite service;

Recommendation ITU-R M.1450, Characteristics of broadband radio local area networks;

Recommendation ITU-R M.1808, Technical and operational characteristics of conventional and trunked land mobile systems operating in the mobile service allocations below 869 MHz to be used in sharing studies.  Section 2.1 of Annex 1 contains the protection criteria for the mobile service to be used in the sharing studies “that an interference-to-noise ratio of I/N = −6 dB be used to determine the interference impact for land mobile systems, and that for applications with greater protection requirements, such as public protection and disaster relief (PPDR), an I/N of −10 dB may be used”;

Recommendation ITU-R M.1823, Technical and operational characteristics of digital cellular land mobile systems for use in sharing studies;

Recommendation ITU-R M.1824, System characteristics of television outside broadcast, electronic news gathering and electronic field production in the mobile service for use in sharing studies;

Recommendation ITU-R M.1732, Characteristics of systems operating in the amateur and amateur-satellite services for use in sharing studies;

Report ITU-R M.2116, Characteristics of broadband wireless access systems operating in the land mobile service for use in sharing studies.

(NOTE - At its May 2012 meeting, WP 5A agreed to initiate a revision of Report ITU-R M.2116.)

# 3/1.15/4	Analysis of the results of studies

[Text of the analysis of the studies]

# 3/1.15/5	Method(s) to satisfy the agenda item

[Text of the Method(s) to satisfy the agenda item]

# 3/1.15/6	Regulatory and procedural considerations

[Example(s) of regulatory text relating to the Method(s) to satisfy the agenda item]

(1) 	Relevant Working Party(ies) to be indicated by the Study Group.