e-NAV12	Input

Agenda item	opening plenary

Task Number	- all e-Navigation related work items

Author(s)	David Patraiko, Jan-Hendrik Oltmann; in consultation with John Erik Hagen, Jon Leon Ervik, and Omar Fritz Erikson

The ‘e-Navigation Platform Suite’ – an introduction

# Summary / Purpose

This document describes and proposes a common understanding of present e-Navigation working fields. It then introduces the notion of an “e-Navigation Platform Suite” as a coherent suite of those working fields in order to assist in the co-ordinated work on the further development and implementation of the IMO e-Navigation strategy.

The document in particular addresses the question how the scope of e-Navigation work can be correctly limited and how the more abstract e-Navigation definition can be transformed into concrete and tangible working areas, thus also reducing the complexity of the e-Navigation strategy implementation. To that end, well known concepts from the previous e-Navigation work are being arranged specifically to form the “e-Navigation Platform Suite”.

The document provides draft input to the IMO NAV Correspondence Group on e-Navigation and also a proposed structure to populate the “e-Navigation.net” website hosted by the Danish Maritime Authority (DMA). Thus, a core element for an e-Navigation communication plan is provided.

This common understanding is intended to be a contribution to the IMO NAV e-Navigation Correspondence Group.

Also, it may also provide guidance to the IALA e-NAV Committee, to other IALA Committees, to the Policy Advisory Panel (PAP), and to Council when considering their work in regard to e-Navigation.

It should be noted that this exercise is for clarification purposes and does not introduce any new concepts within the development of e-Navigation.

# Background

The development of the “e-Navigation Platform Suite” was prompted by the IMO NAV Correspondence Group on e-Navigation on the occasion of a presentation given by the Chair of that Correspondence Group, Mr. John Erik Hagen, at the IALA VTS 2012 Symposium, Istanbul, Turkey, 07-14 September 2012.  Mr. Hagen expressed a need to more clearly define the scope of e-Navigation to allow practical solutions being investigated and developed to answer the IMO determined User Needs and the IMO recognized gaps. At the same conference, the notion of Maritime Service Portfolios (MSPs) became prominent, and it appeared that in particular this concept has now to be developed with priority.

# Related Documents and References

- IMO e-Navigation strategy, as adopted by MSC86, 2008

- Documentation of the IMO NAV e-Navigation Correspondence group

- Terms of Reference for the HGDM (IMO/IHO Harmonization Group on Data Modelling)

- Work plan, Working Group structure and Terms of References of the IALA e-Navigation Committee

- Conclusions of the IALA VTS 2012 Symposium

# ACtion requested of the Committee

The Committee is requested to:

- Consider the proposal for a “e-Navigation Platform suite” as included in the Annex;

- Develop this proposal into a liaison note to the IMO NAV e-Navigation Correspondence Group;

- Align the work of the e-Nav Committee, as related to the IMO e-Navigation strategy, to the “e-Navigaiton Platform suite”;

- Invite other IALA Committees, PAP and eventually Council to consider the “e-Navigation Platform suite” and to align their work accordingly.

- the “e-Navigation Platform Suite” of coherent working fields and Initial e-Navigation Communication Plan

- Introduction and Rationale

The development of the “e-Navigation Platform Suite” was prompted by the IMO NAV Correspondence Group on e-Navigation.  There was recognized a need to more clearly define the scope of e-Navigation to allow practical solutions to be investigated and developed to answer the IMO defined ‘User Needs’ and the IMO recognized ‘Gaps’.

A common understanding of present e-Navigation working fields is proposed: To that end the notion of an “e-Navigation Platform Suite” as a coherent suite of those working fields is introduced in order to assist in the co-ordinated work on the further development and implementation of the IMO e-Navigation strategy.

In particular, the following questions are addressed:

- How can the scope of e-Navigation work be limited without inappropriately truncating the aspirations, goals and substance of the “e-Navigation strategy”?

- How can the more abstract e-Navigation definition be transformed into concrete and tangible working areas, thus also reducing the complexity of the e-Navigation strategy implementation.

To those ends, well known concepts from the previous e-Navigation work are being arranged specifically to form the “e-Navigation Platform Suite”.

Also a structure for a e-Navigation focal point is proposed to populate the existing “e-Navigation.net” website. Thus, a core element for an initial e-Navigation communication plan is provided.

This common understanding is intended to be a contribution to the IMO NAV e-Navigation Correspondence Group.

Also, it may also provide guidance to the IALA e-NAV Committee, to other IALA Committees, to the Policy Advisory Panel (PAP), and to Council when considering their work in regard to e-Navigation.

- The ‘e-Navigation Platform Suite’ - Overview

Seven working fields have been identified, which together constitute the “e-Navigation Platform Suite”. These working fields are:

- Overarching architecture

- Shipboard equipment

- Maritime Service Portfolios (MSPs)

- Communications

- PNT (Position, Navigation, Timing)

- Shore-based infrastructure

- Common Maritime Data Structure (CMDS)

- The Goals for the working fields of the “e-Navigation Platform Suite”

Also, concise goal statements for these working fields have been drafted, as follows:

- Overarching architecture
Goal: To provide the overarching structural framework for operational functions and technical systems, to allow parallel but coherent and harmonized development and implementation of the IMO e-Navigation strategy.

- Shipboard equipment
Goal: To create an effective and user-friendly environment where maritime information and decision support is enhanced through integration and quality assurance of electronic data.

- Maritime Service Portfolios (MSPs)
Goal: To provide a consistent, common architecture of sets of services provided from ashore to shipping, including service level and quality parameters for all relevant operational and technical services in the maritime domain.

- Communications
Goal: To establish an integrated system of communication technologies to provide robust and reliable communications to support maritime operations, safety and emergency response. This system architecture will be transparent to the end users, intelligent when selecting the most effective and efficient physical communication links, and continuously updateable.

- PNT (Position, Navigation, Timing)
Goal: To provide a robust and reliable electronic Position, Navigation & Timing service suitable for the demands of navigation safety. Such a service will seamlessly combine space based and terrestrial systems.

- Shore-based infrastructure
Goal: To provide a common, harmonized, flexible, and generic service oriented system architecture for shore-based technical services, and a common generic architecture for shore-shore-data exchange and able to support a single windows concept. Thus the shore-based infrastructure implements the technical aspects of the Maritime Service Portfolios (MSPs).

- Common Maritime Data Structure (CMDS)
Goal: To provide a common data model and a common architecture for data exchange formats and protocols in the maritime domain, to facilitate accurate, robust, reliable, and efficient data exchange on-board ships, ship-ship, ship-shore, shore-ship, shore-shore, and within shore-based systems.

- Features of the “E-Navigation Platform Suite”

The “e-Navigation Platform Suite” and the working fields have the following features:

- The notion of a “working field” stresses the long established fact, that “e-Navigation” is neither a new operational domain, nor a new operational procedure, nor a new technical system, nor a new technical component, or the like. Rather “e-Navigation” is a paradigm influencing all existing, and also new, operational and technical domains within the maritime domain.

- The above working fields have emerged during the past years of development of the “e-Navigation strategy”; hence, they are put together in one suite of working fields in a natural way. They are not “artificially imposed”.

- The working fields are supported by distinct international, regional, and national communities of experts, who work together coherently towards the common goals of the “e-Navigation strategy”.

- The above working fields comprise the full perceived scope of the “e-Navigation strategy”, as individual specific functions and topics can be mapped to the working fields (see below).

- The number of seven can be easily memorized and handled, while a larger number may no longer be memorisable and hand able without human support.

- The introduction of the working fields reduces complexity.

- The working field “Maritime Service Portfolios (MSP)” stands out in one regard, as it has the potential to address operational services beyond the scope of “e-Navigation”, which – by its very definition – is confined to services provided or supported “by electronic means”.

- Some of the working fields develop aspects, which are needed in a ground-laying way for all other working fields. Such literally fundamental working fields are the “Overarching architecture” and the “Common Maritime Data Structure (CMDS)”. Hence they have received or need to receive some priority.

- Some of the working fields are more technological in orientation, such as the working fields “Shipboard Equipment”, “Communications”, “PNT”, and “Shore-based infrastructure”.

- Topics to be addressed by each working field of The ‚e-Navigation Platform Suite’

In order to reduce complexity further while at the same time covering all relevant aspects, it is proposed that the e-Navigation Platform Suite would be described further by “topical sheets”.

The construction rule for this would be: 
“One topic per sheet, all working field’s answers to that topic on the same sheet”.

All working fields of the e-Navigation Platform Suite would be described concurrently in the same sheet addressing the same topic.

The present list of “topical sheets” is:

- Introduction / Definition

- Goals (see above)

- Major requirements

- Additional requirements

- Elaborate description (allows for more details)

- IMO user needs addressed (provides traceability with IMO identified user needs)

- IMO identified ‘gaps’ addressed (provides traceability with IMO identified gaps)

- Roadmaps (for implementing the working field)

- Stakeholders - global: Who (global)? the distribution of responsibilities / involvement of international organisation would be introduced in an overview manner. Additional topical sheets would allow individual international organisations to show the specifics of their involvement as shown by the following example of IALA.

  - IALA involvement: shows the appropriate IALA groups/bodies engaged in work in the working field (= ‘IALA e-Navigation phone book’)

  - IALA documentation mapping: which existing/draft IALA documentation addresses the working field and how.

- Stakeholders regional: Who (regional)? for each region the stakeholders responsible / involved in the e-Navigation working field would be entered in the appropriate column. Additional topical sheets would allow individual regional organisations to show the specifics of their involvement, similar to the example of “global stakeholders”.

  - European

  - Asian

  - American

  - …

- Stakeholders national: Who (national)?

  - Each country may wish to populate the columns of the e-Navigation Suite, on their individual page, with the organisation responsible / involved  in their country.

- HEAP conformity aspects 
(= “key strategy element” of IMO e-Navigation strategy: “Human element”)

- Usability requirements 
(at Human Machine Interfaces; = “key strategy element” of IMO e-Navigation strategy: “Human element”)

- Portrayal

- Candidate technologies

- Scalability aspects 
(= “key strategy element” of IMO e-Navigation strategy: “Scalability”)

- Testbeds

- Identified training needs 
(aspect of “key strategy element” of IMO e-Navigation strategy: “Human element”)

- Cost aspects (specific to the working field)

- Standardization options 
(= “key strategy element” of IMO e-Navigation strategy: “Equipment standization”)

- Affected IMO instruments 
(= “key strategy element” of IMO e-Navigation strategy: “Convention & Standards”)

- Other regulatory aspects 
(= “key strategy element” of IMO e-Navigation strategy: “Convention & Standards)

- Legacy aspects

- Migration path considerations

- Contraints 
(what limits the development of the working field, either temporary or permanently)

- Known suggestions for improvement of the concept 
(Example: Rename Common Maritime Data Structure (CMDS) into Common Maritime Data Model (CMDM) or even Universal Maritime Data Model (UMDM).)

When considering the above topical list, the following should be noted:

1. The above list is by no means meant to be exclusive or final; new topics may be amended as they appear. This provides a great degree of flexibility.

2. By introducing these topical sheets, not only complexity is reduced, but the work flow for populating the different sheets can be arranged in parallel and/or sequentially, as need may be.

- Mapping the previous “key strategy Elements” to the “e-Navigation Platform Suite”

In the IMO e-Navigation strategy, there are listed eight “key strategy elements” as follows:

- “Architecture”

- “Human element”

- “Conventions & standards”

- “Position Fixing”

- “Communications technology & information systems”

- “ENCs”

- “Equipment standardization”

- “Scalability”

Some of those are still identically contained as a working field in its own right in the “e-Navigation Platform Suite”, as indicated by the following table.

| “key strategy element” | Working field in the “e-Navigation Platform suite” |
| --- | --- |
| “Architecture” | “Overarching architecture” |
| “Position fixing” | “PNT (Position, Navigation, Timing)” |
| “Communications technology & information systems” | “Communications” |

The other “key strategy elements” are either attributes, which apply to any and all of the working fields of the “e-Navigation Platform suite”, and are thus included in the above list of topics under each of the working fields: “Human element” and “scalability” considerations would be required by any and all the working fields. The need to consider the amendment of existing “Conventions & Standards” or the need to create new ones also applies to all working fields. “Equipment standardization” applies to all technologically oriented working fields.

Or a “key strategy element” names a specific products or services, such as “ENCs”. The “ENCs” would be dealt with by co-herent work of different working fields, each working field contributing an aspect, while the Common Maritime Data Structure (CMDS) would be major working field since the CMDS will use the IHO Registry concept, as defined in their S-100/S-99 Standards, which is also the basis for the IHO “product” ENC.

- “e-Navigation.Net” as the Focal Point for the “e-Navigation Platform Suite” – The practical Aspects

While the “e-Navigation Platform Suite” would be essentially practical in itself, as it is a suite of working fields, i. e. of the working efforts of interested communities, there needs to be one focal point to store and provide the above topical sheets to the e-Navigation community at large, and also to the interested general public. (The latter aspect is the starting point for the initial e-Navigation Communication Plan; see below).

Hence, it is proposed that the “e-Navigation Platform Suite” is not kept in paper format, but rather electronically in one dedicated website. The above topic sheets would be web site tab sheets instead of book pages. There would be one entry point into the “e-Navigation Platform Suite”, from where all relevant tabs could be retrieved.

It should be noted, that the web site tab sheets (‘pages’) would have a uniform layout: The column headings of each ‘page’ are the ‘working fields’ which make up the ‘e-Navigation Platform Suite’.

Also, it would be essential to adhere to the principle: “One ‘page’ - one topic”.

For each ‘working field’ there would be created an iconic picture, which would capture the essence of that ‘working field’ in a graphical way.

Each topical page would have a uniform heading field looking like follows:

| Topic of this page (see list below) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Overarching architecture | Shipboard equipment | Maritime Service Portfolios (MSPs) | Communications | PNT (Position, Navigation, Timing) | Shore-based infrastructure | Common Maritime Data Structure (CMDS) |
| < graphical icon> | < graphical icon> | < graphical icon> | < graphical icon> | < graphical icon> | < graphical icon> | < graphical icon> |
| … content regarding topic of the page as relevant to “overarching architecture” … | … content regarding topic of the page as relevant to “shipboard operations + equipment” … | … content regarding topic of the page as relevant to “MSPs” … | … content regarding topic of the page as relevant to “Communications” … | … content regarding topic of the page as relevant to “PNT” … | … content regarding topic of the page as relevant to “Shore operations and infrastructure” … | … content regarding topic of the page as relevant to “CMDS” … |

It would be possible

- to embed links, icons, graphics etc. into the ‘sheets’ which would link to further detailed information, also to off-site locations;

- to share the workload of setting up the “e-Navigation suite” topical sheets amongst different stakeholders in the e-Navigation community;

- to gain – eventually – a central, dynamic, living, “e-Navigation directory” for the whole of the e-Navigation initiative.

It would be crucial to control the list of limited number of working field, which should be easy to remember (that is why not more than seven working fields were chosen).

It is proposed that IALA offers to IMO NAV – via their NAV e-Navigation Correspondence Group – to set up and co-ordinate the e-Navigation Platform Suite as described above. This would be done in close co-operation with IMO since IMO has asserted the governance of the e-Navigation strategy and its implementation.

Danish Maritime Authority (DMA) has offered their existing ‘e-Navigation.net’ internet domain to that end.

The look of the such composed web front end may look like follows (with topic ‘goals’ selected for example purposes):

| Introduction | Goals |  | Requirements |  | … |  | User needs | Gaps |  | User interfaces | Portrayal | … |  | Testbeds |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overarching architecture |  | Shipboard equipment |  | Maritime Service Portfolios (MSPs) |  | Communications |  |  | PNT (Position, Navigation, Timing) |  | Shore-based infrastructure |  | Common Maritime Data Structure (CMDS) |  |
| < graphical icon> |  | < graphical icon> |  | < graphical icon> |  | < graphical icon> |  |  | < graphical icon> |  | < graphical icon> |  | < graphical icon> |  |
| To provide the overarching structural framework for operational functions and technical systems, to allow parallel but coherent and harmonized development and implementation of the IMO e-Navigation strategy. |  | To create an effective and user-friendly environment where maritime information and decision support is enhanced through integration and quality assurance of electronic data. |  | To provide a consistent, common architecture of sets of services provided from ashore to shipping, including service level and quality parameters for all relevant operational and technical services in the maritime domain. |  | To establish an integrated system of communication technologies to provide robust and reliable communications to support maritime operations, safety and emergency response. This system architecture will be transparent to the end users, intelligent when selecting the most effective and efficient physical communication links, and continuously updateable. |  |  | To provide a robust and reliable electronic Position, Navigation & Timing service suitable for the demands of navigation safety. Such a service will seamlessly combine space based and terrestrial systems. |  | To provide a common, harmonized, flexible, and generic service oriented system architecture for shore-based technical services, and a common generic architecture for shore-shore-data exchange and able to support a single windows concept. Thus the shore-based infrastructure implements the technical aspects of the Maritime Service Portfolios (MSPs). |  | To provide a common data model and a common architecture for data exchange formats and protocols in the maritime domain, to facilitate accurate, robust, reliable, and efficient data exchange on-board ships, ship-ship, ship-shore, shore-ship, shore-shore, and within shore-based systems. |  |

- Features of the “e-Navigation PlatForm Suite” 
@ “e-Navigation.Net”

By setting up such a “e-Navigation Platform Suite” in the way described above, the topical contents would be also available to the general public. Thus the “e-Navigation Platform suite” would

- serve as an up-to-date “e-Navigation directory” for – eventually – any and all relevant aspects of e-Navigation;

- serve as the core of any future “e-Navigation Communication Plan”, as this will point to and build upon the “e-Navigation Platform Suite”;

- serve as a “blackboard”, thus allowing co-ordination of work in the appropriate working fields.

- provide a “historical development track” of “e-Navigation” when frozen images of the  “e-Navigation Platform Suite” are archived from time to time.

- An initial e-Navigation Communication Plan

An initial e-Navigation Communication Plan would strive to popularize the above “e-Navigation Platform Suite” by

- disseminating the relevant information on the above working fields;

- pointing to the above focal point web site “e-Navigation.net”.