e-NAV12	Input

Agenda item	9.3

Task Number	14

Author	Joa Hyoung Lee

## Limitation of CSTDMA

# Summary

This document focuses on the limitation of Carrier Sensing Time Division Multiple Access (CSTDMA).

# Background

CSTDMA is defined in ITU-R M.1371-4 as the channel access scheme for AIS Class B and is fully interoperable with SOTDMA transmissions by AIS Class A whilst ensuring the priority is given to SOTMDA.

CSTDMA is also considered as access scheme for Digital VHF described in ITU-R M.1842. ITU-R M.1842-1 describes characteristics of VHF radio systems and equipment used for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels. It also provides a guideline on the use of digital technologies by VHF systems of different bandwidth in the maritime mobile service.

In VHF Data Exchange (VDE), the four contiguous channels 25, 85, 26, 86 will be used for data exchange using the modulation technique described in Annex 4 to Recommendation ITU R M.1842-1.

# Limitation of CStdma

## CSTDMA

Stations using CSTDMA continuously monitor the AIS radio channels background noise level. This background level is used as a reference for a received signal strength measurement at the start of each TDMA slot. When a transmission is required a TDMA slot is randomly selected and the signal strength at the start of the slot measured. If the signal strength is significantly above the background level the slot is assumed to be in use and the transmission is deferred. If the signal strength at the start of the slot is close to the background level the slot is assumed to be unused and the transmission is made. The ‘listen before transmit’ or ‘carrier sense’ scheme works on a slot by slot basis; this limits CSTDMA transmissions to a single TDMA slot. Multiple consecutive slots cannot be allocated using this technique.

The procedure of CSTDMA is defined in ITU-R M.1371-4 as follows. Within a time window of 1 146 µs starting at 833 µs and ending at 1 979 µs after the start of the time period intended for transmission (T0) the AIS Class B “CS” should detect if that time period is used (CS detection window).  Signals within the first 8 bits (833 µs) of the time period are excluded from the decision (to allow for propagation delays and ramp down periods of other units).

The Class B “CS” AIS should not transmit on any time period in which, during the CS detection window, a signal level greater than the “CS detection threshold” is detected. The transmission of a CSTDMA packet should commence 20 bits (TA = 2 083 µs + T0) after the nominal start of the time period.

Figure 1

Carrier sense timing

The CS detection threshold should be determined over a rolling 60 s interval on each Rx channel separately. The threshold should be determined by measuring the minimum energy level (representing the background noise) plus an offset of 10 dB. The minimum CS detection threshold should be –107 dBm and background noise should be tracked for a range of at least 30 dB (which results in a maximum threshold level of –7 dBm).

The transmitter should begin transmission by turning on the RF power immediately after the duration of the carrier sense window (TA). The transmitter should be turned off after the last bit of the transmission packet has left the transmitting unit (nominal transmission end TE assuming no bit stuffing).

3.2 Problem of CSTDMA

When CSTDMA is used with other TDMA access schemes such as SOTDMA or RATDMA which transmits a packet without the startup delay for carrier detection, the carrier detection procedure in CSTDMA should work properly since CSTDMA can detect the RF signal from stations using other TDMA scheme. However, if there are only stations using CSTDMA, the carrier detection procedure may not work properly since all stations with packets to send should wait for RF signal from other stations and thus there is no RF signal during CS detection window. In this case, the carrier detection procedure in CSTDMA decides that there is no station to send a packet and thus start the packet transmission procedure, which may lead to packet collision if there are stations with packet to send in RF range.

Figure 2 “Problem of CSTDMA in Carrier Detection” shows the flowchart of CSTDMA and the problem of carrier detection problem.

FIGURE 2

Problem of CSTDMA in Carrier Detection

# Collison avoidance method for CSTDMA

In CSTDMA procedure, CS detection starts after reception delay for propagation of RF signal from other ship (Figure 3 (A)). When CSTDMA is used in parallel with other TDMA such as SOTDMA, a station using other TDMA starts the RF power up and packet transmission during this period while a station using CSTDMA just waits. When there are only stations using CSTDMA, all stations just wait and thus there is no signal in the air.

As a method for collision avoidance in CSTDMA, we propose an enhanced CSTDMA, CSTDMA/CA (Carrier Sensing Time Division Multiple Access/Collision Avoidance). The concept of CSTDMA/CA is very similar with CSMA/CA which is used in WLAN. In CSMA/CA, to avoid the collision, a station with packet to send packets first transmits RTS (Ready to Send) packet to indicate that it will transmit a packet. Other stations which receive the RTS packet, should postpone the packet transmission to avoid collision.

In CSTDMA/CA, to avoid the collision, a station turns on the RF power during reception delay and transmits carrier signal shortly as a RTS. After reception delay, the station changes to RX mode and performs the CS detection procedure (Figure 3 (B)). If there is no signal during CS detection window, it means that there is no station with packet to send and thus the station can transmit a packet. On the other hand, if carrier signal was detected, it means that there is at least one station with a packet to send and thus the station should stop the packet transmission (Figure 4).

FIGURE 3

Timing in CSTDMA and enhanced CSTDMA/CA

FIGURE 4

Flow chart of CSTDMA/CA

# REFERENCES

[1]	Recommendation ITU-R M.1371-4 Technical characteristics for an automatic identification system using time-division multiple access in the VHF maritime mobile band.

[2]	Recommendation ITU-R M.1842-1 Characteristics of VHF radio systems and equipment for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels.

[3]	e-NAV11/output/10 Plan for AIS and VHF Data Communications in e-Navigation.

[4]	e-NAV11/output/08 Liaison Note to ITU-R Working Party 5B – Plan for Future VHF Data Communications.

# Action requested of the Committee

The Committee is invited to consider the discussion above.