- ANNEX 2

- Working document towards a preliminary draft new report on AIS VHF Data Link (VDL) loading

- Purpose

The purpose of this document is:

- To provide an assessment of AIS VDL loading in an example high vessel traffic area,

- To identify how AIS VDL loading affects the various functions of the AIS,

- To support WRC-15 Agenda Item 1.16 studies on additional AIS channels to support AIS applications and to mitigate AIS VDL loading and

- To consider an example AIS implementation using two additional channels for AIS applications that would also provide sufficient capacity for the future of AIS.

- WRC-15 Agenda Item 1.16 and its intended results

- WRC-15 Agenda Item 1.16 “to consider regulatory provisions and spectrum allocations to enable possible new Automatic Identification System (AIS) technology applications and possible new applications to improve maritime radiocommunication in accordance with Resolution 360 (WRC-12)” addresses the excess AIS VDL loading problem and its potential solution, i.e., that the prospective addition of channels could provide a means to support AIS applications such that the harmful effects of excess AIS VDL loading for both the present and the future are mitigated.

- Background

- Automatic Identification System (AIS) is a maritime communication and safety of navigation system operating in the VHF band and is used for vessel collision avoidance as well as the delivery of information about specific details of the vessel. Further, consequential to the introduction of the AIS-SART for search and rescue operations, the AIS channels were added to Appendix 15 of the International Radio Regulations.

With increasing demand for maritime VHF data communications, AIS has become heavily used for maritime safety, maritime situational awareness and port security. As a result, overloading of AIS1 and AIS2 has created a need for additional AIS channels. International Maritime Organization (IMO) Resolution MSC 74(69) required that AIS, “…improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements: 1) in a ship-to-ship mode for collision avoidance; 2) as a means for littoral States to obtain information about a ship and its cargo; and 3) as a VTS tool, i.e. ship-to-shore (traffic management)”. The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) has advised in its Maritime Radio Communication Plan (MRCP) that additional AIS channels are required for ship-to-ship and ship-to-shore maritime safety information (MSI) and general data communications (i.e. Area Warnings, Meteorological and Hydrographic Data, Channel Management of AIS, future VHF Digital Data Channels, and Ship-shore Data Exchange).

Although satellite detection of AIS on AIS 1 and AIS 2 was proven to be possible, its effectiveness was determined to be unacceptably limited where VDL loading is high. The need for a separate dedicated service on separate dedicated channels was confirmed by WRC-12 and two additional channels were designated.  While this new designation solves the problem for satellite detection, AIS VDL loading remains a serious issue to an increasing degree in many parts of the world due to the proliferation of AIS applications, message types, services and equipment types plus the unanticipated increase in user volume.

IMO also recently expressed its concerns on this subject in Resolution MSC.347(91) (adopted on 30 November 2012) “Recommendation for the Protection of the AIS VHF Data Link.” To solve this problem and protect the integrity of the AIS VDL, AIS subject matter experts in IALA are considering an approach in which Application Specific Messages (ASM) would be moved to two additional AIS channels. WRC-12 acknowledged this concept in its revision of Appendix 18 and provided four candidate channels on an experimental basis for this evaluation.

- Effects on safety and the exchange of information

The harmful effects of AIS VDL loading and its degradation to “maritime radiocommunication,” as stated in WRC-15 Agenda Item 1.16 are described in IALA Recommendation A-124 “Automatic Identification System (AIS) Shore Station and Networking Aspect relating to the AIS Service,” which states that when AIS VDL loading exceeds 50%:

- Safety (the reduced ability of the shipborne AIS to find free slots) is impaired, and

- Information exchange (the reduced ability of the AIS shore infrastructure to exchange information with the shipborne AIS) as required by IMO is impaired.

- Requirements for the exchange of information

In addition to safety requirements, requirements for the exchange of information with an AIS shore infrastructure can be determined from the IMO AIS Regulations and Performance Standards shown below, and including, as noted in Section 3, IMO’s concern for the protection of the integrity of the AIS VDL to ensure that these requirements are met. The technical standard for AIS, Recommendation ITU-R M.1371-4, addresses the AIS shore infrastructure elements of AIS Base Stations, AIS Aids to Navigation and AIS Repeaters, all of which have accompanying IEC performance standards and the AIS infrastructure standard, IALA Rec. A-124 (ref. Section 4).

IMO's SOLAS Convention, as revised, Regulation 19, §2.4.5, states with regard to the purpose of the AIS:

‘AIS shall

1. 	provide automatically to appropriate equipped shore stations, other ships and aircraft information, including ship's identity, type, position, course, speed, navigational status and other safety-related information;

2. 	receive automatically such information from similarly fitted ships;

3. 	monitor and track ships; and

4. 	exchange data with shore-based facilities.’

In addition, the IMO Performance Standards for the AIS state:

‘1.2 	The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic

Services (VTS), by satisfying the following functional requirements:

.1 	in a ship-to-ship mode for collision avoidance;

.2 	as a means for littoral States to obtain information about a ship and its

cargo; and

.3 	as a VTS tool, i.e. ship-to-shore (traffic management).

1.3 	The AIS should be capable of providing to ships and to competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking. Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.

1.4 	The installation, in addition to meeting the requirements of the Radio Regulations, applicable ITU-R Recommendations and the general requirements as set out in resolution A.694(17), should comply with the following performance standards.’

- Excess AIS VDL loading and its harmful effects in high vessel traffic areas

As noted in Section 4, safety and the exchange of information, the two main requirements of AIS, are adversely effected when AIS VDL loading exceeds 50%. The critical 50% threshold has already been exceeded in some areas of the world where vessel traffic is high, for example, the Northern Gulf of Mexico in the United States, as shown in Table 1. There is also a problem of accurately measuring AIS VDL loading, as shown in Table 2, which shows that the indicated loading value of 64% shown in Table 1, based on AIS messages received, is artificially low (presumably because of AIS slot reuse), and must actually be, based on AIS SOTDMA structured behavior, as high as 133%. This also shows that information exchange between vessels and the AIS infrastructure is highly impaired, due to a significant loss (133% - 64% = 69%) of information. This loading problem, as bad as it already is, is increasing as more vessels are being fitted, more types of devices containing AIS are introduced, and more AIS applications and ASM (Application Specific Messages) are implemented.

Table 1

VDL Loading Analysis

United States Nationwide AIS, Liverpool site, Northern Gulf of Mexico

| Site: Liverpool-1680 |  | 18 Dec 2012 (24hours) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Observed VDL Content | Total Messages | Total Slots Used | Total VDL Load (%) | Type 1 VDL Load (%) | Type 1 < 0.5 VDL Load (%) |  |
| AIS "A" | 1,929,553 | 2,078,421 | 64.15 | 49.0 | 29.7 |  |
| AIS "B" | 1,966,550 | 2,109,737 | 65.12 | 50.0 | 30.4 |  |
| Total | 3,896,103 | 4,188,158 | 64.63 | 49.5 | 30.1 |  |
| Message Type (comm state) | AIS "A" Channel | AIS "B" Channel |  |  | Total (both channels) |  |
|  | Messages | Vessels | Messages | Vessels | Messages | Vessels |
| 1 - Class A position report (SOTDMA) | 1,586,142 | 2,059 | 1,618,620 | 2,049 | 3,204,762 | 2,201 |
| 1 - SOG = 0.0 | 695,262 | 1,430 | 710,974 | 1,408 | 1,406,236 | 1,513 |
| 1 - SOG = 0.1 | 215,381 | 1,187 | 219,702 | 1,180 | 435,083 | 1,282 |
| 1 - SOG = 0.2 | 27,284 | 930 | 27,795 | 949 | 55,079 | 1,043 |
| 1 - SOG = 0.3 | 14,528 | 843 | 14,980 | 820 | 29,508 | 934 |
| 1 - SOG = 0.4 | 11,032 | 777 | 11,336 | 787 | 22,368 | 896 |
| 1 - SOG >= 0.5 | 617,645 | 1,535 | 628,707 | 1,543 | 1,246,352 | 1,658 |
| 1 - SOG not available | 5,010 | NA | 5,126 | NA | 10,136 | NA |
| 2 - Class A assigned position report (SOTDMA) | 3 | 3 | 3 | 3 | 6 | 6 |
| 3 - Class A special position report (ITDMA) | 96,223 | 1,299 | 98,859 | 1,325 | 195,082 | 1,468 |
| 4 - Base station report (SOTDMA) | 21,567 | 16 | 24,035 | 16 | 45,602 | 20 |
| 5 - Class A static / voyage data (none) | 148,867 | 1,366 | 143,182 | 1,342 | 292,049 | 1,508 |
| 6 - Addressed Binary (none) | 16 | 8 | 10 | 7 | 26 | 11 |
| 7 - Acknowledge 6 reception (none) | 7 | 6 | 6 | 6 | 13 | 12 |
| 8 - Binary broadcast (none) | 22,340 | 30 | 26,382 | 25 | 48,722 | 31 |
| 9 - SAR Aircraft position (ITDMA/SOTDMA) | 718 | 7 | 574 | 4 | 1,292 | 8 |
| 10 - Request UTC / date (none) | 19 | 7 | 12 | 4 | 31 | 10 |
| 11 - UTC / date response (SOTDMA) | 19 | 12 | 26 | 14 | 45 | 20 |
| 12 - Addressed Text (none) | 2 | 2 | 4 | 4 | 6 | 6 |
| 13 - Acknowledge addressed text (none) | 10 | 9 | 8 | 8 | 18 | 17 |
| 14 - Text broadcast (none) | 5 | 5 | 4 | 4 | 9 | 9 |
| 15 - Message request (none) | 19,016 | 20 | 15,383 | 19 | 34,399 | 25 |
| 16 - Command report behavior (none) | 1 | 1 | 0 | 0 | 1 | 1 |
| 17 - DGNSS broadcast (none) | 3 | 3 | 1 | 1 | 4 | 4 |
| 18 - Class B position report (ITDMA/SOTDMA) | 5,642 | 65 | 5,382 | 65 | 11,024 | 68 |
| 19 - Class B Position & static data (none) | 1 | 1 | 5 | 5 | 6 | 6 |
| 20 - Command FATDMA slot reservations (none) | 24,146 | 15 | 29,234 | 11 | 53,380 | 16 |
| 21 - AtoN position & status (none) | 54 | 7 | 54 | 9 | 108 | 12 |
| 22 - Command AIS channel & mode (none) | 4 | 4 | 6 | 6 | 10 | 10 |
| 23 - Command AIS unit's behavior (none) | 3 | 3 | 5 | 5 | 8 | 8 |
| 24 - Additional AIS unit information (none) | 4,675 | 68 | 4,679 | 63 | 9,354 | 69 |
| 25 - One slot binary data (none) | 68 | 8 | 74 | 12 | 142 | 19 |
| 26 - Scheduled binary data (ITDMA/SOTDMA) | 2 | 2 | 2 | 2 | 4 | 4 |

Table 2

Estimated Actual VDL Loading from Table 1

| Estimated loading of VDL (average over 24 hours) |  |
| --- | --- |
| Day | 18 Dec 2012 |
| Total Type 1 messages >= 0.5 knot received | 1,238,283 |
| Estimated Total Type 1 messages >= 0.5 knot | 2,556,678 |
| Percentage of Est. Total Type 1 >= 0.5 knot received | 48.43% |
| Total received slots - all messages | 4,188,158 |
| Estimated Total of all message slots broadcast | 8,647,273 |
| Percentage of VDL slots received over day | 64.63% |
| Estimated 24-hour average loading of VDL | 133.45% |

- Application of additional channels to mitigate excess AIS VDL loading

Since safety and the exchange of information are the two main requirements of AIS which must be accommodated and protected, and since the current arrangement results in an unacceptable failure (ref. Section 6) to meet these requirements in critical areas of high vessel traffic where these requirements are most needed, it is essential that:

- the Appendix 18 channels identified for additional AIS technology applications, e.g. ASM (application Specific Messages) be designated for this purpose,

- the AIS utilizes the additional channels in a new arrangement that will ensure that the requirements of safety and exchange of information are satisfied and

- the resulting arrangement will protect the integrity of the AIS VDL and is sufficient to accommodate the expected future growth.

- Considerations for the utilization of additional channels

The new arrangement noted in Section 7 should consider technical advances, administrative realities and constraints, including, but not limited to the following:

- There will be a need for a transition period and to ensure backwards compatibility during this period;

- There is strong motivation to minimize equipment and installation costs;

- There are technical advances that can provide significantly improved spectrum efficiency, e.g. advanced modulation waveforms (ref. En 300392-2 v3.2.1, Sec. 5, 76.8 kbps) which achieve an 8X increase in data rate per channel;

- The prospective expanded multi-channel arrangement may provide a means to separate the requirements of safety and the exchange of information on different channels with different communications protocols;

- The various AIS message types have different priorities, purposes and communications access schemes;

- There have been technical advances, e.g., SDR (Software Defined Radios), that can provide highly complex multi-channel multi-function digital functionality in a very small physical size and at very low cost.

This data and analysis is courtesy of the US Coast Guard Research and Development Center.

This data and analysis is courtesy of the US Coast Guard Research and Development Center.