e-NAV13	Input

Agenda item	12.2

Task Number	32

Author	N Ward

AtoN Information – Product Specification

# Summary

This is a further development of the Draft Product Specification on AtoN Information, formerly titled AtoN Metadata (earlier versions superseded e-NAV 13-14 and e-NAV 13-17).

## Purpose of the document

The Committee is invited to note this new version, which will be developed further inter-sessionally, with the aim of completion by e-NAV 14.

## Related documents

IHO S-100, IALA Guidelines 1087 & 1088.

AtoN Information – Product Specification

Draft  0.0.4 – January 2013

Contents	Page

1 Overview	1

1.1    Introduction	1

1.2   References	1

1.3   Terms, definitions and abbreviations	1

1.4   S-???  General Data Product Description	2

1.5   Data specification metadata	3

2 Specification Scopes	3

3 Data Product Identification	3

4 Data Content and structure	5

4.1   Application Schema 	5

4.2   Feature Catalogue 	6

4.3   Feature Types	7

4.3.1   Meta Feature Types	7

4.3.2   Geographic Feature Types 	7

4.4   Attributes	7

4.4.1   Numeric Attribute Values 	7

4.4.2   Text Attribute Values 	7

4.4.3   Mandatory Attribute Values 	7

4.5   Geometry	7

5 Coordinate Reference Systems (CRS)	7

5.1   Introduction 	7

5.2   Horizontal Geodetic Datum 	7

5.3   Vertical Datum 	8

5.4   Units of Measure	8

6 Data Quality	9

6.1   Quality, Reliability and Accuracy of Data	9

7 Data Capture and Classification	10

8 Data Maintenance	12

8.1   Introduction	12

8.2   S-1XX Product Specification Maintenance	12

8.2.1   Maintenance Procedures	12

8.3   Data Maintenance	12

9 Portrayal	13

10 Data Product format (encoding)	14

11 Data Product Delivery 	15

11.1   Feature and Portrayal Catalogue Delivery	15

12 Metadata	16

13 Language 	17

# Overview

## Introduction

The Aids to Navigation (AtoN) Information Product Specification provides a common structure for the exchange of information about AtoNs. This includes buoys, beacons, racons and lights. The product contains the positions, properties, operational status and general comments related to an AtoN.

The Product Specification can be used to exchange AtoN information in a consistent form between Lighthouse Authorities, Hydrographic Offices and other organizations (to include commercial and professional agencies).

This product schema is conformant with IHO S-100 Sections 8-A-1.1 and 8-A-1.4.

## References

### Normative references

IHO S.100 IHO Universal Hydrographic Data Model, January 2010

ISO 8601:2004 Data elements and interchange formats _ Information interchange _ Representation of dates and times

ISO/TS 19103:2005 Geographic information - Conceptual schema language

ISO 19111:2003 Geographic information - Spatial referencing by coordinates

ISO 19115:2003 Geographic information – Metadata

ISO 19115-2:2009 Geographic information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic information - Data product specifications

ISO/IEC 19501:2005, Information technology — Open Distributed Processing - Unified Modelling Language Version 1.4.2

### Informative references

ISO, 2005. ISO 19123 Geographic information - Schema for coverage geometry and functions.

ISO, 2006. ISO 19109 Geographic Information - Rules for Application Schema.

ISO, 2007. ISO 19135 Geographic Information – Procedures for Item Registration.

ISO, 2009. ISO 19136 Geographic Information - Geography Markup Language (GML).

IMO, 2008. Safety of Navigation Circular SN/Circ.243

GLA, 2010. R&RNAV Report 03/NW/10, AtoN Information Systems.

IALA, 2012. Guideline 1088 on an Introduction to Preparing S-100 Product Specifications.

## Terms, Definitions & Abbreviations

Terms and definitions have been taken from the normative references cited in clause 1.1. Only those which are specific to this document have been included and modified where necessary. Additional terms are defined in this document.

### coordinate

one of a sequence of numbers designating the position of a point in N-dimensional space [ISO 19111]

### coordinate reference system

coordinate system which is related to the real world by a datum

### coverage

feature that acts as a function to return values from its range for any direct position within its spatial, temporal, or spatiotemporal domain [ISO 19123] EXAMPLE Examples include a digital image, polygon overlay, or digital elevation matrix. NOTE In other words, a coverage is a feature that has multiple values for each attribute type, where each direct position within the geometric representation of the feature has a single value for each attribute type.

### coverage geometry

configuration of the domain of a coverage described in terms of coordinates [ISO 19123]

### direct position

position described by a single set of coordinates within a coordinate reference system [ISO 19107]

### domain

well-defined set [ISO 19103] NOTE Domains are used to define the domain set and range set of operators and functions.

### elevation

the altitude of the ground level of an object, measured from a specified vertical datum. [IHO:S100 GFM]

### feature

abstraction of real world phenomena [ISO 19101] NOTE A feature may occur as a type or an instance. Feature type or feature instance should be used when only one is meant.

### feature attribute

characteristic of a feature [ISO 19109] NOTE A feature attribute type has a name, a data type and a domain associated to it. A feature attribute instance has an attribute value taken from the value domain of the feature attribute type.

### function

rule that associates each element from a domain (source, or domain of the function) to a unique element in another domain (target, co-domain, or range) [ISO 19107] NOTE The range is defined by another domain.

### geometric object

spatial object representing a set of direct positions [ISO 19107] NOTE A geometric object consists of a geometric primitive, a collection of geometric primitives, or a geometric complex treated as a single entity. A geometric object may be the spatial characteristics of an object such as a feature or a significant part of a feature

### grid

network composed of two or more sets of curves in which the members of each set intersect the members of the other sets in a systematic way [ISO 19123] NOTE The curves partition a space into grid cells.

### grid point

point located at the intersection of two or more curves in a grid [ISO 19123]

### height

distance of a point from a chosen reference surface measured upward along a line perpendicular to that surface [ISO 19111:2006] NOTE Height is distinguished from elevation in that it is a directional measurement.

### range

<coverage> set of values associated by a function with the elements of the spatiotemporal domain of a coverage [ISO 19123]

### record

finite, named collection of related items (objects or values) [ISO 19107] NOTE Logically, a record is a set of pairs <name, item >.

### rectified grid

grid for which there is a linear relationship between the grid coordinates and the coordinates of an external coordinate reference system [ISO 19123] NOTE If the coordinate reference system is related to the earth by a datum, the grid is a georectified grid.

### referenceable grid

grid associated with a transformation that can be used to convert grid coordinate values to values of coordinates referenced to an external coordinate reference system [ISO 19123]

### spatiotemporal domain

<coverage> domain composed of geometric objects described in terms of spatial and/or temporal coordinates [ISO 19123] NOTE The spatiotemporal domain of a continuous coverage consists of a set of direct positions defined in relation to a collection of geometric objects.

### surface

connected 2-dimensional geometric primitive, representing the continuous image of a region of a plane [ISO 19107] NOTE The boundary of a surface is the set of oriented, closed curves that delineate the limits of the surface.

### tiling scheme

a discrete grid coverage that is used to partition data into discrete edge matched sets called tiles

### uncertainty

The interval (about a given value) that will contain the true value of the measurement at a specific confidence level [IHO S44] NOTE Errors exist and are the differences between the measured value and the true value. Since the true value is never known it follows that the error itself cannot be known. Uncertainty is a statistical assessment of the likely magnitude of this error.

### vector

quantity having direction as well as magnitude [ISO 19123] NOTE A directed line segment represents a vector if the length and direction of the line segment are equal to the magnitude and direction of the vector. The term vector data refers to data that represents the spatial configuration of features as a set of directed line segments.

## S-???  General Data Product Description

Title:	Aids to Navigation Information (IALA – AtoN)

Abstract: 	Positions and signals provided by Aids to Navigation. Additional general information can also be included.

Content: 	This Product Specification is a complete description of all the appropriate features, attributes and their relationships necessary to define an AtoN Information product. The detailed content is documented within the Feature Catalogue and the relationships defined in the Application Schema. Details of how these features should be symbolised are contained in the associated Portrayal Catalogue.

Spatial Extent:

Description: Global, marine areas only

East Bounding Longitude: 180

West Bounding Longitude: -180

North Bounding Latitude:  90

South Bounding Latitude:   -90

Temporal Extent:      Not Applicable

Specific Purpose: 	This product specification is intended to be used for encoding and exchanging AtoN Information.

## Data specification metadata

Title:		Aids to Navigation Information Product Specification

S-100 Version:		1.0.0

S-??? Version: 	0.0.2

Date:			31 July, 2012

Language: 		English

Classification: 	Unclassified

Contact: 		IALA e-NAV Committee

Producer

IALA/AISM, 10 rue des Gaudines, St Germain en Laye, 78100, France

Tel +33 1 345 17001
Fax: +33 1 345 18205

URL: 			http://www.iala-aism.org

Identifier: 		??? TBD

Maintenance:				  This Product Specification is maintained by the IALA e-NAV Committee 		  as required.

# Specification Scopes

Scope ID: 		General Scope

Level:			General Scope

Level name:	General Scope

# Data Product Identification

A data set that conforms to this product specification will be identifiable by the discovery metadata that supports it.

Title: 				         Aids to Navigation Information

Alternate Title:	                     IALA - AtoN

Abstract: 	Positions and signals provided by Aids to Navigation. Additional general information can also be included.

Topic Category:	HUMAN DIMENSIONS > INFRASTRUCTURE > TRANSPORTATION

Geographic Description:	Areas specific to marine navigation.

Spatial Extent:

Description: Global, marine areas only.

eastBoundLongitude: 180

westBoundLongitude: -180

northBoundLatitude: 90

southBoundLatitude -90

Vertical Extent:

minimumValue:	Not Applicable

maximumValue:	Not Applicable

unitOfMeasure:	Not Applicable

Temporal Extent:

TM_Primitive:		Not Applicable

Spatial Resolution:	(may not be required)

Purpose:	The purpose of an AtoN Information dataset is to exchange positions and signals provided by Aids to Navigation. Additional general information can also be included.

Language: 							English

Classification: 						Unclassified

Spatial Representation Type: 				Vector

Point of Contact: 					           Producer

Use Limitation: 						None

# Data Content and structure

An AtoN Information dataset is a feature-based product.  The following sub-sections contain the product application schema expressed in UML and an associated feature catalogue.  The feature catalogue provides a full description of each feature type including its attributes and attribute values in the data product.

## Application Schema

Figure 1 – AtoN Information Application Schema

[Figure 1: Comments – The following items need further consideration;

- Multiplicity needs to be defined for all attributes.

- The structure for lights may need further consideration to cover complex light characteristics and sectored lights as a single feature type.

- Daymark should be treated as a fixed AtoN as it may exist on its own (unlike topmark), but it can also be an attribute of a Fixed or Floating AtoN.

- Enumeration lists and definitions need to be included to complete this model.

- ‘Surveyed by’ should be an information type]

## Feature Catalogue

(To be produced once application schema is stable)

This feature catalogue defines the features and attributes permitted in this product.  The Feature Catalogue is supplied in XML form and as an HTML version accompanying this document.

Name: AtoN Information Feature Catalogue

Scope:

Field of application:

Version Number: 0.1

Version Date:

Producer: IALA

Functional Language: English

## Feature Types

### Meta Feature Types

No Meta Feature Types are included in this product.

### Geographic Feature Types

Geographic feature types form the principal content of the data product and are fully defined in the Feature Catalogue.

## Attributes

### Numeric Attribute Values

Floating point or integer attribute values must not be padded by non-significant zeroes.

### Text Attribute Values

Character strings must be encoded using the character set specified in Unicode Transformation Format-8 (UTF-8).

### Mandatory Attribute Values

Where attributes determine the display of a Feature they may be mandatory, all mandatory attributes are identified in the feature catalogue.

## Geometry

The attribute geometry has the value class GM_Point which is a position that shall locate the tracking list value.

# Coordinate Reference Systems (CRS)

## Introduction

The coordinate reference systems are separated into the horizontal and vertical components.

## Horizontal Geodetic Datum

WGS84 (World Geodetic System of 1984) shall be used for the horizontal reference system for spatial data. WGS84 should be used as the reference ellipsoid. The conversion must be undertaken by the submitting organization.

## Vertical Datum

Mean High Water? If a single datum or vertical datums can be identified based on a list of enumerations, a vertical datum attribute will be required on all relevant features.

## Units of Measure

Metres

# Data Quality

## Quality, Reliability and Accuracy of Data

Attributes at the feature level indicate method of positioning and date of observation or confirmation of operation. These attributes are defined in the Feature Catalogue.

In dataset metadata the overall positional accuracy and date to which the dataset has been validated must be provided.

# Data Capture and Classification

It needs to be agreed that a guide is required and production of the guide needs to be considered. Existing ENC data classification and encoding guide DCEG could be used as a template.

# Data Maintenance

## Introduction

Maintenance shall be defined in two parts: maintenance of the product specification and maintenance of the data.

## Product Specification Maintenance

The procedures for maintaining and publishing the various parts of S-1XX are determined by the Product Manager and approved by the Domains Manager. The specification will be reviewed not less than twice yearly and revised when required. Publication will be

## Maintenance Procedures

This specification is owned by the IALA XYZ Committee and assigned to a Product Manager, as recorded by the IALA Domains Manager. The procedures in place to maintain the Product Specification document and associated documents, Feature Catalogue, Encoding Guide will also be recorded and overseen by the IALA Domains Manager.

## Dataset Maintenance

Datasets defined in this specification are maintained by the producer on an as required basis.

# Portrayal

A Portrayal Catalogue defines the display rules and symbology which apply to the Features defined in this specification. The Portrayal Catalogue will be developed, using ENC portrayal as an input and reference to IMO SN/Circ. 243.

# Data Product format (encoding)

Encoding options include XML and GML (Geography Markup Language).

For some products a web service such as an OGC Web Feature Service (WFS) may replace traditional encoding formats.

The following example shows an XML encoding for buoys, taken from a model produced by the General Lighthouse Authorities, put in a form of XML being developed by the UK Hydrographic Office for S-100 Product Specifications.

<?xml version="1.0" encoding="utf-8" ?>

<s100:FeatureCollection xmlns:s100="http://www.iho.int/S-100" xmlns:a104="http://www.iala-aism.org/A-104" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:xlink="http://www.w3.org/1999/xlink" xsi:schemaLocation="http://www.iala-aism.org/A-104 A-104XMLSchema.xsd">

<s100:featureMember>

<a104:BuoySpecialPurposeGeneral s100:id="F1">

<a104:featureName>AFAN OUTFALL INNER</a104:featureName>

<a104:buoyShape>spherical</a104:buoyShape>

<a104:categoryOfSpecialPurposeMark>pipeline mark</a104:categoryOfSpecialPurposeMark>

<a104:colour>yellow</a104:colour>

<a104:depth>8.1</a104:depth>

<a104:topmark>

<a104:topmarkShape>x-shape (St. Andrew's cross)</a104:topmarkShape>

<a104:topmarkColour>yellow</a104:topmarkColour>

</a104:topmark>

<s100:Point><s100:pos>-3.90093 51.58994</s100:pos></s100:Point>

</a104:BuoySpecialPurposeGeneral>

</s100:featureMember>

<s100:featureMember>

<a104:Lights s100:id="F2">

<a104:signalPeriod>10</a104:signalPeriod>

<a104:signalGroup>(1)</a104:signalGroup>

<a104:colour>yellow</a104:colour>

<a104:lightCharacteristic>flashing</a104:lightCharacteristic>

<a104:lightDescription>Fl.Y.10s</a104:lightDescription>

<s100:Point><s100:pos>-3.90093 51.58994</s100:pos></s100:Point>

</a104:Lights>

</s100:featureMember>

</s100:FeatureCollection>

# Data Product Delivery

| Item Name | Description |
| --- | --- |
| unitsOfDelivery | Datasets |
| transferSize | Undefined |
| mediumName | Undefined |
| otherDeliveryInformation |  |

## Feature and Portrayal Catalogue Delivery

A Feature Catalogue in XML format shall be supplied/available from IALA with an accompanying XSL Stylesheet.

The Portrayal Catalogue shall be supplied/be available from IALA in an XML form with accompanying symbols.

# Metadata

Core metadata from S-100 listed in Annex B.

# Language

## The exchange language must be English.

# Note on Future development

The draft Product Specification will be further developed within IALA Committees and when finalised, will be submitted to IHO for inclusion in the Registry.

Once such standards are in use, with suitable security measures, administrations will

be able to coordinate management of AtoN and other related information in a digital environment and this will in turn facilitate sharing of information.

As more and more of the information needed for navigation and related tasks is

available on-line and continuous internet access becomes available, it will be

possible to obtain the relevant information more rapidly and reliably using intelligent search techniques.

Annex  A – Example of S-100 XML encoding for AtoN Information

The following example shows an XML encoding for buoys, taken from a model produced by the General Lighthouse Authorities, put in a form of XML being developed by the UK Hydrographic Office for S-100 Product Specifications.

<?xml version="1.0" encoding="utf-8" ?>

<s100:FeatureCollection xmlns:s100="http://www.iho.int/S-100" xmlns:a104="http://www.iala-aism.org/A-104" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:xlink="http://www.w3.org/1999/xlink" xsi:schemaLocation="http://www.iala-aism.org/A-104 A-104XMLSchema.xsd">

<s100:featureMember>

<a104:BuoySpecialPurposeGeneral s100:id="F1">

<a104:featureName>AFAN OUTFALL INNER</a104:featureName>

<a104:buoyShape>spherical</a104:buoyShape>

<a104:categoryOfSpecialPurposeMark>pipeline mark</a104:categoryOfSpecialPurposeMark>

<a104:colour>yellow</a104:colour>

<a104:depth>8.1</a104:depth>

<a104:topmark>

<a104:topmarkShape>x-shape (St. Andrew's cross)</a104:topmarkShape>

<a104:topmarkColour>yellow</a104:topmarkColour>

</a104:topmark>

<s100:Point><s100:pos>-3.90093 51.58994</s100:pos></s100:Point>

</a104:BuoySpecialPurposeGeneral>

</s100:featureMember>

<s100:featureMember>

<a104:Lights s100:id="F2">

<a104:signalPeriod>10</a104:signalPeriod>

<a104:signalGroup>(1)</a104:signalGroup>

<a104:colour>yellow</a104:colour>

<a104:lightCharacteristic>flashing</a104:lightCharacteristic>

<a104:lightDescription>Fl.Y.10s</a104:lightDescription>

<s100:Point><s100:pos>-3.90093 51.58994</s100:pos></s100:Point>

</a104:Lights>

</s100:featureMember>

</s100:FeatureCollection>

ANNEX B - core metadata from S-100

| Name |  | Path |  | Obligation |  |
| --- | --- | --- | --- | --- | --- |
| Metadata file identifier |  | MD_Metadata.fileIdentifier |  | M a |  |
| Metadata language |  | MD_Metadata.language |  | C b |  |
| Metadata character set |  | MD_Metadata.characterSet |  | C c |  |
| Metadata file parent identifier |  | MD_Metadata.parentIdentifier |  | C d |  |
| Metadata point of contact |  | MD_Metadata.contact > CI_ResponsibleParty |  | M |  |
| Metadata date stamp |  | MD_Metadata.dateStamp |  | M |  |
| Metadata standard name |  | MD_Metadata.metadataStandardName |  | O |  |
| Metadata standard version |  | MD_Metadata.metadataStandardVersion |  | O |  |
| Dataset title |  | MD_Metadata.identificationInfo > MD_DataIdentification.citation > CI_Citation.title |  | M |  |
| Dataset reference date |  | MD_Metadata.identificationInfo > MD_DataIdentification.citation > CI_Citation.date |  | M |  |
| Abstract describing the data |  | MD_Metadata.identificationInfo > MD_DataIdentification.abstract |  | M |  |
| Dataset responsible party |  | MD_Metadata.identificationInfo > MD_DataIdentification.pointOfContact > CI_ResponsibleParty |  | O |  |
| Spatial representation type |  | MD_Metadata.identificationInfo > MD_DataIdentification.spatialRepresentationType |  | O |  |
| Spatial resolution of the dataset |  | MD_Metadata.identificationInfo > MD_DataIdentification.spatialResolution > MD_Resolution.distance or MD_Resolution.equivalentScale |  | O e |  |
| Dataset language |  | MD_Metadata.identificationInfo > MD_DataIdentification.language |  | M |  |
| Dataset character set | MD_Metadata.identificationInfo > MD_DataIdentification.characterSet |  | C f |  |  |
| Dataset topic category | MD_Metadata.identificationInfo > MD_DataIdentification.topicCategory |  | M |  |  |
| Geographic location of the dataset (by four coordinates or by description) | MD_Metadata.identificationInfo > MD_DataIdentification.extent > EX_Extent > EX_GeographicBoundingBox or EX_GeographicDescription |  | C g, h |  |  |
| Temporal extent information for the dataset | MD_Metadata.identificationInfo > MD_DataIdentification.extent > EX_Extent.temporalElement |  | O |  |  |
| Vertical extent information for the dataset | MD_Metadata.identificationInfo > MD_DataIdentification.extent > EX_Extent.verticalElement > EX_VerticalExtent |  | O |  |  |
| Lineage | MD_Metadata.dataQualityInfo > DQ_DataQuality.lineage > LI_Lineage |  | O |  |  |
| Reference system | MD_Metadata.referenceSystemInfo > MD_ReferenceSystem.referenceSystemIdentifier > RS_Identifier |  | O |  |  |
| Distribution Format | MD_Metadata.distributionInfo > MD_Distribution > MD_Format |  | O |  |  |
| On-line resource | MD_Metadata.distributionInfo > MD_Distribution > MD_DigitalTransferOption.onLine > CI_OnlineResource |  | O |  |  |