e-NAV13	Input

Agenda item	8.2

Task Number	6

Author(s)	Bjørnar Kleppe/Norwegian Coastal Administration

Proposed method for validation of service availability in IALA-DGPS

# Summary

The document is an input to the ongoing revision of IALA Recommendation R121 in WG 2 in the committee. It regards calculations used during validation of service availability. A simple method is proposed to calculate the average service availability for a national IALA DGNSS service where a number of beacons are used to provide service to a large sea area and where there are complex patterns of overlapping radio coverage from different numbers of beacons.

## Purpose of the document

The method was provisionally proposed during the WG2 intersessional workshop regarding R121 in Helsinki 15-17.january 2013. This document contains the same proposal for the benefit of anyone that were not present there.

## Related documents

IALA Recommendation R-121 On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz

# Background

IALA Recommendation R-121 On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz is currently under revision by the committee. The R121  state recommendations on methodology for validation of service performance and minimum performance levels are quantified. The current R121 seems to be open to different interpretations regarding methods for validation of service availability, particularily if the DGNSS-service is large and coverage is provided by a network of multiple beacon-sites where different parts of the service area are within range from different numbers of beacons. See annex A.  Considering the strict  performance requirements and the substantial cost involved in providing such services the methodology for validating performance is important.

# Discussion

Many IALA DGNSS services cover vast sea areas that may contain an infinite number of interconnected shipping routes, fairways, tracks or waterways. Such services contain areas where a user is within radio range from multiple beacons and areas within range from only one beacon. IALA DGNSS receivers in use today are capable of switching automatically from one beacon to another in case of unplanned beacon failure.

The coverage overlaps between beacons give important contributions to service availability as seen by the user and this should be accounted for when availability is validated.  For reasons of simplicity an IALA DGNSS service provider may wish to compute a single availability number for its service and based on simple and robust measurements and calculations.

Proposed method:

- The portion of the total designated national service area where a user is within range of only one beacon is estimated. Also the portion of the total designated national service area where the user is within range of any two beacons is calculated, and the same for any three beacons. These portions will be static as long as the beacons are not moved or changed.

- The availabilities for individual beacons are measured and an average beacon availability is calculated

- The probabilities of simultaneous unavailability of one, two or three beacons are multiplied with the proportion of the total service area serviced by one, two, three or more beacons and these products sums up to an average service availability for the total service area.

The method requires that unavailability is uncorrelated between sites and therefore the administration should not schedule simultaneous maintenance work on sites with overlapping coverage.

Average availability = P1 *A1 + P2 *A2 + P3*A3 + …. +Pn*An, where

Pn = Portion of the designated DGNSS service area within radio range from a number of any n DGNSS beacons

An = Availability of DGNSS signal where service is provided by any n DGNSS sites and where

An = 1 – (1 – A1)^n

Example calculation:

Portion of the designated national service area within radio range from any one DGNSS site = 25%

Portion of the designated national service within radio range from any two DGNSS sites = 50%

Portion of the designated national service area within radio range from any three DGNSS sites = 25%

Measured average DGNSS beacon availability = 0,99

Probability that at least one of any two DGNSS sites are available is = 1-(1-0,99)*(1-0,99) = 0,9999

Probability that at least one of any three DGNSS sites are available is = 1-(1-0,99)*(1-0,99)*(1-0,99) = 0,999999

In this case the average availability performance is 0,25*0,99 + 0,5*0,9999 + 0,25*0,999999 = 0,9974

# References

- IALA Recommendation R-121 On the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz

- IALA LISTS OF RADIONAVIGATION SERVICES/ Table of DGNSS Stations (http://www.iala-aism.org/iala/publications/publications.php?LeTypePub=15)

# Action requested of the Committee

The Committee is requested to:

- 1	Consider the document during revision of R121.

- Annex EXAMPLE OF IALA DGPS PATTERNS OF OVERLAPPING COVERAGE

Source: IALA LISTS OF RADIONAVIGATION SERVICES/ Table of DGNSS Stations (http://www.iala-aism.org/iala/publications/publications.php?LeTypePub=15)