e-NAV13	Input paper

Agenda item	9.1

Task	12

Author(s)	WG 3 + WG 4

- Applications of AIS Message 27

- Draft Liaison Note to ITU-R Working Party 5B, Revision of Rec. ITU-R M.1371-4

- Purpose

The purpose of this document is:

- To provide an updated assessment (ref: Report ITU-R M.2169, December 2009), based on the development of new technology, of the loading capacity of the WRC-12 designated channels (RR Ap 18 channels 75 and 76) for the long range detection of AIS using AIS Message 27,

- To propose, based on the benefits, that Recommendation ITU-R M.1371-4 be revised to permit, in addition to the AIS Class A, the transmission of Message 27 by the AIS Class B-SOTDMA and by other AIS devices, e.g., AIS-SART, EPIRB-AIS and AIS-MOB, when utilizing Navigation Status 14 to indicate an active SAR status,

- To explain how this proposed extended use of Message 27 can be implemented within the loading capacity of the designated channels.

- The Message 27 capacity in Report ITU-R M.2169 has been increased by 4X

- Report ITU-R M.2169 proposed the technical approach using Message 27 that was confirmed in Recommendation ITU-R M.1371-4 for “long range detection of AIS” for operation on the two new channels that were designated by WRC-12. This Report assessed the satellite AIS detection capacity based on a single satellite pass (see Figure 1 below). Note that the approach adopted by ITU using Message 27 on the two channels designated by WRC-12 follows the curve for “3-minutes; 2-channels.” However, since the publication of this Report, satellite AIS service providers have confirmed a new “slot de-confliction technology” that provides a 4X increase in this capacity.

FIGURE 1

- Proposal to allow Class B-SOTDMA and AIS SAR devices to use Message 27

- The AIS Class B-SOTDMA was recently introduced for use by professional non-SOLAS vessels, some of which, e.g., fishing vessels, may frequently operate beyond the range of AIS share infrastructure. Its behavior is similar to the AIS Class A, and its population is expected to be modest. Since it uses SOTDMA, it could follow the same procedure and access scheme as the AIS Class A. Like the Class A, the Message 27 can be suppressed when the vessel is within the range of an AIS base station. Thus, the impact on the capacity of long range detection would be nil, and the benefit would be significant if the Class B-SOTDMA could also use Message 27.

- If AIS devices, e.g., AIS-SART, EPIRB-AIS and AIS-MOB, when utilizing Navigation Status 14 to indicate an active SAR status, could also use Message 27, it would be a great benefit to SAR operations when these operations are beyond the range of AIS shore infrastructure. Since these devices could only transmit Message 27 in the active state (Navigation Status = 14), and only in accordance with the schedule set for Message 27, the impact to the capacity of long range detection would be nil, and the benefit to SAR operations would be significant if these devices could also transmit Message 27.