Draft IALA Recommendation 
e-NAV-210

Generic e-Navigation Service Engineering Model Template

(2009-12-22)

# IALA Recommendation Preamble

--- include here the Preamble by e. g. borrowing it from 101

# Revision

| # | Date | Description | Initials |
| --- | --- | --- | --- |
| 1 | 20.Dec 2007 | Version 1.01 | Arch TWG |
| 2 | 24 Oct 2008 | Version 1.02 (Content definition) | Arch TWG |
| 3+4 | 08 Apr 2009 | Version 1.03+1.04 (Transfer of general material from AIS Service; A-124 Ed 2.0) | AIS TWG + Arch TWG |
| 5 | 03 Jul 2009 | Version 1.05 (Transfer of general material from AIS Service; A-124 Ed 2.0 completed; adjustment to data distinction) | AIS TWG + Arch TWG |
| 6 | 22 Dec 2009 | Version 1.06 (Transfer from A-124 continued) | AIS TWG + Arch TWG |

# Table of Contents

IALA Recommendation Preamble	2

Revision	3

Table of Contents	4

1	Introduction	5

2	Context of the document	5

3	Overview on shore-based services	7

4	Fundamental service design considerations	8

4.1	The notion of the Basic Services (BS)	8

4.2	The data a service delivers and how it is being modeled	14

4.3	The parts of a service and their external interactions in general	15

4.4	The influence of geographical topology on a service	18

5	The layered structure of a service - Structure Model	19

5.1	Services with and without Physical Layer	19

5.2	Services without Physical Layer	22

6	Resulting service setup and its component’s tasks	22

6.1	Resulting service setup	22

6.2	Resulting tasks of the functional components of the service	25

7	Interaction between services using the Structure Model	29

8	Overview description of the parts of the generic service engineering model	29

8.1	Deliverables of a service to the common shore-based e-Navigation system	29

8.2	Parts of the generic service model related to the layout and setup of a service	30

8.3	Parts of the generic service model related to components of a service	31

8.4	Parts of the generic service model related to installation and life-cycle management issues	33

# Introduction

The main building block of the common shore-based e-Navigation system architecture [0.2] is the so-called shore-based technical e-Navigation service. This Recommendation provides a common generic engineering model template for the technical e-Navigation services of the common shore-based e-Navigation system architecture.

This common generic engineering model template views the shore-based technical e-Navigation service from a variety of angles of relevant perspectives, which complement each other. To present this bounty of material in a reader-friendly, orderly and consistent manner, this Recommendation consists of this main part and a variety of Annexes. This main part introduces the different angles of perspectives and thereby the different Annexes and ties them together.

This engineering model, as composed by the different relevant perspectives complementing each other, is meant to be the template for shore-based technical e-Navigation services, i.e. all shore-based technical e-Navigation services described within the common shore-based e-Navigation system architecture should adhere to this engineering model.

As a well developed example for the application of this generic engineering model to a specific technology, the AIS Service description has been developed in parallel (refer to IALA Recommendation A-124, Ed. 2). The AIS Service provides an example of a shore-based technical e-Navigation service with a Physical Layer and also exhibits a very large technology specific complexity. Hence, this generic engineering model was validated by using a “difficult” application.

For other technologies which exhibit a lesser degree of complexity in functional and architectural terms, this generic engineering model may be tailored to need while maintaining its core concepts and tenets.

Note: For the purpose of this generic engineering service model, the term “technical e-Navigation service” is abbreviated with “service”. Once a specific service is addressed, it is capitalized, e.g. in “Basic Service” or “AIS Service”.

# Context of the document

Figure 1 (overleaf) provides the reader with the relative place of the present document within the IALA e-Navigation documentation framework: This document builds upon the documents “above it” in Figure 1, in particular upon

- the general e-Navigation architecture, in particular its shore aspects, as explained in IALA Recommendation e-Nav 101 [0.1]

- the system layout definition of the common shore-based e-Navigation system architecture, as defined in IALA Recommendation e-Nav 201 [0.2].

This document informs the documents “below it” in Figure 1, in particular all of the technology-specific descriptions of the individual shore-based technical e-Navigation services at the bottom of Figure 1.

| e-Nav Recommendations on generalities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Table of Contents for “e-Navigation compliant” IALA Documentation |  |  |  |  |
| Shore-based e-Navigation system terminology (= glossary) |  |  |  |  |
| e-Nav Recommendations on e-Navigation conceptional aspects and user requirements viewed from ashore |  |  |  |  |
| The e-Navigation Architecture – the Shore-Based Perspective |  |  |  |  |
| e-Navigation system requirement analysis and e-Navigation applications descriptions: arranged in fields of e-Navigation applications as defined by OPS WG |  |  |  |  |
| e-Nav Recommendations on technical generics of the common shore-based e-Navigation system architecture |  |  |  |  |
| common shore-based e-Navigation system architecture |  |  |  |  |
| technical e-Navigation services (generic engineering model) (= this Recommendation) |  |  |  |  |
| The Universal Maritime Data Model and the data model of the common shore-based e-Navigation system architecture |  |  |  |  |
| Interface considerations with shore-based systems and between shore-based systems |  |  |  |  |
| Infrastructure considerations |  |  |  |  |
| Life-Cycle-Management Considerations: Technical Operation Personnel + Tasks/ Technical Development Personnel + Tasks |  |  |  |  |
| Quality Management and Certification Considerations |  |  |  |  |
| e-Nav Recommendations on shore-based e-Navigation applications mapping to e-Navigation services involved - examples |  |  |  |  |
| Shore-based e-Navigation applications involving “visual” Aids-to-Navigation |  |  |  |  |
| Shore-based e-Navigation applications involving VTS |  |  |  |  |
| … |  |  |  |  |
| e-Nav Recommendations on technology-specific e-Navigation services – examples |  |  |  |  |
| AIS Service | Radar Service | “Visual” AtoN Service | User Interaction Service | … other technical e-Navigation services … |

Figure 1: [Future] IALA e-Navigation documentation layout

# Overview on shore-based services

The layout of the common shore-based e-Navigation system architecture [0.2] reflects the user-requirement driven top-down approach of IMO’s e-Navigation concept. The functionality and data delivery required from the technical common shore-based e-Navigation system by the various users are fulfilled by arranging the necessary functionality and data storage in a so-called Service-Oriented Architecture (SOA).

In a technical Service-Oriented Architecture a technical “service” is the main building block and its composition is developed starting from the service-delivery to the requesting entity, which may be the (human) user or another technical service or system. So, the very name of a “service” reflects the user-requirement driven approach stipulated by IMO.

IALA members are tasked with the deployment and continuous operation of the user-required functionality and data delivery of their shore-based e-Navigation system. When deployment and continuous operation is considered, the user requirements are not the only aspects to consider, however. Cost efficiency of deployment as well as minimization of expenditure over the life-cycle of the technical system need to be taken into account, for instance. These aspects are summarized by the term “Life Cycle Management”, and they have a large impact on the engineering design of the technical services besides the user requirements.

Analyses have shown, that there are also certain structural and functional commonalities between different shore-based technical e-Navigation services, which must be exploited taking into account the above cost efficiency requirements. These commonalities have lead to the development of the present generic engineering model for the shore-based technical e-Navigation services.

The present generic engineering model was developed based on in-depth understanding of a variety of different technologies. The shore-based AIS Service was one very important technology in this regard (and it still serves as application example of this generic service model to a complex technology). Now that the generic engineering model exists, it may be applied to any other technology contained in the common shore-based e-Navigation system architecture. Hence, it serves as a generic engineering model template.

The benefits of that approach are:

- A pool of well defined generic solutions exists, and these solutions may easily be applied to different technologies;

- Thereby this generic engineering model template also serves as a “checklist” for the international community when addressing the deployment of any new technology for the shore side of the e-Navigation concept;

- A common understanding between international engineers for the shore-based technical e-Navigation services is achieved internationally; this is not only one of the objectives of IMO’s e-Navigation concept but also serves as a common ground between IALA National members and industry;

- This generic engineering model template provides a powerful tool for management decisions within IALA national members, i.e. Aids-to-Navigation authorities.

- [there may be more benefits…]

# Fundamental service design considerations

## The notion of the Basic Services (BS)

Definition: A Basic Service (BS) is a discrete service of a shore-based technical e-Navigation service and represents the functional delivery of a defined set of data objects or a defined set of functionality performed by that service. Basic implies that it is one consistent fundamental functionality of the service under consideration. The notion of the Basic Services is essentially driven by a requirement-fulfillment (i.e. client-server) relationship from the shore’s point of view, eventually driven by the shore-based user’s point of view.

Every service of the common shore-based e-Navigation system comprises a minimum set of Basic Services. A Basic Service is always represented by a name and a mnemonic meaningful in the context of that service.

### The mitigation of complexity by the definition of External Basic Services

Any service is part of the common shore-based e-Navigation system. Hence it would be of primary interest to any other shore-based service, what capabilities the specific service under consideration contributes to the shore-based e-Navigation system. These capabilities can be defined as External Basic Services (E-BS) of the specific service under consideration. They are called “external” because data and/or the effects of some functionality cross the boundary of the service under consideration.

External Basic Services are all justified by at least one desired interaction between some entity outside the boundary of the service under consideration. These entities are other services within the common shore-based e-Navigation system which request data and/or functionality, and are hence called “requesting service”. In the case of the User Interaction Service and the Gateway Service, both of which interface the common shore-based e-Navigation system to shore-based users (humans; external systems), these users could also be that entity requesting External Basic Services.

All External Basic Services of the service under consideration taken together provide the summary capability statement of that service from the perspective of the shore-based e-Navigation system or – in the case of the User Interaction Service and the Gateway Service – by the shore-based user. They thus contribute to the reduction of complexity by encapsulation (compare Figure 2).

Shore-based

e-Nav

system

Point of view

from ashore

Encapsulation of complexity

for shore-based e-Nav applications

Data

Collection and Data Transfer Service

Physical Link

E-Nav

Appli-cation

E-Nav

Appli-cation

E-Nav

Appli-cation

External Basic Services of the service as its functional interface to the shore-based e-Navigation system

Figure 2: External Basic Services of a Data Collection and Transfer Service (by example) as comprehensive capability statement of that service

For every service a list of its External Basic Services is given.

For each of the External Basic Services one functional description needs to be developed. The full description of the External Basic Service consists of

- its name,

- a mnemonic,

- its External Basic Service Category indication (see below),

- the data objects involved

- a standardized table describing the functions of the interacting components of the service under consideration,

- a figure, which maps it to the appropriate components of its service, and

- additional functional detail description as appropriate.

The encoding format of the data objects will be detailed in the appropriate Basic Service definition.

External Basic Services may request the support of other services of the shore-based e-Navigation system, i.e. rendering them “requested services” and turn the service under consideration into a “requesting service”.

For a description of the minimum set of External Basic Services refer to Annex 1.

### Basic Service Categories

The definition of BS Categories is useful throughout this Recommendation for the following reasons:

- BS Categories reflect the different net data flow mechanism within the service under consideration:

- There are different data paths and data processing requirements for receiving and transmitting within the service under consideration with associated consequences for design and operation.

- Although apparently the same data object being treated in different BS Categories, the meaning of that data object depends on the net data flow direction, i e. its context. That means, that although the data object looks the same and there is a similarity in appearance and therefore even identity of description, the data objects are different due to their role in the data flow from an ontological point of view.

- To illustrate this, an example from the AIS Service is given: Data received from AtoN AIS stations by ATON_DAT (received) means something different than transmitting AtoN messages via the AIS Service by ATON_DAT (transmit), although the data object description looks identical. In the first case there is a physically existing AtoN AIS Station involved, while in the second case, there is a synthetic or even virtual AtoN involved.

- BS Categories best reflect the data orientation paradigm (as opposed to the technological orientation), which is a fundamental feature of the e-Navigation concept.

- BS Categories reduce the complexity of the service under consideration by employing functional similarities to the largest extent possible.

- BS Categories, once defined, provide a fixed reference point throughout the description of the service under consideration for statements like: “For BS Category x the following applies: ...”.

- BS Categories increase description efficiency of the functionality of the service under consideration substantially, i.e. even allow for a minimum description.

- BS Categories make the description of the service under consideration robust to amendments of the precise data object content.

- BS Categories simplify the maintenance of the documentation of the service under consideration at large because additional data contents can be added easily to the appropriate BS Category without the need to change much of the otherwise BS category-related description: The amendment to the BS Category description will have an immediate effect on all individual BS of the same BS Category.

### Internal Basic Services and their categories

There are also Basic Services, which are not justified or requested by an external “requesting service” of the shore-based e-Navigation system or – in the case of the User Interaction Service or the Gateway Service – by a shore-based external user. These Basic Services are called Internal Basic Services (I-BS): They are supportive in nature, but are essentially required. Examples of Internal Basic Services are recording of service data (pre-requisite to any replay), service initialization, service termination and component management of that service. Internal Basic Services may not be directly visible outside the service, neither on their appropriate Physical Link (in the case of a service with a Physical Layer) nor within the shore-based e-Navigation system.

But also more sophisticated functionality related to the specific technology of the service under consideration often is an Internal Basic Service: Internal Basic Services often interact with the Technical Operation Personnel responsible for the service under consideration, while this functionality is not requested by the shore-based e-Navigation system, i.e. while there is no “requesting service”.

The Internal Basic Services are also categorized into Internal Basic Services Categories in a similar fashion to the External Basic Services.

An Internal Basic Service may request the support of other services of the shore-based e-Navigation system, i.e. rendering them “requested services” and turn the service under consideration into a “requesting service”.

For a description of the Internal Basic Services which every service needs to have refer to the description of the minimum set of Basic Services in Annex 1.

### Example for a Data Collection and Data Transfer Service

Figure 3 provides an example overview on the External Basic Services of a Data Collection and Data Transfer Service, using the above external Basic Service Categories.

Figure 3: Abstract example of External Basic Services for a Data Collection and Data Transfer Service

Note: The dotted lines indicate inheritance relationship.

Figure 4 shows an overview on the Internal Basic Services of a Data Collection and Data Transfer Service as an example in overview.

Figure 4: Overview on the Internal Basic Services of a Data Collection and Data Transfer Service

Note: The dotted lines indicate inheritance relationship.

## The data a service delivers and how it is being modeled

### Introduction

The above Basic Services define the functionality provided by the service under consideration to the shore-based e-Navigation system in an abstract manner. They are operations performed on certain data objects, or: they are f(x), to express it in mathematical terms. The data objects treated by the Basic Services are the operands, i.e. the ‘x’, of these functions. To provide a complete picture of what the service under consideration delivers to the shore-based e-Navigation system, it is necessary to consider those data objects and their structure and mutual relationship. Eventually, it is the data, which is only relevant for other services.

### Usage of the Universal Maritime Data Model (UMDM)

--- Introduce Universal Maritime Data Model (UMDM)

The Data Model of any technical e-Navigation service provides the well-structured and abstract (functional) description of the data objects, the service under consideration exchanges with requesting services of the common shore-based e-Navigation system, [and of the data objects the service under consideration uses for internal purposes]. Thus the Data Model of a service is an application of the UMDM: In the UMDM all data objects are expressively described in one place together, with data attributes.

Also, the Data Model of the service under consideration shows the contribution of its different Basic Services to the overall knowledge the common shore-based e-Navigation system. Hence, the Data Model shows the contributions of the service under consideration to the UMDM. It also assigns the Universal Data Object Identifiers (U.DOI) of the data objects used by the service under consideration.

The Data Model of the service under consideration provides the designer of requesting services with precise data on what data the technical e-Navigation service under consideration may provide (receive) or expect (transmit), without the need to know the technological and/or algorithm details of the service under consideration . Hence the data model helps to encapsulate the “science” of the service under consideration on behalf of the shore-based e-Navigation system as a whole.

Although, the Data Model of any service does not contain encoding data, encoding is required on application level (i.e. regarding the Basic Services). The Data Model supports a variety of general purpose encoding techniques on application level. Hence, the Data Model provides flexibility in selecting an encoding technique as deemed appropriate by the competent authority. A portfolio of appropriate and compatible encoding techniques to be used with data objects of the service under consideration is given in its Interfacing Model (refer to Annex 5).

## The parts of a service and their external interactions in general

Figure 5 presents the parts of a service and their external interactions.

The material parts of a service are called components of the service, which are further sub-divided into so-called service components and so-called service-owned infrastructure (components).

The service components may be functional software modules or technology-specific hardware components (such as technology-specific base stations, antennas and the like). The service components carry the functionality and/or the data delivery to create the Basic Services, while the service-owned infrastructure components (such as computers and Local Area Networks) are involved in that process but only as a tool and should remain “invisible” in the process. This distinction of service components and their respective functional description from the service-owned infrastructure and their respective requirement statement allows to decouple the more steadily changing or even lasting definition of the actually desired functionality from the rapid technology change at the service-owned infrastructure (compare for example the short-lived computer and IT network generations). Thus this distinction effectively and efficiently supports life-cycle management.

The service components are described in this IALA Recommendation as far as generic features are concerned. The service-owned components are described in some other IALA Recommendation as well as the on-site infrastructure (compare Figure 1). The technology-specific functionalities of the service components are described in the appropriate description of the individual services, which are informed by this IALA Recommendation.

There are immaterial parts of the service as well, such as various immaterial “models” mentioned in Figure 5. Their purpose is mainly to show the interactions between the material components from different angles of perspective.  The description of the immaterial parts of a service forms the largest part of this Recommendation. Refer to a dedicated chapter below for a more detailed description.

Finally, there are interactions between both material and immaterial parts of the service with external entities (association indicated by lines in Figure 5). “External” may be either entities external to the service under consideration or even external to the shore-based e-Navigation system. These interactions are taken into account in the appropriate description of the part.

Note, that the possible interactions with external entities depend on the relative position of a service within the common shore-based e-Navigation system architecture [0.2], as indicated by the footnotes in Figure 5. From this also the distinction between services with and without Physical Layer is derived, which will be used throughout this Recommendation. Compare IALA Recommendation on the system layout [0.2] for a list of the different services in this regard.

--- make sure this is true for the system layout document (= eNAV201)

Figure 5: Parts of a service and their interactions with external entities

## The influence of geographical topology on a service

It is necessary to consider the influence of geographical topology on the design of the service under consideration. Figure 5 provides a generic geographic distribution setup of a shore-based e-Navigation system and the place of a Data Collection and Transmitting Service, as an example. The full development of the different generic geographic distribution setups are contained in the Annex 3 (Distribution model).

Figure 6:  Generic geographic distribution setup of a shore-based e-Navigation system and the place of the Data Collection and Transmitting Service

--- maybe „Data Transfer Services“ may suffice (instead of “Data Transfer and Collection Service”), once it is defined somewhere that „transfer“ also includes „sensory collection“ => bearing on eNAV201 as well

Despite the differences in individual geography and topology there are functional similarities regarding the distribution of components of the different services of the common shore-based e-Navigation system architecture, and also certain criteria to select a specific Service Distribution Configuration. The Distribution Model of a service describes some standard distribution configurations which would apply to most administrations, and also provides technology-specific rationales and criteria.

The Distribution Model is essentially functional and generic. It needs to be applied (“tailored”) to the actual geography and topology by every administration. To that end guidance is provided how to arrive at a proper individual distribution model taking into account the following aspects:

- coverage requirements and coverage planning

- coastal topology (concave, convex, line, one spot, ...)

- required functionality setup of the service under consideration, i.e. required Basic Services

- distribution of receive / transmit functionalities (if appropriate)

- maintenance and life-cycle considerations (rationalization)

- data transfer network considerations

- infrastructure requirements

In most cases, due to coverage area requirements, the service of an administration spans a larger part of its coast or even the whole of its coast(s). The actual topology of the coastlines may vary from country to country. The actual number of sites needed depends on a variety of aspects. In some cases, the service under consideration may shrink to an “one spot service”.

The Distribution Model is given in Annex 3.

# The layered structure of a service - Structure Model

The Structure Model describes the layered structure of a service and thereby defines in a general way their specific functionality.

## Services with and without Physical Layer

As indicated above, there are services with Physical Layer and without. Hence, the Structure Model will be different in this regard, while there is a lot of commonality regarding the other aspects of the Structure Model.

### Services with Physical Layer

Each and every service with a Physical Layer consists of three main functional layers as follows (refer to Figure 7):

- Logical Layer

- Physical Layer: comprises all Remote Shore Stations (RSS) of the service under consideration

- Service Management Layer

The Logical Layer of a technical e-Navigation service is the layer which contains all functionality of the service, which treats system-requested net data of the service (as opposed to e.g. management and/or maintenance data) and which is not tied to a specific location at the waterway (as opposed to the Physical Layer). Hence, by definition, the Logical Layer resides at a Node site (compare Distribution model). The Logical Layer comprises the functional component Logical Shore Station (LSS) and other supportive service-owned components.

The Logical Shore Station (LSS) is the functional component of a technical e-Navigation service, where the actual net data treatment is done. Hence, by definition, the LSS resides at a Node site (compare Distribution model). In most cases it is a software module residing in a computer (= service-owned infrastructure component).

The Physical Layer directly interacts with its Physical Link. The Physical Link could be a radio link or a visual link or an acoustic link, depending on the actual technology of the service under consideration. The Physical Link as such does not belong to the service, although the service may influence it by its usage and/or management. The Physical Link in most cases is outside the shore-based e-Navigation system.

For the description of Remote Shore Stations (RSS), the Physical Layer can be further subdivided into three layers as follows:

- Physical Shore Station (PSS) Layer

- Physical Link Terminal Equipment (PLTE) Layer

- Physical Link Couplers (PLC) Layer

For reasons of space/direction diversity in coverage of a RSS under consideration, the lowest two layers are often sub-divided into Sectors, while their stacked or hierarchical relationship remains. A Sector comprises always Physical Link Terminal Equipment and Physical Link Couplers together which are dedicated to a certain space direction regarding coverage. Conceptually wise, omnidirectional coverage ranges can be construed as a One-Sector RSS. Well known examples for the above concept of sectorization are e.g. radio services using directional antennas or fixed visual aids with sector lights.

The Physical Layer of the service under consideration may serve different Physical Links or different physical link technologies simultaneously, which would lead to another subdivision of the two lowest Layers of the Physical Layer together: Physical Link Terminal Equipment X together with Physical Link Couplers X would together and in conjunction serve the Physical Link X. Similarly, the Physical Link Terminal Equipment Y together with the Physical Link Couplers Y would together and in conjunction serve Physical Link Y. A well known example for a service with a Physical Layer serving different Physical Links concurrently is Floating Visual Aids Service, and buoys as the RSSs of that service often operate on various Physical Links simultaneously, namely on the visual link (colour and lights), the acoustic link (fog horn), the radar link (radar reflector and RACON), and on the AIS VHF Data Link (AIS AtoN station).

All different Physical Links served and/or all sectors of a RSS are co-ordinated by the PSS alone and concurrently, and all data delivered to/from all different Physical Links served and/or to/from all sectors are processed by the PSS concurrently.  Hence, the PSS Layer is not subject to the sectorization, but it needs to take into account different Physical Links, if present. Also, in order to avoid ambiguity and discrepancies, the Service Management does interact directly with the PSS alone, and not with any component within the Physical Link Terminal Equipment and the Physical Link Couplers and within any sector.

The Service Management Layer is the layer which contains all functionality of the service to configure it, both from the requesting service and from the Technical Operation Personnel, and manage its components during run-time. The Service Management Layer is represented at the Nodes and at the Remote Sites. Service components within the Service Management Layer are a Service Management Master, which contains all centralized management functionality and data storage and which resides at a Node, and Service Management Agents at a Node (SM-Node) and at a Remote Shore Station (SM-RSS).

Each layer comprises:

- service component, which provides the required functionality in terms of service-specific data processing;

- supporting components and resources, which are exclusively used by that service, such as computers and local networking devices, i.e. the so called service-owned infrastructure.

- (remote) Human Machine Interfaces (HMI) to allow for access to Technical Operation Personnel

Each layer is supported by on-site infrastructure, such as energy supply, which may be shared with other services co-located on the same site.

Figure 7: Layered structure of a service with Physical Layer – Structure model

Technical Operation Personnel (master control)

HMI

Logical Layer -

Logical Shore Station (LSS)

HMI

technical e-Navigation service

Net Data

Status

Service

Management Layer

HMI

Physical Shore Station (PSS)

Layer

HMI

Physical Link Terminal Equipment (PLTE) Layer

HMI

Physical Link Couplers (PLC)

Layer

Physical Link

(= medium)

Technical Operation Personnel

(there are tasks for each layer in principle)

Configuration

Physical

Layer /

Remote

Shore

Station

* Note: This symbolic representation does not imply any inference as to the amount of required personnel for the technical operation of the service. This symbolic representation aims at indicating that human interaction with the largely automated service is required as the last resort - and in some cases possibly on a regular basis - on all layers of the service.

Boundary of shore-based e-Nav system

## Services without Physical Layer

Each and every service without a Physical Layer consists of two main functional layers as follows (refer to Figure 8):

- Logical Layer

- Service Management Layer

--- introduce description (only in terms of differences to above)

--- introduce simplified drawing without Physical Layer: Figure 8 --- Placeholder ---

# Resulting service setup and its component’s tasks

## Resulting service setup

So far, the most fundamental aspects for a service have been introduced such as its deliverables to the shore-based e-Navigation system, as well as its functional parts, the influence of geographical distribution, and its layered structure. These elements have been introduced largely independent of each other. This chapter aims at tying these elements together and provides some synthesis in general.

The Figure 9 shows a typical setup of a service with Physical Layer (for services without Physical Layer, simply drop the statements related to the Physical Layer and replace its role by the contribution of all of the requested services of the shore-based e-Navigation). It is one possible translation of the more abstract layered structure depiction of the service in Figure 7 (see above) into one example setup using specific physical components on specific functional sites.

Figure 9 shows a so-called One-Node AIS Service Configuration setup. Figure 9 shows the functional components, the functional site designations, where these components reside, and also gives some indication regarding the interfacing of a service to its context within a shore-based e-Navigation system. Figure 9 shows some important features of a service as follows:

- The setup of the Remote Shore Station (RSS), i.e. the components internal to it, will be described in Annex 10.1 in more detail. Regarding Figure 9 it is sufficient to note, that there is Physical Link Terminal Equipment (symbolized by antenna on roof top of RSS), that allows the service to access its Physical Link(s). The depiction of the RSS as a house also gives an indication of the relevance of on-site infrastructure consideration.

- A Node site with the Logical Shore Stations is shown, and how they are connected to the components of the other services of the shore-based e-Navigation system at the same Node site. As an (typical) example, particularly the User Interaction Service of the shore-based e-Navigation system is shown. The “other services” may be any other service of the shore-based e-Navigation system.

- The components of the Service Management (SM) that reside at the Node site are shown. (It should be noted, that components of the SM also reside at the Remote Shore Station; see Annexes 10.1 and 11).

- Also the support for Technical Operation Personnel by Human Machine Interface(s) (HMI) provided by the service is shown.

- Note the concept of the functional sites in combination with the premises concept.

- All components at the premise N where the Node site, the Technical Operation Personnel site and the User site reside in the example are connected with LAN functional links / LAN technology components. Only, when it comes to connecting the components of the service at the Node site with its Remote Shore Stations on the Remote sites, WAN feeder links will be used.

Cautionary note: Figure 9 should not be construed as normative or the only recommended way to set up an service. Figure 9 does claim however to be correct and consistent with the normative statements in this Recommendation in regard to what it shows.

Cautionary note: While Figure 9 describes one example of a workable service setup, it should be noted, that the available features and the available options in setting up a service can only precisely be captured by the more abstract descriptions provided in this Recommendation.

Figure 9: Example of a service setup (One-Node Service Distribution Configuration within shore-based e-Navigation system context)

## Resulting tasks of the functional components of the service

### Tasks of the functional components in the context of their interaction

The functional components can now (more) precisely defined by their individual set of tasks in the context of their interaction with each other and with entities external to the service. The Figure 10 shows the different tasks of the different functional components of the service in context.

Please note

- the different task orientation of functional components, i.e.

- the orientation of the Logical Layer towards the requesting / requested services of the shore-based e-Navigation system and their requirements;

- the orientation of the Physical Layer towards the Physical Link(s) served/employed by the service and its specific physical requirements;

- the need to have a service wide data storage which allows service-wide visibility and access of relevant data objects;

- the configuration and control capabilities of the Service Management (SM).

- that the instances are now implied, namely

- that the Physical Layer consists of all Remote Shore Stations (RSS) of the individual service under consideration as deployed by an individual administration;

- that the Logical Layer consists of all Logical Shore Stations (LSS) set up as individual software processes of the individual service under consideration as deployed by an individual administration;

- that there are necessary functionalities, such as the service wide data storage, which are invisible to the net data flow between the Physical Link(s) and the requesting services, and vice versa. These functionalities are supported by infrastructure components owned by the AIS Service (so called service owned infrastructure), namely by distributed data bases, local area networking, replicator devices, etc.

To specify the precise features and attributes of those functional components, such as data processing capabilities, interaction capabilities, interfacing requirements, timing constraints, and reliability figures, the appropriate service models need to be considered as introduced above.

These models have discussed the service on the class level. In order to set up a service by an individual administration, it is necessary to eventually arrive at specific components which materialize at specific sites, i.e. at the instance level. This is where the Distribution model fits in.

All these different aspects need to be merged within the description of the functional components. This will be done in the appropriate Annexes, describing the particular functional component. In the following sections, a short introduction is provided.

Figure 10: Overview over tasks of the functional components of the service

This Figure 10 will be used and explained in several Annexes which elaborate certain aspects of this figure.

### Mapping of service capabilities to component functionalities and component requirements

To finally arrive at the required functionalities and at the requirements of the components internal to the service specifically, it is necessary to map the Basic Services (both external and internal) to the relevant individual components internal to the service. In accordance with the layered structure of the service there are three main components, namely the Logical Shore Station (LSS), Physical Shore Station (PSS), and Service Management (SM) to be considered for the mapping.

In the case of services without Physical Layer there is no PSS. In the case of services with Physical Layer the Physical Link Terminal Equipment, the Physical Link Couplers (PLC), and also the Physical Link as such are omitted from the consideration at this point since they are transparent in regard to the net data flow. They will re-appear when considering secondary issues of the service, such as component management proper or management of the Physical Link.

As an example for the mapping in the case of a service with a Physical Layer, the Figure 11 shows the mapping of an External Basic Service (Category E-1) to the component functionalities of the above components. As an example for the mapping in the case of a service without a Physical Layer, the Figure 12 is given.

Figure 11: Sample mapping of External BS (Cat. E-1) to component functionality in the case of service with at Physical Layer

Figure 12: Sample mapping of External BS (Cat. E-1) to component functionality in the case of service without at Physical Layer

The required component functionalities of the service under consideration are collected in Annexes 9, 10 and 11 of this document which deal with the functional descriptions of the respective components.

### Type approval considerations for PSS, LSS, and SM software modules

IMO, within its e-Navigation strategy [---Reference---], has formulated certain requirements regarding quality level of services offered by a competent shore authority:

--- Provide examples/references

Ultimately these requirements can only be achieved and validated by passing the procedure of type approval for the functional components involved. This section elaborates on the type approval aspect using the above introduction of the functionality descriptions for the PSS, the LSS, and the SM.

Regarding type approval the following notions should be taken into account:

- Type approval for software modules should only apply to the service components of the PSS, LSS, and SM software modules. Ideally these specific functionalities would be plug-in software modules. Type approval would not address or include sevice-owned infrastructure components, such as computers, LAN components etc.

- The description of the service components of PSS, LSS, and SM in Annexes 9, 10, and 11 would be precise in terms of functionality while not being prescriptive regarding implementation method by a manufacturer. Thus, the functional components PSS, LSS, and SM would be modeled as “black boxes”, rendering open interface definitions.

- The following aspects and the associated degree of impact should guide the selection of specific functionality of these functional components subject to type approval:

| Aspect | Degree of impact |  |
| --- | --- | --- |
|  | HIGHER | LOWER |
| Local impact of Physical Link(s) related run-time configurations (1) | PSS | SM |
| Coastal-wide impact of Physical Link(s) related run-time configuration (1) | SM (in particular SM-Master functionality) | PSS |
| Quality of data delivery to/from shore-based e-Navigation system | LSS + PSS | SM |

(1) for services with Physical Layer only, and only when Physical Layer has capability to influence Physical Link(s) as such.

# Interaction between services using the Structure Model

--- introduce in an overview manner (with Figures), how the Structure Model helps in understanding the interaction of different services of the shore-based e-Navigation system and how it shows, what layers are involved in what interaction and (eventually) what interfaces are needed to that end => Pointer to Interfacing Model…

# Overview description of the parts of the generic service engineering model

The previous chapters provided a solid introduction and overview. The individual parts of the generic service model need to be introduced in more detail to arrive a that precision level necessary for specific use by IALA, IALA membership and the supply industry at large for planning, procurement, deployment and operation of any specific service. There is an Annex dedicated to one of the following parts each (compare List of Annexes). The following introduction also serves as an overview content definition for the Annexes.

The generic service engineering model is described by the following parts, which complement each other:

## Deliverables of a service to the common shore-based e-Navigation system

Basic Services Categories and Minimum Set of Basic Services

Introduces the normative definitions of the categories of both External and Internal Basic Service Categories and introduces the Minimum Set of Basic Services, i.e. a comprehensive normative definition of Basic Services generally required for each service of the common shore-based e-Navigation system. Refer to Annex 1 for further discussion.

Data objects of a service and their properties (data model):

Introduces abstract descriptions of the data provided by the service at its Basic Services level to the common shore-based e-Navigation system, i.e. defines data objects and their properties of a service. Refer to Annex 2 for further discussion.

## Parts of the generic service model related to the layout and setup of a service

Distribution model

The Distribution model deals with issues arising from geographical distributed service layouts, including generic aspects of coverage planning as one major factor if appropriate. The Distribution model is given in Annex 3.

Interaction and Data Storage model

The Interaction model introduces the dynamics of the interaction of individual components of the service and in particular within the service (data flow dynamics, such as push or pull concepts) and deals with associated data storage issues in an abstract fashion, on a generic level.

The Data Storage Model deals with storage entities of the service on an abstract level. It is relevant in particular due to the geographical distribution of its components. It deals with aspects like visibility of data objects, age of data objects, and rule base for data processing in the service, and consequences of geographically distributed service distribution configuration regarding distributed data storage, such as replication.

The Interaction Model and Data Storage models together are given in Annex 4.

Interfacing model

The Interfacing model of the service describes the interfaces of the service to other services of the shore-based e-Navigation system and the interfaces between the components of the Service. Following aspects are taken into account:

- the application encoding methods; a portfolio of encoding techniques is recommended to suit the data objects of the Data Model;

- the protocol stacks recommended for data transfer internal and external of the service.

All application encoding methods and protocol stacks recommended are in keeping with the open system philosophy of the shore-base e-Navigation system architecture. The Interfacing Model is given in Annex 5

Internal Time Latency model

The Internal Time Latency Model of the service describes the time latency limits regarding the Basic Services (by external and internal BS Category). In particular the resulting permissible latency of data delivery of the service operating within the shore-based e-Navigation system is discussed. This is relevant, for example, when correlating time-critical data from different Data Collection and Data Transfer Services within the Value Added Data Processing Services of the shore-based e-Navigation system. From the timing requirements of the Basic Services, concrete timing requirements for components of the service and their mutual interconnection are derived. The Internal Time Latency Model is given in Annex 6.

Internal Reliability model

The Internal Reliability Model of the service analyses the internal reliability aspects of the service and describes how to measure internal reliability during run-time. The Internal Reliability Model is based on the Structure Model, the Distribution Model and the Interaction Model. Details are given in Annex 7.

Test model

The Test Model for the service:

- describes what are good test strategies the service as a whole for Technical Operation Personnel, and

- provides guidance how to ascertain the proper function of an service after cold or warm start.

The Test Model does not generally deal with testing of individual components. Details are given in Annex 8.

## Parts of the generic service model related to components of a service

Generic functional description of the Logical Shore Station

The generic functional description of the LSS describes the functionality needed on the Logical Layer to provide the External and Internal Basic Services, employing the concept of the Basic Services Category. In particular, this part contains the contribution of the LSS to the Minimum Set of Basic Services. For the description of an individual service, the technology-specific functionalities of the LSS of this individual service is contained in this part. See Annex 9.

Generic functional description of the Physical Layer as a whole – the Remote Shore Station

The generic functional description of the Physical Layer as a whole describes how the different Sub-Layers of the Physical Layer interact to achieve the contribution of the Physical Layer to the External and Internal Basic Services. Also the additional components of the service and their interactions at the Remote site are introduced, thus rendering a comprehensive description of the Remote Shore Station of the service under at a Remote site consideration. See Annex 10.1.

Generic functional description of the Physical Shore Station

The Physical AIS Shore Station (PSS) performs the following main tasks:

- Pre-processing of data in one or both directions (receive and/or transmit), depending on configuration

- Control of the Physical Link Terminal Equipment Layer of the RSS depending on the configuration preset of the Service Management; this includes also control of the Sectors, if defined.

The PSS essentially is a software process running as application software on a computer as a physical entity or as an integral firmware of a dedicated unit, such as a smart switch.

Details are described in Annex 10.2.

Generic functional description of the Physical Link Terminal Equipment

The Physical Link Terminal Equipment comprises all devices and/or functionality needed to – regarding “receiving“ - transform a signal received from the Physical Link Couplers into a data object as appropriate for the specific technology of the service under consideration. The data object is forwarded to the Physical Shore Station (PSS).

Regarding “transmission”, a data object received from the Physical Shore Station is transformed into an appropriate signal which is forwarded to the Physical Link Couplers for eventual transmission on the Physical Link. Examples of PLTE are radio transceiver devices in the case of radio based services or light generating/receiving devices in the case of visual services. Details are described in Annex 10.3.

Generic functional description of the Physical Link Couplers

Regarding “transmission”, Physical Link Couplers are all devices and/or functionality needed to effectively couple a signal generated by the Physical Link Terminal Equipment into the appropriate Physical Link(s) and/or – regarding “receiving” – to extract signal(s) present in the Physical Link(s) and forward it/them to the Physical Link Terminal Equipment. Examples of Physical Link Couplers are radio antennas for radio wave signals or optical lenses for light signals. Details are described in Annex 10.4.

Generic functional description of the Service Management

The Service Management of a competent authority performs the following tasks:

- It acts as a management entity for the whole of the service,

- it manages the Internal Basic Services, and

- it is the last resort regarding faulty behaviour.

Specifically, the SM:

- invokes, initialises, configures and terminates all the instances of LSS and PSS at run-time;

- determines the functional connections between PSS and their associated LSS for them to use during run-time;

- determines the functional connections between the LSS and the requesting services’ logical interface associated with them, i.e. this top level acts as a "switch-board" for the data exchange relationships between the different processes;

- provides the Human Machine Interface for the Technical Operation Personnel to monitor the current status of the service and potentially of the appropriate Physical Link(s) and configure them accordingly.

For most Service Distribution Configurations the SM consists of several distributed agents and of one centralized master functionality, all of which are functional sub-components of the SM and are generally software processes. There are the following functional sub-components:

- SM-Agent at the Remote Site, which represents the ASM at the Remote Shore Station of the service,

- SM-Agent at the Node site, representing the SM to the LSS,

- SM-Master (regularly only one SM master per service).

Details are described in Annex 11.

## Parts of the generic service model related to installation and life-cycle management issues

On-site infrastructure considerations and site co-location issues

These two aspects are related:

- All the components of a service, i.e. the service components proper and the service-owned infrastructure components have requirements regarding the on-site infrastructure, such as housing, protection against a variety of external influences, energy supply, source for a time base, and so forth. These requirements are discussed in a generic fashion in this part.

- Also, at Remote Sites usually several different services of the common shore-based e-Navigation system architecture are present. For example, there are Remote Sites typically populated by a variety of radio services, such as AIS, VHF voice communication, and GMDSS DSC service. Sharing on-site infrastructure may be a economical necessity in most cases. This results in site co-location issues, which are addressed in a generic fashion in this part.

Refer to Annex 12.

Recommendation regarding efficient operation and maintenance of a service

In this part all generic aspects are discussed regarding the efficient operation by Technical Operation Personnel as well as the efficient maintenance and further development by Technical Maintenance/Development Personnel. By this part, the concept of the technical shore-based e-Navigation service is tied into the well established conceptual framework of technical operation and life-cycle management. Means to maintain and improve efficiency of the service delivery over the full life-cycle are described here generically.  Refer to Annex 13.

Runtime configuration management of a service

This part addresses generic runtime configuration of the service by the Technical Operation Personnel. In this Recommendation the conceptual framework is provided, which is used by the individual descriptions of the technology-specific individual services. Refer to Annex 14.

List of Annexes

The following list is the list of Annexes of the present Recommendation on the generic service engineering model template.

Since it is meant to be a template to inform IALA Recommendations on individual, i.e. technology-specific services (such as AIS Service or Radar Service), the order of appearance and the numbering scheme was developed to be used in an identical fashion throughout the IALA Recommendations on individual, i.e. technology-specific services. This would allow for ease of reference to this Recommendation while providing some flexibility regarding in particular technology-specific aspects related to installation and life-cycle management: Additional annexes on e.g. technology-specific runtime configurations may be added as Annexes 15ff. Hence, designers of technology-specific service documentations are strongly encouraged to adhere to this setup of Annexes.

Annex 0 – References, Glossary of Terms, Abbreviations

This Annex 0 contains all references, terms and abbreviations from throughout this Recommendation.

Deliverables of a service to the common shore-based e-Navigation system:

Annex 1 – 	Basic Service Categories definitions and Minimum Set of Required Basic Services

Annex 2 – 	Data objects of a service and their properties (data model)

Annexes related to the layout and setup of a service:

Annex 3 – 	Distribution model

Annex 4 – 	Interaction and data storage model

Annex 5 – 	Interfacing model

Annex 6 – 	Internal Time Latency model

Annex 7 – 	Internal Reliability model

Annex 8 – 	Test model

Annexes related to functional components of a service

Annex 9 – 	generic functional description of the Logical Shore Station

Annex 10.1 – 	generic functional description of the Physical Layer as a whole

Annex 10.2	- generic functional description of the Physical Shore Station

Annex 10.3 – generic functional description of the Physical Link Terminal Equipment

Annex 10.4	- generic functional description of the Physical Link Couplers

Annex 11 – 	generic functional description of the Service Management

Annexes related to installation and life-cycle management issues

Annex 12 – 	Site co-location issues and on-site infrastructure considerations

Annex 13 – 	Recommendation regarding efficient operation and maintenance of a service

Annex 14 - 	Runtime configuration management of a service

(for further details in particular on the mechanisms refer to Annex 5)

Note: “Service components” and “components of a service” is not the same, since the latter comprises the former plus the service-owned infrastructure components.

Refer to Distribution model in Annex 3 for further details

Refer again to the Distribution model, Annex 3, for further explanation regarding the sites.

For further detail refer to the Interfacing Model, Annex 5.

For details regarding the setup and installation of an RSS also in infrastructure terms refer to the Installation guidelines for RSS, Annex 12.

The term “class” is used as defined by the object-oriented paradigm.