IALA eNav2 Input Document

Proposed changes to IALA Recommendation R-129

On GNSS Vulnerability and Mitigation Measures

Introduction

Edition 1 of the above document was issued in December 2004 and is considered to be a valuable input into discussions in this field. However, with the introduction of the concept of eNavigation, it is proposed that the document requires revision.

Addition to Section 1.2 - Scope

“The key structural components of a safe and comprehensive E-Navigation policy are……accurate and reliable electronic positioning signals, with ‘fail-safe’ performance (probably provided through multiple redundancy, e.g. GPS, Galileo, differential transmitters, Loran C and defaulting receivers or onboard inertial navigation devices)”  (1)

Addition to Section 3 - Sources of Vulnerability

Inset additional 4th paragraph:

‘A less commonly observed failure mode is the permanent or temporary disablement of GNSS receiver antenna subjected to high power radar transmissions, owing to microwave damage to internal components’ (2)

Amendment to Section 3.3 – Risk Analysis

To bring this table into line with other documents and extend its scope, it is proposed to replace the table with:

| Threat | Risk | Consequences | Mitigation difficulty/cost |
| --- | --- | --- | --- |
| System failure | L | H | H |
| Power supply failure | M | H | L |
| Receiver/antenna failure | M | H | L |
| Onboard interference | M | M | L |
| External interference | L | H | M |
| Ionospheric | L | M | M |
| Jamming | L | H | M |
| Spoofing | L | H | H |
| Radar burn-out | L | H | L |

Amendment to Section 3.5.2 – Backup Systems

First bullet; replace ‘Loran-C2’with ‘Loran-C / Enhanced Loran (eLoran)’; delete footnote.

Additional line after ‘….precise timing reference’: ‘A number of demonstration projects in the US and Europe(3) have shown that eLoran can meet the IMO navigation performance requirements of the Worldwide  Radionavigation System for harbour entrances, harbour approaches and those coastal waters with a high volume of traffic and/or a significant degree of risk (4).’

Additional 4th paragraph: ‘An important bonus of using eLoran – something GNSS cannot provide - is the eLoran compass. When the receiver is used with an H-field (Magnetic Loop) antenna it can be employed as an automatic direction-finder taking bearings on the transmitting stations. From these, the receiver calculates the ship’s heading, generally with an accuracy of better than 1, and is independent of the ship’s movement.’

- Development of an E-Navigation Strategy, Submission to IMO MSC81, December 2005

- Performance Standards for Shipborne Galileo Receiver Equipment, Submission to IMO NAV52, April 2006

- The General Lighthouse Authorities’ Loran programme and current status in Europe. Basker.S., Proc. International Loran Association 2006 Convention and Technical Symposium, Groton CT, October 2006.

- Worldwide Radionavigation System. International Maritime Organisation. Resolution A 953(23). December 2003.