# Comments by the ANM Committee on IALA Recommendation R 135 - The Future of DGNSS

## Background

The IALA Council, at its 37th Session (Dec 2005) noted that “the document was missing the user’s needs and that the ANM Committee could be asked to explore this aspect.”  The suggestion was approved for the future ANM Committee work programme and the document was accepted in its present form.

As the ANM Committee met after the e-NAV Committee, their input could not be considered by the later committee, when finalising the IALA Recommendation.

The ANM Committee has now provided the relevant input (E-NAV2/8/2 refers).

## Action requested

1) The E-NAV Committee is requested to consider the Council’s comments and the ANM Committee’s input regarding user needs, and decide as appropriate.

2) The E-NAV Committee is also requested to consider minor corrections proposed by the ANM Committee below:

a)    Correction on Page 6 of Recc

| ## System | Provider cost | User cost | Marine Standard |
| --- | --- | --- | --- |
| IALA DGNSS | moderate | low | Yes |
| SBAS | very high | very low | No |
| AIS | low | low | Yes |
| Pseudolites | high | moderate | No |
| Eurofix | low* | moderate | No |
| RTK | moderate | high | No |

b)    Correction on Page 10 of Recc

messages for 9 satellites[1] every 6.3 seconds at 100 bps. The repetition rate of these

[1] Generally, Reference Stations are able to acquire and track 12 SV, but broadcast corrections for a maximum of 9 satellites (see, for instance, US Department of Transportation, USCG, “Broadcast Standard for the USCG DGPS Navigation Service (COMDTINST M16577.1)”, April 1993, Chapter 2, Paragraph I.1.d., p. 2-8). If there are more than 9 satellites above the mask angle then differential corrections are broadcast to the highest 9.

Additional comments:

The IALA DGNSS beacon system originally was developed by the U.S. Coast Guard, and that prototype DGPS broadcasts were sent from the Montauk Point, New York radiobeacon from 15 August 1990. To provide for the transmissions a form of phase modulation identified as Minimum Shift Keying (MSK) were introduced, as well as the Radio Technical Commission for Maritime Services (RTCM) SC-104 Differential GPS standard. These were later adopted by IALA. An important consideration at that time was the Selective Availability feature of the GPS system, which degraded the accuracy for civilian use to within plus / minus 100 metres. On May 1, 2000 President Clinton announced that the policy of degrading should be discontinued. This in principle meant that accuracies of plus / minus 18 metres would be generally available. It should be noted that the U.S. Department of Defense has reserved the right to turn SA back on

**********************

[