| From: IALA e-Navigation Committee (e-NAV1) | e-NAV1-output-02 rev2 |
| --- | --- |
| To: IMO COMSAR and ITU-WP8B (information) | October 18, 2006 |

Liaison Note

AIS SEARCH AND RESCUE TRANSMITTER (AIS-SART) STATION

1	IALA thanks IMO for the liaison statement on the proposed AIS Search and Rescue Transmitter (AIS-SART) and considered the matter at the e-NAV Committee meeting in September 2006 and at an intersessional meeting on this subject in October 2006.

2	IALA envisages that an AIS-SART station potentially has the capability to exceed the performance characteristics of existing radar SARTs in certain sea conditions and environments that reduce the effectiveness of radar.

3	There are many options for meeting the performance requirements whilst complying with ITU-R M.1371-2 and being backward compatible with existing AIS stations, in particular Class A mobile AIS stations.  It is assumed that at least the MKD on existing vessels equipped with a Class A AIS station should alert the OOW and display the position and a safety related text message from the AIS-SART station.

4	The AIS Technical Working Group of the IALA e-NAV Committee organised an intersessional meeting to consider the matter further in detail to ensure that the device operates cooperatively and effectively on the VDL and has the following recommendations:

4.1	It is recommended that an AIS-SART should operate in pre-defined VDL time slots.  It is also recommended that the AIS-SART be restricted to AIS1 and AIS2 frequencies.  This will reduce power consumption, “on” time, and is more commercially achievable (reduce manufacturer costs and extends battery operating time).

4.2	AIS-SART stations in operation must transmit a position report with position and identification of the station.  An additional safety related broadcast message with a pre-defined text of 16 ASCII characters maximum can be transmitted (message content to be defined by IMO e.g. ‘Mayday’). To guarantee that these type of messages will be displayed on the MKD of all Class A stations and that the messages are available for external connected equipment like Radar and ECDIS, existing position reports and text messages as described in ITU-R M.1371-1 must be used.

4.3	AIS-SART stations must have a GNSS receiver for synchronisation and position determination.

4.4	AIS-SART stations must have a unique identification number.

4.5	AIS-SART stations should not be subject to Channel Management.

4.6	Once the performance standards are accepted by IMO, liaison between IMO, ITU, IEC and IALA is needed to incorporate the specifications of AIS-SART stations in the appropriate documentation.

5	The preliminary draft performance standards are a sound basis for developing the technical specifications for an AIS-SART station. IALA suggests the following changes:

5.1	All different types of AIS applications are called ‘AIS stations’ (ITU language). Therefore it is suggested to speak about ‘AIS-SART stations’ (suggested changes all over the document).

5.2	The MKD may not be used once AIS is connected to Radar and/or ECDIS. The following change in the introduction of section 2 is suggested:

The AIS-SART station should be capable of indicating the location of a unit in distress on the assisting AIS stations by means of visual indication on the Minimum Keyboard and Display (MKD) orand other navigational equipment capable of processing and displaying AIS information such as ECDIS, Radar or an integrated navigation system (INS) available on board the ship.

5.3	In the IMO Performance Standard section 2.1.15 and 16, ‘be capable’ can be understood that another value can be applied. Therefore it is suggested to change section 2.1.15 and 2.1.16 as follows:

.15	be capable of transmitting transmit its position during operation with an repetition rate of at least 1 minute;

.16	be capable of updating update its position during operation with an update rate of maximum 5 minutes.

5.4	The range of detection is also dependent on the sea state, in particular for transmitters just above water level. Therefore it is suggested to change section 2.5 as follows:

2.5	The AIS-SARTs An AIS-SART station in operation should be detectable at a distance of at least 5 nautical miles by another AIS station with an antenna height of 15 meters and a sea state of [to be determined], complying with resolution MSC.74(69), with an antenna height of 15 meters. AIS-SARTs  stations should also be detectable by airborne AIS stations at a distance of up to 50 nautical miles, at a height of 3,000 ft.

6	IALA is of the opinion that the current draft performance standards with the suggested modifications are adequate and is looking forward to IMO for the completion of the specifications.