REQUEST TO COUNCIL

FROM:	e-nav committee

regarding:	LIAISON STATEMENT TO ITU

The e-NAV Committee has reviewed a proposal to ITU that calls for the reorganization of ITU Radio Regulations (RR) Appendix 18. RR Appendix 18 is essential to the GMDSS, VTS radiocommunications, the AIS and other necessary maritime services.

The proposal, ITU-R Document 8B/TEMP/212(Rev. 1), 11 September 2006, “Characteristics of VHF Radio System and Equipment for the Exchange of Data and E-Mail on Maritime Appendix 18 Channels,” poses a threat to the essential services supported by RR Appendix 18, including the existing VTS radiocommunications and the AIS. This proposal has been advanced to the status of a preliminary draft new Recommendation by ITU-R Working Party 8B (WP8B) and is currently being considered under World Radio Conference (WRC) 2007, Agenda Item 1.14. This matter urgently needs to be addressed in the next meeting of ITU-R WP8B, 13-21 June 2007 in Geneva.

Alternative technologies are available that could more efficiently provide the prospective new “VHF Radio System and Equipment for the Exchange of Data and E-Mail on Maritime Appendix 18 Channels” without harmful interference to the VTS radiocommunications and the AIS and disruption to Appendix 18 and the services it supports, including SOLAS.

In its 37th Session, December 6th – 7th, 2005, the IALA Council approved a report on the future of maritime radiocommunications. This report included guidance to “Develop Guidance on the Future of Maritime Radio Communications” and to “review the driving factors, new technologies and the maritime applications likely to be affected. It was proposed that the study should continue during the next Committee cycle.” This report is included as Annex 2, and the expert radiocommunications technical analysis is included as Annex 1.

ACTION

The e-NAV Committee asks that Council forwards the attached liaison statement to ITU-R WP8B on this subject. The Liaison Statement, with the informative Technical Analysis (Annex 1) and the IALA Report on Future Radiocommunications (Annex 2) are attached.

DRAFT

LIAISON STATEMENT TO ITU

VHF Radio System and Equipment for the exchange of data and e-mail on maritime appendix 18 channels

under WRC-07 agenda item 1.14

IALA requests that ITU-R WP8B consider the potential harmful effects on the VTS radiocommunications and the AIS from the proposed draft new recommendation (Document 8B/TEMP/212(Rev. 1), 11 September 2006) “Characteristics of VHF Radio System and Equipment for the Exchange of Data and E-Mail on Maritime Appendix 18 Channels” and the preliminary draft report to which it refers (Document 8B/524, August 28, 2006) which includes the technical characteristics of the radiocommunications system.

This system has potential harmful effects on ITU RR Appendix 18 and the other important services it supports, including the AIS which is now a global mandate for SOLAS. Administrations are obligated to support the AIS, and many Administrations are in the process of implementing AIS shore infrastructure that could prospectively be negatively impacted if the proposed wideband VHF data system is implemented with its shore infrastructure. This liaison statement also proposes that careful consideration be given to technologies that can more efficiently provide this much-needed VHF Data Service that do not require the reorganization of Appendix 18 and that do not pose an interference threat.

Annex 1 provides an electromagnetic compatibility (EMC) analysis of the harmful effects of the proposed system compared to another available proven technology that provides a more efficient data service and does not pose incompatibility with Appendix 18 channels.

In view of this analysis, WP 8B should consider what technologies may be available that could provide the prospective new “VHF Data Service” without disruption to Appendix 18 and the critical services it now provides, including SOLAS.

ITU is also requested to note that the requirements within the report in Annex 2 calls for the worldwide protection of the frequencies AIS 1 and AIS 2. Therefore, in view of the technical analysis provided in Annex 1, it is further recommended that ITU develop interference protection criteria for AIS.

Annex 1

Technical Analysis

Electromagnetic Compatibility (EMC) Analysis of Proposed VHF Digital Radiocommunications Systems

The proposal from Norway to reorganize ITU RR Appendix 18 is in two parts. The first part (Document 8B/TEMP/212(Rev. 1), 11 September 2006) is a proposal for the reorganization of Appendix 18 to support a new VHF Data System, and the second part (Document 8B/524, August 28, 2006) consists of a proposed new Recommendation for a VHF Data Network. The second part contains the technical details of the digital radiocommunications system and the characteristics of the system.

ITU RR Appendix 18 proposed Reorganization:

Note that this proposal calls for the consolidation of 9 duplex (shore-ship) channels into one wideband duplex (shore-ship) channel. Note also that the interleaved adjacent channels to the AIS are proposed to be designated for voice VHF Public Correspondence (VPC) service.

Transmitter Characteristics (based on ETSI standard ETS300113):

NOTE – The following paragraphs contain portions of text from the ETS300113 standard.

Types of radios

The system operates in full duplex mode where bursts of data are transmitted over the RF channel. All data radios used are required to be capable of full-duplex operation.

Though base and mobile data radios are used in the system, the RF performance of both types shall meet the same requirements. For the purpose of this document only one wideband data radio type is assumed. Whenever a referred standard defines different parameter limits for different radio types, the limit value pertinent to base stations shall be applied.

Channel spacing

The wideband system is assumed to operate in a pair of 225 kHz channels created by joining 9 standard 25 kHz channel. The duplex spacing is assumed to be 4.6 MHz.

Emission designator

200KF1DAN

Modulation symbol rate

The modulation symbol rate shall be 133 000 symbols per second, which corresponds to raw data bit-rate of 133 kbps.

Test conditions, power sources and ambient temperatures

For definition of normal and extreme test conditions and for requirements on the test power sources see ETS300113-1v.1.5.1 clause 6.

Transmitter frequency error

The frequency error of the transmitter is the difference between the measured carrier frequency in the absence of modulation (or with modulation, provided that the presence of modulation allows sufficiently accurate measurement of the carrier frequency) and the nominal frequency of the transmitter.

The transmitter frequency error shall not exceed 3 kHz under any combination of allowable operating conditions. For method of measurement see ETS300113-1 v 1.5.1., clause 8.1.2.

Transmitter carrier power

The transmitter carrier power is the mean power delivered to the artificial antenna during a radio frequency cycle.

The rated output power is the carrier power (conducted) of the equipment declared by the manufacturer.

The transmitter carrier power shall stay within +2.0 dB and –3.0 dB from the rated power under any combination of allowable operating conditions. For method of measurement see ETS300113-1 v 1.5.1., clause 8.2.2.1.

Transmitter spectral mask

IALA Liaison Note: This transmitter spectral emissions mask is not compatible with the 25 KHz channel structure of ITU RR Appendix 18 in that it does not meet the adjacent channel power limit of -70 dBc/± 25 KHz specified by IEC until ± 250 KHz.

AIS 1

161.975 MHz

AIS 1

161.975 MHz

@-33dBc

AIS 2

162.025 MHz

-70 dBc IEC Limit for Appendix 18 Adjacent Channel Power

CH 27B

161.950 MHz

CH 28B

162.000 MHz

Proposed 225 KHz channel

Center Frequency of proposed new 225 KHz channel @ 161.825 MHz

-70 dBc @ ± 250 KHz

The transmitted spectrum shall stay within these limits under any combination of allowable operating conditions. The spectrum is measured by a spectrum analyser using 10 kHz read bandwidth, the zero reference level equals to the unmodulated carrier power.

Example of an alternative method for VHF data transmission (based on ETSI Standard TETRA-TEDS):

ETSI has implemented a standard for radio data transmission for the Trans-European Trunked Radio Association (TETRA) in the land mobile radio services on 25 KHz channels. The standard includes two modulation types for use on 25 KHz channels, π/4 DQPSK at 36 KBPS and π/8 D8PSK at 54 KBPS. The higher data rate is the newer one, and it is used for TEDS (TETRA Enhanced Data Service). This is proven technology, and both base station and mobile equipment is currently in widespread use. RTCM (the Radio Technical Commission for Maritime services) has formed a Special Committee (RTCM SC 123) to assess this and other technologies for use in the VHF marine band (ITU RR Appendix 18) for a prospective new VHF Small Messaging Service. The following figures illustrate RTCM SC 123 test results of the TETRA-TEDS transmission methods.

Figure 1

RTCM SC123 Test Results for TETRA-TEDS Modulation

Figure 2

RTCM Test Results for Slightly Reduced Data Rates to fit IEC Emissions Mask

Comparison of the two ETSI standards:

Comparison of the impact on ITU RR Appendix 18:

The Telenor proposal requires the restructuring of ITU RR Appendix 18 to consolidate 9 contiguous duplex channels of 25 KHz each, specifically the channels 82, 23, 83, 24, 84, 25, 85, 26 and 86. Furthermore, it requires the designation of channels 27 and 28 for voice service.

Alternatively, the approach proposed by RTCM SC 123 is to keep the 25 KHz channel structure of ITU RR Appendix 18 by using the IEC emissions mask to allow the channels to be used separately (independently) for any purpose by Administrations.

Comparison of spectrum efficiency (bits/second per Hz of bandwidth):

The ETSI standard ETS300113-1v.1.5.1 proposed by Telenor achieves 133 KBPS in a 225 KHz channel (a combination of 9 contiguous duplex channels of 25 KHz each, specifically the ITU RR Appendix 18 channels 82, 23, 83, 24, 84, 25, 85, 26 and 86).

Alternatively, the test results shown in Figure 2 above where the ETSI standard for TETRA-TEDS is slightly reduced to fit for Appendix 18 achieves 43.2 KBPS in a single 25 KHz channel without consolidating multiple channels. The performance advantage of the TETRA-TEDS method is 43.2/25 = 1.73 vs. 133/225 = 0.59 or 1.73/0.59 = 2.93, a factor of almost 3:1.

Comparison of EMC (electromagnetic compatibility):

The ETSI standard ETS300113-1v.1.5.1 proposed by Telenor violates the IEC adjacent channel power limit of – 70dBc for the adjacent ± 5 ITU RR Appendix 18 channels, which compromises the performance and usefulness of these channels, including the AIS channels AIS 1 and AIS 2. The problem for the AIS is of global impact due to its use for maritime safety and security.

Alternatively, the test results shown in Figure 2 above indicate that the IEC adjacent channel power limit of -70 dBc is met with a data rate as high as 43.2 KBPS on a 25 KHz channel. This, along with the IEC requirement for 70 dB adjacent channel rejection ratio for receivers operating in Appendix 18, insures EMC between the channels. Thus, the ETSI TETRA-TEDS method is demonstrated to be without compromise to the EMC of Appendix 18 for all services, voice, data and AIS.

Comparison of service effectiveness:

Shore-based services are regulated by Administrations, and international standards are preferred because ships are better equipped to use these services when there is global acceptance of equipment, technical standards and the use of radio channels. Administrations vary in their use of ITU RR Appendix 18 channels, e.g. the United States has assigned some of these channels to the land mobile radio services. Any proposed new standard for VHF data service must consider that using the channels one at a time is preferred because it allows flexibility amongst the various Administrations. There is also a practical advantage to keeping the channels separated in that multiple users can access the services simultaneously on different channels rather than having to be multiplexed on one consolidated channel.

Comparison of the EMC between the AIS service and the VPC service:

Traditionally, assignment of channels in the land mobile radio services considers that geographical separation is needed between uses of the same channel (channel re-use) and also the adjacent channel. This is because base stations are situated on high elevations to achieve coverage of the service area. For shore-based VHF marine radio services, specifically the VHF Public Correspondence (VPC) service, this tradition has been followed. There has always been avoidance of the use of adjacent VPC channels in the same service area because of the potential for interference between the base stations.

The Telenor proposal relegates VPC voice services to channels 27 and 28 which are interleaved and adjacent to AIS 1 and AIS 2. This is problematical because the VPC voice service raises a constant CW (continuous wave) carrier on these shore-based frequencies that present strong interference potential for the AIS when VPC base stations are situated in close proximity to AIS base stations.

An illustrative sample calculation follows:

An EMC analysis (in free space) based on the AIS receiver sensitivity of -107dBm, VPC transmitter with an ACPR = 70dB (in accordance with IEC 62320-1 Ed.1, the AIS base station standard) and an assumed AIS base station antenna (and a VPC base station antenna) selection of the DB 222E (a popular typical marine VHF base station antenna) is as follows:

20 log D = Pt – Ps – 36.6 – 20 log F - ACPR + 10 log N + NA + CCRR + Gr – L + Gt, where

D = Distance separation between AIS and SeaSmart stations in land (statute) miles;

Pt = VPC transmitter carrier power (50 Watts at antenna base) = +47dBm;

Ps = AIS minimum receiver sensitivity specification level (for a 20% PER) = -107dBm;

20 log F = 44.18dB, where F = Frequency in MHz = 161.975 MHz (AIS1);

ACPR = adjacent channel power ratio of the VPC transmitter = 70dB;

10 log N = 10 log 2 (N = number of VPC adjacent channels = 2 for AIS1) = 3dB;

NA = noise level allowance = 3dB;

CCRR = AIS receiver co-channel rejection ratio = 10dB;

Gr = gain of the AIS base station antenna in the VPC direction (at broadside) = 6.5dBi;

L = total loss of AIS feed-line, connectors, Polyphaser and triple-cavity filter = 4dB; and

Gt = gain of the VPC antenna in the AIS direction (at broadside) = 6.5 dBi.

Thus, for Ps = -107 dBm,

20 log D = 47 – (-107) – 36.6 – 44.18 – 70 + 3 + 3 + 10 + 6.5 – 4 + 6.5 = 28.22dB

D = 10(28.22/20) = 25.8 miles

This example shows that installation of two base stations on shore with adjacent 25 KHz ITU RR Appendix 18 channels must be with a geographical separation of greater than 25.8 miles. If the Telenor proposal is implemented, AIS shore stations must be separated from the proposed voice VPC base stations by a greater distance than this, especially if the AIS receiver has better sensitivity than the IEC standard minimum value. Typically, AIS base stations are capable of receiver sensitivity of -115 dBm or better. If this receiver sensitivity value is used and needed to be protected, the required distance separation is raised as follows (Note that this may actually be beyond line-of-site, depending on antenna height above sea level, but in principle the distance is still very large.):

Then, for Ps = -115 dBm,

20 log D = 47 – (-115) – 36.6 – 44.18 – 70 + 3 + 3 + 10 + 6.5 – 4 + 6.5 = 36.22dB

D = 10(36.22/20) = 64.7 miles

Alternatively, if the VPC voice service were implemented on the lower VPC channels far below AIS 1 and cavity filters were used to reduce the transmitter noise floor, this problem could be greatly reduced. The advantage of the proposal to use the ETSI TETRA-TEDS data transmission method in this case is that it allows the channels to be used separately and thus the VPC voice service could be implemented at the lowest VPC channel(s) available within the Administration and the VPC data services to be implemented on a step-by-step basis incrementing upward as the channels are occupied. This would allow the interleaved adjacent channels to AIS to be used by VPC data services only when the lower channels are all occupied, which would be less frequent. Furthermore, the VPC data service would not raise a constant CW carrier but only have a RF carrier raised for the duration of the data transmission. This would be less of an imposition on the AIS.

Conclusion:

The system proposed by Telenor has potential harmful effects on ITU RR Appendix 18 and the other important services it supports, including VTS radiocommunications and the AIS which is now a global mandate for SOLAS. Administrations are obligated to support the AIS, and many Administrations are in the process of implementing AIS shore infrastructure that could prospectively be negatively impacted if the proposed wideband VHF data system is implemented with its shore infrastructure.

This Annex provides an electromagnetic compatibility (EMC) analysis of the harmful effects of the proposed system compared to another available proven technology that provides a more efficient data service and does not pose incompatibility with Appendix 18 channels.

Annex 2

IALA Report on Future Maritime Radio Communications

“In 2004 the IALA Council approved a new work item for the Radionavigation Committee, “Develop Guidance on the Future of Maritime Radio Communications”. This initial report reviews the driving factors, new technologies and the maritime applications likely to be affected. It is proposed that the study should continue during the next Committee cycle.”

In its 37th Session, December 6th – 7th, 2005, the IALA Council approved the following report.

Report to The IALA Council on the Future of Maritime Radio Communications

Introduction

In 2004 the IALA Council approved a new work item for the Radionavigation Committee, “Develop Guidance on the Future of Maritime Radio Communications”. This initial report reviews the driving factors, new technologies and the maritime applications likely to be affected. It is proposed that the study should continue during the next Committee cycle.

Drivers

- Spectrum: Increasing pressure on the spectrum for commercial mobile telecom applications. In particular radar designers are being pushed towards technology that will use less bandwidth and reduce spurious emissions. This will eventually free up spectrum and make possible alternative uses of adjoining bands. This has major consequences for racon design.

- Two-way Data Communication: Increasing need for a common communication platform for two-way data communication between ship and shore.

- Internet Access: There is a growing requirement for Internet access on ships, at sea as well as in port.

- e-Navigation: great interest is being shown in the potential for integrating sources of navigation information to provide the user with the optimum, relevant data on a single display, e.g. the IALA e-ANSI project – communicating AtoN status information to the navigator.

- Commercial pressures: (e.g. 3G providers) commercial providers are increasingly anxious to find markets for their services. This may provide opportunities for new ways of communicating and presenting navigation information to the user.

- Non-safety related communication: pressures on existing systems, e.g. messaging on AIS

- MF/HF data exchange: ITU is developing technical requirements for MF/HF data exchange services as a possible alternative to NBDP (radio telex) and has also initiated studies on VHF digital technology.

Candidate Technologies

- IP Networks: New services are envisaged for the broadcast over IP networks of Maritime Safety Information (MSI), using HF and satellite services such as Inmarsat BGAN (Broadband Global Area Network). Other IP network systems might also be applicable.

- UWB (Ultra Wide Band): Spread spectrum applications include short range radars and wireless networks. This technology presents opportunities, but may also pose a threat to existing services, since it raises the noise level.

- Digital radio technology (including CDMA and TDMA): applicable to many uses in maritime data communication, where spectrum is scarce and error-free communication is required. At the same time discussion are taking place regarding the possible sub-division of maritime mobile communication bands, especially VHF.

Administrative measures

A major factor in the growing problems for maritime radio communication is the lack of radio spectrum. There are several regulatory measures that could be applied, e.g. subdivision of present channels as well as applying channel sharing between services. The introduction of digital technologies may also help to overcome these problems. Because of the substantial user base in some applications, these measures will require a long lead-time as well as a long implementation time. This highlights the need to inform ITU of future spectrum requirements for maritime services.

Maritime Applications

- Communication:

  - AIS:

    - ITU has assigned two VHF frequencies, 161.975 MHz and 162.025 MHz worldwide for AIS purposes on a primary, non-exclusive basis. Interference problems in some parts of the world have indicated that more protection may be required.

    - There is increasing pressure to utilise AIS for non-safety related messages thus potentially overloading the AIS channels. This type of communication is important but should be taken care of by other communication methods than AIS. Agreement is needed on the technology to use.

  - Long Range Identification and Tracking (LRIT)

  - SAR:

    - Search and Rescue applications need priority. The current attitude among  regulators is to allow market demand to dictate spectrum allocation. If allowed to go unchallenged, this approach could lead to serious degradation of SAR services and consequent risk to life.

  - VTS:

    - VTS-stations handle a large amount of voice traffic on several different VHF-channels. The problem is the severe lack of free VHF-channels, increasing VHF-traffic and new VHF service requirements. The situation is particularly bad in areas of overlapping coverage from different VTS-areas, even from different countries. The introduction of AIS may reduce the need for VTS VHF voice traffic.

  - NAVTEX:

    - Discussions are taking place on the future of NAVTEX. The development or eventual replacement of NAVTEX will have an effect on maritime services and available spectrum.

  - GMDSS:

    - The long gestation period of GMDSS saw many problems with training and false alarms. These appear to have been addressed by maritime administrations, but there is likely to be continuing pressure for improvement.

  - SafetyNet:

    - This Inmarsat MSI service would be one possible replacement for NAVTEX.

  - Maritime Voice Communication on VHF:

    - The main development here is likely to be reduced channel spacing and the move towards digital technology

  - Maritime Data Communication

- Radionavigation

  - GNSS:

    - Additional signals on GPS – L2C, L5 and eventually GPS 3 will increase the applications and improve accuracy and integrity

    - It is planned to restore GLONASS to a full 18 satellite constellation by 2008

    - Galileo is scheduled to become operational by 2008 and will provide several levels of service on multiple signals

  - DGNSS

    - DGNSS can be accomplished using several different methods: IALA DGNSS, AIS, Eurofix, e-Loran, SBAS

    - The frequency band 283.5-325 kHz is allocated for Radio-navigation, but regulatory changes may be needed to ensure its continued availability for the  IALA DGNSS

    - AIS may be used as a communication channel for a DGNSS service

    - Loran transmissions may be used as a communication channel for a DGNSS service, i.e. Eurofix or 9th pulse modulation

    - Additional means of radio communication may be required for fast update rate DGNSS applications

- Radar

  - VTS

    - Shore based radars are likely to be affected by changes in technology required to meet more stringent limits on spurious and out of band emissions

  - Radar AtoN

    - The racon continues to be useful on both the X-band (3 cm) and S-band (10 cm).

    - However, IMO MSC 79 has issued new radar performance requirements that no longer require S-band (10 cm) radars to trigger or detect racons. New ITU rules are forcing changes to S-band (10 cm) radars and existing racons will not respond to new S-band (10 cm) radars.

    - It is too soon to know which direction future radar development is going to take and therefore it is too soon to decide which direction future racon development should take. CIRM is forming a special interest working group to study this problem.

Recommendations

The Council is invited to note this report and approve the inclusion of this item in the future work program of the Committee. This would involve continued studies and periodic reporting on the future developments in maritime radio communications and preparation of standards as appropriate.

References

- RNAV22/6/1, Draft Submission to WRC07 –AIS matters/J. Hersey

- RNAV22/4/3, IMO Report – COMSAR9 / J.C. LeClair

- RNAV23/9/1, Future of Radar Aids to Navigation