## 3.5	Long Range Identification and Tracking

Aids to Navigation, Port Security, and other shoreside authorities are faced with the responsibility of maintaining surveillance of maritime approaches to their ports and port facilities for safety, security, and environmental protection.

These authorities are pursuing vessel tracking technologies to assist in the detection, classification, identification, and tracking of vessels. Among these technologies, Long Range Identification and Tracking (LRIT)  is being implemented internationally for tracking ships globally.

Long range identification and tracking (LRIT) is a cooperative surveillance capability. In the basic LRIT concept (Figure 1, courtesy of Inmarsat), a ship carries radio communications equipment that reports identification, position and time to authorities tracking that ship.

Figure 1. Early LRIT Concept

The International Maritime Organization (IMO) Maritime Safety Committee (MSC), at its 81st session in May 2006 (MSC 81), adopted amendments to the Safety of Life at Sea Convention (SOLAS) for the long-range identification and tracking (LRIT) of ships. (IMO MSC.202(81)). MSC 81 also approved performance standards and functional requirements for LRIT and established an ad hoc working group on the engineering aspects of long-range identification and tracking.

The SOLAS regulation on LRIT

The new regulation 19-1 of SOLAS Chapter V (Safety of Navigation) enters into force on 1 January 2008, with most ships required to transmit LRIT information by 31 December 2008. Industry representatives and others had voiced concern about the potential need to install, upgrade, or re-fit shipboard equipment by 1 January 2008; hence the delay in the start of operations.

The regulation requires cargo ships of 300 gross tons and above, passenger ships, and mobile offshore drilling units on international voyages to be fitted with a system to automatically transmit LRIT information. Ships that operate exclusively within sea area A1 (essentially within VHF range of shore) and fitted with an automatic identification system (AIS) are not required to comply with the regulation.

Contracting governments, subject to certain restrictions, can receive LRIT information transmitted by ships as follows:

- Flag states: All flag ships worldwide.

- Port states: All ships indicating an intention to enter a port facility, at a distance or time set by the port state, but not in internal waters of another contracting government.

- Coastal states: All ships, regardless of flag, within a distance of 1000 nautical miles of the coast, but not in internal waters of another contracting government, nor in the territorial sea of the contracting government whose flag the ship is entitled to fly.

Administrations (the government of the state whose flag the ship is entitled to fly) may deny coastal states access to LRIT information at any time. Despite the regulation’s broad reach to 1000 nautical miles for coastal states, it is important to note the first provision of the regulation:

Nothing in this regulation or the provisions performance standards and functional requirements adopted by the Organization in relation to the long-range identification and tracking of ships shall prejudice the rights, jurisdiction or obligations of States under international law, in particular, the legal regimes of the high seas, the exclusive economic zone, the contiguous zone, the territorial seas or the straits used for international navigation and archipelagic sea lanes.

Contracting governments must bear all costs for long-range identification and tracking information that they request and receive. A master of a ship may, for the protection of navigational information or when he considers LRIT operation may compromise the safety or security of his ship, switch off the LRIT shipboard equipment. Search and rescue (SAR) services of a contracting government may receive long-range identification and tracking information free of charge for SAR purposes.

LRIT Performance Standards and Functional Requirements

The long-range identification and tracking performance standards and functional requirements were also approved at MSC 81. These lay out the LRIT system architecture (Figure 2) and describe how the long-range identification and tracking system will work.

Figure 2. LRIT System Architecture

# In this architecture, the administration determines whether its ships will report to a national, regional/cooperative, or the international LRIT data center. Each of these types of centers may use multiple communications service providers. The architecture is also designed to accommodate multiple application service providers. There are a number of existing ship reporting and vessel monitoring systems that may be able to function as national LRIT data centers within the LRIT system architecture. Examples of these include Amver (Amver is a name that formerly stood for Automated Mutual-Assistance VEssel Rescue. [It is a former acronym/abbreviation that is now a term of art, like radar or sonar.); Victoria (Russian Federation); GICOMS (Republic of Korea); and AUSREP (Australia).

Engineering Aspects of LRIT

MSC 81 established an ad hoc working group on the engineering aspects of LRIT and directed it to develop technical details needed for successful implementation of LRIT. Technical specifications for the international LRIT data center and data exchange, as well as for communications within the LRIT system network were developed. These include communications between LRIT data centers and the data exchange, in accordance with the LRIT data distribution plan.  IMO also developed protocols for development testing of the LRIT system and for testing the integration of new LRIT data centers into the system.