- Rapporteurs Report

- Development in non-radio navigation systems

- (Task M2)

- 1. Background

This report, prepared for the ENAV Committee, provides information about ongoing developments in the field of non-radio navigation systems. Furthermore developments of INS (Integrated Navigations Systems) which combines radio navigation and non-radio navigation systems will be reported. Most of the information is based on published material, conference proceedings and contacts with manufacturers in this area.

- 2. Architectures of GNSS/INS Integrations

Today integrated GNSS/INS systems are used for many applications. Based on the complementary error behaviour of both sensors, higher performance levels are possible. The improvement in accuracy, availability, integrity and continuity depends also on the architecture of integration. In general the possible methods can be separated into three different integration categories, “loose-”, “tight-“ and “deep”

According to its measurement principle INS systems provide an autonomous solution for position, velocity and attitude with high data rate and bandwidth. The sensors used in an INS are a triad of gyros for measuring rotation and accelerometers for measuring accelerations. Its typical error behaviour, however, causes only short term stability with respect to accuracy. A GNSS receiver, on the other hand, generates position and velocity solutions which are characterized by long time error stability and a high accuracy level. These are the main reasons for the successful use of integrated GNSS/INS systems in many applications, providing a higher performance level in comparison to the stand-alone solutions. The following sections shall provide a short overview about the typical methods of GNSS/INS integration.

- 2.1 Loose GNSS/INS Integration

In the loose architecture, the INS and GNSSS receiver operate as independent navigation systems (Figure 1). In this case the sensors are completely independent of each other. The INS calculate an own solution for position, velocity and attitude. Also the GNSS sensor provides an independent navigation result containing position, velocity and time.  Further the information from both sensors will be combined in a Kalman filter process to provide a third navigation solution.

- Benefit from Loose GNSS/INS integration

- Higher accuracy of the navigation system, caused by the different error behaviour of both sensors.

- The INS can act as an active low pass filter for GNSS noise effects

- Availability can be improved during short GNSS outages

- Mutual control of navigation results is possible in order to increase the integrity of the combined sensor system.

(PVT)

(PVA)

(PVTA)

- Figure 1:  Loose GNSS/INS Integration

- 2.2 Tight GNSS/INS Integration

In this architecture, the INS and GNSS are reduced to their basic sensor functions (Figure 2). They provide pseudoranges, pseudorange rate, accelerations and gyro measurements. All measurements will be used to provide a combined navigation solution. The most important goal of the tight GNSS/INS integration is the improvement of GNSS availability, especially in high dynamic environment or in observation conditions with low GNSS signal-to-noise ratio.

- Figure 2:  Tight GNSS/INS Integration

- Advantages

- In general the tight architecture provides a more accurate solution than the loose integration method

- Better availability in high dynamic applications, in low SNR environments and in situations where fewer than 4 satellites are visible

- Disadvantages

- No redundant navigation solutions as provided from loose integration method

- It is necessary to modify the signal processing of the GNSS receiver

- 2.3 Deep GNSS/INS Integration

The deep coupling principle represents the highest level of GNSS/INS integration (Figure 3). The architecture of the GNSS receiver is fundamentally different from typical receivers. Furthermore the INS is used as an integral part of the GNSS receiver. Thus the deep method can not longer be viewed as a navigator independent from INS.

- Figure 3:  Tight GNSS/INS Integration

- Advantages

- The deep integration enhances the robustness of GNSS to interference and jamming

- Optimal fusion of the measurements from GNSS and INS

- Disadvantages

- No independent position solutions available

- 3. Conclusion

As shown before the integration of GNSS and INS sensors can be implemented using three different general principles. The gain in performance of the integrated system depends not only on the sensors types but also on the way of coupling. If accuracy or integrity requirements are emphasized, the loose coupling is the preferred method. If the applications are focussed on a high availability, especially in conditions of high dynamics or low signal-to noise ratios, the tight or deep architecture should be taken into account. In this case only short time stability must be guaranteed by the inertial sensor. So primarily low-cost units based on MEMS technology can provide the inertial data.

- M. Hoppe, March 2007

[1] 	Kreye, C., Eisfeller, B., Ameres, G.,”Architectures of GNSS/INS Integrations”, 
 	Institute of Geodesy and Navigation University FAF Munich, 2004

[2]	Dr, Gebre-Egziabher, “What is the difference between loose, tight, ultra-tight and 
 	deep integration strategies for INS and GNSS”, Inside GNSS, February 2007