# Draft Vi.1

# IALA eAtoN Services

BASIC AtoN SERVICES

# BASIC AtoN Services

## Introduction

This paper presents a method for defining and describing electronic aids to navigation (eAtoN). The purpose of this document is to provide first level discussion of eAtoN Service and the Basic AtoN (BAtoN) Services and the relationship to electronic Navigation (eNav). It will present a relational representation in the form of a relational diagram as they may be represented logically.

E-Navigation can be defined as:

“e-Nav is the digital (electronic) interaction of maritime services, ship to ship, ship to shore, shore to ship and shore to shore”

All Aids to navigation can be represented digitally; this allows a total evaluation and manipulation of Aids to Navigation with respect to eNavigation. IALA has the responsibility of Aids to Navigation, including VTS hence IALA has a significant role to play in the development of eNavigation.

## The relational representation

In the example shown in Figure 1 we have a typical organization chart with the Competent Authority (shore based Aids to Navigation provider) at the head of the organization. It has policies, regulations, resources and provides service (public service in most cases). To provide this service it needs other resources that work within its management framework (policy, regulations etc.). The examples used in this document are Office of Boating Safety (OBS) Service, VTS Service, AtoN Service – it could also have included all services Navigable Waters Protection Agency (NWPA), Fleet, SAR etc. Each of these services use other services to deliver their service i.e. VTS uses Communications (voice) Service to get or distribute their information. Each of these services is a Basic Aid to Navigation Service (BAtoN service). Radar provides the BAtoN service of Radar position, Radar range, point of coincidence. AIS is also a service – it provides AIS position, AIS range, AIS speed over ground etc (in AIS they are have been named Basic AIS Services or BAS). This illustrates the relationship that exists between all of the BAtoN services to the Service providers Competent Authority).

Each service on every level can be described – CCG Service, MCTS Service, etc. There are many methods to describe them – verbal, pictorial, text, combinations of multi-media.

Each service or component will have a relationship to the other Components – the AIS Service “flows to or from” the VTS Service and a BAS “flows to or from” the AIS Service. Each Component then has only to list the interface criteria for each relationship, one from the point of view of an input the other as an output. These criteria need to match.

It therefore follows that if we build a system of components with their interface we describe a total Service eventually.

Figure 1

Figure 2 has taken the organization view of Figure 1 and transformed it into a more “logical” presentation with respect to the relations. The lines in Figure 1 that usually mean “flows to” are replaced by blocks that denote “belongs to”. The system of relationships has to be duplicated for each instance of the service (either other Competent Authorities or delegated authority). It should also be noted that there are often multiple interfaces to a component (manager of it, other services, conditional influence).

Figure 2

## Subdivision of the BAtoN Services

The BAtoN services can be subdivided into "external" and "internal". The "external" BAtoN services are those services that deliver net information from and to ships and shipboard applications, the direction of the data flow depending on the nature of the BAtoN service.

The "internal" BAtoN services are those which are needed in particular to perform the "system management" functionality. They do not deliver net information about ships and shipboard applications. Some "internal" BAtoN services may be needed to provide "external" BAtoN services under certain circumstances. The "internal" BAtoN services are contained and managed within the AtoN Service and may be triggered and / or configured by higher levels of shore-based infrastructure.

## Relationship between eAtoN Services and eNav Services

The following is a working description of e-Navigation:

“e-Nav is the digital (electronic) interaction of maritime services, ship to ship, ship to shore, shore to ship and shore to shore”

From a ships point of view for information flowing from shore to ship and in many cases from ship to ship we can recognize the relationship between information gathered by ships sensors (AIS, voice communications, Radar etc) and eNav. eNav is the collection of such information and its communication to the human controlling a vessel.

Every piece of information presented to the person controlling a vessel shall have an interface to a basic service. The inverse is also true for the shore, every piece of information presented to VTS shall have an interface to a basic service.

The task of IALA with IMO is to define these internationally accepted interfaces.

Figure 3 illustrates the relationship between the basic services as collected by ships sensors and the data presented to the person controlling a vessel. It should be noted that the Basic services from the shore bear a direct relationship to those of the vessel. In AIS the BAS in the shore station are directly related to the BAS on the vessel’s AIS. The interface in this case is the AIS VHF Data Link (VDL).

Figure 3

It should be noted that Figure 2 and Figure 3 are inverted with respect to each other. They interface at the BAtoN services and it is this interface that needs to be defined by both the shore authorities (IALA) and the ship authorities (IMO).

## Description of the BAtoN Services

It is proposed that each of the Basic Aids to Navigation be described in detail.

The description of the BAtoN Services can serve the following purposes.

### The BAtoN Services as the interface between operational and technical considerations

"External" BAtoN produce, transfer, and accept information to and from higher levels that are not themselves part of the AtoN Service.  The functionality of higher level services and AIS applications are built upon the capabilities of these services. This is illustrated by:

The "external" BAtoN services are those the AtoN Service delivers to the recipient's side ashore at the interface between the AtoN Service and "higher", application-oriented levels. Therefore, their description also takes into account the processing at the shore-based applications.

Therefore, the BAtoN services are also the basis for an assessment of the usefulness of the AtoN Service for a particular intended application in terms of accuracy, frequency, reliability etc.

### The BAtoN Services as a means to convey consistent relevant information from the recipient's point of view

When making use of the potential of the AtoNs by setting up an AtoN shore infrastructure, which is integrated into e. g. a VTS environment, the VTS designer, who formerly had not been involved in shipborne equipment issues at all, will now need to receive and use information on the eNav shipborne installation.

This is due to the fact, that the introduction of the eNav as an automated, co-operative system triggers the integration of shipborne and VTS technologies, which formerly existed more or less independently of each other. The same applies – in principle, but more on an operational level with less technical sophistication – to the VTS operator(s).

The BAtoN service descriptions provide relevant information for a given issue to an AtoN Service designer or manager.

This description of the BAtoN service does not make redundant the referenced documents, i.e. the appropriate international standards, nor do they introduce new system features. However, each description of a BAtoN service integrates, in a comprehensive and highly accurate manner, all information items from various sources into functional terms that are essential to understand what this BAtoN service is delivering.

### The BAtoN Services as the functional, technical interface between the AtoN Service and higher technical levels

Besides being the means to assess the operational usefulness and features of the AtoN Service to higher levels and applications, the BAtoN service – in their precise technical description – constitute the functional, technical interface between the AtoN Service and higher technical levels.

### The BAtoN Services as a means to encapsulate complexity

The BAtoN services descriptions are also the basis for an assessment of the characteristics of a particular AtoN service for a particular intended application in terms of accuracy, frequency, reliability, fallback arrangements etc. An individual BAtoN service, considered as one consistent and prudently partitioned set of functionality of the whole AtoN Service may be investigated on its own, thus allowing a concentrated assessment of this particular functionality.

This holds true although there are interactions between certain BAtoN services. Since these interactions will be clearly identified, they may be considered one by one, instead of being overwhelmed by the sheer complexity of the AtoN Service as a whole.

### The BAtoN Services as a means to understand and handle complex configuration states of the AtoN Service as a whole

The BAtoN services may also serve as a convenient means to understand and handle complex configuration states of the AtoN Service as a whole because of the above features of the BAtoN, in particular because they encapsulate complexity.

A common description of services can be used to evaluate risk and aid in making Marine AtoN Moderization (MAM) decisions

## The functional interface between the shore-based applications and the AtoN Service

The requirements for the interface between shore-based applications and the AtoN Service can be defined in functional terms as follows:

- This functional interface should provide the AtoN Service to shore-based applications as a set of BAtoN Services.

- The functional interface should facilitate the integration of the AIS Service shore-based applications, such as radar-based VTS environments.

- The functional interface encapsulates the technical details of both the eAtoN technology and of the layout and local configuration of shore-based AtoN. (This is a state-of-the-art engineering principle. It protects application software from unnecessary changes due to configuration or technology improvements within the AtoN Service.)

- The BAtoN Services should have digital (NMEA format) representations. These sentence should be formatted to be compatible with AIS sentences to facilitate data fusion.

Fused data can be defined as:

The amalgamation of all source data related to a vessel or target

The interface between the AIS Service and its clients e.g. the VTS environment would also comprise a list of available BAtoN Services (selection or all) as set up in this particular VTS environment.

## Conclusion

BAtoN service descriptions and their relationship to eNav will be of benefit to manufacturers, system integrators, and users alike.  It provides a basic understanding of what services eAtoNs can deliver.  What services it cannot deliver, and what are the proper conditions under which the service should be used.  It will describe the BAtoN services that support the design of both the eAtoN and eNav based applications that directly serve the needs of the mariner, vessel traffic service, and littoral state.

## Recommendations

- IALA should define the interface criteria for shore based sensor to operator relationship between BAtoN services and the application that will interface with the BAtoN services.

  - The AIS model should be used for all interfaces to facilitate data fusion.

  - This interface should be offered to IMO to form part of a Performance standard for shipboard requirements to ensure both shore and shipborne interfaces are the same.

- IALA should prepare a description of each BAtoN service.