Proposed Clarification on Short Safety-Related Messages

In the section 3.5.1.4 of the ITU-R M.1371-3, the short safety-related message from Class-B AIS station should use pre-configured contents, however the details are not defined.  The AIS Technical Working Group of IALA e-NAV committee considered the clarification of the pre-configured contents and proposes as follows:

- Recognizing that

- there is no international agreement to handle the safety related messages as the initiation of the SAR activities;

- not all SAR organizations handle these messages as the initiation of the SAR activities;

- nearby ship may not recognize the position and ship’s name easily;

- the use of VHF DSC has the priority in the distress and urgency communications;

the following pre-configured messages can be used:

- MAYDAY

- PAN PAN PAN

- Fire, explosion

- Flooding

- Collision

- Grounding

- Sinking

- Adrift

- Abandoning ship

- Piracy attack

(Note) The above words come from Radio Regulations and the “Nature of Distress” words in the ITU-R Recommendation M.493.

- When sending the above message, the mariner must follow the procedures specified in the ITU-R Recommendation M. 541 shall be introduced to avoid false alert.

- Caution:

- The other pre-configured contents may be defined by the manufacturer.