IALA Register of Binary Messages for Regional Applications

# Introduction

The IALA Register of Binary Messages for Regional Applications is a repository for records of regional binary messages currently in use by competent authorities.

The IALA e-Navigation committee maintains a register of these messages. This register is regularly updated by the committee. One benefit is to encourage standardisation.

Registration process

To register a regional binary message, a competent authority should complete the form at Appendix A and submit it to the IALA secretariat.

The e-Nav committee will maintain the register on behalf of IALA and will update it within its regular meeting schedule.

Manufacturers are encouraged to register regional binary messages via a competent authority.

Use of existing regional binary messages

All competent authorities and manufacturers are encouraged, wherever possible, to use an existing regional binary message, already defined in this register, rather than create a new one. The following chart defines the process:

# Checklist for next meeting:

# Recommended binary messages by IALA

# What level of detail is needed vs generic definition, e.g. analog data field.

# Appendix A: Form that should be used for submission of Binary Messages for Regional Applications

Appendix A is used to propose a new binary message for the register, or the use of an existing registered binary message.

# Message 6

## DAC xxx, AI yy: - purpose

This message ...

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6; always 6 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; 0 = default; 3 = do not repeat anymore |
| Source ID | 30 | MMSI number of source station |
| Sequence Number | 2 | 0-3 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit Flag | 1 | Retransmit Flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted |
| Spare | 1 | Not used. Should be zero. |
| Regional Application Identifier | 16 | DAC = xxx, AI = yy |
| Total number of bits | zzz | Occupies ? timeslots |

### Further explanation

...

# Message 8

## DAC xxx, AI yy: - purpose

This message ...

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8; always 8 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; 0 = default; 3 = do not repeat anymore |
| Source ID | 30 | MMSI number of source station (transmitting station) |
| Spare | 2 | Not used. Should be set to zero |
| Regional Application Identifier | 16 | DAC = xxx, AI = yy |
| Total number of bits | zzz | Occupies ? timeslots |

### Further explanation

...

# Message 25

## DAC xxx, AI yy: - purpose

This message ...

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 25; always 25 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; default  0; 3  do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |
| Binary data flag | 1 | 0 = unstructured binary data (no Application Identifier bits used) 1 = binary data coded as defined by using the 16-bit Application identifier |
| Destination ID | 0 / 30 | If Destination indicator = 0 (Broadcast); no data bits are needed for the Destination ID If Destination indicator = 1; 30 bits are used for the MMSI number of the destination |
| Application identifier (if used) | 0 / 16 bits | DAC = xxx, AI = yy |
| Binary data |  |  |
| Total number of bits | zzz | Occupies 1 slot |

### Further explanation

...

# Message 26

## DAC xxx, AI yy: - purpose

This message ...

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 26; always 26 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; default  0; 3  do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Destination indicator | 1 | 0 = Broadcast (no Destination ID field used) 1 = Addressed (Destination ID uses 30 data bits for MMSI) |
| Binary data flag | 1 | 0 = unstructured binary data (no Application Identifier bits used) 1 = binary data coded as defined by using the 16-bit Application identifier |
| Destination ID | 0 / 30 | If Destination indicator = 0 (Broadcast); no data bits are needed for the Destination ID If Destination indicator = 1; 30 bits are used for the MMSI number of the destination |
| Application identifier (if used) | 0 / 16 bits | DAC = xxx, AI = yy |
| Communication state selector flag | 1 | 0 = SOTDMA communication state follows 1 = ITDMA communication state follows |
| Communication state | 19 | SOTDMA communication state (see § Error: Reference source not found, Annex 2), if communication state selector flag is set to 0, or ITDMA communication state (§ Error: Reference source not found, Annex 2), if communication state selector flag is set to 1 |
| Total number of bits | zzz | Occupies ? slots |

### Further explanation

...

Appendix B 1: Summery of registered Binary Messages for regional applications

Appendix B 1 is the summary of registered Binary Messages for regional applications.

| Registered by; | Purpose/Description | Date registered | DAC/IA | In Use by | Source of full specification of the bits. | Supplementary material source. |
| --- | --- | --- | --- | --- | --- | --- |
| EC | Inland AIS: Inland ship static and voyage related data | 2007/03 | 200/10 | EU RIS Committee | See annex B2; Section x | Tracking and Tracing standard for Inland navigation |

Appendix B 2: Register of Binary Messages for regional applications

Example for a binary message

Inland specific message FI 10: Inland ship static and voyage related data

This message should be used by inland vessels only, to broadcast ship static and voyage related data in addition to message 5. The message should be sent with binary message 8 as soon as possible (from the AIS point of view) after message 5.

Table x: Inland vessel data report

|  | Parameter | Number of bits | Description |
| --- | --- | --- | --- |
|  | Message ID | 6 | Identifier for Message 8; always 8 |
|  | Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Default = 0; 3 = do not repeat any more |
|  | Source ID | 30 | MMSI number |
|  | Spare | 2 | Not used, should be set to zero. Reserved for future use. |
| Binary data | Application Identifier | 16 | As described in Table 2.6 |
|  | Unique European Vessel Identifcation Number | 48 | 8*6 Bit ASCII characters |
|  | Length of ship | 13 | 1 - 8000 (rest not to be used) length of ship in 1/10m 0 = default |
|  | Beam of ship | 10 | 1 - 1000 (rest not to be used) beam of ship in 1/10m; 0 = default |
|  | Ship or combination type | 14 | Numeric ERI Classification (CODES): 1 Vessel and Convoy Type as described in ANNEX E ERI ship types |
|  | Hazardous cargo | 3 | Number of blue cones/lights 0 - 3; 4 = B-Flag, 5 = default = unknown |
|  | Draught | 11 | 1 - 2000 (rest not used) draught in 1/100m, 0 = default = unknown |
|  | Loaded/unloaded | 2 | 1 = loaded, 2 = unloaded, 0 = not available/default, 3 should not be used |
|  | Quality of speed information | 1 | 1 = high, 0 = low/GNSS = default * |
|  | Quality of course information | 1 | 1 = high, 0 = low/GNSS = default * |
|  | Quality of heading information | 1 | 1 = high, 0 = low = default * |
|  | Spare | 8 | Not used, should be set to zero. Reserved for future use. |
|  |  | 168 | Occupies 1 slot |

* shall be set to 0 if no type approved sensor (e.g. gyro) is connected to the transponder