# e-NAVIGATION – Options for Position-fixing Systems

## Introduction

At the first meeting of the IALA e-Navigation Committee, three fundamental elements were identified that should be in place before e-Navigation could be introduced. These are:

- Electronic Navigation Chart (ENC) coverage of all navigational areas,

- A robust electronic position-fixing system (with redundancy), and

- An agreed infrastructure of communications to link ship and shore.

Electronic position-fixing systems can be divided into Global Navigation Satellite Systems, GNSS augmentations, terrestrial radio-navigation systems and non-radio positioning systems.

## GNSS

There are two operational GNSS at present (GPS & GLONASS) and two more planned (GALILEO and COMPASS). The status of each is described briefly below.

GPS

The Standard Positioning Specification for GPS states that it will meet an accuracy target of 13 m 95% of the time, worldwide. General experience is that it performs much better than this, at least in UK latitudes, with 6 m being a more typical figure. Availability also generally exceeds the IMO requirement of 99.8%, but there are questions to be resolved about integrity and continuity.

There are currently 31 operational satellites in the constellation. Three of these are Block IIR-M satellites with the L2C signal. The first Block IIF with the L5 signal has still to be launched. L2C is scheduled to become operational in 2012 and L5 in 2015, with GPS III still at the design stage and due to be fully operational in 2020.

# GLONASS

After reaching 16 operational satellites at the end of August 2006, the number has now dropped back to 11, which is not sufficient to provide an independent positioning service. However, a commitment has been made by the head of the Russian Space Agency to restore an 18 satellite constellation by 2008.

# GALILEO

The European GNSS Supervisory Authority is now established and the first test satellite GIOVE A continues to broadcast signals. GIOVE B has been delayed because of component problems and it is possible that GIOVE A2 may be launched to retain the frequencies. Negotiations between the parties to the concession and the GSA continue. The official operational date is currently 2012.

# COMPASS

The People’s Republic of China is preparing to deploy its own full constellation GNSS, consisting of 5 GEOs and 30 MEOs.

## GNSS Augmentations

It has long been recognised that GNSS require augmentation to achieve the required integrity for safety of life applications and the accuracy needed for specialised navigation and positioning. Augmentation systems fall into two broad categories: Ground Based (GBAS) and Satellite Based (SBAS).

## GBAS

The (IALA) maritime beacon system has been the standard GNSS augmentation system for maritime applications for over a decade. It uses transmissions in the 300 kHz radio-navigation band in accordance with ITU-R Recommendations.

Loran transmissions at 100 kHz can also be used to broadcast differential corrections for GNSS and there are also short-range, high accuracy systems provided commercially for specialised applications. There is also a facility within AIS to broadcast augmentation messages via message Type 17.

## SBAS

There are two operational and two planned public service SBAS, all using geo-stationary satellites to broadcast integrity messages and differential; corrections. These are not standardised for maritime applications.

The US Wide Area Augmentation System (WAAS) is operational for non-safety critical applications. EGNOS is in Initial Operational Phase, following a successful Operational Readiness Review. It is reported to be broadcasting continuously. The two MTSATs forming the space component of Japan’s MSAS have been deployed, but the ground segment has not yet been completed. India is deploying the ground segment of its GAGAN system.

There are also two providers of wide area commercial SBAS services, which are primarily used for survey and dynamic positioning functions.

## Terrestrial Systems

There are many high accuracy, local radio-positioning systems provided, mostly on a commercial basis, for specialised applications. However, the only terrestrial radio-navigation system with widespread, regional coverage is Loran-C.

There are four regional Loran-C systems, all in the Northern hemisphere. The North American system is run by the US Coast Guard and Canadian Coast Guard and covers most of the continental US and substantial parts of Canada. The former NELS (North West European Loran System) covers the Atlantic coast of NW Europe and is presently operated by Norway and France, with the cooperation of Germany and Denmark. The UK is operating a trial station at Rugby. A new international agreement for this system is under consideration. The Far East Radio-Navigation System (FERNS) is provided under an international agreement between PRC, Russia, Korea and Japan and extends from the Bering Straits to the South China Sea. Saudi Arabia also has a system, covering its own territory and the Arabian Gulf.

Initiatives have been taken to develop e-Loran, an enhanced version of Loran-C offering accuracies comparable to GNSS, which would provide a complement and alternative to GNSS.

## Non-Radio Positioning Systems

The main candidate is the Inertial Measuring Unit (IMU), usually integrated with GNSS to enhance it and cope with outages.

For ocean and coastal areas, both navigation and tactical-grade IMUs will give protection for appreciable GNSS outages (over 1 hour and 15 minutes respectively), giving time for reversion to alternative means of positioning.

In the harbour approach phase, accuracy to 10 metres can be obtained for 3.5 minutes with a navigation-grade IMU-based system, giving time for an experienced user to reference the radar to conspicuous ground fixed targets, provided these are available.

Where better than 10 metre accuracy is required, INS coupled with GNSS does not appear to offer any benefit in the event of a GNSS failure.

The price/performance ratios of available IMUs make them unsuitable for commercial application, but low-cost MEMS-based inertial systems with similar performance are likely to become available in the medium term.

Table contributed to Gap Analysis for IMO CG

| Aids to Navigation [IALA] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Aspect | Today |  | E-Navigation |  | Issues |
| Production |  |  |  |  |  |
| GNSS GNSS augmentation Terrestrial RNAV systems Short-range radar Physical AtoN AtoN information |  | GPS, GLONASS DGPS, DGLONASS Loran-C Radar beacons Lighthouses, buoys, beacons Hard copy |  | GNSS (GPS, Galileo, GLONASS etc) DGNSS, SBAS, RAIM eloran Universal radar beacon Interactive AtoNs Available electronically | Uncertain timelines for deployment Harmonisation Need for international support New Technology radars Development needed Data exchange and presentation |
| Capabilities |  |  |  |  |  |
| Availability Integrity Accuracy Continuity | Dependent on GPS Limited integrity Accuracy generally adequate Requirement needs defining |  | Availability enhanced by GNSS/eLoran Integrity met by DGNSS and RAIM Accuracy will meet all needs New requirements will be met |  | System performance, recording & validation, |
| Standards/ Certification |  |  |  |  |  |
| Service provider Shipborne systems Manufacturers | IMO Res. A.953(23) GPS Performance Standard IEC 61108-1/2/4 |  | IMO Res. A.915(22) GNSS Performance Standards IEC 61108-1/2/3/4 |  | Need updating and merging into one resolution Combined RX or separate inputs to INS/IBS Need updating for new GNSS services |
| Security: Confidentiality/Integrity/Availability |  |  |  |  |  |
| Interference Jamming Spoofing | GPS is vulnerable |  | GNSS will reduce vulnerability |  | Alternative systems will still be needed (e-Loran, radar & physical AtoNs) |
| Liability |  |  |  |  |  |
| For consequential damage | Negligence only |  | Not expected to change |  | Some operations (e.g. Collision avoidance) may depend on several systems – shared liability |
| Regulated Standards |  |  |  |  |  |
| Mandatory carriage | EPFS |  | INS/IBS |  | Combined receivers or separate inputs, specification of required inputs |