GNSS User Needs

- User needs for Maritime Operations

There are no universally accepted user needs / requirements for maritime navigation. The accuracy and integrity requirements vary significantly between different phases of marine navigation. Generally, 3 major phases are identified: oceanic navigation (distance to the nearest obstacle greater than 50 nautical miles), coastal navigation (distance to the nearest obstacle between 3 and 50 miles) and pilotage navigation (distance to the nearest obstacle less than 3 miles).

The requirements for oceanic navigation are very broad because there are no physical constraints.  In coastal areas, vessels travelling along the coast and approaching ports demand more stringent requirements because of the need to avoid collisions and groundings.  However, it is in pilotage waters that the requirements are the most demanding because of the close proximity to dangers. Therefore, only these requirements will be discussed below.

Requirements differ also between different countries and organisations. For this evaluation, in which mainly accuracy and integrity requirements will be focused, the following sources will be used:

- The requirements adopted by the International Maritime Organisation (IMO) in the definition of the future Global Navigation Satellite System (GNSS), which were also adopted by IALA in its “NAVGUIDE”;

- The US government requirements, contained mainly in the 2005 edition of “Federal Radionavigation Systems” (FRS).

  - Current Requirements Adopted by IMO for a Future GNSS

Since 1983, IMO has been discussing a World-Wide Radionavigation System, with the objective of amending regulation V/12 of the SOLAS Convention, in order to include a mandatory requirement for ships to carry a receiver of such a system. In 1995, the 19th session of the IMO Assembly approved Resolution A.815(19) [Ref. 1], containing in its Appendix the operational requirements for a World-Wide Radionavigation System. This Resolution, which specified operational requirements relevant to the first generation GNSS, commonly referred to as GNSS-1, was cancelled, in 2003, by Resolution A.953(23) World-Wide Radionavigation System [Ref. 2]. This revised document comprises an annex with an appendix outlining procedures and responsibilities concerning the recognition of systems, policies for shipborne receiving equipment and operational requirements. It is stated that the requirements may be met by individual radionavigation systems or by a combination of such systems.

In 1997, during its 20th Assembly, IMO stated its policy for a future GNSS, approving Resolution A.860(20) – “Maritime Policy for a Future Global Navigation Satellite System (GNSS)” [Ref. 3], which specified top-level maritime requirements more appropriate to the second generation GNSS (GNSS-2). In November 2001, IMO reviewed the requirements for maritime applications, approving Resolution A.915(22) – “Revised Maritime Policy and Requirements for a future GNSS” [Ref. 4], which cancelled Resolution A.860(20). One of the main changes was the inclusion of a “Table of Minimum Maritime User Requirements for General Navigation”, which contains the following requirements for pilotage waters navigation:

- Port approach and restricted waters:

Absolute accuracy: 10m

Integrity [Time To Alarm (TTA)]: 10s

- Port:

Absolute accuracy: 1m

Integrity (TTA): 10s

These accuracy requirements were also adopted by IALA in its NAVGUIDE. It is important to note that IALA does not mention integrity requirements in that publication.

In terms of integrity, IMO anticipates that “more stringent requirements may be necessary for ships operating above 30 knots” .

It is interesting to quote also the proposed accuracy requirements for some non-general navigation applications, included in a “Table Showing the Minimum Maritime User Requirements for Positioning”, namely hydrography: 1-2m; aids to navigation management: 1m; and offshore exploration and exploitation: 1m.

1.2. US requirements

The Federal Radionavigation Plan (FRP) is the official source of radionavigation policy and planning for the U.S. Federal Government. The companion document, entitled Federal Radionavigation Systems (FRS), contains information on government roles and responsibilities, user requirements and systems descriptions, and is published separately from the FRP. The FRS is jointly developed by the US Departments of Defense and Transportation.

Chapter 2, of the last edition of FRS [Ref. 5] contains the civil marine radionavigation requirements for the different phases of navigation, based on a combination of requirements studies, user inputs and estimates. Pilotage navigation is divided in two phases: inland waterways and harbour entrance and approach.

For the inland waterway phase of navigation, the predictable accuracy requirements (2 drms) are 5-10 m for recreational boats and smaller vessels and 2-5 m for other ships. For the harbour entrance and approach phase, the predictable accuracy requirements (2 drms) are 8-20 m for all ships and 1-5 m for resource exploration.

This document warns that “special radionavigation requirements may arise from new environmental laws and regulations designed to reduce marine vessel casualty events ” .

The performance requirements contained in FRS-2005 do not include integrity, but the well-known “Volpe Report” [Ref. 6] quotes, in its Maritime Navigation Requirements section, integrity values adopted by the RTCA Special Committee SC-159, for aviation use. The integrity parameter, which requires a TTA of 10 seconds, is considered to be adequate also for maritime use and will be used in our evaluation.

1.3. Compliance of GNSS / DGNSS with current user needs

Table 1 summarises the user needs / requirements described above. Table 2 evaluates how stand-alone GPS meets, or not, the requirements for pilotage waters navigation, showing the need for extra accuracy afforded by DGNSS. Table 3 shows that DGNSS meets almost all accuracy requirements for pilotage waters navigation, including the most demanding ones. Only the 1m-accuracy requirement of IMO for ports, is difficult to meet, because it is a very stringent requirement. Nevertheless, it can be met in those ports which have a Broadcast Station sited in the close proximity. With carrier-phase corrections, planned to be incorporated in DGNSS services in a near future, it will be much easier to comply with this very stringent requirement.

| SOURCE | PHASE OF NAVIGATION | ACCURACY REQUIRED | INTEGRITY REQUIRED |
| --- | --- | --- | --- |
| IMO | Port approach & restricted waters | 10m | 10s |
|  | Port | 1m | 10s |
| US | Inland waterways | Recreational boats and smaller vessels: 5-10 m Other ships: 2-5 m | 10s |
|  | Harbour entrances and approaches | All vessels: 8-20 m Resource exploration: 1-5 m | 10s |

Table 1 – Requirements for the Various Phases of Pilotage Waters Navigation

| SOURCE | PHASE OF NAVIGATION | ACCURACY REQUIREMENT | GPS | INTEGR. REQ. | GPS |
| --- | --- | --- | --- | --- | --- |
| IMO | Port approach & restricted waters | 10m | +/- | 10s | N |
|  | Port | 1m | N | 10s | N |
| US | Inland waterways | Smaller vessels: 5-10 m (2 drms) | +/- | 10s | N |
|  |  | Other ships: 2-5 m (2 drms) | N |  |  |
|  | Harbour entrances and approaches | All vessels: 8-20 m (2 drms) | Y | 10s | N |
|  |  | Resource explor.: 1-5 m (2 drms) | N |  |  |

Table 2 – Does GPS Meet the Requirements for Pilotage Waters Navigation?  [Ref.7]

| SOURCE | PHASE OF NAVIGATION | ACCURACY REQUIREMENT | DGNSS | INTEGR. REQ. | DGNSS |
| --- | --- | --- | --- | --- | --- |
| IMO | Port approach & restricted waters | 10m | Y | 10s | Y |
|  | Port | 1m | +/- | 10s | Y |
| US | Inland waterways | Smaller vessels: 5-10 m (2 drms) | Y | 10s | Y |
|  |  | Other ships: 2-5 m (2 drms) | Y |  |  |
|  | Harbour entrances and approaches | All vessels: 8-20 m (2 drms) | Y | 10s | Y |
|  |  | Resource explor.: 1-5 m (2 drms) | Y |  |  |

Table 3 – Does DGNSS Meet the Requirements for Pilotage Waters Navigation? [Ref. 7]

1.4. Conclusion

In terms of accuracy it is clear that DGNSS no longer has the dramatic benefit that it used to, before SA was discontinued. Nevertheless, and despite the significant improvement in its accuracy, stand-alone GPS is not yet sufficient to comply with some accuracy requirements, namely for pilotage waters navigation.

However, the main benefit of DGNSS for mariners is the integrity, which gives mariners an extra-assurance that the position is correct. Stand-alone GPS does not offer adequate built-in integrity to its users and the use of DGPS stations enables the timely warning of any satellite anomaly.

With the termination of SA, DGNSS broadcasts are also being able to improve their services by reducing the susceptibility to interference and transmitting safety of navigation and meteorological messages. In the future, the addition of new messages containing phase corrections will allow DGNSS users sub-metre accuracies.

While integrity is now the main justification for DGNSS, there are some occasions when the accuracy improvement of DGNSS is more apparent and significant, namely when few satellites are visible, due to terrain shading, and during ionospheric disturbances (peaks of the solar cycle, severe magnetic storms, etc).

REFERENCES

[1] IMO Resolution A.815(19), “World-Wide Radionavigation System”, 23 November 1995.

[2] IMO Resolution A.953(23), “World-Wide Radionavigation System”, 5 December 2003.

[3] IMO Resolution A.860(20), “Maritime Policy for a Future Global Navigation Satellite System (GNSS)”, 27 November 1997.

[4] IMO Resolution A.915(22), “Revised Maritime Policy and Requirements for a future GNSS”, 29 November 2001.

[5] US Department of Defense and US Department of Transportation, “2005 Federal Radionavigation Systems”.

[6] John A. Volpe National Transportation Systems Center, “Vulnerability assessment of the transportation infrastructurerelying on the Global Positioning System”, Prepared for the Office of the Assistant Secretary for Transportation Policy (US Department of Transportation), 29 August 2001.

[7] Moore, T., Hill, C., Monteiro, L., “The benefits of DGPS for marine navigators”, Proceedings of the 7th European Navigation Conference (ENC-GNSS 2003), Graz (Áustria), 22-25 April 2003

IMO Resolution A.915(22), “Revised Maritime Policy and Requirements for a future GNSS”, 29 November 2001, Appendix 3, Note 2, p. 17.

US Department of Defense and US Department of Transportation, “2001 Federal Radionavigation Systems”, Paragraph 2.3.3, p. 2-20.