e-NAV 2 / 08 / 04

Proposed Modifications to Message Type 27

RTCM SC104 v. 2.3

Introduction:

This paper details a modification to the RTCM standard SC-104 v.2.3.  In particular, it is prposed to modify RTCM Message Type 27 to include Pre- and Post-Broadcast Integrity flags for the broadcast sites in the Extended DGPS Radiobeacon Almanac, as well as flags indicating constellations provided by a DGPS provider.  The modifications proposed herein do not alter the message length.  Rather, it harvests unused bits of the Station Name.

The e-NAV Committee is invited to support this modification, which has been proposed by the US Coast Guard.

Insert into first paragraph, before the last sentence:

This message has been defined to extend reference station selection criteria by providing an indicator of the integrity method utilized at the transmitter as well as flags indicating the satellite constellation(s) being corrected.

Insert between the seventh paragraph and Table 4-34:

The Integrity Mode field of message Type 27 is used to help facilitate transitions between stations by providing another selection criterion - integrity.  This field provides information on the method of integrity monitoring used at the broadcast station.  When using Post-broadcast integrity, correction data is monitored through the reception of the broadcast to the end user: i.e., the end-user obtains the correction information before the integrity monitor has had time to evaluate the integrity.  If an error is detected, the user is made aware on essentially the next broadcast.  If Pre-broadcast integrity monitoring is used, the Reference Station provides the Integrity Monitor with correction data over a local connection, prior to transmission.  This enables the Integrity Monitor to evaluate the data and to make any required changes, prior to broadcast.  This enables the Reference Station to ensure that hazardous or misleading information is not broadcast.  The Post-Broadcast Integrity Mode can be used to validate the transmission data link (e.g., exciter/transmitter validation), thus those employing Pre-Broadcast Integrity will often employ the Post mode as well.

Table Modifications:

- Remove the row for Station Name and insert the following 3 rows:

| INTEGRITY MODE | 2 | -- | “00” – Pre-Broadcast Integrity Mode Only. “01” – Post-Broadcast Integrity Mode Only. “10” – Pre- and Post-Broadcast Integrity Modes. “11” – Reserved. |
| --- | --- | --- | --- |
| CONSTELLATIONS | 7 |  | Each bit represents a different satellite constellation. When a bit is set, it is used to indicate that the associated reference station(s) provides correction data/messages for that particular satellite constellation. Spare bits are used to enable inclusion of future constellations. The lowest bit number is left-most. Bit Constellation - GPS - Galileo - GLONASS - Spare - Spare - Spare - Spare |
| STATION NAME (9 CHARACTERS) | 63 |  | (Note 6) |

2) Replace “Note 6” with the following:

Note 6:	7 bit ASCII (i.e., the 7 least-significant-bits of 8-bit ASCII characters).  The name should conform to the IALA List, Short Form.  Unused character fields should be filled with zeros.

3) Replace “Note 5” with the following:

Note 5:	Should be coded as “0” if the datum used is close enough to WGS84 to be adequate for the intended use.  For example, it is intended that systems such as those using PE90, and ITRF are sufficiently close enough to WGS-84 and should, therefore, use the “0”datum flag.

Figure 4-17 Modifications:

1    2    3    4    5    6    7    8    9   10   11   12   13   14   15   16   17   18   19   20   21   22   23   24   25   26   27   28   29   30

Words  3,

9, 15 etc

1    2    3    4    5    6    7    8    9   10   11   12   13   14   15   16   17   18   19   20   21   22   23   24   25   26   27   28   29   30

1    2    3    4    5    6    7    8    9   10   11   12   13   14   15   16   17   18   19   20   21   22   23   24   25   26   27   28   29   30

1    2    3    4    5    6    7    8    9   10   11   12   13   14   15   16   17   18   19   20   21   22   23   24   25   26   27   28   29   30

1    2    3    4    5    6    7    8    9   10   11   12   13   14   15   16   17   18   19   20   21   22   23   24   25   26   27   28   29   30

1    2    3    4    5    6    7    8    9   10   11   12   13   14   15   16   17   18   19   20   21   22   23   24   25   26   27   28   29   30

Words  4,

10, 16 etc

Words  5,

11, 17 etc

Words  6,

12, 18 etc

Words  7,

13, 19 etc

Words  8,

14, 20  etc

PARITY

PARITY

PARITY

PARITY

PARITY

PARITY

LATITUDE

LONGITUDE

(LOWER BYTE)

LONGITUDE

(UPPER BYTE)

MSB

LSB

MSB

LSB

LSB

MSB

MSB

REF STATION #1 ID

FREQUENCY

(UPPER 6 BITS)

LSB

LSB

MSB

FREQUENCY

(LOWER 6 BITS)

REF STATION #2 ID

O P

BIT

RATE

D

A

T

R

B

C

I M

Constellation

Flags

C

H

3

STATION NAME

CH1

STATION NAME

CH2

STATION NAME

CH3

(LOW 6 BITS)

STATION NAME

CH4

STATION NAME

CH5

CH6

(UPPER 4 BITS)

CH6

(LOW 3

BITS)

STATION NAME

CH7

STATION NAME

CH8

STATION NAME

CH9