# DRAFT

RECOMMENDATION ON AIS EQUIPMENT REQUIREMENTS

(Extracted and amended from the

IALA TECHNICAL CLARIFICATIONS

ON

RECOMENDATION ITU-R M.1371-1

Draft Edition 1.5)

Table of Contents

DRAFT	1

1	Introduction	3

2	Reference documents	6

3	Mapping of sections of ITU-R M.1371-3 to different kind of AIS stations	8

3.1	General	8

3.2	Sections or paragrahs	9

3.3	Section/paragraphs describing Messages	10

Editorial Notes:

1. Document edit is done with “track changes on.”

2. Some deletions in the matrix are not retained by “track changes” and are permanently deleted.

3. Items that have been reviewed/updated are highlighted in yellow.  This is used to mark the progress in creating the new matrix document.

4. Editorial notes are highlighted in light blue.

5. Items noted for future action are highlighted in green.

RECOMMENDATION ON

AIS EQUIPMENT REQUIREMENTS

# Introduction [This material is from the Clarifications remove and edit as necessary]

  - The purpose of AIS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment.

  - It has long been realized that an automatic reporting device fitted to a ship would have the potential to increase significantly the safety of navigation and would allow the improved control and monitoring of the maritime environment.  Taking up this challenge, the International Maritime Organization (IMO) together with the International Telecommunications Union (ITU) and the International Electrotechnical Commission (IEC) developed a new navigation system called the Automatic Identification System (AIS).

  - Recommendation ITU-R M.1371-1 is the document that describes the technical characteristics for a Universal Shipborne Automatic Identification System using Time Division Multiple Access in the VHF Maritime Mobile Band. As with any technical document some concepts are subject to interpretation. This document presents a series of clarifications for some of these concepts as agreed by the IALA AIS Committee.

A separate ITU-R Recommendation is being prepared as an interim step to set out the “Technical Characteristics for a Class B Shipborne Automatic Identification System  using Carrier Sense Time Division Multiple Access (CSTDMA) in the VHF Maritime Mobile Band” (referred to as Class B”CS” AIS), compatible and interoperable with stations complying with Recommendation ITU-R M.1371-1. It is expected that when Recommendation ITU-R M.1371-1 is revised, the ITU recommendation on Class B”CS” AIS should be incorporated.

These Technical Clarifications contain references to the Class B that uses the access methods described in ITU-R M.1371-1, and these references should not be confused with Class B"CS" as described in the ITU-R M.[XXXX] Recommendation.  Class B"CS" uses the CSTDMA access method.

  - Figure 1 (attached) illustrates the inter-relationship of this document to IMO SOLAS Chapter V; the IMO recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS), (MSC.74(69)); the IMO Guidelines for the Onboard Operational Use of the Shipborne AIS (A.917(22)); the ITU Recommendation on the Technical Characteristics for a Universal Shipborne Automatic Identification System (AIS) using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371-1); and the IEC Standard 61993-Part 2: Universal Shipborne Automatic Identification System (AIS) Operational Performance Requirements, Methods of testing and required test results; the IALA Guidelines on Automatic Identification Systems (AIS).

1.5	As shown in Figure 1, each of the referenced documents in paragraph 1.4 including this document has a specific focus in relation to an overall AIS as follows.

This document, "Recommendation on Technical Clarification of ITU-1371-1", deals specifically with the VHF Data Link (VDL) of an AIS. It acknowledges the fact that some of the specific technical paragraphs in ITU-R M.1371-1 might require interpretations as to their precise, intended meaning.  Consequently, this document presents a set of unifying recommendations that will, hopefully, result in a standardized application of these paragraphs by potential manufacturers, system integrators as well as users of the systems.  Adherence to these recommendations will ensure the viability of the VDL and the interoperability among AIS units obtained from different sources.

1.6 However, this document, whose specific focus is shown by the  outline in Figure 1, also covers some other specific, technical details of various possible AIS station or device configurations that are only mentioned in general terms in ITU-R M.1371.  These specific details are presented in section 2 of this Recommendation in the tables entitled "Mapping of M.1371-1 to the functional requirements of different kinds of AIS devices".  The AIS devices/stations covered by this mapping process are:

- class A shipborne mobile stations,

- class B shipborne mobile stations,

- SAR aircraft mobile stations,

- Aids to Navigation stations,

- Base Stations,

- Simplex Repeater Stations, and

- Duplex Repeater Stations.

Mapping the overall requirements of ITU-R 1371 to these specifically identified AIS devices will enable manufacturers, system integrators and users alike to obtain a comprehensive understanding of their similarities and important differences and limitations thus ensuring their most effective and applicable use.

1.7	The scope of the other documents, referred to in para. 1.4 can be derived from the specific borderlines as shown in the Figure 1.

1.8	All of the identified documents have a specific focus and address specific user needs. Nevertheless, IALA also recognized that there is a need for a high level document that consolidates all of technical and operational aspects of the sub-systems described in Figure 1.  Consequently, IALA is at this time also drafting a general IALA Guideline on AIS  which will incorporate all of the elements in Figure 1 in a descriptive manner.

# Reference documents

[Enter here all the documents referenced in considering the content of the matrix]

# Mapping of sections of ITU-R M.1371-3 to different kind of AIS stations

## General

Different kind of AIS devices can be defined:

- Mobile stations

- Class A shipborne mobile AIS stations, confirming the IMO carriage requirements,

- Class B shipborne mobile AIS stations, not falling under the IMO carriage requirements,

- Class B SO - using SOTDMA access scheme

- Class B CA - using carrier sense access scheme

- SAR airborne mobile AIS stations,

- A-to-N (Aid to navigation) stations, mobile based; in addition, there may be an A-to-N transmission process be added to an AIS base station.

- Shore stations

- AIS base stations,

- Dependent base station

- Independent base station

- General base station

- AIS simplex repeater stations,

- AIS duplex repeater stations.

- [AIS Search and Rescue [Transmitter/transceiver] (AIS SART)]

(Note: "Mobile" and "fixed" is referred to as VDL related functionality. A station, which behaves like a "mobile" station on the VDL, may be fixed, and vice versa.)

For Class B, SAR and AtoN AIS stations these tables may not reflect the latest developments. An attempt was made however to indicate what would be required in order to fulfil the requirement for 1371-1 compatibility. Areas of uncertainties have been identified by question mark.  THESE COLUMNS WILL BE UPDATED WITH THE NEXT EDITION (Ed. 1.4) OF THIS DOCUMENT.  In the future additional new types of AIS devices will appear e.g. Class A derivatives for regional applications (e.g. the European inland waterways) or receive only shipborne mobile AIS devices, etc. These are not  included expressively in this document. However, these future types of AIS devices should be designed in accordance with the description given herein which fits best, e.g.  regional derivatives of Class A shipborne mobile AIS equipment should comply to the functional requirements of Class A shipborne mobile AIS equipment, or receiving-only shipborne mobile AIS devices should comply with the receiving functionality of Recommendation ITU-R M. 1371-1 which applies to either Class A or Class B shipborne mobile AIS equipment

The following table maps all sections/paragraphs in Recommendation ITU-R M.1371-1 for the types of AIS devices. The table gives an overview of whether a certain section/paragraph applies compulsory, recommended or only as an option or whether it should not be used by this particular AIS device at all. Reading of the table is as follows:

There are a number of footnotes and remarks in the table which are explained in the 'Remarks' column.

There are some question marks included ,indicating section applications, which have to be discussed further.

When an '@' is given in the paragraph column, also the sub paragraphs are included in that function.

## Sections or paragrahs

For each type of device, there are 4 small columns showing the applicability, where ‘x’ indicates that the section/paragraph is applicable, and ‘—‘ that the section/paragraph is not applicable. The meaning of the four applicability columns is:

C	Compulsory in any configuration

N	Not allowed; must not be done or used or output

R	Recommended, but not part of minimum requirements

O	purely optional

V	Transmit if VDM sentence is received

Example:

| Annex, Paragraph of M.1371-3 | Short description | Cl. A shipborne mobile station |  |  |  | Cl. B shipborne mobile station |  |  |  | SAR aircraft mobile station |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C | N | R | O | C | N | R | O | C | N | R | O |  |
| A2, 3.2.2.11 | Long transmission packets | x |  |  |  |  | x 8) |  |  |  |  |  | x | 8) Limit to 2 slots. |

This section is Compulsory for Cl A, Not allowed for Cl B and Optional for SAR. For Cl B the message is limited to 2 slots.

## Section/paragraphs describing Messages

Each message as described in A2, 3.3.8.2.1……18 can have the following functions:

G	Generate the message and transmit

R	Receive, process and internal use of the message

P	Presentation interface output

T	Transmit by repeater station after receiving it (repeat)

Example:

| Annex, Paragraph of M.1371-3 | Short description | Cl. A shipborne mobile station |  |  |  | Cl. B shipborne mobile station |  |  |  | Repeater station Simplex |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C | N | R | O | C | N | R | O | C | N | R | O |  |
| A8, 3.1 | Message 1, 2, 3: Position reports | GRP | T |  |  | RP | GT |  |  | RT | G |  | P |  |

This message will be generated by a Cl A system only, explicitly not by the others, all systems will receive it and send it to the presentation interface (optional for repeater station). Cl A and Cl B may not repeat it but the repeater station will do that.

Summarized:

| x | Applicable | C | Compulsory in any configuration | G | Generate and transmit |
| --- | --- | --- | --- | --- | --- |
| -- | Not Applicable | N | Not allowed; must not be done or used or output | R | Receive, process and internal use |
| ? | to be discussed | R | Recommended, but not part of minimum requirement | P | Presentation interface output |
| @ | paragraph description, including sub-paragraphs | O | purely Optional | T | Transmit by repeater stations after receiving (repeat). This functionality is not allowed for AIS base stations. |
|  |  |  |  | V | Transmit if VDM sentence is received |

| Annex, Paragraph of M.1371-3 | Short description | Cl. A shipborne mobile station |  |  |  | Cl. B shipborne mobile station |  |  |  | SAR aircraft mobile station |  |  |  | A-to-N station |  |  |  | Class C station delete |  |  |  | Base station |  |  |  | Repeater station Simplex |  |  |  | Repeater station Duplex |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C | N | R | O | C | N | R | O | C | N | R | O | C | N | R | O | C | N | R | O | C | N | R | O | C | N | R | O | C | N | R | O |  |
| A2, 1 | Structure of the AIS | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2 | Physical layer | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.1.1 A2, 2.1.2 | Physical layer, Parameters, General, Constants and Bandwith except Table 3 PH.TXP | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.1.1 | Table 3 PH.TXP High setting of transmit output power Note: Check the TX and RX BT Products against the IEC documents. | x |  |  |  |  |  |  | x |  |  |  | x 1) |  |  |  | x | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | 1) Because high altitude transmitting power need to be considered. The proposal is to switch off the transmitter for high altitude, switch to low transmitting power setting at medium altitude and to normal operation at low altitude. |
|  | Table 3 PH.TXP Low setting of transmit output power | x |  |  |  | x |  |  |  | x 1) |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.1.3 | Transmission media | x 2) |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | X2a |  |  | 2) For Long Range see remark A4 2a) The AIS duplex repeater should operate on a duplex pair as specified by a competent authority. |
| A2, 2.1.4 | Dual channel operation, 2 receivers | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
|  | Transmission on 2 frequencies | x |  |  |  |  |  | x |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A1, 5 | Frequency Band VHF Operation with 25 kHz bandwidth | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.2 | Transceiver characteristics | x |  |  |  | ? 4) |  |  |  | ? 4) |  |  |  | ? 4) |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | 4) receiver characteristic: need to be expanded |
| A2, 2.3 @ | Modulation scheme | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.4 | Data transmission bit rate | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.5 | Training sequence | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.6 | Data encoding | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.7 | Forward error correction | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.8 | Interleaving | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.9 | Bit scrambling | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.10 | Data link sensing | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 2.11 @ | Transmitter settling time | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | X4a) |  |  |  | 4a) the transmitter settling time must be short enough to ensure a total repeater delay, reception on RX-antenna connector to repeating on TX-antenna connector does not exceed two bit times including filter, demodulation, processing, modulation and filter delay. |
| A2, 2.12@ | Transmitter power except default operation on high power level Research requirement | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | X4b |  |  |  | 4b) Default setting can be either high or low power, subject to local requirement (being fixed station equipment) |
|  | default Operation on high power level | x |  |  |  |  | x |  |  |  | x |  |  |  |  |  | x | x |  |  |  | x |  |  |  | x |  |  |  | X 4b |  |  |  |  |
| A2, 2.13 | Shutdown procedure within 2 s | x |  |  |  | x 5) |  |  |  | x 5) |  |  |  | x 5) |  |  |  | x 5) |  |  |  | x 5a) |  |  |  | x |  |  |  | x 5) |  |  |  | 5) if there is no DSC component then shutdown requirement shall be reduced to 0.5 sec 5a) An AIS base station with internal DSC functionality should shut down the transmission after 1.1 seconds. |
| A2, 2.14 | Safety precautions | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 3 | Link layer | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 3.1 | Sublayer 1 Medium Access Control (MAC) | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 3.1.1 | TDMA synchronization, 1st paragraph | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
|  | RX not synchronized to slot boundaries New |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | MAC SyncBaseRate |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  | x |  |  |  |  | x |  |  |  | x |  |  |  |
|  | MAC SyncMobileRate | x |  |  |  |  | x 6) |  |  |  | x 6) |  |  |  | x? 6) |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  | 6) refer to footnote (1) in A1, Table1 |
| A2, 3.1.1.1 | UTC direct | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
| A2, 3.1.1.2 | UTC indirect | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
| A2, 3.1.1.3 | Synchronized to base station | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x 7) |  |  |  | x7) |  |  |  |  | x |  |  | 7) although 3.1.1.3 is only applicable for mobile AIS stations, an AIS base or an AIS simplex repeater station should be allowed to synchronise to an(other) AIS base station. |
| A2, 3.1.1.4 | Number of received stations – capable of semaphore synchronisation | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
|  | Can be a semaphore | x |  |  |  |  | x 6) |  |  |  | x 6) |  |  |  | x? 6) | x |  |  | x |  |  | x |  |  |  |  | x |  |  |  | x |  |  |  |
| A2, 3.1.2 | Time division | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |
| A2, 3.1.3 | Slot phase and frame synchronization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2, 3.1.3.1 | Slot phase synchronization | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
| A2, 3.1.3.2 | Frame synchronization | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
| A2, 3.1.3.3 | Synchronization – Transmitting stations | x |  |  |  |  | x 6) |  |  |  | x 6) |  |  |  | x? 6) |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  | x |  |  |  |
| A2, 3.1.3.3.1 | Base station operation |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  | x |  |  |  |  | x |  |  |  | x |  |  |  |
| A2, 3.1.3.3.2 | Mobile station operation as a semaphore | x |  |  |  |  | x 6) |  |  |  | x 6) |  |  |  | x? 6) |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |
| A2, 3.1.3.4 @ 3.1.3.4.3 | Synchronization – Receiving stations | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
|  | See new tables. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2, 3.1.4 | Slot identification | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
| A2, 3.1.5 | Slot access | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | X6a |  |  |  | 6a) The transmitter should begin transmission by turning on the RF power at slot start. The transmitter should be turned off after the last bit of the transmission packet has left the transmission unit. HDLC flags may be used to turn off the transmitter. |
| A2, 3.1.6 | Slot state | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  |
| A2, 3.2 @ | Sublayer 2 Data link services (DLS) except 3.2.2.11, Long transmission packets | x |  |  |  | x |  |  |  | x 8) |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- 8a) |  |  |  | 8) 3.2.2.8.2 Distance delay may need consideration with respect to SAR. The proposal is to switch off the transmitter for high altitude, switch to low transmitting power setting at medium altitude and to normal operation at low altitude. See also note 1). 8a) The following subparagraphs are "compulsory" for the AIS duplex repeater, however: 3.2.2, 3.2.2.2, 3.2.2.3, 3.2.2.4 (Start flag should be used to detect a valid received message), 3.2.2.7, 3.3.2.8, 3.2.2.8.3, 3.2.2.9, 3.2.2.10, 3.2.2.11. For A2, §3.2.2.6: not allowed (duplex repeater turn around). |
| A2, 3.2.2.11 | Long transmission packets | x |  |  |  | x 9) |  |  |  |  |  |  | x |  |  |  | x | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | 9) Limit to 2 slots? |
| A2, 3.3 | Sublayer 3 Link management entity (LME) | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.1 @ | Access to data link | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.2 | Modes of operation | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 3.3.2.1 | continuous | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |
|  | autonomous | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
| A2, 3.3.2.2 | Assigned | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  | x 10 |  |  | -- |  |  |  | -- |  |  |  | 10) An AIS base station can not be placed in the assigned mode via the VDL using message 16. See also comment 17). |
| A2, 3.3.2.3 | Polled | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 3.3.3 | Initialization | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.4 | Channel access schemes | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.4.1 @ | Incremental TDMA – ITDMA | x |  |  |  | x |  |  |  | x 11) |  |  |  |  | x 12) |  |  |  | x |  |  |  | x |  |  |  | x |  |  | -- |  |  |  | 11) refer to clarification at A2, 3.3.8.2.7, Msg. 9 and Msg. 9 in A2, Table 13 12) ITDMA and FATDMA are only feasible and therefore may be only used if a A-to-N or a Simplex Repeater is combined with a Base Station |
| A2, 3.3.4.2 @ | Random access TDMA – RATDMA | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.4.3 @ | Fixed access TDMA – FATDMA |  | x |  |  |  | x |  |  |  | x |  |  |  | x 12) |  |  |  | x 13) |  |  | x |  |  |  | x | x |  |  | -- |  |  |  | 13) Provide slot designation methods so FATDMA access can be an option. |
| A2, 3.3.4.4 @ | Self-organizing TDMA – SOTDMA | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  | x |  |  | x 14) |  |  |  |  | x |  |  | -- |  |  |  | 14) refer to footnote (2) of A2, Table 13. SOTDMA Communication State operation stays as defined in 3.3.7.2.2, except when slot time out is zero (0), then slot Offset equals 2250. |
| A2, 3.3.5 @ | Autonomous and continuous operation using SOTDMA | x |  |  |  | x |  |  |  | x |  |  |  |  | x 15) |  |  |  | x |  |  |  | X 15a) |  |  |  | x15a) |  |  | -- |  |  |  | 15) the autonomous and continuous mode of operation of a A-to-N is achieved using RATDMA. 15a) An AIS base or an AIS simplex repeater station is not operating in the autonomous mode. |
| A2, 3.3.6 @ | Assigned operation for mobile stations | x 16) |  |  |  | x 16) |  |  |  | x 16) |  |  |  | x 16) |  |  |  |  | x |  |  |  | x 17) |  |  |  | x 17) |  |  | -- |  |  |  | 16) description of the reaction of the mobile units reflects only cl A type of message. The others keep the standard message. An assigned mode flag is indicating the use of assigned mode for Class B, AtoN, SAR airborne mobile AIS stations. 17) According to this description, an AIS base station or an AIS repeater station may not be switched into assigned mode because they are not operating in the autonomous mode. |
| A2, 3.3.7 | Message structure | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.7.1 | Message ID | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.7.2 @ | SOTDMA message structure | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  | x |  |  | x 14) |  |  |  | x 19 |  |  |  | -- |  |  |  | 19) refer to "IALA Recommendation on AIS Shore Stations and networking aspects related to the AIS Service", section 26.4 |
| A2, 3.3.7.3 @ | ITDMA message structure | x |  |  |  | x |  |  |  | x 11) |  |  |  |  | x 12) |  |  |  | x |  |  |  | x |  |  | x 19) |  |  |  | -- |  |  |  |  |
| A2, 3.3.7.4 @ | RATDMA message structure | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- 20) |  |  |  | 20) in A2, §3.3.7.4.2 delete the words "§4.6.2 and", Rational: does not apply |
| A2, 3.3.7.5 | FATDMA message structure |  | x |  |  |  | x |  |  |  | x |  |  |  | x 12) |  |  |  | x |  |  | x |  |  |  | x |  |  |  | -- 21) |  |  |  | 21) in A2, §3.3.7.5 delete the words "§4.6.2 and", Rational: does not apply |
| A2, 3.3.8 | Message types | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.8.1 | Message summary | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.8.2 | Message description | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 3.3.8.2.1 | Message 1, 2, 3 Position report | GRP | T |  |  | R | GT | P |  | RP | GT |  |  | R | GT |  | P | RP | VG |  |  | RP V | G |  |  | RT | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.2 | Message 4 Base station report | RP | GT |  |  | R | GT | P |  | RP | GT |  |  | R | GT |  | P | RP | VG |  |  | GRP | V |  |  | RT 22) | G |  | P | RT | G |  | P | 22) If repeated by an AIS simplex repeater which is indicated by Repeat Indicator, then the UTC given in these messages may not be accurate due to the repeat delay. |
|  | Message 11 UTC and data response | GRP | T |  |  | R | T | P | G? | R | T | P | G? | R | T |  | G?P | RP | VG |  |  | RP | GV |  |  | RT 22) | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.3 @ | Message 5 Ship static and voyage related data | GRP | T |  |  | R | GT | P |  | R | GT | P |  | R | GT |  | P | RP | VG |  |  | RP V | G |  |  | RT | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.4 @ | Message 6 Addressed binary message | GRP 22 bis) | T |  |  | R | T | P 22 bis) | G 9) | G R P 22 bis) | T |  |  | GRP 22 bis) | T |  |  | G R P 22 bis) | V |  |  | GRP V |  |  |  | RT | G |  | P | RT | G |  | P | 22 bis)Presentation Interface output only messages addressed to station MMSI |
| A2, 3.3.8.2.5 | Message 7 Binary acknowledgement Message 13 Safety related acknowledgement | GRP | T |  |  | GR | T | P |  | G R P | T |  |  | GRP | T |  |  | RPG | V |  |  | GRPV |  |  |  | RT | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.6 | Message 8 Binary broadcast message | GRP | T |  |  | R | T | P | G 9) | G R P | T |  |  | GRP | T |  |  | RPG | V |  |  | GRPV |  |  |  | RT | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.7 | Message 9 Standard SAR Aircraft Position report | RP | GT |  |  | R | GT | P |  | G 23) R P | T |  |  | R | GT |  | P | RP | GV |  |  | RPV | G |  |  | R T 24) | G |  | P | R T 24) | G |  | P | 23) How is the static data of a SAR aircraft transmitted? 24) see remark 8): distance delay 3.2.2.8.2 |
| A2, 3.3.8.2.8 | Message 10 UTC time and data inquiry | GRP | T |  |  | GR | T |  | P | G R | T |  | P | GR | T |  | P | RP | VG |  |  | GRP | V |  |  | RT | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.9 3.3.8.2.10 | Message 12 Addressed safety related message Message 14 Safety related broadcast message | GRP 22 bis) | T |  |  | R | T | G 9) P 22 bis) |  | G R P 22 bis) | T |  |  | GRP 22 bis) | T |  |  | RG P 22 bis) | V |  |  | GRP V |  |  |  | RT | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.11 | Message 15 Interrogation | ###### G 25)R P | T |  |  | R G 25) | T |  | P | G 25) R | T |  | P | R | T |  | G 25)P | R 25 bis) G P | V |  |  | G 25) R P V |  |  |  | RT25a | G |  | P | RT | G |  | P | 25) refer to A 2, Table in § 3.3.8.2.11 25a) Only repeat if slot offset = 0. 25 bis) CLASS C can not be interrogated |
| A2, 3.3.8.2.12 | Message 16 Assigned mode command | RP | GT |  |  | R | GT |  | P | R | GT |  | P | R | GT |  | P | R 26 b) P | VG |  |  | GR 26 b) P V |  |  |  | R T 25a 26a | G |  | P | R 26)T | G |  | P | 26) refer to A2, § 3.3.6 26a) Adjust content of message to reflect the new offset value. 26 b) CLASS C and a Base station can not be assignd by msg 16. |
| A2, 3.3.8.2.13 | Message 17 GNSS broadcast binary message | R P | GT |  |  | R 28) | GT |  | P | R | GT |  | P | R | GT |  | P | RP | VG |  |  | R P G 27) V |  |  |  | R T 27a | G |  | P | R T | G |  | P | 27) if implemented 27a) System design should take into account the maximum delays allowed for repeating DGNSS correction messages 17. 28) IALA recognizes that IEC is developing and maintaining a test standard for Class B shipborne mobile AIS equipment. When developing this standard, IEC is asked to consider, that all Class B shipborne mobile AIS stations should have the same fallback arrangements and priorities with regard to the use of EPFS (Electronic Position Fixing System) as provided for in IEC´s test standard for Class A shipborne mobile equipment. This includes also the same use of Msg. 17 as in Class A shipborne mobile AIS equipment. Additionally, when an internal GNSS is used for position reporting it should comply to the same quality level as defined in the test standard for Class A shipborne mobile equipment with regard to position reporting. |
| A2, 3.3.8.2.14 | Message 18 Standard Cl B equipment position report | RP | GT |  |  | R | T | G 27)P |  | R P | GT |  |  | R | GT |  | P | RP | VG |  |  | RPV | G |  |  | R T | G |  | P | R T | G |  | P |  |
| A2, 3.3.8.2.15 | Message 19 Extended Cl B equipment position report | RP | GT |  |  | R | T | G 27)P |  | R P | GT |  |  | R | GT |  | P | RP | VG |  |  | RPV | G |  |  | R T | G |  | P | R T | G |  | P |  |
| A2, 3.3.8.2.16 | Message 20 Data link management message | RP | GT |  |  | R | GT |  | P | R | GT |  | P | R | TG |  | P | RP | VG |  |  | GRP | V |  |  | R T 26a G |  |  | P | R T | G |  | P | see also 3.3.7.5 |
| A2, 3.3.8.2.17 | Message 21 Aids-to-navigation report | RP | GT |  |  | R | GT | P |  | R | GT |  | P | G | T | R | P | RPG | V |  |  | RP V |  |  | G | RT | G |  | P | RT | G |  | P |  |
| A2, 3.3.8.2.18 | Message 22 Channel management | R P | GT |  |  |  | GT | RP |  | R | GT | P |  |  | GT | R | P | R P | V G |  |  | G 27) R PV |  |  |  | R T | G |  | P | R T | G |  | P |  |
| A2, 4 | Network layer | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 4.1 | Dual channel operation and channel management – except DSC component and manual input (4.1.6) | x |  |  |  | x ? |  |  |  | x ? |  |  |  | x ? |  |  |  | x |  |  |  | x 27a 27b |  |  |  | x 27a 27b 27c |  |  |  | -- |  |  |  | 27a) for A2,§4.1: Receiving message 22 not applicable for AIS base station and AIS simplex repeater station; for A2, §4.1.1: Compulsory in general but channel selection is done by configuration sentences only. There is no requirement for storing received regional operating settings; for A2, §4.1.3: by configuration; for A2, §4.1.5: An AIS base station or an AIS simplex repeater should only operate in one designated area and should not be subject to move via transition to another area; for A2, §4.17 and §4.1.8: not allowed 27b) for A2, §4.1.4 : Not allowed; these rules apply for mobile AIS stations only. 27c) A2, §4.1.6, §4.1.7, §4.1.8, §4.1.9 not applicable for AIS simplex repeaters |
| A2, 4.1.1 | Operating frequency channels | x |  |  |  | x 27 d) |  |  |  | x? |  |  |  | x? |  |  |  | x |  |  |  | x 27e) |  |  |  | x |  |  |  |  |  |  |  | 27d) Channel management by DSC may not be required 27e)Compulsory in general but channel selection is done by configuration sentences only. There is no requirement for storing received regional operating settings |
| A2, 4.1.2 | Normal default mode of dual channel operation | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |  |  |  |  |
| A2, 4.1.3 | Regional operation frequencies | x |  |  |  | x 27f |  |  |  | x |  |  |  | x |  |  |  | x 27g |  |  |  | x |  |  |  | x |  |  |  |  |  |  |  | 27f) When not possible does not transmit 27g) configuration |
| A2, 4.1.4 | Regional operation areas | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  | x |  |  |  | x |  |  | -- |  |  |  | These rules apply for mobile stations only |
|  | DSC channel [need to investigate this being entered in this section] | x |  |  |  |  |  |  | x 3) |  |  |  | x? 3) |  |  |  | x 3) |  | x |  |  |  |  |  | x |  | x |  |  |  |  |  |  | DSC channel management is optional for base stations |
| A2, 4.1.5 | Transitional mode operations near regional boundaries | x |  |  |  | x |  |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |  |  | A base station should only operate in one designated area and should not subject be to move via transition to another area |
| A2, 4.1.5.1 | Conditions for changing bandwidth only | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | x |  |  |  | -- |  |  |  |  |  |  |  |  |
| A2, 4.1.6 | Channel management switch by manual input | x |  |  |  | x ? |  |  |  | x ? |  |  |  |  |  |  | x | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 4.1.7 | Resumption of operation after power on | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  |  | x |  |  | -- |  |  |  |  |  |  |  |  |
| A2, 4.1.8 | Priority of channel management commands | x |  |  |  | x |  |  |  | x |  |  |  |  |  |  | ? |  | x |  |  |  | x |  |  | -- |  |  |  |  |  |  |  |  |
| A2, 4.1.9 | Conditions of changing both AIS operational frequency channels | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | x |  |  |  | -- |  |  |  |  |  |  |  |  |
| A2, 4.2 @ | Distribution of transmission packets, except 4.2.2.1 for AtoN position reports (optional) | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x 31) |  |  |  | -- |  |  |  | 31) for A2, §4.2.2 and §4.2.3 not allowed |
| A2, 4.3 | Reporting rates | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 4.3.1 @ | Autonomously changed reporting rate | x 32) |  |  |  | x 32) |  |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  | -- |  |  |  | -- |  |  |  | 32) as applicable |
| A2, 4.3.2 | Assigned reporting rates | x |  |  |  | x |  |  |  | x 33) |  |  |  | x 33) |  |  |  |  | x |  |  |  | x |  |  | -- |  |  |  | -- |  |  |  | 33) only if allowed to be a semaphore |
| A2, 4.4 | Data link congestion resolution | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 4.4.1 | Intentional slot reuse by own station | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 4.4.2 | Use of assignment for congestion resolution | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  | x |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 4.5 | Base station operation |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  | x |  |  |  |  | x |  |  | -- |  |  |  | base station functions are not additional to mobiles |
| A2, 4.6 | Repeater operation |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  | x |  |  |  | x |  |  |  | repeater station is a different kind of AIS device and not a base station |
| A2, 4.6.1.1 | Repeat indicator (Mobile use) | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 4.6.1.2 @ | Repeat indicator (Base/repeater station use) | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  |  |  |  | x ? | x 33a |  |  |  | x 33b |  |  |  | -- |  |  |  | 33a) An AIS base station does not increase the repeat indicator by repeating via received VDM sentence. An AIS base station should pre-set the repeat indicator to a value between 0 and 3, default=0 for generated messages. For A2, 4.6.1.2.1: not allowed 33b) Simplex repeater station should increment the value of the repeat indicator as described in 4.6.1.2.1 |
| A2, 4.6.2 | Duplex repeater mode | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  |  | x |  |  |  | x |  |  | x |  |  |  | x |  |  |  |  |
| A2, 4.6.3 @ | Simplex repeater mode | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  |  | x |  |  |  | x |  |  | x |  |  |  | -- |  |  |  |  |
| A2, 4.7 | Handling errors related to packets | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 5 | Transport layer | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |  |  | x |  |  |  | x |  |
| A2, 5.1 | Transmission packet | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  |  |  |  | x | -- |  |  |  |  |
| A2, 5.2 @ | Conversion | x 34 a) |  |  |  | x 9) 34) 34 a) |  |  |  | x #### 34) |  |  |  | x 34) |  |  |  | x 34 b) |  |  |  | x |  |  |  |  |  |  | x | -- |  |  |  | 34) requires presentation interface 34 a) Mobile slot limitation of 20 slots per frame for sum of messages 6, 8, 12 and 14 slots used. 34 b) CLASS C slot limitation of 20 slots/frame for sum of messages 6, 8, 12, 14, 15, 21 slots used |
| A2, 5.3.1 | Addressed message | x |  |  |  | ? 35) |  |  |  | x 35) |  |  |  | x 35) |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  | 35) see message 6 at A2, 3.3.8.2., - configuration possibilities to reduce number of retries for certain categories of AIS devices. Proposed is Cl. B one (1) retry - channel load? - MMSI available for all Cl. B? |
| A2, 5.3.2 | Broadcast message | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |
| A2, 5.3.3 | Conversion to presentation interface | x |  |  |  | x 28) |  |  |  | x 27) |  |  |  |  |  |  | x | x |  |  |  | x |  |  |  |  |  |  | x |  |  |  | x |  |
| A2, 5.4 | Presentation interface protocol | x |  |  |  |  |  | x ? |  |  |  | x ? |  |  |  | x ? |  | x |  |  |  | x |  |  |  |  |  |  | x |  |  |  | x | there are no compulsory protocols for the presentation interface beside cl A. It is recommended to have it for all types of AIS. Proposed is future development by: Cl. B: IEC standard for Cl. B AIS SAR: IALA (with guidance from IMO) AtoN: IALA AIS Committee base station: IALA AIS Committee repeater stations: IALA AIS Committee |
| A3 @ | DSC | x |  |  |  |  |  |  | x 3) |  |  |  | x 3) |  |  |  | x 3) |  |  |  | x 3) |  |  |  | x 3) | -- |  |  |  | -- |  |  |  |  |
| A4 @ | Long range | x |  |  |  |  |  |  | x |  |  |  | x |  |  |  | x |  | x |  |  |  | x |  |  | -- |  |  |  | -- |  |  |  |  |
| A5 @ | Application specific messages | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | -- |  |  |  | Application specific messages are put through transparently. |
| A6 @ | Sequencing of transmission packets | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | x |  |  |  | -- |  |  |  | -- |  |  |  |  |