AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation eNAV-101 

On

The E-Navigation Concept
(eNAV Concept)

Working towards Edition 1.0

2007-03-21

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

IALA Recommendation on the E-Navigation Concept (eNAV Concept)

(Recommendation eNAV-101 )

- THE COUNCIL

RECALLING that one of the aims of the Association is to foster safe, economic and efficient movement of vessels and the protection of the environment through the improvement and harmonisation of aids to navigation, vessel traffic services and other means world-wide.

NOTING ...;

- NOTING ALSO the studies carried out by  ...;

NOTING FURTHER that IALA has adopted:

- ...<summary statement regarding the bulk of existing, technology-oriented IALA documentation developed over the years>...;

- ...;

- ...;

RECOGNISING that the eNAV concept will assist in the development and maintenance of a peer-to-peer applications between ship and shore, shore and ship and between shore and shore, particularly with respect to the:

- ...<specific benefits of eNAV concepts regarding peer-to-peer applications>...;

- ....; and,

- .....

RECOGNISING ALSO that the eNAV concept will assist administrations in the efficient deployment of their services to the mariner and to the maritime community:

- ... <specific benefits for administrations> ....;

- ....; and,

- .....

HAVING CONSIDERED the various applications of the eNAV-Concept that have been identified by IMO, ITU, IEC and IALA;

RECOMMENDS that National Members and other appropriate Authorities providing marine aids to navigation services adopt the methodology in accordance with the principles set out in the Annex to this recommendation.

*    *    *

Annex

The e-Navigation Concept (eNAV Concept)

Table of Contents

# Fehler! Es wurden keine Einträge für das Inhaltsverzeichnis gefunden.

- Background

  - IALA’s general mandate

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- enhance the protection of the maritime environment.

The functions of IALA include, among other things:

- developing international cooperation by promoting close working relationships and assistance between members;

- collecting and circulating information on communicating recent developments and matters of common interest;

- liaison with relevant inter-governmental, international and other organisations.  For example, the International Maritime Organisation (IMO), the International Hydrographic Organisation (IHO), the Commission on Illumination CIE, and the International Telecommunications Union (ITU);

- liaison with organisations representing the aids to navigation users;

- address emerging navigational technologies, hydrographic matters and vessel traffic management;

- provide specialist advice or assistance on aids to navigation issues (including technical, organisational or training matters);

- establishing Committees or Working Groups to:

- formulate and publish appropriate IALA recommendations and guidelines;

- contribute to the development of international standards and regulations;

- study specific issues;

- encouraging IALA members to develop policies that address the social and environmental issues associated with establishing and operating aids to navigation.  This includes issues such as:

- preservation of historic lighthouses, and;

- use of aids to navigation as a base for the collection of data or other governmental or commercial services;

- organise Conferences and Seminars relevant to aids to navigation activities.

  - IALA’s mandate and IMO’s eNAV concept

IMO (MSC 81) accepted a proposal by seven member countries to add a new item on the development of an e-navigation strategy to its work programme.  The NAV and COMSAR Sub-Committees were asked to consider the issue.

In July 2006, at the NAV Sub-Committee meeting, several delegations reiterated the role that IALA could play in the development of e-navigation, particularly the shore aspects of aids to navigation.  They also pointed out that IALA had created an e-navigation Committee at its 2006 Conference in Shanghai.  IALA was prepared to play a key role, working closely with IMO, IHO and other relevant international organisations.  Indeed, IMO had invited IALA and other international organisations to participate in its work and provide relevant input.

Also a main characteristic of e-navigation was the interaction between ship and shore.  In this regard, the Officer of the Watch (OOW) and the shore authority were not considered to be discrete entities.  Rather, equipment on board and ashore interacted with each other.  This was not a new concept – such systems were in use for years.  Today, with the proliferation of new technologies, there was a need to amalgamate these and integrate equipment for the benefit of the OOW and for enhancing the efficiency of aids to navigation service.

  - External changes leading to the development of the present eNAV concept

External changes that prompted the development of an eNAV concept for use by administrations are:

- Increased demand for improved information procession in maritime services: The maritime community find itself facing an increased demand for improved maritime services in order to enhance safety and security at sea and protection of the marine environment.

- Increased demand for communication: The maritime community requires simple and efficient communication systems that ensure confidentiality, integrity and availability of information being transmitted and received shore-ship, ship-shore and shore-shore.

- Advent of digital information technologies: Most information is now available in digital format both onboard ships and within shore-based systems. It is also a requirement that more data is collected and stored in order to facilitate data comparison, data exchange and statistic evaluation.

- Globalisation: The maritime community has identified the need to track vessels from berth-to-berth for strategic vessel management, Search-and-Rescue, and security. This resulted also in an increased demand for international data exchange.

- Extended coverage: There is a requirement for extended range which is can be fulfilled more readily by eNAV technologies.

- International standards: International standardization has been recognized as a state-of-the-art description for technology. Hence, there is an increased need for information and documentation for a common, international and public understanding of system functions that constitute the eNAV concept

- Open architecture: Modular and open system architecture design is the preferred solution for software-dominated systems. Open system architectures are more scaleable and better maintainable. When individual components need to be replaced due to life-cycle management, there is a requirement to maintain the functional requirements of a shore-based system more efficiently even.

- Reduction in staffing level: Due to a steady reduction in staffing level, Administrations require an eNAV concept with increased capability for vessel reporting, situational awareness and communications to assist shore-based operators and maintenance personnel.

- Cost/benefit: Administrations face an increased public awareness for cost/benefit of their operation.

Therefore, IALA has a responsibility to the maritime community to provide shore services (ship to shore, shore to ship and shore to shore) that enhance maritime safety, security and the environment. IMO has clearly identified the relationship of ship to shore and a common approach for e-Navigation is essential.

- Introduction

IALA describes its e-Navigation (eNAV) concept in a working definition as follows :

“e-Navigation is the collection, integration and display of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services and security at sea and protection of the marine environment.”

Inferences from this eNAV concept definition are:

- Operational requirements (user requirements) need to be analysed in the framework of the overarching eNAV architecture, i.e. internationally analysed and consolidated operational requirements need to be represented in a way meaningful to the individual parts of the overarching eNAV architecture.

- Three mutually interacting parts of an overarching eNAV architecture can be identified:

- shipboard integration of information/data processing devices;

- application-to-application [information/data] exchange in a peer-to-peer fashion;

- shore-based eNAV system architecture that integrates a variety of shore-based technologies and information/data processing devices;

- Since berth-to-berth service to a ship’s voyage is envisioned, the eNAV concept requires global commonality between the shore-based eNAV system architecture of different national competent authorities, and also a certain degree of international standardisation.

- Also common service levels and standardised minimum capabilities are implied, both for the participating shipboard integrated equipment, for the application-to-application [information/data] exchange means and for the services provided from applications ashore.

From a more technical point of view the concept of logically representing the above relationship eNAV can be described as:

“eNAV is the digital (electronic) interaction of maritime services, ship to ship, ship to shore, shore to ship and shore to shore.”

All Aids to navigation can be represented digitally; this allows a total evaluation and manipulation of Aids to Navigation with respect to eNAV. IALA has the responsibility of Aids to Navigation, including VTS, hence IALA has a significant role to play in the development of the eNAV concept.

- The application-to-application of [information/data] exchange

The key element of the eNAV concept is thinking in terms of [information/data] flow between applications, which constitute sources and sinks for [information/data] or both simultaneously. Also, [information/data] storage over longer periods of time and retrieval are part of the eNAV concept.

To achieve that end, operational requirements (user requirements) need to be represented in terms of appropriate state-of-the-art data models and abstract descriptions of the processes and of the functionality which is required to properly manipulate that [information/data].

Also, the co-operation in [information/data] exchange between ships and shore, shore and ships, shore and shore, ships and ships is at the core of the eNAV concept. Hence the co-operative nature is a core value of eNAV concept.

To provide seamless service, the existing non-co-operative services and aspects must be integrated in this predominantly co-operative eNAV concept by proper engineering models.

Also, thinking in virtual links between participating applications is appropriate for the abstract flow of [information/data] between applications.

While the above abstract statements constitute the core values of the eNAV concept in analysis and design, eventually the [information/data] exchange and the processes and functionalities using that data need to be manifested in physical links, e.g. a physical communication link, and physical entities and devices, e. g. computer systems. Hence, requirements and limitations stemming from that physical world will have their impact on the modelling.

Finally, the different dimensions of application-to-application [information/data] exchange should be considered, when considering the eNAV concept, e. g.:

- Locations of participating applications: on-board ships, on board floating AtoN, onboard aircrafts interacting with the maritime community, on shore, etc. This dimension gives rise to the requirement of a high connectivity within the eNAV concept.

- Distributed responsibilities of stake holders in the eNAV concept, e.g. different authorities ashore operating on the same area, but with different tasks.

- The large variety of the nature and amount of [information/data] to be exchanged between applications will result in a large variety of appropriate technologies to be considered.

- Quality requirements: Here attributes like accuracy, integrity, reliability, continuity, time behaviour etc. need to be defined when analysing the operational requirements (user requirements).

- The overarching eNAV architecture

All the dimensions introduced above (and some more) need to be observed when developing the eNAV concept. This can only be done appropriately by introducing different elements of the eNAV concept, hence an eNAV architecture. Each element of the eNAV concept then contributes in a specific way to the fulfilment of the  requirements. The first element to be considered is the overarching eNAV architecture.

The overarching eNAV architecture is shown in Figure 1.

To the right of Figure 1 there is, for simplicity’s sake, a single “ship technology environment”. From the eNAV perspective the relevant devices within the ship technology environment are the transceiver station, the data sources and the data sinks connected to the transceiver station, the Integrated Navigation System (INS) and the Integrated Bridge System (IBS). The transceiver station is given as a single station for simplicity’s sake, although there may be several transceiver stations. The transceiver station interfaces with the physical link(s) to the eNAV services ashore.

The eNAV services provide the shore-based interfaces to the physical link(s), and they also encapsulate the technology proper to the whole of the common shore-based eNAV system architecture. The entities which are involved with the specifics of the “link technology proper” are confined by the dotted line.

The eNAV services ashore interface to an data transfer network, which distributes the data to applications. For their precise structure see the common shore-base eNAV system architecture below.

Both the shipboard entities, the physical link(s) and the shore-based entities are included in the overarching eNAV architecture, which is indicated in Figure 1 by the surrounding line.

Finally, the GNSS is given as an “external system”, which provides position and time information. For a further discussion of the dependency see below.

IBS

Ship technology environment

"External" systems, such as GNSS and/or terrestrial radionavigation systems

Appli-cation 1

Application 2

Application 3

Application 4

Link technology proper

Scope of eNAV concept

Shipborne Rx/Tx station

Data

source

Data

sinks

INS

eNAV

Ser-vice

Application-to-

application

(peer-to-peer)

virtual

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

Figure 1: 
Overarching eNAV architecture

- Virtual and physical links

  - The relationship between virtual and physical links

The bold dotted arrow in Figure 1 indicates one virtual connection for [information/data] exchange between the “application 4” ashore and the “ship’s technology environment” (data sink), and vice versa (from data source respectively), as an example. The physical path of the [information/data] exchange actually uses the physical links and the various physical interfaces between the entities involved, as indicated by the small arrows. The virtual connection is the abstract statement in regard to requirement analysis for the application-to-application [information/data] exchange. The physical path may take a completely and more sophisticated path. To use the abstraction of the virtual connection hence facilitates the requirement analysis.

Physical links between (fixed) shore stations and (mobile) shipboard equipment each employ one appropriate electromagnetic signal (radio waves) or light.

  - Relevant physical link technologies

Figure 2a shows a variety of physical link technologies which are relevant for the overarching eNAV architecture. It also shows that well-known technologies such as VTS radar can be modelled as a physical link: the data exchanged is the ship’s radar echo signature. Figure 2b shows, that also “visual” Aids-to-Navigations (either fixed or floating) can be modelled in that way. In fact, “visual” Aids-to-Navigation may employ a variety of state-of-the-art physical link technologies simultaneously, not just light.

Every eNAV service employs at least one physical link technology.

Figure 2a/2b: Variety of physical link technologies relevant for the overarching eNAV architecture

- The common shore-based eNAV system architecture

In accordance with the mandate of IALA the focus of attention now moves toward the shore side of the overall eNAV system architecture. There, the eNAV Concept manifests itself in a common shore-based eNAV system architecture.

  - Goals for the common shore-based eNAV system architecture

The concrete goals for the common shore-based eNAV system architecture are:

- Quality goals:

- providing the truly required functionality to the various users (including accuracy, integrity, reliability, and continuity): not less, but also not (much) more

- apply objective criteria for each technical service provided

- same level of service for different users with similar requirements

- extensive usage of applicable international technical standards

- minimize response time for fulfilment of  new user requirements

- Cost related goals:

- Full life-cycle cost evaluation

- Minimize costs for planning and development by maximum usage of applicable international standards

- Minimize costs for implementing new systems due to (rapid) change of technology

- Cut investment costs by x%

- Cut running costs over full life cycle by y%, taking into account cut in staffing level

- Staff related goals:

- Maintain and improve qualification of personnel

- Secure core competence for administration on the long run

- Organisation related goals:

- Identify and describe all processes involved, optimize those processes as applicable, and implement the optimized processes completely

- Further optimize organisational set-up

- Public relations:

- Contribute to an improved public relation of the administration as a whole

  - What (for) instead of how! - the paradigm for the common shore-based eNAV system

The most important part of the paradigm of the common shore-based eNAV system is the Object-oriented Engineering Process (OEP) (see Figure 3 next page). It describes the concrete methodology applied to the engineering task at hand in the following step-by-step procedure.

- List and specify all primary operational requirements for the common shore-based eNAV system as a whole.

- List and specify the secondary requirements for the common shore-based eNAV system as a whole from a management and technical point of view, such as life-cycle-management requirements and resources usage requirements.

- Use state-of-the-art engineering tools to do an engineering analysis of requirements to the shore-based eNAV system. The well attested use case approach provides such state-of-the-art engineering methodology.

- Derive the essential system requirements by exploitation of similarities between primary of different users and also of secondary requirements as identified above.

- Design a common shore-based eNAV system layout predominantly based on eNAV services.

- For each and every essential system requirement identify the interactions of the relevant individual eNAV services needed to fulfil that specific essential system requirement.

- Draft a precise functional description of an individual technology as an eNAV service in accordance with an generic eNAV service model. Include tertiary requirements now.

- Derive all component requirements for all relevant components of an individual eNAV service (primary, secondary, tertiary).

- Capture all the above descriptions in a documentation system and prepare for the submission to Quality Management audits.

Figure 3: Engineering analysis of requirements to the shore-based eNAV system

  - Fundamental design principles for the holistic common shore-based eNAV system architecture

To achieve the above goals the following fundamental principles were introduced:

- user-requirement driven system design: Only clearly and consistently stated user requirements result in technical service provided.

- shift from technology-orientation to information-orientation while designing the system layout: All technical solutions based on data modelling as a rule.

- introduction of the Object-Oriented Paradigm (OOP) for all technical aspects.

- application of a uniform engineering model for all technical services provided by the system, regardless of technology.

- implementation of functional specifications, based on international standards to the largest extent possible: Procurement of technical solutions based on functional specifications as a rule.

- open system architecture and focus on open and standardised interfaces between components and services adhered to: No proprietary interfacing as a rule.

- employment of remote control techniques where feasible at all in order to allow for one coastal-wide technical operation and maintenance centre: Components without remote control capabilities as a rule not permissible (the control capabilities must allow for open interfacing, too).

- implementation of life-cycle management in order to prevent “quick-and-dirty solutions” with huge long-term costs associated: Full life cycle coverage of technical proposals required for their approval.

- documentation of each and every functional aspect in a uniform, overall documentation system: This is a pre-requisite for any quality management system.

- separation between technical operation and maintenance tasks and personnel on one hand (no licence to change functionality and/or structure of component) and system development and optimization tasks and personnel on the other hand (licence to change functionality and/or structure).

Figure 4: common shore-based eNAV system architecture (overview)

  - Overview over common shore-based eNAV system architecture

Figure 4 provides an structural overview representation of the common shore-based eNAV system architecture.

The main building block of the new system model is the so-called eNAV service. An eNAV service encapsulates all primary functionalities dealing with a specific technology or with a specific user, depending on the kind of eNAV service. All eNAV services of the common shore-based eNAV system architecture adhere to the same object-oriented eNAV service engineering model. It will be described in a different IALA Recommendation (see below). All eNAV services are self-contained and provide for themselves all capabilities needed for their tasks, including their own eNAV service management.

In Figure 4 there are some individual eNAV services given, or blocks of eNAV services. Basically all eNAV services shown in Figure 4 do as their task, what is implied by their name. Therefore, the Figure 4 should be self-explanatory to a certain extent. The following explanations should be given, though:

- The “Data Collection and Data Transfer services” are a bundle of eNAV services interfacing the eNAV system to the physical links towards the traffic objects, the natural environment and the waterways as such. They do, what is implied by their name. For example, the AIS Service, the Radar Service, the “Visual” Aids-to-Navigation Service belong to this group. Some operate bi-directionally (e.g. the AIS Service), while others operate uni-laterally on their appropriate physical links.

- The “Value Added Data Processing Services” also are a bundle of individual eNAV services. Their main task is to add value to (raw) data e.g. by processing, combination, comparison etc., store information/data and provide it upon request to other eNAV services.

- The “User Interaction Service” – a single individual eNAV service – is specialised to provide the Man-Machine-Interface to the primary users of the shore-based eNAV system, i. e. such users who are supported immediately by the eNAV system.

- The “Gateway Service” – another single individual eNAV service – is specialised on [information/data] exchange shore-to-shore. Mainly it interfaces to external systems of “third parties”, which provide [information/data] needed within the own eNAV system or which request relevant [information/data] to be forwarded to them. The “Gateway Service” can also interface different shore-based eNAV systems of different organisations operating in the same area as well as provide interfacing to supra-national systems.

- It should be noted, that from the common shore-based eNAV system architecture is modelled in a client-server-fashion; the individual eNAV services can regularly assume both roles, depending on their present role in a given interaction chain to support a given application-to-application [information/data] exchange within the overarching eNAV architecture.

From Figure 4 also one further important conclusion can be derived: It is the plurality of the eNAV services in their system-wide interaction, which provide the required functionality of the whole of the system to its users. It should also be noted, that there is no such thing like a “super-service”, which would manage or control all services. Such a “super-service” is both unnecessary when adhering properly to the object-oriented paradigm and would also constitute an Achilles heel of the eNAV system architecture. (This does not contradict a centralised control of the technical operation of the eNAV system or its centralised maintenance, however.)

For further details refer to the appropriate IALA documentation as given below.

  - The common shore-based system architecture in IALA documentation

In IALA documentation several Recommendations are covering various aspects of the eNAV Concept. Basically, there are two categories of IALA Recommendations (compare Figure 5):

- Framework Recommendations (horizontal blocks in Figure 5): They contain the general aspects of the eNAV concepts from a shore perspective, and they define the framework for all individual eNAV services and their recommendation. They are organised in a top-down fashion.

- Recommendations for individual technologies as eNAV services (vertical blocks in Figure 5): They describe the specific aspects of the given individual technology as an eNAV service within the framework of the common shore-based eNAV system architecture.

| eNAV Concept |  |  |  |  |
| --- | --- | --- | --- | --- |
| common shore-based eNAV system architecture |  |  |  |  |
| eNAV System Requirement Analysis |  |  |  |  |
| eNAV services (engineering model) |  |  |  |  |
| AIS Service as an eNAV service | Radar Service as an eNAV service | “Visual” AtoN Service as an eNAV service | VHF Voice Communication Service as an eNAV service | ... other individual eNAV Services ... |

Figure 5: [Future] IALA eNAV documentation layout

- Dependency on external systems and on infrastructure

  - Dependency on GNSS for position and time

As indicated in Figure 1, there is a dependency of the eNAV Concept on external systems for position fixing and for timing. This may pose a certain vulnerability for all eNAV applications, since dynamic position information is involved in most eNAV applications and time information is required in every eNAV application. Hence, mitigation methods are important. Annex A shows mitigation methods currently being employed or are at hand.

  - Dependency on infrastructure

Though functionally apparently “invisible”, the supporting infrastructure as a module internal to the common shore-based eNAV system architecture has an ever increasing importance for the proper and efficient function of the eNAV applications. Hence, the dependency of the common shore-based eNAV system on its infrastructure.

The infrastructure consists of supporting devices, which e.g. allow application software to be executed (i.e. computers, computer periphery, operation systems, local area network components, etc.), provide a reliable energy supply, provide temperature control, provide housing for components, protection against harmful interference, tampering, unauthorized access, environmental conditions, etc.

Infrastructure becomes more and more sophisticated and in itself subject to increasing requirements. Reduction of energy consumption, environmental friendliness, centralised management, maintenance cost, staffing level, staff responsibilities, and reliability level are just a few key words in this regard.

When setting up a shore-based eNAV system, the infrastructure should be planned carefully to mitigate harmful consequences of the dependency of the shore-based eNAV system on its infrastructure.

- Conclusion

[To be developed in due course]

- Glossary of terms

eNAV concept: [To be developed in due course]

overarching eNAV architecture: [To be developed in due course]

common shore-based eNAV system architecture: [To be developed in due course]

eNAV services: [To be developed in due course]

individual eNAV Services: [To be developed in due course]

virtual link: [To be developed in due course]

physical link: [To be developed in due course]

peer-to-peer connection: [To be developed in due course]

- References

Reference 1

Reference 2

Reference 3

- Annex A – Mitigation methods for the dependency of the eNAV concept on GNSS (position, time)

Task note: The “Technical Working Group” is asked to draft the Annex in accordance with the following guidance:

- this recommendation represents a very prominent, fitting and lasting place (and maybe the only place in existing IALA documentation) to discuss the issues related to the dependency of the eNAV concept on the external GNSS and also feasible mitigation methods.

- position and time should both be covered

- Position in ship-shore-applications: determination methods on-board (transmitting side), the knowledge of which should be of interest to the receiving application (here in particular: ashore); candidates are given in the attached working documents

- Time in shore-ship-applications, shore-shore-applications applications: determination methods ashore, e. g. combinations of GNSS with fixed time sources (time normals)

- The mitigation methods shown should be feasible and should be cost-efficient; optimum would be a survey of methods, which already have entered standardisation, or are on the brink to; academic solutions should not be covered.

- The annex should not exceed 2-5 pages max., please

------------------------ working paper out of eNAV 1 follows ---------------------------

At the first meeting of the IALA e-Navigation Committee, three fundamental elements were identified that should be in place before e-Navigation could be introduced. These are:

Electronic Navigation Chart (ENC) coverage of all navigational areas,

A robust electronic position-fixing system (with redundancy), and

An agreed infrastructure of communications to link ship and shore.

Electronic position-fixing systems can be divided into Global Navigation Satellite Systems, GNSS augmentations, terrestrial radio-navigation systems and non-radio positioning systems.

The Committee is invited to consider this paper as the basis for an input to the IMO Correspondence Group on e-Navigation.

GNSS

There are two operational GNSS at present (GPS & GLONASS) and two more planned (GALILEO and COMPASS). The status of each is described briefly below.

GPS

The Standard Positioning Specification for GPS states that it will meet an accuracy target of 13 m 95% of the time, worldwide. General experience is that it performs much better than this, at least in UK latitudes, with 6 m being a more typical figure. Availability also generally exceeds the IMO requirement of 99.8%, but there are questions to be resolved about integrity and continuity.

There are 31 operational satellites in the constellation. Three of these are Block IIR-M satellites with the L2C signal. The first Block IIF with the L5 signal has still to be launched. L2C is scheduled to become operational in 2012 and L5 in 2015 with GPS III, still at the design stage and due to be fully operational in 2020.

GLONASS

After reaching 16 operational satellites at the end of August 2006, the number has now dropped back to 11, which is not sufficient to provide an independent positioning service. However, a commitment has been made by the head of the Russian Space Agency to restore an 18 satellite constellation by 2008.

GALILEO

The European GNSS Supervisory Authority is now established. And the first test satellite GIOVE A continues to broadcast signals. GIOVE B has been delayed because of component problems and it is possible that GIOVE A2 may be launched to retain the frequencies. Negotiations between the parties to the concession and the GSA continue. Official operational date remains 2012.

COMPASS

The People’s Republic of China is preparing to deploy its own full constellation GNSS, consisting of 5 GEOs and 30 MEOs.

GNSS Augmentations

It has long been recognised that GNSS require augmentation to achieve the required integrity for safety of life applications and the accuracy needed for specialised navigation and positioning. Augmentation systems fall intoi two broad categories: Ground Based (GBAS) and Satellite Based (SBAS).

GBAS

The (IALA) maritime beacon system has been the standard GNSS augmentation system for maritime applications for over a decade. It uses transmissions in the 300 kHz radio-navigation band in accordance with ITU-R Recommendations.

Loran transmissions at 100 kHz can also be used to broadcast differential corrections for GNSS and there are short-range, high accuracy systems provided commercially for specialised applications.

SBAS

There are two operational and two planned SBAS, all using geo-stationary satellites to broadcast integrity messages and differential; corrections. These are not standardised for maritime applications.

The US Wide Area Augmentation System (WAAS) is operational for non-safety critical applications. EGNOS is in Initial Operational Phase, following a successful Operational Readiness Review. It is reported to be broadcasting continuously. The two MTSATs forming the space component of Japan’s MSAS have been deployed, but the ground segment has not yet been completed. India is deploying the ground segment of its GAGAN system.

Terrestrial Systems

There are many high accuracy, local radio-positioning systems provided, mostly on a commercial basis, for specialised applications. However, the only terrestrial radio-navigation system, with widespread, regional coverage is Loran-C. There are four regional Loran-C systems, all in the Northern hemisphere. The North American system is run by the US Coast Guard and Canadian Coast Guard and covers most of the continental US and substantial parts of Canada. The former NELS (North West European Loran System) covers the Atlantic coast of NW Europe and is presently operated Norway and France, with the cooperation of Germany and Denmark. The UK is operating a trial station at Rugby. A new international agreement for this system is under consideration. The Far East Radio-navigation System (FERNS) is provided under an international agreement between PRC, Russia, Korea and Japan and extends from the Bering Straits to the South China Sea. Saudi Arabia also has a system, covering its own territory and the Arabian Gulf.

Initiatives have been taken to develop e-Loran, an enhanced version of Loran-C offering accuracies comparable to GNSS, which would provide a complement and alternative to GNSS.

Non-Radio Positioning Systems

The main candidate is the Inertial Measuring Unit (IMU), usually integrated with GNSS to enhance it and cope with outages.

For ocean and coastal areas, both navigation and tactical-grade IMUs will give protection for appreciable GNSS outages (over 1 hour and 15 minutes respectively), giving time for reversion to alternative means of positioning.

In the harbour approach phase, accuracy to 10 metres can be obtained for 3.5 minutes with a navigation-grade IMU-based system, giving time for an experienced user to reference the radar to conspicuous ground fixed targets, provided these are available.

For docking and where sub-10 metre accuracy is required, INS coupled with GNSS does not appear to offer any benefit in the event of a GNSS failure.

The price/performance ratios of available IMUs make them unsuitable for commercial application, but low-cost MEMS-based inertial systems with similar performance are likely to become available in the medium term.

Table contributed to Gap Analysis for IMO CG

| Aids to Navigation [IALA] |  |  |  |
| --- | --- | --- | --- |
| Aspect | Today | E-Navigation | Issues |
| Production |  |  |  |
| GNSS GNSS augmentation Terrestrial RNAV systems Short-range radar AtoN Physical AtoN information | GPS, GLONASS DGPS, DGLONASS Loran-C Radar beacons Lighthouses, buoys, beacons Hard copy | GNSS (GPS, Galileo, GLONASS etc) DGNSS, SBAS, RAIM eloran Universal radar beacon Interactive AtoNs Available electronically | Uncertain timelines for deployment Harmonisation Need for international support New Technology radars Development needed Data exchange and presentation |
| Capabilities |  |  |  |
| Availability Integrity Accuracy Continuity | Dependent on GPS Limited integrity Accuracy generally adequate Requirement needs defining | Availability enhanced by GNSS/eLoran Integrity met by DGNSS and RAIM Accuracy will meet all needs New requirements will be met | System performance, recording & validation, |
| Standards/ Certification |  |  |  |
| Service provider Shipborne systems Manufacturers | IMO Res. A.953(23) GPS Performance Standard IEC 61108-1/2/4 | IMO Res. A.915(22) GNSS Performance Standards IEC 61108-1/2/3/4 | Need updating and merging into one resolution Combined RX or separate inputs to INS/IBS Need updating for new GNSS services |
| Security: Confidentiality/Integrity/Availability |  |  |  |
| Interference Jamming Spoofing | GPS is vulnerable | GNSS will reduce vulnerability | Alternative systems will still be needed (e-Loran, radar & physical AtoNs) |
| Liability |  |  |  |
| For consequential damage | Negligence only | Not expected to change | Some operations (e.g. Collision avoidance) may depend on several systems – shared liability |
| Regulated Standards |  |  |  |
| Mandatory carriage | EPFS | INS/IBS | Combined receivers or separate inputs, specification of required inputs |

------------------------ 2nd working paper out of eNAV 1 follows ---------------------------

Evaluation of integrated GNSS and INS for marine navigation (was ENAV02-06-07)

Introduction

This paper is a summary of a study carried out for the General Lighthouse Authorities of the UK & Ireland by the IESSG, University of Nottingham.

The objective of the project was to provide an assessment of the capabilities of GNSS-INS to assist with future rationalisation of marine aids to navigation, taking into consideration the e-navigation concept. It examined the price and performance of Inertial Measurement Units (IMU) and considered whether a proliferation of ship-borne inertial systems could change the demands placed on GLA services.

The Committee is invited to note the paper for information and consider whether the findings have any consequences for IALA Policy.

INS for Commercial Marine Applications

Inertial navigation systems are not currently used in most commercial marine applications. This has been because of their historically high cost and the lack of any convincing cost benefit analyses for these applications. Additionally, there has not been a high profile accident where a subsequent investigation showed that it would have been prevented solely by the mandatory use of INS; therefore there has been no pressure for the introduction of INS.

A fundamental advantage of an inertial sensor is that it is ship based and independent of external systems. In this respect it is similar to radar - they both give relative positional data.

Integration of an inertial sensor into a commercial marine navigation system has the potential to produce the following benefits:

Improved navigational solution, with positional and velocity estimates available at a rapid update rate

Automatic warnings that other sensors, including GNSS, have failed or are giving reduced navigational accuracy

Maintain an acceptable navigation solution in the event of a failure or degradation of other sensors

This study concentrated on quantifying the accuracy of navigation in the event of a GNSS failure, using different grades of IMU (‘Navigation’, ‘Tactical’ and ‘Low Cost’) and various (simulated) GNSS failure periods.

Results

It was clear from trials that none of the options considered was suitable to maintain navigation accuracy during a major outage of GNSS. However, the following observations could be made:

For ocean areas, both navigation and tactical-grade IMUs will give protection for appreciable outages (over 1 hour and 15 minutes respectively).

For coastal areas, 15 minutes of good accuracy would be maintained using a navigation-grade IMU, giving good time for reversion to traditional navigational techniques, in particular, using radar for absolute position fixing and increasing the frequency of positioning using visual sights.

For the harbour approach phase, accuracy to 10 metres could be obtained for 3.5 minutes with a navigation-grade IMU-based system.  This would give time for an experienced user to reference the radar to conspicuous ground fixed targets, provided these were available.

For docking and where sub-10 metre accuracy is required, INS coupled with GNSS does not appear to offer any benefit in the event of a GNSS failure.

Unfortunately, the price/performance ratios of the available IMUs make it unlikely that they could become an acceptable solution. Future MEMS-based INS with similar performance and lower price could prove to be a useful addition to a ship’s navigation system.

Conclusions

Although INS may offer tangible advantages to improve a ship’s navigation it is unlikely that its adoption alone would lead to a reduction in the need for conventional AtoNs.

Because the duration of a GNSS outage is indefinable, INS cannot be considered as a primary backup to GNSS

Its main safety advantage is in allowing accurate navigation for a defined time after GNSS failure to enable the navigation of the ship to revert to alternative techniques in an orderly manner.

When GNSS is available it is speculated that the extra motion information obtainable from INS could prove useful to enhance the performance of autopilots, thereby reducing fuel consumption.

The INS units considered in the study do not appear to provide any useful reversionary capability when used in applications needing accuracies of much better than 10 metres.

An INS coupled with GNSS and other existing shipborne sensors, such as a gyrocompass, may give superior performance. This is discussed further below.

Recommendations for Further Work

Trials should be carried out on a vessel with lower dynamics, representative of larger, commercial craft, such as a GLA buoy tender,.

Any further trials should consider the possible benefits to the integrated solution of combining the outputs of a ship-borne heading reference system.

This study has not attempted to draw any conclusions about the improved integrity that may derive from the integration of GNSS and INS.  A rigorous study of the performance of integrity monitoring algorithms for an integrated GNSS/INS solution should be carried out, possibly using data from the above proposed vessel trial.

------------------------ 3rd working paper eNAV2-5-2 follows ---------------------------

Development in non-radio navigation systems

1. Background

This report, prepared for the ENAV Committee, provides information about ongoing developments in the field of non-radio navigation systems. Furthermore developments of INS (Integrated Navigations Systems) which combines radio navigation and non-radio navigation systems will be reported. Most of the information is based on published material, conference proceedings and contacts with manufacturers in this area.

2. Architectures of GNSS/INS Integrations

Today integrated GNSS/INS systems are used for many applications. Based on the complementary error behaviour of both sensors, higher performance levels are possible. The improvement in accuracy, availability, integrity and continuity depends also on the architecture of integration. In general the possible methods can be separated into three different integration categories, “loose-”, “tight-“ and “deep”

According to its measurement principle INS systems provide an autonomous solution for position, velocity and attitude with high data rate and bandwidth. The sensors used in an INS are a triad of gyros for measuring rotation and accelerometers for measuring accelerations. Its typical error behaviour, however, causes only short term stability with respect to accuracy. A GNSS receiver, on the other hand, generates position and velocity solutions which are characterized by long time error stability and a high accuracy level. These are the main reasons for the successful use of integrated GNSS/INS systems in many applications, providing a higher performance level in comparison to the stand-alone solutions. The following sections shall provide a short overview about the typical methods of GNSS/INS integration.

2.1 Loose GNSS/INS Integration

In the loose architecture, the INS and GNSSS receiver operate as independent navigation systems (Figure 1). In this case the sensors are completely independent of each other. The INS calculate an own solution for position, velocity and attitude. Also the GNSS sensor provides an independent navigation result containing position, velocity and time.  Further the information from both sensors will be combined in a Kalman filter process to provide a third navigation solution.

Benefit from Loose GNSS/INS integration

Higher accuracy of the navigation system, caused by the different error behaviour of both sensors.

The INS can act as an active low pass filter for GNSS noise effects

Availability can be improved during short GNSS outages

Mutual control of navigation results is possible in order to increase the integrity of the combined sensor system.

(PVT)

(PVA)

(PVTA)

Figure 1:  Loose GNSS/INS Integration

2.2 Tight GNSS/INS Integration

In this architecture, the INS and GNSS are reduced to their basic sensor functions (Figure 2). They provide pseudoranges, pseudorange rate, accelerations and gyro measurements. All measurements will be used to provide a combined navigation solution. The most important goal of the tight GNSS/INS integration is the improvement of GNSS availability, especially in high dynamic environment or in observation conditions with low GNSS signal-to-noise ratio.

Figure 2:  Tight GNSS/INS Integration

Advantages

In general the tight architecture provides a more accurate solution than the loose integration method

Better availability in high dynamic applications, in low SNR environments and in situations where fewer than 4 satellites are visible

Disadvantages

No redundant navigation solutions as provided from loose integration method

It is necessary to modify the signal processing of the GNSS receiver

2.3 Deep GNSS/INS Integration

The deep coupling principle represents the highest level of GNSS/INS integration (Figure 3). The architecture of the GNSS receiver is fundamentally different from typical receivers. Furthermore the INS is used as an integral part of the GNSS receiver. Thus the deep method can not longer be viewed as a navigator independent from INS.

Figure 3:  Tight GNSS/INS Integration

Advantages

The deep integration enhances the robustness of GNSS to interference and jamming

Optimal fusion of the measurements from GNSS and INS

Disadvantages

No independent position solutions available

3. Conclusion

As shown before the integration of GNSS and INS sensors can be implemented using three different general principles. The gain in performance of the integrated system depends not only on the sensors types but also on the way of coupling. If accuracy or integrity requirements are emphasized, the loose coupling is the preferred method. If the applications are focussed on a high availability, especially in conditions of high dynamics or low signal-to noise ratios, the tight or deep architecture should be taken into account. In this case only short time stability must be guaranteed by the inertial sensor. So primarily low-cost units based on MEMS technology can provide the inertial data.

[1] 	Kreye, C., Eisfeller, B., Ameres, G.,”Architectures of GNSS/INS Integrations”, 
 	Institute of Geodesy and Navigation University FAF Munich, 2004

[2]	Dr, Gebre-Egziabher, “What is the difference between loose, tight, ultra-tight and 
 	deep integration strategies for INS and GNSS”, Inside GNSS, February 2007

Due to that development it is strictly speaking a misnomer to use the word “visual” aids-to-navigation. Since “visual” aids-to-navigation is a well-known term, it will be used throughout the description of the eNAV concept and the common shore-based eNAV system architecture.

Primary aspects are those, which justify the existence of the system component under consideration. Secondary aspects are those, which arise only as “second thoughts”, e.g.: To replay data, there must have been a data logging beforehand. Tertiary are those, which arise only when considering faulty behavior or optimization of basically healthy functional concepts.

It should be noted, that by convention the individual eNAV services are capitalised (“Service”) as opposed to “eNAV service” or “eNAV services” in a generic sense.

This Recommendation.

This framework Recommendation describes in more conceptional and technical detail the common shore-based eNAV system architecture and its features.

This framework Recommendation contains a) an eNAV system requirement analysis, i. e. an analytic engineering representation of the operational requirements for various applications; b) a mapping of these applications to a pattern of interactions of the individual eNAV services involved to generate this application. This framework Recommendation thus provides a seamless and stringent derivation of the interaction of the individual eNAV services from the analytic engineering representation of the operational requirements.

This framework Recommendation lays out the generic engineering model of an eNAV service based on the definitions and statements given in the other framework Recommendations. It provides the guidelines to fit any relevant technology into the common shore-based eNAV system architecture as an individual eNAV service.