IALA TECHNICAL CLARIFICATIONS

ON

RECOMENDATION ITU-R M.1371-3

Edition 2.0

Table of Contents

1	Introduction	3

2	Technical Clarifications of ITU-R M.1371-3	7

3	Mapping of sections of M.1371-3 to different kind of AIS stations (Informative)	8

3.1	General	8

3.2	Sections or paragrahs	8

3.3	Section/paragraphs describing Messages	9

Technical Clarifications of

Recommendation ITU-R M.1371-3

(Draft Edition 2.0)

# Introduction

  - The purpose of AIS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment.

  - It has long been realized that an automatic reporting device fitted to a ship would have the potential to increase significantly the safety of navigation and would allow the improved control and monitoring of the maritime environment.  Taking up this challenge, the International Maritime Organization (IMO) together with the International Telecommunications Union (ITU) and the International Electrotechnical Commission (IEC) developed a new navigation system called the Automatic Identification System (AIS).

  - Recommendation ITU-R M.1371-3 is the document that describes the technical characteristics for a Universal Shipborne Automatic Identification System using Time Division Multiple Access in the VHF Maritime Mobile Band. As with any technical document some concepts are subject to interpretation. This document presents a series of clarifications for some of these concepts as agreed by the IALA AIS Committee.

  - Figure 1 (attached) illustrates the inter-relationship of this document to IMO SOLAS Chapter V; the IMO recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS), (MSC.74(69)); the IMO Guidelines for the Onboard Operational Use of the Shipborne AIS (A.917(22)); the ITU Recommendation on the Technical Characteristics for a Universal Shipborne Automatic Identification System (AIS) using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371-1); and the IEC Standard 61993-Part 2: Universal Shipborne Automatic Identification System (AIS) Operational Performance Requirements, Methods of testing and required test results; the IALA Guidelines on Automatic Identification Systems (AIS).

1.5	As shown in Figure 1, each of the referenced documents in paragraph 1.4 including this document has a specific focus in relation to an overall AIS as follows.

This document, "Recommendation on Technical Clarification of ITU-1371-3", deals specifically with the VHF Data Link (VDL) of an AIS. It acknowledges the fact that some of the specific technical paragraphs in ITU-R M.1371-3 might require interpretations as to their precise, intended meaning.  Consequently, this document presents a set of unifying recommendations that will, hopefully, result in a standardized application of these paragraphs by potential manufacturers, system integrators as well as users of the systems.  Adherence to these recommendations will ensure the viability of the VDL and the interoperability among AIS units obtained from different sources.

1.6 However, this document, whose specific focus is shown by the  outline in Figure 1, also covers some other specific, technical details of various possible AIS station or device configurations that are only mentioned in general terms in ITU-R M.1371.

1.7	The scope of the other documents, referred to in para. 1.4 can be derived from the specific borderlines as shown in the Figure 1.

1.8	All of the identified documents have a specific focus and address specific user needs. Nevertheless, IALA also recognized that there is a need for a high level document that consolidates all of technical and operational aspects of the sub-systems described in Figure 1.  Consequently, IALA is at this time also drafting a general IALA Guideline on AIS  which will incorporate all of the elements in Figure 1 in a descriptive manner.

# Technical Clarifications of ITU-R M.1371-3

Important General Remark

Every clarification of this Chapter is presented in a uniform way:

- Every clarification is introduced in the appropriate context of Recommendation ITU-R M.1371-3, which is always quoted as a whole section. This is done to ensure, that all clarifications are to be understood in their appropriate context.

- The verbatim text of Recommendation ITU-R M.1371-3 is given in italics; additions by the clarification are given upright and underlined as of previous clarification editions and upright and double underlined in the present edition. Deleted text portions are identified with strike-out as of previous clarification editions and double strike out in the present edition.

- For every clarification a rationale is given to allow complete understanding as to why a clarification was needed also. Eventually, the clarifications will be incorporated into a future revision of Recommendation ITU-R M.1371-3).

- A date of when the clarification was drafted is given.

- A comment contains additional information on the clarification itself. For instance, some clarifications have already been incorporated in the first edition of the IEC standard for Class A shipborne mobile AIS stations (IEC 61993-2latest edition). A future substantial change of the clarification result in the creation of a legacy issue. Any future change for those clarifications should therefore be done with the utmost caution.