Tools for integrating the Mariner’s use of traditional navigation techniques

within e-navigation

Submitted to the IALA e-Navigation Committee (e-NAV3) for consideration

by

The Australian Maritime Safety Authority (AMSA) and The Nautical Institute (NI)

##### Summary

The development of e-navigation is to be user needs led so as to enhance navigational safety through electronic integration of systems onboard ships and in shore organisations such as Vessel Traffic Services. This paper focuses on the needs of mariners in particular, and addresses the need to ensure that mariners remain engaged in the process of navigation.

In particular, the authors of this paper recommend the development of tools that will enable the effective integration of key traditional navigation techniques within new e-navigation systems.  These tools will help facilitate situational awareness, enable an independent position fixing capability and preserve the ability to assess and monitor the risk of collision.

An “electronic pelorus” is proposed, which would enhance the functionality of the traditional shipboard compasses for taking bearings of fixed and floating objects, by electronically capturing bearings that would then be automatically recorded for use within the electronic navigation system.  Also proposed is enhanced radar functionality enabling the electronic capture of radar bearings and ranges for use within the electronic navigation system, and also for the positive alignment of radar images with known geographical reference points on electronic charts using proposed “tie point tagging”.

The tools proposed within this paper are commended to the IALA E-navigation Committee for consideration of their operational and technical suitability within the development of e-navigation.

# E-navigation

With the advent of e-navigation where greater use of electronic integration is envisioned to reduce human error and workload, it will be essential that mariners have tools and procedures to maintain engagement with the navigation system and so provide a high level of situational awareness in order that good decisions can be made and any navigation system anomalies or failures can be readily detected.

# Electronic Pelorus

# Background

The compass has been the primary navigation tool for mariners for many years.  Under the SOLAS convention the standard compass must be magnetic but most ships also carry gyro compasses for steering and navigation.  In order to take azimuths and bearings to distant fixed and floating objects a sight vane is used.  Alternatively, and typically when there was no gyro compass, a separate pelorus could be used to observe relative bearings which where then adjusted to true bearings to enable plotting on the navigational chart.

In navigation terms, observing the bearing of a fixed object ahead or astern assists in the monitoring of a vessel’s position with respect to a planned track (i.e. either on track or off track to port or starboard).  The observing of beam bearings helps to determine progress along a planned track, and the observing of bearings of distant objects roughly parallel to a new course help in the determination of when to execute an alteration to the new course.  Multiple bearings from identified fixed geographical objects can be plotted on a chart and the resulting intersection provides a position fix.

In collision avoidance terms, there is a risk of collision when the true bearing of a target remains constant, or nearly constant, and the range is reducing.  Monitoring the visually observed bearing of such a target has always been a highly reliable means of assessing risk of collision.

Although the use of a compass has long been an essential tool for marine navigation and collision avoidance, in recent times its use has tended to wane due to the difficulty of transferring an observed bearing onto an electronic display.  In addition, enclosed bridge wings and a general overconfidence in GNSS electronic position fixing technology has resulted in a reduced dependence on the use of visual bearings.  This can lead to mariners becoming disengaged from the process of actively monitoring a vessel’s safe navigation, which in turn results in a loss of situational awareness and as a direct consequence, incidents of grounding and collision.

# User Needs

User needs within e-navigation that could be met, at least in part, by the use of an integrated electronic pelorus include:

- The need for mariners to be engaged and motivated to actively participate in the practice of navigation.

- The need to identify fixed and floating objects and to be able to plot visually observed bearings (relative to true north) of these fixed and floating objects.

- The need to identify risk of collision by all available means in accordance with COLREGS Rule 7.

- Masters and pilots need to assess the alertness and effectiveness of bridge watchkeepers.

- In the future, shore authorities may need to assess the alertness and effectiveness of bridge teams.

Functional requirements of an electronic pelorus

An electronically integrated electronic pelorus must be capable of visually sighting an object and transferring a line of position (LOP) to a navigation system display.  Further functionality may include:

- The ability to transfer a LOP to an electronic navigation system display, possibly by pulling a ‘trigger’ or pressing a button.

- The ability, once an object has been sighted, to automatically cross-reference it through a graphical means with charted features, or using databases such as lists of lights, sailing directions or a world fleet database.  This might result in the cross reference being made through AIS technology so that once an object, such as a lighthouse or another vessel, has been sighted a photographic image and characteristics of the object is simultaneously displayed.

- Bearings of other vessels to assess risk of collision may be able to associate themselves” with other target tracking features from radar or AIS to offer more effective tracking and to provide information redundancy and internal confirmation and checking.

- Visual images from the pelorus may be able to be enhanced with low-light or infrared technology.

- LOPs taken with the pelorus may be recorded within the Voyage Data Recorder (VDR) or other equipment so that a master can assess the level of activity of watchkeepers.

## Integration of radar derived information

#### Background

At present a mariner is able to collect range and bearing information from a ship's radar and use it to help navigate their vessel.  This is one of the current fundamental functional requirements necessary to facilitate the safe navigation of ships fitted with radar.  The functions for which radar information is currently used include:

- monitoring of range and bearings of contacts to help determine risk of collision,

- monitoring of a vessel's progress along a planned track (parallel indexing),

- position fixing by enabling the manual plotting of intersecting radar ranges and/or bearings taken from known or identified radar conspicuous geographic features,

- assessing conditions of visibility,

- warning of heavy rain, with associated reduction of visibility, and

- enhancing situational awareness by the provision of a birds’ eye (PPI) perspective.

At present these functions typically involve manual plotting on paper charts, or the manual entry of radar range and bearing information into an ECDIS where the information can be electronically plotted and displayed. The performance of these functions helps engage mariners in the process of navigation, assists with the maintenance of situational awareness and facilitates interaction and engagement with onboard navigation equipment.  Within e-navigation it will be vital to address these same Human Element (HE) issues and so maintain the involvement of mariners in the process of navigation whilst ensuring the optimal use of technology.

### User needs and corresponding functional requirements

As the onboard functional requirements for e-navigation are developed there will be a need to define functional requirements that capture the above radar related functions.  In designing these functional requirements the opportunity should be taken to include enhancements and additional functionality to help improve mariner engagement and navigation safety, which might be able to be easily realised by virtue of better integration of radar and electronic charts.

The onboard functional requirements for the use of radar information within an e-navigation environment could be described as follows:

- radar contacts must be able to be selected, or electronically tagged, and there must be a capability for the automatic monitoring of bearings and ranges of these same radar contacts so as to provide advice to mariners about the likely risk of collision (this functional requirement should basically capture and preserve ARPA functions).

- it must be possible to select, or electronically tag, radar conspicuous geographic features and the subsequent use in the automatic monitoring of a ship's progress along a planned track using predefined parallel indexes associated with the current leg of the planned track.

- it must be possible to select identified geographic features and to electronically capture their radar ranges and bearings for subsequent use to resolve a relative position fix and determine a calculated geographic position.

- it must be possible to designate, or electronically tag, known radar conspicuous geographic features (known as tie points) and for the navigation system to use these tie points to continuously calculate a ship's geographic position (this would require the navigation system to obtain from electronic chart data the geographic positions of the identified tie points and to maintain graphical coincidence of the radar and charted features).

- it must be possible for the navigation system to provide a comparison between positions determined via functional requirements c. and d. with positions obtained from external GNSS or other independent electronic means, and to provide the operator with an easy to interpret graphical presentation of these compared positions.

# Procedures & Training

If an integrated electronic pelorus and radar functionality are used to help effectively engage mariners, add redundancy for position fixing, improve risk of collision assessment and help document activity levels, best practice procedures and operational training will need to be developed in unison with any technical implementation.

##### Conclusion

The development of an integrated electronic pelorus and radar functionality, including manual and automatic “tie point tagging”, will provide tools that allow mariners to effectively interact with onboard navigation systems using proven techniques and the best practice of good seamanship.

These improved interactions will result in mariners being more engaged, motivated, current, and competent in the process of navigation, rather than being relegated to the role of being a system monitor.  In addition mariners will be provided with an effective visual tool for assessing risk of collision, additional and independent position fixing capability, and will have access to auditing tools to monitor physical activity during watchkeeping.

The IALA E-navigation Committee is invited to consider the value of such tools, assess the technological feasibility of development, identify procedural and training needs, and make recommendations for its implementation within the scope of e-navigation.

Nicholls’s Concise Guide Vol 1, Brown, Son & Ferguson, Ltd Glasgow, 1944 p179