| From: U.S. Coast Guard | e-NAV3 / 15 / 01 |
| --- | --- |
| To: e-NAV Committee | August 16 2007 |

Interference Protection Criteria for AIS channels

- Introduction

Although AIS is ‘Safety of Navigation’ equipment required on board most ships —SOLAS V/19— and relied upon by Administrations for vessel traffic services (VTS) and navigation safety, the International Telecommunications Union (ITU) currently does not recognize the frequencies used by AIS as having a safety designation.  The 2007 Word Radio Conference will be reconsidering AIS’s safety designation.

AIS systems operate on two frequencies 50 kHz apart largely to ensure protection from interference; however, such separation does not ensure its protection.  Further, even transmitters operating on radio frequency frequencies adjacent to the AIS frequencies can cause interference.  The two default radio channels allocated for Automatic Identification System (AIS) purposes (161.975 MHz and 162.025 MHz, AIS 1 and AIS 2 respectively) are also allocated by the ITU Radio Regulations for fixed and mobile purposes, under the condition that “Any use of frequencies in these bands by stations of other services to which they are allocated should be avoided in areas where such use might cause harmful interference to the maritime mobile VHF radiocommunication service.”  Thus AIS channels can be shared with other users, who potentially can also cause interference.

2.   Discussion

ITU-R has instituted interference protection requirements for many systems having a safety function.  To accomplish this, ITU-R has developed interference protection criteria for such systems as global navigation satellite systems, 406 MHz EPIRBs and radar navigation systems, identified frequencies used for safety purposes such as done for the Global Maritime Distress and Safety System, or as is often done to protect aeronautical systems, place them in exclusively allocated bands having a recognized safety status.  Since systems such as GPS and radar are used by both the aeronautical and maritime service, interference protection criteria developed by the aviation community has generally served to protect maritime users as well.  However, that is not the case for AIS.

The purpose of developing interference protection criteria would be primarily to ensure that International Maritime Organization requirements for safe intership navigation and vessel traffic service operation using AIS is maintained.

## Recommendation

The United States respectfully proposes that the IALA E-Navigation Committee include in their work program the development of draft AIS interference protection criteria for submission to ITU-R Working Party 8B for their consideration.  The U.S. stands ready to assist in this effort.

Article 5.226 of the ITU Radio Regulations.

See Report ITU-R M.[EMC2AIS] EMC Assessment of Shore-Based Electronic Navigation (ENAV) Infrastructure and New Draft Standards for Data Exchange in the VHF Maritime Mobile Band (156-174 MHz)

Appendix 15 to the ITU Radio Regulations.