IALA TECHNICAL CLARIFICATIONS

ON

RECOMMENDATION ITU-R M.1371-3

(presently Draft Revision Recommendation ITU-R M.1371-2)

Edition 2.0

Table of Contents

1	Introduction	3

2	Technical Clarifications of ITU-R M.1371-3	7

3	Mapping of sections of M.1371-3 to different kind of AIS stations (Informative)	8

3.1	General	8

3.2	Sections or paragrahs	8

3.3	Section/paragraphs describing Messages	9

Technical Clarifications of

Recommendation ITU-R M.1371-3

(Draft Edition 2.0)

# Introduction

  - The purpose of AIS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment.

  - It has long been realized that an automatic reporting device fitted to a ship would have the potential to increase significantly the safety of navigation and would allow the improved control and monitoring of the maritime environment.  Taking up this challenge, the International Maritime Organization (IMO) together with the International Telecommunications Union (ITU) and the International Electrotechnical Commission (IEC) developed a new navigation system called the Automatic Identification System (AIS).

  - Recommendation ITU-R M.1371-3 is the document that describes the technical characteristics for an Automatic Identification System using Time Division Multiple Access in the VHF Maritime Mobile Band. As with any technical document some concepts are subject to interpretation. This document presents a series of clarifications for some of these concepts as agreed by the IALA e-NAV Committee.

  - Figure 1 (attached) illustrates the inter-relationship of this document to IMO SOLAS Chapter V; the IMO recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS), (MSC.74(69)); the IMO Guidelines for the Onboard Operational Use of the Shipborne AIS (A.917(22)); the ITU Recommendation on the Technical Characteristics for an Automatic Identification System (AIS) using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371-3); and the IEC Standard 61993-Part 2: Universal Shipborne Automatic Identification System (AIS) Operational Performance Requirements, Methods of testing and required test results; the IALA Guidelines on Automatic Identification Systems (AIS).

  - As shown in Figure 1, each of the referenced documents in paragraph 1.4 including this document has a specific focus in relation to an overall AIS as follows.

This document, "Recommendation on Technical Clarification of ITU-1371-3", deals specifically with the VHF Data Link (VDL) of an AIS. It acknowledges the fact that some of the specific technical paragraphs in ITU-R M.1371-3 might require interpretations as to their precise, intended meaning.  Consequently, this document presents a set of unifying recommendations that will, hopefully, result in a standardized application of these paragraphs by potential manufacturers, system integrators as well as users of the systems.  Adherence to these recommendations will ensure the viability of the VDL and the interoperability among AIS units obtained from different sources.

  - However, this document also covers some other specific, technical details of various possible AIS station or device configurations that are only mentioned in general terms in ITU-R M.1371.

  - All of the identified documents have a specific focus and address specific user needs. Nevertheless, IALA also recognized that there is a need for a high level document that consolidates all of technical and operational aspects of the sub-systems described in Figure 1.  Consequently, IALA is at this time also drafting a general IALA Guideline on AIS  which will incorporate all of the elements in Figure 1 in a descriptive manner.

| IALA Documents |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NAVGUIDE [AIS Manual] |  |  |  |  |  |  |
| Guidelines: AIS as an eNAV service |  |  |  |  |  |  |
| A-123 Provision of shore based AIS | A-124 AIS Service |  |  | A-126 AIS in marine aids to navigation | A-xxx Matrix of AIS stations (out of IALA Technical Clarifications) | A-yyy Future recommendation |
|  | Annex FATDMA planning | Annex …. | Annex …. |  |  |  |
| Non IALA Documents |  |  |  |  |  |  |
| IEC 61993-2 Class A | IEC 62287-1/-2 Class B “CS”/”SO” |  |  | IEC 62320-1 Base station | IEC 62320-2 AtoN station | Future IEC standards |
| ITU-R documentation on AIS, e.g. ITU-R M.1371-3 |  |  |  |  |  | IALA Technical Clarifications on 1371-3 |
| IMO documentation on AIS, e.g. AIS Performance standards |  |  |  |  |  |  |

Figure 1  Documents Matrix

# Technical Clarifications of ITU-R M.1371-3

Important General Remark

Every clarification of this Chapter is presented in a uniform way:

- Every clarification is introduced in the appropriate context of Recommendation ITU-R M.1371-3, which is always quoted as a whole section. This is done to ensure, that all clarifications are to be understood in their appropriate context.

- The verbatim text of Recommendation ITU-R M.1371-3 is given in italics; additions by the clarification are given upright and underlined as of previous clarification editions and upright and double underlined in the present edition. Deleted text portions are identified with strike-out as of previous clarification editions and double strike out in the present edition.

- For every clarification a rationale is given to allow complete understanding as to why a clarification was needed also. Eventually, the clarifications will be incorporated into a future revision of Recommendation ITU-R M.1371-3).

- A date of when the clarification was drafted is given.

- A comment contains additional information on the clarification itself. For instance, some clarifications have already been incorporated in the first edition of the IEC standard for Class A shipborne mobile AIS stations (IEC 61993-2 latest edition). A future substantial change of the clarification result in the creation of a legacy issue. Any future change for those clarifications should therefore be done with the utmost caution.

## A8; §3.2.1 Message 23: Group Assignment Command

### Proposed Clarifying Text:

The Group Assignment Command is transmitted by a Base station when operating as a controlling entity (see § 4.3.3.3.2 Annex 7 and § Error: Reference source not found). This message should be applied to a mobile station within the defined region and as selected either by its position, by its “Ship and Cargo Type” or by its “Station Type”. The receiving station should consider all selector fields concurrently.  It controls the following operating parameters of a mobile station;

- transmit/ receive mode;

- reporting interval; and

- the duration of a quiet time.

Table 1

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 23; always 23 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; default = 0; 3 = do not repeat any more |
| Source ID | 30 | MMSI of assigning station |
| Spare | 2 | Spare. Shall be set to zero |
| Longitude 1 | 18 | Longitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (180, East = positive, West = negative) |
| Latitude 1 | 17 | Latitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (90, North = positive, South = negative) |
| Longitude 2 | 18 | Longitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (180, East = positive, West = negative) |
| Latitude 2 | 17 | Latitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (90, North = positive, South = negative) |
| Station type | 4 | 0 = all types of mobiles (default); 1 = Class A mobile station only reserved for future use; 2 = all types of Class B mobile stations; 3 = SAR airborne mobile station; 4 = Class B “SO” mobile stations only AtoN station; 5 = Class B “CS” shipborne mobile station only; 6 = inland waterways; 7 to 9 = regional use and; 10 to 15 = for future use |
| Type of ship and cargo type | 8 | 0 = all types (default) 1...99 see Error: Reference source not found 100...199 reserved for regional use 200...255 reserved for future use |
| Spare | 22 | Not used. Should be set to zero. Reserved for future use. |
| Tx/Rx mode | 2 | This parameter commands the respective stations to one of the following modes: 0 = TxA/TxB, RxA/RxB (default); 1 = TxA, RxA/RxB , 2 = TxB, RxA/RxB, 3 = reserved for future use |
| Reporting interval | 4 | This parameter commands the respective stations to the reporting interval given in Table 2 |
| Quiet time | 4 | 0 = default = no quiet time commanded; 1-15 = quiet time of 1 to 15 min. |
| Spare | 6 | Not used. Should be set to zero. Reserved for future use. |
| Number of bits | 160 | Occupies one-time period |

Table 2

Reporting interval settings for use with Message 23

| Reporting interval field setting | Reporting interval for Message 23 |
| --- | --- |
| 0 | As given by the autonomous mode |
| 1 | 10 minutes |
| 2 | 6 minutes |
| 3 | 3 minutes |
| 4 | 1 minute |
| 5 | 30 seconds |
| 6 | 15 seconds |
| 7 | 10 seconds |
| 8 | 5 seconds |
| 9 | 2 seconds (not applicable to the Class B “CS”) |
| 9 | Next shorter reporting interval |
| 11 10 | Next longer reporting interval |
| 11 | 2 seconds (not applicable to the Class B “CS”) |
| 12-15 | Reserved for future use |

NOTE – When the dual channel transmission is suspended by Tx/Rx mode command 1 or 2, the required reporting interval should be maintained using the remaining transmission channel.

### Rationale for Clarification:

Clarify the concurrent evaluation of parameters position, ship and cargo type and station type:

This proposed change harmonises the Message 23 reporting interval settings with the Class-B “CS” testing standard (IEC 62287-1 Ed.1).

### Date of amendment: 21 September 2007

### Comment

.No comment.

## A8; § 3.22 Message 24: Static data report

### Proposed Clarifying Text:

Message 24 Part A may be used by any AIS station to associate a MMSI with a name.

Message 24 Part A and Part B shall be used by Class B “CS” shipborne mobile equipment. The message consists of two parts. Message 24B shall be transmitted within 1 min following Message 24A.

In case of an interrogation for a Class B “CS” on a Message 24, the response shall include Part A and Part B.

Table 3

Message 24 Part A

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 24; always 24 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Part number | 2 | Identifier for the message part number; always 0 for Part A |
| Name | 120 | Name of the MMSI-registered vessel. Maximum 20 characters 6-bit ASCII, @@@@@@@@@@@@@@@@@@@@ = not available = default |
| Number of bits | 160 | Occupies one-time period |

Table 4

Message 24 Part B

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 24; always 24 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Part number | 2 | Identifier for the message part number; always 1 for Part B |
| Type of ship and cargo type | 8 | 0 = not available or no ship = default 1-99 = as defined in § Error: Reference source not found 100-199 = reserved, for regional use 200-255 = reserved, for future use |
| Vendor ID | 42 | Unique identification of the Unit by a number as defined by the manufacturer (option; “@@@@@@@“ = not available = default). See Table XX |
| Call sign | 42 | Call sign of the MMSI-registered vessel. 7 X 6 bit ASCII characters, “@@@@@@@“ = not available = default |
| Dimension of ship/reference for position. Or, for unregistered daughter vessels, use the MMSI of the mother ship. | 30 | Dimensions of ship in meters and reference point for reported position (see Error: Reference source not found and § Error: Reference source not found). Or, for an unregistered daughter vessel, use the MMSI of the associated mother ship in this data field |
| Spare | 6 |  |
| Number of bits | 168 | Occupies one-time period |

Table XX – Vender ID field

| Bit | Information | Description |
| --- | --- | --- |
| (MSB) 41 …...... 24 (18bits) | Manufacturer’s ID | The Manufacturer's ID bits indicate the manufacture’s mnemonic code consisting of three 6 bit ASCII characters. 1) |
| 23 …...... 20 (4bits) | Unit Model Code | The Unit Model Code bits indicate the binary coded series number of the model. The first model of the manufacture uses “1” and the number is incremented at the release of a new model. The code reverts to “1” after reaching to “15”. The “0” is not used. |
| 19 …...... 0 (LSB) (20bits) | Unit Serial Number | The Unit Serial Number bits indicate the manufacture traceable serial number. When the serial number is composed of numeric only, the binary coding should be used. If it includes figure(s), the manufacture can define the coding method. The coding method shall be mentioned in the manual. |

1) IALA is presently considering to create an open international register of manufacturers ID’s

### Rationale for Clarification:

In the ITU-R M.1371-3, the vender ID field (42bits) is prepared in the Message 24B for the Class-B AIS station, however the detail is not defined.

The Table XX proposes the clarifications needed for the additional details.

### Date of amendment: 21 September 2007

### Comment

No comment.