PROPOSAL ON THE INTERNATIONAL STANDARDIZATION OF “ELECTRONIC NAVIGATION SUPPORT SYSTEM”

1. Purpose

In order to assist ships to move more safely and further efficiently, as this statement might already be known but briefly, in Japan, a new form of navigation system called Electronic Navigation Support System (ENSS), which was intended to utilize AIS in a further effective manner, had been placed under consideration of an study committee specifically set up for the subject for three years from July 2004. The committee conducted a study-and-research work and also a field test for verification of the expected system.

As a result of these efforts, the committee developed a trial version of ENSS which could demonstrate a capability to present, in terms of characters and images, information provided by AIS binary messages and static information about sea areas of mariners’ interest on a remotely located display unit. It was confirmed through the field test and other studies that the system was, even on a trial base, sufficiently effective for marine navigation.

These findings gave us a confident that the need for ENSS will be increasingly enhanced in future as a one of e-Navigation assisting tool of ship operators onboard,.

This paper will present outline of the ENSS trial version and of the field test at sea, and propose that ENSS be brought up to discussions at an international level with the aim to make it a global standard.

2. Notes on Actions

2.1. Basic design of the system

In designing of the ENSS trial version, the basic design was challenged to develop by;

-  identifying problems which may occur in utilization of AIS information,

-  determining information items to be provided to users, and

-  taking account of the performance requirements in both hardware and software point of view.

(Refer to Annex 1/prop.3.)

2.2. Assessment by prototype software

Based on the concept of system designing, a test system in a laboratory level was formed up by adopting software developed for assessment of the expected system. Evaluation of the laboratory version system was carried out in a style of demonstration to the people who had a carrier of ship crew and engaged in shipping industry at the time. The results of the evaluation were further analyzed to obtain additional fundamentals of the system structure (see Annex 2/prop.3).

2.3. Field Test at sea

According to results of the analysis above, an electronic navigation support system still on a trial base but at sea, i.e., the trial system was structured in a region selected for the field test. The final version of the trial system in field could present binary messages and/or virtual AtoN AIS information on a display in front of the people contributed to the project. Since the system should essentially serve to the users at sea and on land, a communication test was conducted, as planned in advance, between a shore and a ship station.

The results in field were evaluated by the hands of the people concerned, i.e., ex-crew belonging to the shipping society (see Annex 3/prop.3).

3. Proposal

Almost all of the AIS equipment (MKD) onboard are, at present, in a status of being unable to effectively and clearly indicate an AtoN AIS information and/or a binary message on a display. Consequently, the AIS environment is considered in a situation that useful information provided by AIS functions are not sufficiently utilized.

In light of this, the study committee had managed to design an electronic navigation support system, by developing a laboratory version test system and then the trial system at sea, and by conducting a field test with a tentatively structured trial system. Results of the field test demonstrated that the system is, even on a trial stage, appreciable with a high score of evaluation and quite effective for marine navigation.

Since binary massages transferred through AIS or navigation support information delivered via an AtoN AIS are considered to become a significant factor to contribute to the efforts of ensuring safety of navigation and protection of marine environment, it is essential to develop a system which has the capability to effectively provide such messages and/or information on a display.

However, to make the expected system popular worldwide, it is important to have the user equipment internationally standardized and available at a reasonable price. In this context, it seems that IALA is requested to take actions of promoting ENSS up to the point of a global view as a standard navigation system and of making the system to be used as a universal tool everywhere in the world.