# Background

Current symbology for AIS AtoN’s does not differentiabe between Physical and Virtual AtoNs, and does not reveal the type of AtoN, which it represents.

For this reason, the RDANH would like to propose solutions to the problems raised regarding confusion / clutter, and add this into the development of future display standards, in relation to the AIS technology as part of e-Navigation.

## Current symbology

Current symbology requires AIS AtoN’s to be displayed as a diamond with centered cross hairs indicating the position of the AIS AtoN:

Virtual AIS AtoN :

(Indicates the presence of an AIS AtoN, further information must be derived by selecting the AtoN target)

Physical AIS AtoN

(The view of the originally charted symbol of the Physical AtoN is obscured by the AIS AtoN symbol)

# Proposal for an improved symbology for display of AIS AtoN’s

The RDANH proposal will address three issues:

- Physical AIS AtoN symbols should not clutter or confuse the display of the charted AtoN symbol which they associate.

- AIS AtoN’s have the capability of conveying important status information like ‘off position’ or ‘malfunction’ of lights, RACON or similar, and symbology should support an easy mean of detecting these error states.

- The meaning of a Virtual AIS AtoN is currently not expressed in the symbol.  Operational input suggests that in stressful situations the Mariner will often not go trouble the of selecting a virtual AIS AtoN target and possibly flipping through menus to find out which type of AtoN this is.  Therefore, symbology should indicate this using existing symbology.

## Physical AtoN symbology

Our proposal for displaying a physical AIS AtoN is to use the current symbol, the diamond with crosshairs, but to reduce the size and line thickness of the symbol such that it becomes significantly smaller than the charted object, thus not obscuring the object itself, and using blue color:

However, when zooming out to a larger scale, where little chart detail information is available, the AIS AtoN symbol should still be visible, as other AIS targets.:

In this scale additional information of the AIS AtoN could be reached by selecting the AIS AtoN.

On large overview scales, where only RADAR or AIS targets are shown with only coastline or similar information, AIS AtoN symbols could  be left out, to reduce cluttering of the display.

### Physical AIS AtoN’s, off positon

Physical AIS AtoN stations will indicate their current position, and may thus move slightly around the charted position.  This would look like this, but should generally not be obscure the view, since the AIS symbol is small.

Physical AIS AtoN stations may detect and indicate, if a floating AtoN has in fact drifted so far from it’s position, that it is to be considered ‘off position’.  We propose to display this by indicating the text ‘OP!’ directly north-east of the physical AIS AtoN symbol..

OP!

This way, it is evidently clear that some floating AtoN object is in a place, where it shouldn’t be, and one of the near by charted AtoN objects may be expected to be missing, without the user consulting any menus.

### Physical AIS AtoN’s, Malfunction

The AIS AtoN may in it’s AIS AtoN report message use status indicators to report that light or RACON has failed.  In case a malfunction is indicated in the AIS AtoN report, we suggest to display the the text ‘M!’ directly north-east of the physical AIS AtoN symbol. Details of the malfunction (light, racon) should be available by selecting the AIS AtoN target.

M!

## Virtual AtoN symbology

Virtual AIS AtoN’s are markers, which do not have a physical AtoN counterpart, and are intended to be used as markers for special or temporary situations, where physical AtoN’s cannot be in place (yet). They are thus useful for alerting mariners to new or temporary dangers which are not usually there.

However, the current symbol only reveals an AIS AtoN, it does not reveal to the mariner, which type of AtoN is at hand, as does charted physical AtoN’s.  If display users wish to interpret the meaning - the type of AtoN - currently the user has to select the AtoN target and look through the information associated with the target.  Operational input indicate to us, that this operation is too complex. For the type of AtoN for at virtual AIS AtoN to be useful information, the type of AtoN should be indicated by symbology, while the virtual nature should also be easily recognizable.  Additional information relating to the AtoN could be available by selecting the AtoN.

We propose the following display symbology dependant on display scale:

On large overview scales, where only RADAR or AIS targets are shown with only coastline or similar information, or scales with only little chart information, virtual AIS AtoN symbols should be displayed due to the special or temporary nature of the virtual AtoN.

We propose the use of a diamond with dotted outline, with a ‘V’ inside, to indicate that the AtoN is a virtual one.  A circle inside the ‘V’ indicates the precise position of the virtual AtoN at the centre of the diamond, similarly to the crosshairs of the physical AtoN symbol, and the symbol is of same size as for physical AtoN’s.

The illustration on the right indicates 2 virtual and one physical AIS AtoN on an overview scale.

On a more detailed scale, where chart symbols become visible, the meaning of the virtual AtoN should be illustrated by symbology resembling charted, physical AtoN’s, but with a distinction that enables the mariner to determine, that the object is virtual – it is not physically there.

We propose to use existing AtoN symbology to illustrate the type of AtoN, but to replace the black outline (not the fill colours) with a blue, dottet outline, and to add a blue ‘V’ east of the object.  Example Illustrations are shown below.