| From: AIS-TWG |  |
| --- | --- |
| To: e-NAV Committee | May 29, 2007 |

Informative Note

ITU-R M.585-3

“Assignment and use of maritime mobile service identities”

- Introduction

The e-NAV Committee has considered ITU-R M.585-3, “Assignment and use of maritime mobile service identities” and fully supports the principles laid down for the selection MMSI. In particular, section 4.2 of IALA Recommendation A-126 on “The Use of the Automatic Identification System (AIS) in Marine Aids to Navigation” Edition 1.3, gives guidance to competent authorities on how to select an MMSI for an AtoN AIS Station in accordance with ITU-R M.585-3.

The e-NAV committee have identified the need for an MMSI that is not allocated to a specific AIS Station but is a unique identifier for a competent authority. The purpose of this type of MMSI is to allow addressed messages to be sent to the competent authority.

Two applications of what could be called a “System or Virtual” MMSI are given below.

- VTS Example.

A VTS with 5 geographically separate AIS Base Stations requires vessels entering port to send an addressed binary message with the number of persons on board. See IMO SN/Circ 236 Application 6 message “NUMBER OF PERSONS ON BOARD”. The vessels transmit the message which will be received by one or more of the 5 AIS Base Stations. The AIS Base Stations output the message to the shore side network where it will be read by the VTS.

The proposal is to use a “System or Virtual” MMSI of the format 00MID9XXX as the address of the VTS. A specific example would be 002329000 for the Port of London.

- AtoN Authority Example

Floating AIS AtoN Stations transmit an addressed binary message 6 that provides the AtoN Authority with information on the status of the systems on the AtoN. The AtoN Authority does not have a network of AIS Base Stations but receives the AIS data from another competent authority. In this case the AtoN Authority needs a “System or Virtual” MMSI so that the addressed binary messages can be sent to the relevant unique address.

Following the same principles as the VTS example above a specific example would be 002329011 for the Northern Lighthouse Board in the UK.