Annex 3/prop.3

Summary of a Field Test

on a Trial System with AtoN AIS Information

1. Outline of Trial System

The trial system to be evaluated in the field was structured by mainly; a radar site, an ARPA, a trial message generator (personal computer), a trial shore station and an AtoN AIS station. Within the system, information on virtual, synthetic and real AtoN AIS and pseudo AIS targets were transferred via a radio link. In addition, meteorological and hydrological data were also sent out through the function of an operation console and an AIS shore station, both of which were co-located in Tokyo-wan Maritime Traffic Advisory Service Center (Tokyo MARTIS).

The arrangement of the trial system is shown in Fig. 1.

Fig.1 Arrangement of the trial system

2. Transfer of Virtual AtoN AIS Information

The trial shore station transmitted the virtual AtoN AIS information which was written into an internal memory of the station itself by using setup software.

The transfer procedure followed the definition that;

“A virtual AtoN AIS information is provided by AIS message 21. The difference between a virtual and a synthetic AtoN is a real AtoN ‘does or does not physically exist’ at the location indicated by the AIS message. Since a physical AtoN does not exist, the field of ‘Virtual AtoN Flag’ should be filled out with ‘1’ for the virtual AtoN AIS.”

3. Transfer of Real AtoN AIS Information

The real AtoN AIS station transmitted the AtoN AIS information of itself which was written into an internal memory of the station by using setup software.

The transfer procedure followed the definition that;

“A real AtoN AIS information is provided by AIS message 21, which is transmitted by an AtoN AIS station collocated with a physical AtoN. Since a physical AtoN does exist, the field of ‘Virtual AtoN Flag’ should be filled out with ‘0’ for the real AtoN AIS.”

4. Transfer of Synthetic AtoN AIS Information

The trial AIS shore station transmitted the synthetic AtoN AIS information which was written into an internal memory of the shore station by using setup software.

The transfer procedure followed the definition that;

“A synthetic AtoN AIS information is provided by AIS message 21. The difference between a virtual and a synthetic AtoN is a real AtoN ‘does not or does physically exist’ at the location indicated by the AIS message. Since a physical AtoN does exist, the field of “Virtual AtoN Flag” should be filled out with “0” for the synthetic AtoN AIS.

5. Transfer of Pseudo Target Information

An X-band radar site located on shore was regarded as the trial radar station in the test. The radar was used to detect a ship, which was capturing a radar echo of other ship navigating in the area around, and pursue the detected target with its ARPA tracker. Then, the radar station transmitted a tracking information about the target as a target data to a PC message generator prepared for the trial.

The PC message generator converted the target data into a format of AIS message, whereas the trial radar station transmitted the pseudo AIS target in a format prepared for the trial.

6. Transfer of AIS Binary Messages

An AIS shore station transmitted meteorological/hydrological data using AIS message 8, i.e., a binary message, to be sent out through broadcasting. The original data was prepared with the defined format and inputted to the trial system at an AIS console installed in Tokyo MARTIS.

The message formats used to provide information about virtual and real AtoN AIS were prepared according to IALA Recommendation on AtoN AIS, A-126. The format for the information about pseudo targets and binary messages were identical to the ones described in Agenda item 18 of the 49th session of IMO Sub-committee on Safety of Navigation.

４

２

Test of Virtual AtoN

Type: Safe Water Marking

Lat: 34-35-00.000

Lon: 139-06-00.000

Position Accuracy: High (< 10m)

Off Position: None (in place)

Ref. of Position: A0 B0 C0 D0

Type of Elec. Posi. Equip.:

Not defined

Time Stamp: 11

RAIM: Not used

Mode: Autonomous

Fig.2 Sample Display of a Scene in the Field Test at Sea