# FATDMA for Simplex Repeater Stations

A Competent Authority should exercise great care when implementing a Simplex Repeater Station as inappropriate configuration could have a detrimental effect on the VDL.

The VDL access schemes for Simplex Repeater Stations are either FATDMA or RATDMA. There are 3 types of Simplex Repeater Station

- FATDMA only

- RATDMA only

- FATDMA  and RATDMA

Simplex Repeater Stations can be configured to repeat either all received messages or  selected sub-sets of the received messages. The filtering to select a sub-set of messages for repeating can be by one or more of the following examples

- By MMSI

- By message type

- By range

- By location

- By vessel type

- By vessel speed

- By cargo

- By country

- By class of mobile

- By value of the repeat indicator

The above list gives some of the possible means of filtering, many others are possible.

The greatest impact on the VDL is when a Simplex Repeater Station is configured to repeat all messages. A Simplex Repeater Station should repeat the message within 4 seconds (150 slots) and if it cannot find free slots within 4 seconds the message should not be repeated. It should be noted that a Simplex Repeater that is configured to repeat all messages effectively doubles the VDL loading. Hence the Simplex Repeater should have a limit put on how much it increases the VDL loading to. It is considered that the Simplex Repeater Station should not increase the VDL load by

- more than 25% for a FATDMA only device or

- beyond 50% for a device capable of RATDMA,

Above 50% VDL loading starts to have an impact on Mobile AIS Stations ability to find free slots.

## FATDMA only Simplex Repeater Station

A FATDMA only Simplex Repeater Station could have a maximum of [25%] of the VDL slots reserved for its use to comply with the maximum VDL loading rules. Hence if the VDL loading from original messages is between 0% to 25% then most messages can be repeated in FATDMA slots. However if the VDL loading from original messages exceeds 25% then some messages cannot be repeated. In such circumstances the FATDMA only Simplex Repeater Station should select which messages to repeat by the following priority

- [Mobile position reports

- By distance rejecting the most distant first

- Channel management messages

- Base Station messages

- Safety related messages

- Binary messages

- AtoN messages

- By repeat indicator giving priority to original messages.]

The Competent Authority should monitor the use of FATDMA only Simplex Repeater Stations to ensure that the VDL loading does not exceed 50% in the vicinity of the Station for any significant period as this would adversely affect the ability of mobile stations to find free slots for their own transmissions.

## RATDMA only Simplex Repeater Stations

A RATDMA only Simplex Repeater Station should not increase the VDL loading beyond 50%. If the VDL loading from original messages exceeds 25% then the RATDMA only Simplex Repeater Station should restrict the number of messages it repeats to keep the VDL loading below 50%.

In such circumstances the RATDMA only Simplex Repeater Station should select which messages to repeat by the following priority

- [Mobile position reports

- By distance rejecting the most distant first

- Channel management messages

- Base Station messages

- Safety related messages

- Binary messages

- AtoN messages

- By repeat indicator giving priority to original messages.]

As a RATDMA only Simplex Repeater Station reduces the number of messages it repeats during high VDL loading there is less need for the Competent Authority to monitor the VDL loading in the vicinity of a RATDMA only Simplex Repeater Station.

## Combined FATDMA and RATDMA Simplex Repeater Station

A Combined FATDMA and RATDMA Simplex Repeater Station should follow the 50% VDL loading rule in the same manner as a RATDMA only Simplex Repeater Station. To achieve this a proportion of its transmission slots are FATDMA, the remainder being RATDMA.

A Combined FATDMA and RATDMA Simplex Repeater Station uses FATDMA slots for its transmissions and only resorts to RATDMA if it cannot find an available FATDMA slot within 4 seconds. If it then cannot find a free RATDMA slot within 4 seconds it does not repeat the message.

An example of a Combined FATDMA and RATDMA Simplex Repeater Station could have 10% of the VDL reserved for its FATDMA transmissions and could use up to a further 15% of the VDL for RATDMA transmissions. However if the VDL loading exceeds 50% then the number of RATDMA transmissions would be reduced to keep the VDL loading below 50%.

## Example of FATDMA Reservations for Simplex Repeater Station

A Combined FATDMA and RATDMA Simplex Repeater Station or a FATDMA only Simplex Repeater Station that requires 10% of the VDL could have the following FATDMA reservation.

### Channel A

Slots 6, 7,8 & 9 with an in increment 150

Slots 51,52 & 53 with an increment of 150

Slots 79, 80, 81 & 82 with an increment of 150

Slots 111, 112, 113 & 114 with an increment of 150

### Channel B

Slots 12, 13,14 & 15 with an in increment 150

Slots 61,62 & 63 with an increment of 150

Slots 99, 100, 101 & 102 with an increment of 150

Slots 121, 122, 123 & 124 with an increment of 150

These FATDMA reservations would require only 2 Data Link Management messages (Message 20), one on each channel.