SUB-COMMITTEE ON							         COMSAR 12/11/**

RADIOCOMMUNICATIONS AND					                           1 February 2008

SEARCH AND RESCUE							        Original:  ENGLISH

12th session

Agenda item 11

DEVELOPMENT OF AN e-NAVIGATION STRATEGY

Report from the e-Navigation Correspondence Group

Submitted by the United Kingdom

## SUMMARY

Executive Summary:  NAV 53 re-established the intersessional Correspondence Group for e-Navigation under the co-ordination of the United Kingdom. The Group’s terms of reference included a requirement to submit a document to COMSAR 12 raising specific questions that should be addressed by COMSAR and prepare a final comprehensive report for submission to NAV 54.

The paper summarises progress made by the Correspondence Group, with assistance from IALA and IFSMA, in identifying the main requirements for the e-Navigation user.

Action to be taken:	Paragraph 19

Related documents:  	COMSAR 11/WP.4, NAV53/WP.8, NAV53/13/00

### Introduction

- The United Kingdom has coordinated the work of an intersessional Correspondence Group, set up by NAV52 and re-established under NAV53, for the development of an e-Navigation strategy.  The Group consists of around 70 members, with representatives from different flag states, maritime agencies and non-governmental organisations.  Flag states who participated include Poland, Panama, Sweden, China, Norway, the Marshall Islands, Brazil, Bolivia, Spain, Denmark, the Bahamas, Turkey, South Africa, the USA, the Republic of Korea, the Netherlands, Finland, Germany, Singapore, the United Kingdom, Portugal, Japan, Australia and Canada.  Other governmental and non-governmental organisations that participated include IALA, IHO, CIRM, ICS, IEC, APA, CIRM, the British Chamber of Shipping, IFSMA, BIMCO, INMARSAT, IMPA, SHOM, TRANSAS, IHB, IHMA, INTERCARGO, INTERTANKO, MENAS, the Nautical Institute, OCIMF and UKHO.

- In accordance with the recommendations made from the COMSAR and NAV Sub-Committees, with assistance from IALA and IFSMA, the Correspondence Group has been developing the user requirements for the e-Navigation strategy.  This paper takes account of input from COMSAR 11, prompted by specific questions submitted by the Correspondence Group.

- The Correspondence Group would like COMSAR to consider the implications for developing a common information data source, delivering resilient communications, data provision and integrity based on the requirements outlined below.

Main outcomes from NAV 53:

- NAV 53 considered document NAV 53/13 submitted by the United Kingdom [on behalf of the Correspondence Group] on the development of an e-Navigation strategy.  The Sub-Committee agreed that the definition of e-Navigation would be:

- “e-Navigation is the harmonized collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment.”

- The Sub-Committee also [provisionally] agreed eleven core objectives and endorsed the broad expectations of e-Navigation in relation to improved communications and developing user requirements for shore-based and on-ship stakeholders.  The Sub-Committee also agreed that, to progress the work for NAV 54, the Intersessional Correspondence Group should be re-established under the co-ordination of the United Kingdom.  The terms of reference for the Group were modified, to reflect the most immediate priority - to identify user needs and services.  NAV 53 also asked the Correspondence Group to submit a document to COMSAR 12 raising specific questions that may need to be addressed by COMSAR and to prepare a comprehensive report for submission to NAV 54.

Response to COMSAR 11 and NAV53:

- The Correspondence Group submitted a report (NAV53/13/) to the NAV Sub-Committee which included the recommendations from the COMSAR 11 Sub-Committee.

- The NAV Sub-Committee at its 53rd session noted that COMSAR 11 had agreed that:

- the user requirements should be clearly defined by the NAV Sub-Committee before the COMSAR Sub-Committee could review the technical improvements that might be required if GMDSS equipment was to be utilized as a data communication network for e-Navigation;

- the development of e-Navigation should be user-driven and not technology driven;

- there should be equipment performance standardization, including a standard mode of operation (S-mode) for shipboard equipment; and

- the software installed in operating systems should follow a formal change control process to ensure that all elements of the e-Navigation system would operate efficiently.

- It also noted that COMSAR 11 had agreed that several issues were within its remit; notably, search and rescue, data communication links, and operation of the GMDSS.  The COMSAR Sub-Committee had also agreed that the existing GMDSS infrastructure supported SAR services and communications; but with respect to e-Navigation, broadband communication on a global basis using satellite technology would be necessary.

- The NAV Sub-Committee had agreed with COMSAR 11 that the e-Navigation strategy should be user, rather than technology driven and was of the view that it was first necessary to identify and define the user requirements before considering any technology standards.  The Sub-Committee noted the ongoing work of the Correspondence Group to develop the gap-analysis and system architecture for e-Navigation, but concluded that the user requirements should be developed first.  NAV also agreed that it was necessary to determine the present limits of the e-Navigation strategy, recognizing that this strategy had to be updated as and when necessary.

- With respect to user requirements, the NAV Sub-Committee noted the information provided by IFSMA on the project being undertaken by the Nautical Institute, aimed at developing a standard presentation of information using a standard menu system for shipboard units (S-mode). The Sub-Committee welcomed this initiative and noted the support of COMSAR 11, but agreed that pending further development, it would be premature at this stage to endorse a S-mode for mariners.

- The Sub-Committee further agreed that there was a need to provide an internationally agreed alternative system for complementing the existing satellite navigation, positioning and timing services to support e-Navigation and recognized that potential back up systems could be made available.  It concluded however, that it was premature to identify any specific system before the users’ requirements for e-Navigation had been finalized.

##### Developing the User requirements

- In September 2007, the Correspondence Group was involved in drafting and approving a list of maritime users that would be affected by e-Navigation.  The Correspondence Group also endorsed IALA's methodology and template, developed to identify the user need.  The method was based on the five categories in the definition of e-Navigation (provisionally agreed at NAV53), for the harmonized collection, integration, exchange, presentation and analysis of maritime information, on-board and ashore by electronic means.  A sixth category, covering human element issues was subsequently added to address issues such as motivation, engagement, training, standardisation and ergonomics.  The Correspondence Group was also involved in consulting maritime users and its members made a significant contribution in terms of feedback to IALA.

- The Correspondence Group was also involved with the deliberations in IALA's Strategy and Operations Working Group, which analysed responses to the exercise.  The Working Group agreed that the high-level user requirement should focus on major functional issues with a minimum reference to any specific technologies, solutions or existing standards; although it was agreed that these would need to be considered when it came to implementation.

- General conclusions from the preliminary user needs analysis were that e-Navigation will need to address the following high level user needs at a minimum:

Common Maritime Information Data Structure: Mariners expressed a clear need for all maritime information pertaining to the planning and execution of voyages, the assessment of risk, and compliance with regulation to be accessible from a single integrated system. Furthermore, shore users expressed a clear need for all information pertaining to their maritime domain, vessels and their voyages, to be contained in an internationally agreed common maritime information data structure. Such a data structure was also seen as an essential requirement for the sharing of information between shore authorities on an international basis.

Automated and Standardised Reporting Functions: A clear need was expressed for e-Navigation to provide more automated and standardised reporting functions. There is a clear need for increased communication of ship and voyage information to shore, and for safety related information to be sent from shore to all maritime users. These reporting requirements have proved to be highly distracting to all users, and should be automated or pre-prepared as much as possible both in terms of format, content and communications technology. The group recognised that security, legal and commercial issues will have to be addressed.

Human Centred Presentation Needs: All users expressed a clear need for navigation displays to be designed to clearly indicate risk and to optimise support for decision making by both mariners and shore based users. Strong support was given to an integrated ‘alert management system’ such as contained in the revised IMO Integrated Navigation System (INS) performance standards (alerts being structured as alarms, warnings and cautions). Further consideration should be given to the use of a decision support system that offers suggested reactions to certain alerts, and the integration of navigation alerts onboard ships within a whole ship alert management system.

There is a need among mariners for standardised presentation and operation functionality to enhance the effectiveness of internationally standardised training, certification and familiarisation. The concept of S-Mode was supported for the ship.

Shore users expressed a clear need for their displays to be fully flexible supporting both a Common Operating Picture (COP) and a User Defined Operating Picture (UDOP). Strong support was given for the use of Geographical Information Systems (GIS) and for layered and/or tabulated displays.

Effective and Robust Ship and Shore communications: A clear need was expressed for there to be an effective and robust means of communications for ship and shore users. Relevant shore authorities need an effective means of communicating with vessels to provide safety and operational information with an automated communications selection system. Communication with vessels and between vessels should be effective, make best use of audio/visual aids and standard phrases, minimise linguistic challenges and distraction to operations.

Human Machine Interface: e-Navigation systems must be designed to engage and motivate the user while managing workload. It was clear that as electronic systems take on a greater role, facilities should be developed for the capture and presentation of non-electronic data such as visual observations and user knowledge and experience. The presentation of information for all users should be designed to reduce ‘single person errors’ and enhance team operations. On board ship this includes the entire navigation watch, including the lookout, as well as the Master and, when one is aboard, the pilot. There is a clear need for good ergonomics both in the physical layout of equipment and in the use of light, colours, symbology and language.

Data and System Integrity: A clear user need was expressed for e-Navigation systems to be resilient and to take into account issues of data validity, plausibility and integrity, and for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems, should be considered.

Analysis: There is a clear need for an effective use of analysis functions within e-Navigation to improve performance and prevent single person error.  Shipboard systems should include analysis functions that support the user in complying with regulations; identifying risks, and avoiding collisions and groundings including the calculation of real-time under keel clearance (UKC) and air drafts.  Shore based systems should support environmental impact analysis, operational planning, hazard/risk assessment, reporting indicators and prevention.  Consideration should also be given to the use of analysis for activities including: response recovery, risk assessment and planning, detection and prevention, mitigation, preparedness, resource management and communication.  e-Navigation analysis should be designed to complement users’ capabilities while compensating for any limitations.

- The IALA working group recognised that further feedback will be needed from other user groups and that an IMO strategy will have to address the development and implementation of e-Navigation in order to meet these stated needs.

- The IALA WG also compiled two ‘high level’ user needs analyses, one focusing on a generic SOLAS ship mariner’s needs and the other a generic Shore Authorities’ needs (including VTS, Coastal Surveillance, SAR, Counter Pollution, Port Authorities, and other maritime services).    From these two generic high level analyses, further and more specific user needs would be sought.  It was noted that at this particular stage, some major e-Navigation user needs were still to be addressed, such as Pilots, Leisure Craft, Fishing Vessels, Naval Ships and Ship-owners. It was also noted that the IALA WG was awaiting specific feedback on port VTS from the IALA VTS Committee.  Every effort will be made for these to be addressed prior to the submission of the CG report to Nav 54.

- The output from the WG will be examined at the 4th Session of IALA's e-NAV 4 Committee, scheduled to meet in Shanghai, China on 25-29 February 2008.  Thereafter, conclusions from e-NAV 4 will be submitted formally to the Correspondence Group for consideration and final submission to NAV54.

##### Action requested of the Sub-Committee

- There is a clear user requirement for common maritime information data structure, effective and robust ship and shore communications and data and system integrity - all of which fall within COMSAR sphere of expertise.  The Sub-Committee is invited to consider the following questions:

  - What existing international regulations and standards are relevant to the high level communications related user needs identified for e-Navigation in paragraph 15?

  - Of the existing international regulations and standards identified above, which would need to be addressed, or further developed, to provide a harmonised resilient system which offers high integrity for communications and data provision to satisfy the e-Navigation user needs?

  - What are the existing technical constraints and limitations, in terms if bandwidth, frequency and power consumption, which will need to be addressed to overcome any identified gaps to achieve the e-Navigation user needs?

  - How should communications and information systems be developed and co-ordinated internationally and within technical standards for data structure, technology, bandwidth and frequency allocations - including the steps that need to be taken?

  - What are the potential regulatory and technical problems that will need to be overcome considering that e-Navigation is to be scaleable across small and large vessels alike?

IALA paper: "Methodology for capturing e-Navigation user needs" ref: e-Nav-3 output-10