| IALA eNAV Liaison to ITU-R WP5B (2008) Fact Sheet |  |
| --- | --- |
| ITU-R Working Party: WP 5B | Document No: IALA eNAV XXX |
| References: - IALA Liaison Statement 8B/584-E, Annex 2 - Resolution 357 (WRC 07) - Report ITU-R M.2122 (2007) - Doc. 8/BL/43 (3 October 2007) - Footnote o) of Appendix 18 as modified by WRC-07 | Date: 15 January 2008 |
| Document Title: Preliminary Draft New Recommendation: “AIS Protection Criteria Related to the Maritime Mobile Service” |  |
| Purpose/Objective: IALA requested (liaison statement) that ITU-R WP8B develop “EMC assessment techniques” to evaluate new candidate technologies for Appendix 18. It also requested that these techniques be used to develop “AIS protection criteria.” In response, Report ITU-R M.2122 “EMC assessment of shore-based electronic navigation (eNAV) infrastructure and new draft Standards for data exchange in the VHF maritime mobile band (156-174 MHz)” was developed and approved (2007). It now provides the means for the development of the “AIS Protection Criteria.” |  |
| Abstract: This proposed Preliminary Draft New Recommendation should provide: - EMC assessment of the technologies currently used and those have been proposed for the transmission of voice, data, and AIS radiocommunications in the VHF maritime mobile band; - Protection criteria for the AIS based on this EMC assessment of AIS and the various other technologies that may be used by the services that share the VHF maritime mobile band. |  |

Justification for this Preliminary Draft New Recommendation

“AIS Protection Criteria Related to the Maritime Mobile Service”

- Discussion

The VHF maritime mobile was originally designed for voice radiocommunications, but there is now an increasing demand for data communications in the band. Data transmission was first introduced using DSC (digital selective calling), and channel 70 was designated exclusively for selective calling and distress calling using DSC. Due to its designation by IMO for the GMDSS (Global Maritime Distress and Safety Service), DSC is not available for data communications.

More recently, AIS was introduced as a navigation safety system, and two VHF channels were assigned and dedicated for AIS on the high seas. These AIS channels were originally taken from the upper legs of the duplex channels 87 and 88 that had been previously assigned to the VHF Public Correspondence (VPC) service. Channels 87B and 88B are now designated as AIS 1 and AIS 2, and the lower legs, 87A and 88A, are now re-designated as channels 87 and 88. Although AIS is a digital system, its use is exclusively dedicated by IMO for navigation safety, and it contains a limited set of defined navigation and safety-related messages for use by ships and shore stations. It should also be noted that WRC-07 added a secondary mobile-satellite service allocation on AIS-1 and AIS-2 frequencies to allow satellite reception of AIS on a secondary basis. Thus, AIS is not available for data communications, and there is a need for a VHF data exchange service other than the AIS.

In 2007, a new Recommendation for VHF data communication [Doc. 8/BL/43] was approved by ITU-R SG 8 for use on 25 kHz channels. A proposal to reorganize Ap 18 (Appendix 18 of the Radio Regulations) to provide a consolidated block of 9 contiguous channels in a 225 kHz block for a wideband data service was tabled pending a review of the consequential EMC (electro-magnetic compatibility) effects on the services in Ap 18. The new Recommendation addressed e-mail and data messaging for both in ship-shore and ship-ship data applications. The Recommendation also specifically referred to the channels designated with footnote o) in Ap 18 which are all duplex channels. A large ship-shore coverage range on multiple adjacent channels is possible because of the inherent invulnerability of duplex channels to adjacent channel interference.

EMC between voice and data can not apply the same emissions masks and adjacent channel performance requirements to both systems. But, the organization of Ap 18, e.g., both the 25 kHz channel spacing and the duplex/simplex channel groupings, must also be considered in order to provide the full coverage range required by the ship-shore services.

This proposed Draft new Recommendation uses Report ITU-R M.2122 (2007) for assessing EMC between voice, data and AIS systems. It examines of the use of AIS on the designated simplex channels AIS 1 and AIS 2 by shore stations that are in close geographical proximity to shore stations providing voice and/or data communications on interleaved adjacent duplex channels. Consequentially, protection criteria in the form of calculations based on the technical parameters of the AIS and the various other systems are proposed to protect the victim shore-based AIS service.

- Proposal

IALA requests that ITU-R provide a new Recommendation for “AIS Protection Criteria” that also serves a means for:

  - Assessment of new technologies and their EMC-compatibility with the AIS;

  - Integration of AIS with other technologies for data and voice radiocommunications in Appendix 18; and

  - Implementation of shore-based infrastructure to provide an appropriate electro-magnetic environment to support the EMC requirements of the AIS and the other distress and safety-related services in Appendix 18.

“AIS Protection Criteria Related to the Maritime Mobile Service”

# Introduction

This PDNR (Preliminary Draft New Recommendation) considers only the AIS protection criteria with respect to the maritime mobile service. It is recognized that the AIS may also be subject to adjacent channel interference from the LMR service, by sharing the maritime mobile frequencies and/or by the LMR frequencies immediately above the maritime mobile band. Consideration of AIS protection from the LMR is outside of the scope of this report. The VHF maritime mobile was originally designed for voice radiocommunications, but there is now an increasing demand for data communications in the band. Data transmission was first introduced using DSC (digital selective calling), and channel 70 was designated exclusively for selective calling and distress calling using DSC. Due to its designation by IMO for the GMDSS (Global Maritime Distress and Safety Service), DSC is not available for data communications.

More recently, AIS was introduced as a navigation safety system, and two VHF channels were assigned and dedicated for AIS on the high seas. These AIS channels were originally taken from the upper legs of the duplex channels 87 and 88 that had been previously assigned to the VHF Public Correspondence (VPC) service. Channels 87B and 88B are now designated as AIS 1 and AIS 2, and the lower legs, 87A and 88A, are now re-designated as channels 87 and 88. Although AIS is a digital system, its use is exclusively dedicated by IMO for navigation safety, and it contains a limited set of defined navigation and safety-related messages for use by ships and shore stations. Thus, AIS is not available for data communications.

In 2007, IALA requested (liaison statement) that ITU-R WP8B develop “EMC assessment techniques” to evaluate new candidate technologies for Appendix 18. It also requested that these techniques be used to develop “AIS protection criteria.” In response, Report ITU-R M.2122 “EMC assessment of shore-based electronic navigation (eNAV) infrastructure and new draft Standards for data exchange in the VHF maritime mobile band (156-174 MHz)” was developed and approved (2007).  This Report was approved for EMC assessment of the current systems and proposed new systems (voice, data and AIS) operating in Ap 18 (Appendix 18 of the Radio Regulations) and for the development of “AIS Protection Criteria.”

A new Recommendation for VHF data communication [Doc 8/BL/43] was also approved by ITU-R SG8 for use on 25 kHz channels. A proposal to reorganize Ap 18 to provide a consolidated block of 9 contiguous channels in a 225 kHz block for a wideband data service was tabled pending a review of the consequential EMC effects to be assessed by the new Report. The approved Recommendation addressed e-mail and data messaging for both in ship-shore and ship-ship data applications. The Recommendation also specifically referred to the channels designated with footnote o) in Ap 18 which are all duplex channels. A large ship-shore coverage range on multiple adjacent channels is possible because of the inherent invulnerability of duplex channels to adjacent channel interference.

Without the benefit of the new Report, it may seem logical to conclude that it would be sufficient to insure EMC between voice and data radiocommunications to apply the same emissions masks and adjacent channel performance requirements to both systems. Although the emissions masks are necessary, the organization of Ap 18, e.g., both the 25 kHz channel spacing and the duplex/simplex channel groupings, must also be considered in order to provide the full coverage range required by the ship-shore services.

This PDNR uses Report ITU-R M.2122 (2007) to further assess the EMC between voice, data and AIS systems. It examines of the use of AIS on the designated simplex channels AIS 1 and AIS 2 by shore stations that are in close geographical proximity to shore stations providing voice and/or data communications on Ap 18 channels 27 and 28 that are the interleaved adjacent duplex channels. It also considers the potential adverse effects to the ship-borne AIS from shore stations that are near navigable waterways. Consequentially, protection criteria are proposed to protect the both the shore-based AIS and the ship-borne AIS.

# Rationale for the EMC assessment of eNAV infrastructure

## Compatibility of various eNAV infrastructure on adjacent RR Appendix 18 channels

It is particularly important to address the possibility of interference from other maritime stations that use the channels adjacent to AIS for VPC radiocommunications, since this system could raise a constant CW carrier transmitter for the entire duration of the transmission. For example, VPC stations receive on the A-side of the duplex channels (e.g. channel 27A = 157.350 MHz, and channel 28A = 157.400 MHz) and transmit on the B-side (e.g. channel 27B = 161.950 MHz, and channel 28B = 162.000 MHz), while the AIS both transmits and receives on the B-side (AIS 1 = channel 87B = 161.975 MHz, and AIS 2 = channel 88B = 162.025 MHz). Because of the frequency separation (±0.025 MHz), these VPC shore transmitters could potentially interfere with AIS shore station receivers if they are located in close geographical proximity to each other, but the AIS transmitters have a large frequency separation (+4.625 MHz) from the VPC receivers and thus pose little interference threat to those stations. Likewise, the VPC mobile stations are not an interference threat to the AIS because they transmit 4.625 MHz below the AIS. Thus, this EMC analysis only needs to consider the interference from the VPC base stations to the AIS base stations.

## Compatibility of the transmitter emissions used for the eNAV

Transmitter emissions should be evaluated to assess the EMC between the shore-based eNAV:

•	Voice radiocommunications,

•	Data exchange, and

•	AIS.

Transmitter emissions for AIS base stations are now defined by International Standard IEC 62320-1 Ed.1. The IEC also has emissions mask requirements for radio transmitters operating in Appendix 18, and these requirements have been considered for the AIS. Presumably, voice radiocommunications also consider the IEC emissions mask requirements, but the Reference Example in Report ITU-R M.2122 for a 225 kHz wideband VHF data exchange system based on ETS300113-1v.1.5.1 does not consider the need to meet these requirements and thus will not be further evaluated.

# EMC reference example of shore-based AIS and VPC stations on interleaved adjacent channels

Assignment of channels for base stations considers that geographical separation is needed between uses of the same channel (channel reuse) and also the adjacent channel. This is because base stations are situated on towers and other high elevations to achieve coverage of the service area. For shore-based VHF marine radio, specifically the VPC, this tradition has been followed. There has always been avoidance of the use of adjacent VPC channels in the same area because of the potential for interference between the base stations, but now that the VPC and the AIS are separate services that share the same spectrum, there is a need to mitigate the potential interference, and an EMC analysis is needed to establish protection criteria for the AIS which is a critical navigation safety service.

The EMC analysis examples in this report evaluate VPC channels 27 and 28 which are interleaved and adjacent to AIS 1 and AIS 2. The second adjacent channel to AIS 1 is channel 86, and this channel could also be studied, but the first upper adjacent channel to AIS 2 is the frequency 162.050 MHz, which is in the LMR service (outside of Ap 18) and is far more critical. The main purpose of this report is to determine the minimum safe distance between AIS base stations and VPC base stations. In practice, such determinations should take into account the details of the particular situation being considered, as described in Section 5.  Separation distance determinations may vary when taking into account additional factors of the specific sharing situation or applying mitigation techniques.

## Channels for voice and data exchange

The interleaved adjacent channels within Appendix 18 present concerns to all maritime systems applications (voice, data exchange and AIS). A comprehensive EMC analysis based on the technical characteristics of the systems has not been performed within ITU-R and is required. There may be some question whether a digital VPC application could be impaired on the ship station side by the ship-borne AIS on the internationally-designated AIS channels. No suitable off-the-shelf solution has been identified that solves this interference problem, although technology is currently available to develop a solution.

Thus, it may seem logical to conclude that VPC voice communications could be introduced on channels 27 and 28 (the VPC channels adjacent to and interleaved with AIS) in order to provide an immediate commercial application that was invulnerable to AIS and to reserve the channels farther removed from the AIS (and therefore less vulnerable to degradation from the AIS) for the new digital application. This argument may be supported by the rationale that a multi-channel bandwidth may be needed for the data exchange application that would necessitate combining contiguous VPC channels that could not include those interleaved and adjacent to the AIS; however, it does not clearly address the potential EMC issues to the VHF maritime mobile, including the AIS.

An alternative approach using the high data rate provided in Annex 1 of the new Recommendation [Doc. 8/BL/43] could analyse the notion of having each channel independently available for any functional use, which would not require the upper interleaved VPC channels to be the only channels designated for voice radiocommunications in order to keep the lower VPC channels available as one contiguous block for the proposed new digital service. This could allow the voice services to be implemented on any of the channels currently available in Appendix 18 for VPC voice service thus alleviating the pressure to use channels 27 and 28.

  - EMC analysis of a typical shore station scenario for AIS and VPC channels 27 and 28

The following EMC example is based on an AIS receiver sensitivity of –107 dBm (in accordance with IEC 62320-1 Ed.1, the AIS base station standard), VPC transmitter emissions with an ACPR (adjacent channel power ratio) = 70 dB (referred to equipment standards for base stations) and an assumed base station antenna selection, a popular typical marine VHF base station antenna shown in Figure 1 for both the AIS and the VPC. The distance separation (Ds) between the two shore stations (the VPC voice station and the AIS station) is derived based on free-space propagation between the stations, assuming that the antennas are situated on high-sites. For lower antenna elevations, propagation models such as ITU-R P.525 and NTIA ITM (irregular terrain model) can be used to more accurately assess the separation distance.

Figure 1

Base Station Antenna

Typical high-gain antenna used for marine VHF shore stations

Note:

This antenna is typical of what is used on VHF base stations for VPC and AIS services. Sites may use smaller antennas with less gain and reduced coverage range.

    - Effective Incident Radiated Power (EIRP) for VPC systems

The effective incident radiated power (EIRP) of the VPC system at the authorized carrier frequency is +58dBm (631 Watts). In logarithmic terms, the transmitter power of +47dBm (+47dBm = 50Watts, the maximum level permitted for shore stations), plus the antenna gain of +11dBi = +58dBm.

    - Adjacent Channel EIRP (based on radio regulations and equipment standards)

The total EIRP from the VPC system is -9dBm on AIS1 (two equal contributions (+3dB) of -12dBm from VPC channel 27 and VPC channel 28, assuming that VPC channels 27 and 28 are active from two VPC stations and that an AIS station is positioned midway between the VPC stations to achieve the maximum possible distance separation) and -12dBm on AIS2 (-12dBm from VPC channel 28) based on IEC equipment standards. Power on the ± 25 kHz adjacent channels is -70dB with respect to the VPC carrier frequency (based on standards), although actual performance could possibly be better (refer to §3.4.4).

    - Effects of VPC Adjacent Channel EIRP on AIS Base Stations

The effects on the AIS base station receivers using AIS1 and AIS2 from the adjacent channel power EIRP from the VPC operations on channels 27 and 28 are as follows:

For AIS1, the VPC signal appears as two adjacent channel signals (one on each side, lower and upper) with sidebands that are co-channel at -70dBc each (-67dBc total with respect to each adjacent channel). For AIS2, only one VPC signal appears at -70dBc (on the lower side). The AIS receiver co-channel signal rejection ratio specified by the IEC standards is -10dB for a 20%PER (packet error rate). The minimum AIS receiver sensitivity specified by IEC is -107dBm, but since better performance is available, some Administrations are requiring -115dBm for increased coverage range. This is further evaluated in §3.3.

Thus, for an AIS base station to meet its specified minimum performance for AIS1, the maximum signal levels that appear on AIS 1 from the VPC channels 27 and 28 must be below -120dBm (13dB below -107dBm, two equal signals that are each 10dB below -107dBm for a 20%PER). For AIS 2, considering only one VPC adjacent channel (the other adjacent channel is not in the maritime mobile band), the level is -117 dBm (10 dB below -107 dBm for a 20% PER).

Since the AIS base stations are expected to cover the coastline as well as the offshore, the selection of the antenna would be typical for both the VPC and the AIS base stations. The reference level for AIS detection is at the AIS receiver input at the base of the antenna feed-line (allowing for an antenna gain of 11dBi and a feed-line loss factor of 2dB). The geographical distance separations (in free space between) between the VPC sites and the AIS base station sites can be calculated from the free space radio range equations as follows.

20 log Ds = Pt – Pr – 36.6 – 20 log F – ACPR + Gt + Gr – L, where

Ds = 		Distance separation (in free-space) between AIS and VPC sites in statute miles;

Pt = 		VPC transmitter carrier power at antenna base = +47dBm (50Watts);

Pr = 		power at AIS receiver input in dBm = -120dBm (for a 20% PER at -107dBm);

F = 		Frequency in MHz = 161.975 MHz (AIS1);

ACPR = 	adjacent channel power ratio of assumed VPC transmitter = 70dBc;

Gt = 		antenna gain of the assumed VPC antenna = 11dBi;

Gr = 		antenna gain of the assumed AIS base station antenna = 11dBi; and

L = 		feed-line loss of the assumed NAIS base station antenna feed-line = 2dB.

Then, 	20 log Ds = 47 + 120 – 36.6 – 44.18 – 70 + 11 + 11 – 2 = 36.22 and

Ds = 		10((36.22)/20) = 64.7 miles.

Refer to Section 3.3 for a scenario using lower gain antennas and broadside pattern (off-axis antenna discrimination) that reduces this range and better receiver sensitivity that increases it. This second example follows in 3.3. The radio horizon (the limit for free-space propagation) should also be considered. When the free-space distance calculation is near or beyond the radio horizon, computerized earth propagation models should be used to more precisely determine this separation as shown in 3.2.4 below. A radio horizon calculation for this example follows:

Dh = 		radio horizon distance in statute miles = √ (3KH1/2) + √ (3KH2/2), where

K = 		diffraction factor of the earth’s atmosphere = 1.33 (the conventional “4/3 earth model”),

H1, H2 = 	Antenna elevations (ft), H1 (VPC) = 150m = 492ft and H2 (AIS) = 200m = 656ft,

Dh = 		31.3 + 36.18 = 67.48 miles.

    - Propagation Model Simulations of the EMC Effects

Computerized propagation models ITU-R P.525 and NTIA ITM (irregular terrain model) were used to more accurately predict the distance separations using the parameters from §3.2.3. The results shown in Figure 2 differ slightly (57.1 miles and 47.8 miles) because of the different treatment of geographic and atmospheric variables (the K-factor), but they both confirm that large geographical separations (approximately 50 miles) are necessary to satisfy the this EMC scenario.

Figure 2

Sample Propagation Model Simulation

Sea water area

VPC

Land Area

| Color | Propagation model | Separation distance between AIS and VPC stations |
| --- | --- | --- |
| Yellow | NTIA ITM | 47.8 miles |
| Blue | ITU-R P.525 | 57.1 miles |

Simulation Model Input Data (using the set of technical parameters in §3.2.3)

- E.I.R.P(ACPR): -12 dBm, RF power on AIS1 (161.975 MHz) adjacent to VPC Ch 27

- Frequency: VPC Ch 27 (161.950 MHz)

- Received Power at NAIS base station: -120 dBm (for a 20 % PER at -107 dBm)

- TX VPC antenna height: 150m (from one example VPC site)

- RX AIS antenna height: 200m (from one example AIS receiving site)

  - EMC analysis for the general case for shore stations for AIS and VPC channels 27 and 28

The “typical scenario” in the EMC analysis of 3.2 prescribes distance separations of 47.8 miles to 57.1 miles (refer to 3.2.4) between AIS base stations and VPC base stations using channels 27 and 28. As was noted, the technical analysis (for the baseline calculation in 3.2.3) could be more generalized and the parameters adjusted for different types of antennas and improved AIS receiver performance to re-assess the necessary distance separation. For example, lower gain antennas (for both the VPC and the AIS), lower CCRR (for the AIS) and higher ACPR (for the VPC) would reduce the distance separation and better AIS receiver sensitivity would increase it. Obviously, coverage range for the AIS and the VPC makes higher antenna gain very desirable, and if lower gain antennas are used (refer to Figure 3), then improved receiver sensitivity (e.g., -115dBm vs. -107dBm) is needed to increase the coverage range.

Figure 3

VHF marine base station antenna

Typical low-gain antenna used for shore stations with limited space and reduced coverage range

For the antenna in Figure 3,

20 log Ds = Pt – Pr – 36.6 – 20 log F − ACPR + 10 log N + NA + CCRR + Gr – L + Gt, where:

Ds = 		distance separation between AIS and VPC voice stations in land (statute) miles

Pt = 		VPC transmitter carrier power (50 Watts at antenna base) = +47 dBm;

Pr = 		AIS minimum receiver sensitivity (for a 20% PER) = −107 dBm

20 log F = 	44.18 dB, where F = Frequency in MHz = 161.975 MHz (AIS 1)

ACPR = 	adjacent channel power ratio of VPC transmitters, channels 27 and 28 = 70 dB

10 log N = 	10 log 2 (N = number of VPC adjacent channels = 2 for AIS1) = 3 dB

NA = 		noise level allowance = 3 dB

CCRR = 	AIS receiver co-channel rejection ratio = 10 dB

Gr = 		gain of the AIS base station antenna (at broadside) = 6.5 dBi

L =		total loss of feed-line, connectors, lightning arrestors and filters = 4 dB, and

Gt = 		gain of the VPC antenna (at broadside) = 6.5 dBi.

Thus, for 	Pr =	–107 dBm,

20 log Ds = 47 – (–107) – 36.6 – 44.18 – 70 + 3 + 3 + 10 + 6.5 – 4 + 6.5 = 28.22 dB

Ds =		10(28.22/20) = 25.8 miles

Since AIS base stations that have been certified to IEC 62320-1 Ed.1 exhibit receiver sensitivity of –115 dBm or better, Administrations have begun to specify this level in order to achieve the required minimum coverage range for AIS shore stations. If this receiver sensitivity value is used with the low gain antennas, the required distance separation is raised. Note that this may actually be beyond the free-space propagation range, depending on antenna height, but the distance is still very large (refer to 3.2.4).

Then, for 	Pr =	–115 dBm,

20 log Ds = 47 – (–115) – 36.6 – 44.18 – 70 + 3 + 3 + 10 + 6.5 – 4 + 6.5 = 36.22 dB

Ds = 		10(36.22/20) = 64.7 miles

But it is also necessary to include the high-gain antenna of Figure 1. The gain of this antenna at broadside (the assumed nominal direction of the other VPC or AIS shore station) is about 6 dBd (8 dBi).

For the antenna of Figure 1,

For 		Pr =	–107 dBm,

20 log Ds = 47 – (–107) – 36.6 – 44.18 – 70 + 3 + 3 + 10 + 8 – 4 + 8 = 31.22 dB

Ds =		10(31.22/20) = 36.4 miles

And for		Pr =	–115 dBm,

20 log Ds = 47 – (–115) – 36.6 – 44.18 – 70 + 3 + 3 + 10 + 8 – 4 + 8 = 39.22 dB

Ds = 		10(39.22/20) = 91.4 miles

Table 1 shows the results for both antenna types and both levels of AIS receiver sensitivity.

Table 1

Distance separation between AIS base stations and VPC stations on adjacent Ap 18 channels as a function of AIS receiver sensitivity and antenna type

| AIS base station receiver sensitivity | Antenna type | VPC voice channels | Calculated distance to AIS base station |
| --- | --- | --- | --- |
| –107 dBm (minimum standard) | Low-gain antenna (Figure 3) | 27 & 28 | 25.8 miles |
| –115 dBm (improved) | Low-gain antenna (Figure 3) | 27 & 28 | 64.7 miles |
| –107 dBm (minimum standard) | High-gain antenna (Figure 1) | 27 & 28 | 36.4 miles |
| –115 dBm (improved) | High-gain antenna (Figure 1) | 27 & 28 | 91.4 miles |

  - VPC transmitter emissions spectrum issues

The EMC analysis in 3.3 assumes that the ACPR (adjacent channel power ratio) performance of the VPC transmitter is 70 dB or better. This level is specified for the transmission of data in accordance with the new Recommendation for VHF data communication [Doc. 8/BL/43], and the relationship between ACPR performance, transmitter emissions spectrum and channel mask is illustrated in Figures 4, 5 and 6 that are taken from Annex 1.

# Transmitter emissions mask requirements

Some Administrations allow maritime VPC channels to be shared with the LMR (land-mobile radio) services. This raises potential EMC concerns for the use of the channels 27 and 28 that are the interleaved adjacent channels to the AIS. For example, ETSI has implemented a standard for radio data transmission for the Trans-European Trunked Radio Association (TETRA) in the land mobile radio on 25 kHz channels. The standard includes two modulation types for use on 25 kHz channels, π/4 DQPSK at 36 kbps and π/8 D8PSK at 54 kbps. The higher data rate (54 kbps) standard is the most recent one, and it is used for TEDS (TETRA Enhanced Data Service). This is proven technology, and both base station and mobile equipment is currently in widespread use. RTCM (the Radio Technical Commission for Maritime Services) has formed a Special Committee (RTCM SC 123) to assess this and other technologies for use in the VHF marine band (Appendix 18) for a prospective new VSMS (VHF Small Messaging Service). The following figures illustrate RTCM SC 123 test results of the TETRA-TEDS transmission methods. Figure 4 presents the spectra for TETRA modulation at the normal 36/54 kbps data rates, along with the IEC mask for Appendix 18. It is apparent that these data rates slightly fail to meet the mask limit of –25 dBc at ±10 kHz offset from the carrier.

Figure 4

Transmitter emissions mask

RTCM SC123 test results: TETRA modulation at 36/54 kbps

Somewhat lower data rates (32/48.8 kbps and 28.8/43.2 kbps) were then tested. Figure 5 overlays these test results with those of Figure 4. It is evident that both 28.8 kbps π/4-DQPSK modulation and 43.2 kbps π/8-D8PSK modulation comfortably fit the IEC mask for Appendix 18. This modulation method is now approved for maritime use in Annex 1 of the new Recommendation.

Figure 5

Transmitter emissions mask

RTCM test results: Slightly reduced data rates to fit IEC emissions mask

    - Adjacent channel power ratio (ACPR) requirements

Another important consideration in addition to the transmitter emissions mask is the adjacent channel power. This determines the net amount of power received by separate systems on adjacent 25 kHz channels. The Adjacent Channel Power (a.k.a. Adjacent Channel Power Ratio, ACPR) was measured for the 28.8 kbps π/4-DQPSK and 43.2 kbps π/8-D8PSK modulations using the ETSI EN 300 113 16 kHz bandwidth criterion. The results are shown in Figure 6, which indicates compliance with the IEC maritime requirement of –70 dB ACPR for operation in Appendix 18.

Figure 6

Adjacent channel power ratio (ACPR) performance

RTCM test results: 28.8 kbps π/4-DQPSK and 43.2 kbps π/8-D8PSK modulation

The conclusions of RTCM SC123 are:

1)	ETSI TETRA modulations at 28.8 kbps π/4-DQPSK and 43.2 kbps π/8-D8PSK are efficient methods for transmitting data in the marine VHF band, and

2)	These methods are compatible with the 25 kHz channels in Appendix 18.

# Narrowband channel implementations by VPC stations

As stated in §3.4.1, some Administrations allow maritime VPC channels to be shared with the LMR (land-mobile radio) services. The LMR also implements narrow-banding to 12.5 kHz and/or 6.25 kHz channels in some Administrations. Although Recommendation ITU-R M.1084-4 tabulates these channels, it also shows  that significant geographical separations are needed between these channels and the adjacent 25 kHz AP 18 channels due to the overlapping channel masks. Narrow-banding may reduce the adjacent channel interference if it reduces the ACPR on AIS 1/2. These effects are assessed in §3.5.

    - VPC voice radiocommunications transmitter emissions

Voice radio-telephony is the legacy VPC service on the VPC-designated channels. The transmitter emissions mask specified by IEC shown in Figure 5 should be required as the minimum standard for the VPC channels for both data and voice transmissions. Some advances in transmitter technology, e.g. DSP-based Cartesian vector feedback, provide significantly improved VPC transmitter emissions such as that shown in Figure 7. The EMC effects of this prospective improvement are shown in §3.5.

Figure 7

VPC linear FM voice transmitter emissions spectrum

Using DSP-based Cartesian vector feedback

+43.2 dBm unmodulated carrier 0 dBc reference level.

+ 25 kHz adjacent channel centre

–80 dBc ACPR (assumed typical) level based on projection of adjacent channel emissions

Projection of adjacent channel emissions

+ 25 kHz adjacent channel bandwidth (±10 KHz) at –70 dBc ACPR specified level

  - Expanded EMC analysis to include optimized VPC and AIS base station performance

Although Administrations have acquired AIS receivers with a receiver sensitivity of –121 dBm, AIS base stations have additional receiver requirements that limit the receiver sensitivity to -115 dBm. The following EMC analysis example considers using the best available AIS base station receiver sensitivity, the best available VPC transmitter technology, and the optimization of the antenna selection and placement to provide maximum coverage range and inter-system isolation. Free-space distance calculations are used, but computer propagation model simulations, e.g. ITU-R P.525 and NTIA ITM, can be applied to further refine these calculations and to adjust to irregular terrain. The AIS receiver has a sensitivity of –115 dBm and co-channel rejection ratio of 10 dB. In the co-channel performance test, a 3 dB allowance for noise is made such that the interfering co-channel signal and the noise are assumed to be at equal levels. Thus, to insure the AIS sensitivity of –115 dBm, the maximum tolerable co-channel interference from the VPC transmitter adjacent channel power is (–115 dBm) –3 dB –10 dB = –128 dBm. The AIS receiver adjacent channel rejection ratio (ACRR) is 70 dB, and this is referenced to a PER (packet error rate) of 20% which is the PER specified for receiver sensitivity. If we take the same 3 dB allowance for noise in this case, then the maximum allowable VPC transmitter power (on the VPC adjacent channel frequency) received by the AIS site is (–115 dBm) –3 dB + 70 dB = –48 dBm. Note that these levels are 80 dB apart and that these are two separate candidate contributors to the degradation of AIS sensitivity performance. If both ACPR and ACRR were at the 70 dB specification thresholds set by the IEC standards, then the predominant contributor of these two independent contributions would be the ACPR of the VPC transmitter. However, if the ACPR of the VPC transmitter were 80 dB (referring to Figure 7), then the two contributions would be equal and the EMC analysis could be made based on either parameter (ACPR or ACRR) with an additional 3 dB allowance for the other parameter. For example, the ACRR case analysis which indicates a maximum level of –48 dBm would be decreased to –51 dBm to account for both of these effects. An EMC analysis (in free space) based on the AIS receiver sensitivity of –115 dBm, VPC transmitter selected with ACPR = 80 dB and the AIS base station antenna of Figure 1 is as follows:

20 log D = Pt – Ps – 36.6 – 20 log F − ACPR + 10 log N + NA + CCRR + Gr – L + Gt

D = 	distance separation between AIS and VPC stations in land (statute) miles

Pt =	 VPC transmitter carrier power (50 Watts at antenna base) = +47 dBm

Ps =	 AIS minimum receiver sensitivity specification (for a 20% PER) = –121 dBm

20 log F = 	44.18 dB, where F = Frequency in MHz = 161.975 MHz (AIS1)

ACPR = 	adjacent channel power ratio of the VPC transmitter = 80 dB

10 log N =	 10 log 2 (N = number of VPC adjacent channels = 2 for AIS1) = 3 dB

NA =	 noise level allowance = 3 dB

CCRR = 	AIS receiver co-channel rejection ratio = 10 dB

Gr = 	gain of the AIS antenna in the VPC direction (broadside) = 8 dBi

L = 	loss of AIS feed-line, connectors, lightning arrestor and filters = 4 dB, and

Gt =	 gain of the VPC antenna in the AIS direction (at broadside) = 8 dBi.

Thus, for ACPR = 80 dB,

20 log D = 47 – (–115) – 36.6 – 44.18 – 80 + 3 + 3 + 10 + 8 – 4 + 8 = 29.22 dB

D = 10(29.22/20) = 28.9 miles

Table 1 of §3.3 can now be modified to include VPC transmitter ACPR as shown in Table 2 below.

Table 2

Distance separation between AIS base stations and VPC stations on adjacent Ap 18 channels as a function of AIS receiver sensitivity, antenna type and VPC transmitter ACPR

| AIS base station receiver sensitivity | VPC transmitter ACPR performance | Antenna type | VPC voice channels | Distance to AIS base station |
| --- | --- | --- | --- | --- |
| –107 dBm (minimum standard) | -70 dBc | Low-gain antenna (Figure 3) | 27 & 28 | 25.8 miles |
| –115 dBm (improved) | -70 dBc | Low-gain antenna (Figure 3) | 27 & 28 | 64.7 miles |
| –107 dBm (minimum standard) | -70 dBc | High-gain antenna (Figure 1) | 27 & 28 | 36.4 miles |
| –115 dBm (improved) | -70 dBc | High-gain antenna (Figure 1) | 27 & 28 | 91.4 miles |
| –115 dBm (improved) | -80 dBc | High-gain antenna (Figure 1) | 27 & 28 | 28.9 miles |

- Conclusions from the EMC analysis of the shore-based VPC and AIS services

The EMC analysis examples in this report evaluate VPC channels 27 and 28 which are interleaved and adjacent to AIS 1 and AIS 2. The second adjacent channel to AIS 1 is channel 86, and this channel could also be studied further, but the first upper adjacent channel to AIS 2 is the frequency 162.050 MHz, which is in the LMR service (outside of Ap 18) and is far more critical, but is outside of the scope of this report. The main purpose of this report is to determine by calculation the minimum safe distance between AIS base stations and VPC base stations. The distance separations in Table 2 are determined to protect the shore-based AIS service from interference from the VPC service operating on channels 27 and 28. Note that these separations depend on the levels of AIS and VPC performance and the antennas used for these services. In practice, such calculations should be performed taking into account the details of the particular situation being considered.  Separation distances may be reduced or increased by taking into account additional factors of the specific sharing situation or applying mitigation techniques. The distance separation in the geographical area of interest also depends on the terrain and the antenna elevations ASL (above sea level), and the appropriate propagation models can be used to consider these factors as shown in Figure 2 of §3.2.4.

# EMC assessment of the eNAV and advanced technologies

In developing an “eNAV systems and technologies assessment,” Administrations should consider information such as that shown below describing the base station locations:

•	Antenna tower address, latitude and longitude

•	Antenna height above ground and above sea level

•	Antenna make and model

•	Antenna gain, radiation pattern and EIRP

•	Antenna orientation

•	Transmitter characteristics (applicable standards)

•	Transmitter make and model

•	Transmitter output power

•	Transmitter emissions spectrum

•	Any equipment/operations to limit interference

This information should be processed (in an EMC analysis similar to that provided in this report) to determine if the EMC scenario is likely to cause interference. If there is a probability of interference, then recommendations should be made (based on the EMC analysis) on how to minimize/alleviate the interference.

## EMC assessment of area-based eNAV infrastructure

Assessment of eNAV infrastructure should also consider that the VPC and/or the AIS may have administration-wide authorization. In that case, the EMC assessment should consider the different technical characteristics of the two systems, e.g. simplex packet data operation (as in the AIS service) vs. duplex FM-CW voice operation (as in the VPC voice radiotelephony services). Technical details of the radio systems should include receiver sensitivity, adjacent channel power ratio, adjacent channel rejection ratio, co-channel rejection ratio, any data packet error correcting scheme (e.g. interleaving/FEC (forward error correcting)) and the coverage requirements of the VPC and AIS base stations.

This is a good example showing the need for an eNAV infrastructure EMC assessment because these assessments are customarily designed around coverage areas with defined coverage contours where base station transmitter signal field strength levels are significantly above the receiver sensitivity thresholds of the mobile receiving equipment. For this technical assessment, all of the technical characteristics of the VPC transmitters and the VPC sites along with the receivers and the sites must be considered to achieve acceptable EMC levels.

## Technical possibilities for interference mitigation

This notional example EMC analysis illustrates that operating VPC voice radiocommunications on channels 27B and 28B with AIS1 and AIS2 may be difficult under certain scenarios. Further study into mitigation techniques, such as signal discrimination or antenna pattern discrimination, is needed.

## The benefit of automatic continuous interference detection capability in the AIS

Some AIS receivers have the inherent capability to detect and record the RSSI (received signal strength indication) levels for each AIS time period (typically referred to as a time slot) on both AIS frequencies and to report the value of the RSSI level on the serial input/output port. This inherent capability needs to be further exploited by the development and integration of special software that can be installed in the AIS receivers and the connected on-site AIS computers that can continuously monitor the “background noise level” on the AIS VDL (VHF data link) on the AIS-designated frequencies AIS1 and AIS2. For this proposed special capability, in a time period when no signal is present, the RSSI level reported by the AIS receiving system should be approximately NF + KTB + I, where NF is the noise figure of the AIS receiver, KTB is the value of thermal noise in the AIS receiver bandwidth and I is the value of on-channel interference received on the AIS frequency.

This proposed special software also has the benefit of determining AIS VDL loading and detecting slot reuse. Base stations otherwise see “reused” slots as unused due to the fact that multiple simultaneous AIS messages can not be detected by base stations whenever (which is usually the case) the difference in received signals is not much higher than 10 dB, the AIS co-channel signal detection minimum threshold. While VDL management may be beyond the scope of administrations’ requirements, this capability is urgently needed to enable this proposed maritime eNAV systems EMC assessment.

eNAV – “E-Navigation is the harmonised creation, collection, integration, exchange and presentation of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”.