German Federal Waterways and Shipping Administration,

Waterways and Shipping Directorates North and Northwest

Traffic Technology and Telematics Division

c/o Hindenburgufer 247

24106 Kiel

Germany

Input document to the 4th IALA eNAV Committee meeting, February 2008, Shanghai

Contribution to the IALA World Wide Radio Navigation plan (IALA-WWRNP) / Recapitalization of MF DGNSS Systems

- Or: A holistic vision for terrestrial maritime GNSS augmentation and backup compatible with the emerging e-Navigation concept

Main Part (Part I)

# Executive Summary

This document presents a holistic vision for world-wide terrestrial maritime GNSS augmentation and backup. This document contributes to the IALA Council requested IALA World-Wide Radio Navigation Plan (IALA WWRNP) and to its accompanying IALA radio navigation strategy.

The starting point are the following questions:

- Which terrestrial augmentation and backup radio navigation systems are already today truly globally distributed and may thus be considered globally relevant? (This is the global distribution aspect.)

- How can these systems be brought to a new plane of usefulness (added-value) by only a relatively small or moderate investment on the part of the operating administration(s)?  (This is the recapitalization aspect.)

The document identifies the IALA MF DGNSS system and the AIS Service as the prime candidates. Both systems already presently allow for the transmission of DGNSS data (so-called D-Mode), i.e. for terrestrial GNSS augmentation. Certain amendments with a moderate investment of the above systems (so-called Ranging-Mode or R-Mode) are suggested, which would allow for an additional world-wide terrestrial backup in coastal waters.

Also, both systems allow for transparent transmission of other data handed over to them (so-called T-Mode). The maximization of available MF radio link capacity for the T-Mode would allow for a data broadcast avenue shore-to-ship, which could carry some of the increasing demand for data transmission within the e-Navigation concept.

The degree to which the vision is implemented is scalable. Also, certain elements of the vision can be implemented short-term (such as the shore-based re-engineering of the IALA MF DGNSS system), while other elements would need a medium- or long-term perspective (such as the global implementation of an amended shipboard radio navigation  receiver, which would be required for the implementation of the Ranging-Mode).

# Table of contents

Executive Summary	1

Table of contents	2

1	Goals of this document	2

2	Background	2

3	Candidate systems for the IALA WWRNP	3

4	Harmonisation with overarching e-Navigation architecture	5

5	The methodology of the D-Mode, T-Mode, and R-Mode	7

6	Added value for the shore-side by re-engineering of the IALA MF DGNSS system	9

7	The vision of the R-Mode for MF and for AIS	11

8	Implications for an integrated shipboard radio navigation augmentation and backup receiver	11

9	Conclusions	12

# Goals of this document

This document presents a holistic vision for world-wide terrestrial maritime GNSS augmentation and backup. It is compatible with and supportive of the emerging e-Navigation concept. This document contributes to the IALA Council requested IALA World-Wide Radio Navigation Plan (IALA WWRNP) and to its accompanying IALA radio navigation strategy.

This vision was developed primarily from the point of view of a National Member of IALA, i.e. a coastal administration, who presently operates an IALA Medium Frequency DGNSS augmentation system, an emerging AIS Service, and a terrestrial GNSS backup station (Loran/Eurofix). Therefore,

- the questions related to the recapitalization of MF DGNSS systems (IALA Rec. R-135),

- the questions related to the yet unexplored full potential of the AIS Service regarding radio navigation (IALA Rec. A-124 presently under revision), and

- the questions related to the future of the Loran after the demise of the NELS

were the initial starting points for the considerations presented herein.

However, the vision – in order to be truly holistic – needs to take into account the shipboard equipment and the different radio links between shore and ships. Hence, some proposal regarding shipboard equipment is included.

Also, the vision takes into account the emerging insights and knowledge regarding the shore-based e-Navigation system architecture.

It also presents a comprehensive and coherent methodology to facilitate discussions on a functional level. The proposed methodology provides for a seamless usage of the terrestrial radio navigation systems within an emerging e-Navigation communications framework.

# Background

The following facts and concurrent developments contributed largely to the holistic vision presented herein:

- IMO/NAV53 (July 2007) recognized the need for a backup radio navigation system due to the vulnerabilities of the GNSS. IMO did not prescribe a specific backup system, however:

„The Sub-Committee considered the information provided by the United Kingdom (…) and IALA (…) on the need to provide a back-up to the Global Navigation Satellite System (GNSS) because of the vulnerabilities of GNSS. The Sub-Committee agreed that there was a need to provide an internationally agreed alternative system for complementing the existing satellite navigation, positioning and timing services to support e-navigation and recognized that potential back up systems could be made available and that it was premature to identify any specific system before the users’ requirements for e-navigation had been finalized.” (report NAV53, §13.24)

- IALA Council decided, that IALA e-Navigation Committee should prepare a World Wide Radio Navigation Plan (WWRNP) to satisfy the IMO requirements, while resolving pending re-capitalization questions of many IALA’s National Members. It follows, that IALA would need to create an IALA radio navigation strategy to implement that plan.

- The IALA e-NAV Committee prepared a draft initiative to request a mandatory carriage requirement for a shipboard MF DGNSS receiver (compare e-NAV3-WP6).

- An IALA invited meeting on the present world-wide status of the Loran, Haugesund, September 2007, produced the following insights:

- From a global perspective, the present Loran coverage is confined to certain countries of the Northern hemisphere.

- It will require considerable effort in terms of money and labour on the part of those countries to even maintain that coverage.

Also, the concept of the eLoran was introduced in broad terms. The indication was given, that eLoran might be one backup radio navigation system as requested by IMO.

- The VTS Committee has requested (VTS25-output-21 = eNAV3-8-2; second bullet), that relevant GNSS status measurement should be made available to shore-based VTS operators.

- IALA has developed an e-ANSI draft concept, which aims at providing the mariner with status information on Aids-to-Navigation (Notices to Mariners), amongst others, by state-of-the-art electronic means. This work item is now part of the current work programme of the e-NAV Committee.

- Within the e-NAV Committee considerations are taking shape regarding a common shore-based e-Nav system architecture. A special Working Group of the e-NAV Committee is intended to deal with these issues, the so-called Architecture TWG.

- Another special WG is intended to deal with technical solutions to satisfy the increased demand for digital voice and data communications between ships and shore, the so-called Comms TWG.

# Candidate systems for the IALA WWRNP

It is IALA’s goal to develop a WWRNP. It is intended to contribute to IMO’s World Wide Radio Navigation System (WWRS).

The global scope should be the starting point for any consideration of candidate systems:

- Only the GNSS has ocean-wide coverage, and only the GNSS is truly global in the strict meaning of the word. Therefore, GNSS rightly is the basis of IMO’s WWRS, beyond debate. However, it is also beyond debate, that GNSS has an inherent vulnerability (refer to IALA Rec. R-129), and that it should not be relied on exclusively.

- It is in coastal waters where requirements regarding position fixing quality parameters like accuracy, integrity, reliability, and continuity increase considerably. There terrestrial augmentation and terrestrial backup become pressing needs. To provide terrestrial augmentation and terrestrial backup in coastal waters is a major mitigation for the vulnerabilities of the GNSS.

- Therefore, IALA’s National Members started operating such terrestrial augmentation and terrestrial backup systems already in the past. IALA’s international co-ordination of the implementation of these systems proved successful to an extent, that there are now such systems in place practically on a global scale.

- Hence, the following question emerges:

- Which terrestrial augmentation and backup radio navigation systems are already today truly globally distributed and may thus be considered globally relevant? (This is the global distribution aspect.)

- How can these systems be brought to a new plane of usefulness (added-value) by only a relatively small or moderate investment on the part of the operating administration(s)?  (This is the recapitalization aspect.)

Figure 1 shows an assessment of the candidate radio navigation systems.

Figure 1: Assessment of radio navigation systems regarding distribution and range

Regarding Figure 1 the following should be noted:

- The IALA MF DGNSS system is nowadays practically globally available in coastal waters: The IALA master list of reference stations indicates more than 400 stations world-wide. The radio link of this system is internationally defined at ITU (Recommendation ITU-R M.823). IALA is considered the custodian of this definition by ITU. The IALA MF DGNSS system provides today DGNSS correction transmissions as well as a data channel for text messages (ASCII). There is margin to increase the data throughput for that data channel in a legacy-receiver friendly way.

- The AIS was not designed as a radio navigation system in the first place. However, it has the built-in capability to distribute DGNSS correction data (Recommendation ITU-R M.1371; msg 17), besides its many other data transmission capabilities (position reports, safety related messages, application specific data, etc). AIS shore infrastructure is presently distributed globally – even wider than the IALA MF DGNSS system. It takes a relatively small investment to connect a shore-based AIS Service to a DGNSS correction source to transmit DGNSS correction data.

- Loran was designed as a terrestrial radio navigation system. Therefore, it provides a terrestrial backup functionality for GNSS. There exists Recommendation ITU-R M.589, which also specifies additional DGNSS correction transmission and general data transmission capabilities (Annex 2 of Rec. ITU-R M.589). There is an international initiative under way to improve these three capabilities, hence creating an eLoran system. While the (e)Loran could be deployed globally in principle, its present distribution is confined to the Northern hemisphere. To expand coverage would require a larger investment and also a large effort to overcome public opposition. This is mainly due to the specific requirements of this long-wave system regarding antenna towers. Considering its present distribution from a global point of view, (e)Loran can only be considered a regional system at present.

Concluding, at present the two truly globally distributed systems, the IALA MF DGNSS system and the AIS Services should be considered the prime candidates for any IALA WWRNP. Both systems presently allow for DGNSS correction transmissions, thus constituting augmentation systems. Both systems allow for transmission of other data, too. Finally, both systems also have the potential for being global  terrestrial backup systems. (e)Loran will be considered, too, having in mind its present limitations regarding global distribution.

# Harmonisation with overarching e-Navigation architecture

Any IALA WWRNP must take into account the emerging overarching e-Navigation architecture. The overarching e-Navigation architecture consists of three major parts:

- shipboard equipment and applications for mariners (including ECS/ECDIS usage);

- shore-based equipment and applications for shore-based operators (i.e. shore infrastructure);

- relevant radio links, which connect ships and shore.

The IALA WWRNP will affect primarily the shore-based infrastructure and its operation. Due to the co-operative nature of the e-Navigation concept, the radio links used and the shipboard equipment need to be part of the scope. Figure 2 (overleaf) shows the engineering representation of this holistic vision for terrestrial maritime GNSS augmentation and backup compatible with the emerging e-Navigation concept. Figure 2 will be explained below.

Figure 2: Complete engineering representation of the holistic vision for terrestrial maritime GNSS augmentation and backup, supportive of the emerging e-Navigation concept

The lower part of Figure 2 shows the envisaged shore part; here the relevant parts of the emerging common shore-based e-Navigation system architecture are shown. The middle part of Figure 2 shows the relevant radio links in a functional way, i.e. functional connections between shore and ship are indicated. The upper part of Figure 2 shows the relevant shipboard equipment setup, using both present and envisaged elements.

New elements necessary for a future, fully developed vision are indicated by grey shading. Existing elements, which may be used as available presently or with little amendments, are not highlighted. The new elements necessary for a future and fully developed vision are the following:

- Re-engineering of the present IALA MF DGNSS system in accordance with the emerging shore-based e-Navigation system architecture. This would result in the MF Transmission Service and the DGNSS Correction Service;

- interface to shore-based users, e. g. VTS operators; by User Interaction Service;

- Precise timing sources to provide independent and precise timing throughout;

- Connections to internal and/or external data sources to feed the T-Modes of the available radio links;

- Ranging mode (R-Mode) for MF Transmission Service;

- Ranging mode (R-Mode) for AIS Service;

- Shipboard terrestrial augmentation and backup radio receiver for the D-, R-, and T-Modes of all three radio links under consideration;

- Implementation of a shore based Long Wave Transmission Service (eLoran ).

Please note the following:

- The potential of the vision described herein can still be exploited (although to a lesser degree), if only some of the above new elements are introduced.

- There are several options for combination of existing and new elements.

- The AIS mobile stations remains untouched throughout and that their operation will not be compromised by this proposal in any way.

# The methodology of the D-Mode, T-Mode, and R-Mode

One prominent feature of Figure 2 is the functional designator D-Mode, T-Mode, and R-Mode for the distinct functionalities of the three radio links under consideration.

- The distinct functionality of the Differential-Mode is broadcast of DGNSS correction data in accordance with the DGNSS-specific timing requirements.

- The distinct functionality of the Transparent-Mode is the broadcast of other data shore to ship. Examples of that data are e. g. safety-related information, real-time measurement data of the fairway conditions, or status information regarding Aids-to-Navigation as developed in IALA’s e-ANSI concept.

- The distinct functionality of the Ranging-Mode is the provision of Time-Of-Emission (TOE) broadcasts from shore to ship. The shipboard radio receiver may then determine the Time-Of-Arrival (TOA) of the received signals. It can thereby calculate a distance (range) to the transmitter, the absolute position of which should be known. Using several such calculations from a number of different transmitters simultaneously, the shipboard equipment may thus determine its own position. This is called the “All-In-View” method. State-of-the-art digital receiver technology is capable of doing this kind of sophisticated algorithms in real time.

The use of the above functional designator D-, T-, and R-Mode proved useful especially in the context of the e-Navigation concept. It allows for

- a functional and more abstract description of the capabilities of the systems under consideration in the context of the e-Navigation concept.

- expert discussions of the different modes independent of each other to a certain degree, even if the same radio link is used. Thus it is potentially facilitating the parallel work of different teams of experts;

Hence, the concrete sciences of the different modes are encapsulated in the above expert discussions while the e-Navigation application designers may employ the more functional and abstract descriptions to simplify their lives.

Mapping the present functionalities and future potential to the D-Mode, R-Mode and T-Mode renders Table 1.

Table 1: Mapping of present functionalities and future potential to functional modes

| System | D-Mode | T-Mode | R-Mode |
| --- | --- | --- | --- |
| Medium Frequency Transmissions Service | Presently available; ITU-R M.823 in conjunction with RTCM msgs 9,3,7,16 (main functionality) | Presently available; ITU-R M.823 in conjunction with RTCM msg 16; free ASCII strings | Potential new feature; requires additional TOE synchronisation and legacy-friendly signal optimization |
| AIS Service | Presently available; ITU-R M.1371, msg 17, in conjunction with RTCM msgs | Presently available; ITU-R M.1371, various messages for position reports, safety related messages, application specific messages etc. (main functionality) | Potential new feature; while being TOE synchronized by definition, AIS base station transmissions would require more precise time base and legacy friendly signal optimization |
| Long Wave Transmission Service (eLORAN) | Presently available, when using appropriate Annex of ITU-R M.589 in conjunction with RTCM DGNSS correction msgs | Presently available, when using Annex 2 of ITU-R M.589 | Presently available; ITU-R M.589 (main functionality) |

- Note: The DGNSS correction data for the D-Mode is provided by the DGNSS Correction Service after re-engineering of present shore-based MF DGNSS system.

- Note: The data transmitted by using the T-Mode is provided by a shore-based data source (i.e. by a different service within the emerging shore-based e-Nav system architecture (compare Figure 2).

From Table 1 the following conclusions can be drawn:

- All services provide the D- and the T-Mode already presently in principle.

- The LW Transmission Service (eLoran) alone can provide all modes presently, although presently restricted to certain regions from a global point of view.

- For the MF Transmission Service and for the AIS Service it appears feasible to amend them by an R-Mode each. Additional studies would be required.

- The T-Mode of the MF Transmission Service could potentially relax the demand for use of the T-Mode capabilities of the AIS Service. I.e. it could mitigate the potential abuse of the AIS Service for general communications.

- An investigation into transmission requirements for the D-Mode of the MF Transmission Service could provide margin for more T-Mode usage, i.e. the trade-off between D-Mode and T-Mode should be investigated with the goal to maximize MF transmission capacity for the T-Mode.

- In addition, the T-Modes of the MF Transmission Service and of the LW Transmission Service (eLoran) may be used for long range broadcasts of maritime information (beyond the range of the AIS shore infrastructure); the T-Modes of these services may eventually even provide a state-of-the-art and cost-efficient alternative/replacement to Navtex, fully integrated within the overarching e-Navigation architecture.

- IALA’s National Members would be provided a fresh and solid justification within the e-Navigation context for a state-of-the-art re-capitalization of existing MF DGNSS systems based on the enhanced use of the T-Mode of the MF Transmission Service.

# Added value for the shore-side by re-engineering of the IALA MF DGNSS system

This chapter explains the added value for the shore-side by re-engineering of the present MF DGNSS system as indicated in the lower part of Figure 2:

- The most important idea for the re-engineering of the IALA MF DGNSS system is the disentanglement of its functional parts. The goal is to separate the part which generates the (abstract) DGNSS correction data from the part which transmits these data via the MF radio link.

- Both parts would be transformed into individual services of the emerging shore-based e-Nav system architecture.

- Thus the DGNSS correction generation part would be transformed into the DGNSS Correction Service (DGN). It would be potentially enriched by additional functionality related to the GNSS measurement and augmentation (see below).

- Several of the services of the shore-based e-Navigation system architecture may broadcast these DGNSS corrections by their individual D-Mode to the ships. In particular, the MF Transmission Service, the AIS Service, and the LW Transmission Service (eLoran) would receive the DGNSS correction data for broadcast.

- Similarly, the MF transmission part of the IALA MF DGNSS system would be transformed into the MF Transmission Service (MFS).

- It would broadcast the DGNSS correction data received from the DGNSS Correction Service in accordance with the appropriate timing requirements. It would constitute the D-Mode of the MF Transmission Service.

- Also, it would transparently transmit other data, which it would receive from any other service, thus constituting the T-Mode of the MF Transmission Service.

- Further, the MF Transmission Service would be potentially enriched by additional functionality related to the MF radio link, such as the R-Mode functionality (see below).

- Obviously, both services will exchange data between themselves and with other services by means of (standard) computer interfaces.

- The DGNSS Correction Service constantly performs various measurements of the GNSS. Some relevant measurements regarding the present GNSS status could be forwarded to the User Interaction Service (UIS). It is specialized in presenting any data to shore-based operators, e. g. in VTS centres, in an appropriate manner. Thus, the request of the VTS Committee could be fulfilled (compare VTS25-output21 = eNAV3-8-2; second bullet).

- Other services of the shore-based e-Nav system architecture may receive data as well and use them for their purposes: The Gateway Service could forward the GNSS measurements and DGNSS corrections to external systems for their use.

There are at least the following benefits of the above re-engineering process:

- The GNSS measurement and the calculation of the DGNSS corrections would available from one (1) encapsulated source to the whole of the shore-based e-Nav system.

- This would allow for a maximum flexibility when designing e-Nav applications. This is due to the flexible way how the various services interact.

- The MW Transmission Service, by its T-Mode, would offer transparent data transmissions via the MF radio link to a variety of other shore-based services. This allows systematic use of the MF radio link for e-Nav communication applications.

- The encapsulation principle for all services would render them in a life-cycle management friendly way; i.e. each services could and would be subjected to an independent life-cycle management.

- This provides an opportunity to keep costs in bay while continually and independently improving service quality as progress happens.

# The vision of the R-Mode for MF and for AIS

A part of the vision presented in this document is the creation of terrestrial, world-wide backup system(s) by adding R-Mode to one or both MF and AIS transmissions from ashore. This document confines itself to the presentation of the fundamental ideas as follows:

- Regarding R-Mode for the MF Transmission Service:

- Broadcasts via the MF radio link could be synchronized precisely with UTC by means of a high precision, UTC-synchronized fixed time base available to the MF transmitter.

- This would render a Time-of-Emission (TOE) implicit information.

- The shipboard MF receiving modules would determine the Time-of-arrivals (TOA) of all MF transmitters in view and would derive a terrestrial position fix.

- The absolute position would be available to that calculation e.g. by some infrequent D-Mode or T-Mode transmissions containing the surveyed absolute position.

- To achieve the necessary position precision, the MF transmission signal would need to be optimized in a legacy-friendly manner.

- The IALA WWRNP should encourage its National Members to equip as many MF transmitter sites as possible with the R-Mode.

- Regarding R-Mode for the AIS Service:

- The AIS is a Time-Of-Emission (TOE) system by its very definition: It is built completely on a Time Division Multiple Access protocol, i.e. on usage of time slots.

- The absolute position of AIS base stations are transmitted by configuration default (msg 4), hence providing an absolute reference point for the shipboard position calculation algorithm. It would work similar to that of the R-Mode of MF Transmission Service (as explained above).

- The appropriate standard for the AIS, Rec. ITU-R M.1371, states minimum requirements for the precision of UTC synchronization of base station transmissions. They may not be sufficient for high position fix accuracy. Minimum requirements do not prevent introducing a higher precision time base, though.

- Hence, adding R-Mode to the AIS boils down to increasing the precision of the AIS base station transmission synchronization to UTC and to legacy-friendly signal optimization.

It should be noted, that the shipboard position calculation algorithm cold combine results from R-Modes of all three radio links under consideration. It could thus enhance the quality of the terrestrial position fix even further. Further studies on these topics are required.

# Implications for an integrated shipboard radio navigation augmentation and backup receiver

This chapter explains the upper part of Figure 2 in more detail:

- The shipboard MF DGNSS receivers as presently defined in IEC 61108-4 already are capable of the D-Mode and the T-Mode of the MF radio link. Hence, nothing would need to be changed regarding existing shipboard MF DGNSS receivers, if this  part of the vision would be implemented.

- The appropriate shipboard terrestrial augmentation and backup radio receiver modules would only need to be amended

- to use an improved D- and T-Mode of the MF Transmission Service;

- to use the R-Mode of the MF Transmission Service.

- to make the DGNSS corrections distributed by the D-Mode of the AIS an integral part of the position fix for the ship’s navigation.

- to make a terrestrial backup by AIS available, i.e. the R-Mode of the AIS.

- to use the Long Wave Transmission Service (eLoran) at all.

Note: In all cases, the GNSS navigation receiver and also the shipboard mobile AIS station (regularly Class A) remain separate and untouched devices.

# Conclusions

Figure 2 together with the explanations given above contribute to the IALA WWRNP. This contribution makes maximum use of the existing and globally distributed candidate systems already presently operated by IALA’s National Members, i.e. in particular IALA MF DGNSS system and the AIS Service.

Certain amendments are suggested, which would allow for a world-wide terrestrial augmentation and backup in coastal waters with a small or moderate investment of the above systems.

The vision is harmonized with the e-Navigation concept. In particular, the introduction of the T-Mode would allow for data broadcast avenues shore to ship, which would carry some of the increasing demand for data transmission within the e-Navigation concept.

Also, life-cycle-management considerations are supported largely.

The degree to which the vision is implemented is scalable. Certain elements of the vision can be implemented short-term (such as the shore-based re-engineering of the IALA MF DGNSS system), while other elements would need a medium- or long-term perspective (such as the global implementation of an amended shipboard radio receiver). Part II of this document provides short-term resulting activity proposals as well as medium-term and long-term strategic considerations. They are conveniently arranged in “threads” to facilitate parallel work.

The IALA eNAV Committee is invited to consider the above vision and the concrete proposals in Part II and act accordingly.

Jan-Hendrik Oltmann

(Deputy Head of Division; System Engineer)

Michael Hoppe

(System Engineer and Consultant at Traffic Technologies Centre, Koblenz)

Also, (e)Loran is considered. The high associated investment cost to achieve truly global coverage of coastal waters is the major obstacle to become a globally available terrestrial augmentation and/or backup system. It is therefore considered a regional candidate system – from a global point of view.

The IALA Rec. A-124 (presently under revision) provides detailed descriptions and guidelines how to accomplish the transmission of DGNSS correction data by shore-based AIS Service in a co-ordinated fashion.

As the Loophead (Ireland) example has shown. Also, Germany abandoned plans to set up the new LORAN station Eichhof in the Eastern part of the country years ago.

The overarching e-Navigation architecture should not be confused with the shore-base e-Nav system architecture, presently under development at IALA e-Nav Committee.

The data as such is not changed in the process of transmission between its reception from a shore-based data source and the output of the shipboard receiver. Hence, transparent data transmission.

These additional studies would be justified bearing in mind the present factual global distribution of both services. A resulting implementation of an R-Mode would be justified if small or moderate investment would be required, only.

Navtex uses a medium frequency transmission at 500 kHz, thus has a slightly shorter range than the MF DGNSS transmissions (around 300 kHz).

This would be done in accordance with the emerging generic shore-based e-Nav service model (compare work programme of Arch TWG of e-Nav Committee).

The (D)GNSS-specific monitoring aspects would also become part of this service (not the monitoring of the MF radio link transmissions, though).

The monitoring aspects specific to the MF radio link transmissions would be included here. Thus, the MF Transmission Service would guarantee the proper transmission of data received for transmission, if not indicated otherwise to the requesting shore-based service.

This R-Mode would be legacy-friendly, i.e. it would optimize the present transmission characteristic and the present transmission signals without affecting the existing mobile devices.

Note: The reported position of the AIS mobile station (e.g. Class A mobile AIS station)  is available to the INS already presently, as are the DGNSS corrections received by the AIS mobile station. It is neither presently required nor guaranteed, that these data items will be incorporated into the position fix for the navigation of the ship, however.