German Federal Waterways and Shipping Administration,

Waterways and Shipping Directorates North and Northwest

Traffic Technology and Telematics Division

c/o Hindenburgufer 247

24106 Kiel

Germany

Input document to the 4th IALA eNAV Committee meeting, February 2008, Shanghai

Contribution to the IALA World Wide Radio Navigation plan (IALA-WWRNP) / Recapitalization of MF DGNSS Systems

- Or: A holistic vision for terrestrial maritime GNSS augmentation and backup compatible with the emerging e-Navigation concept

Supporting Part (Part II)

Table of Contents

Resulting contribution to IALA radio navigation strategy	1

Thread 0: Spread the vision	2

Thread 1: Re-engineering of the shore-based IALA MF DGNSS Correction system in accordance with the emerging shore-based e-Nav system architecture	3

Thread 2: Revise Rec. ITU-R M.823	5

Thread 3: Develop and implement e-Nav applications using the T-Mode of the MF Transmission Service	6

Thread 4: Make available GNSS status and measurements to shore-based operators	8

Thread 5: Initiate the international development of an “all-in-view” shipboard radio receiver concept for terrestrial augmentation and terrestrial backup	9

Thread 6: Introduce an R-Mode to the MF Transmissions Service	10

Thread 7: Introduce an R-Mode to the AIS Service	11

Thread 8: Develop and implement a viable eLORAN concept	11

# Resulting contribution to IALA radio navigation strategy

It is the goal of this supporting part to derive a contribution to an IALA radio navigation strategy from the proposal for an IALA WWRNP (compare Part I). This can be done best by defining certain “threads”. One thread may be individually pursued. Each thread can be subdivided in short-term, medium- and long-term activities, in principle. There may be certain dependencies between the individual threads, which are indicated.

The vision for an IALA WWRNP as presented in Part I would achieve its maximum effect, if and when all of the threads identified would materialize eventually at the same time. However, the vision presented in Part I should not be construed as a “package deal” – on the contrary, it has value if “only” some threads are pursued, too.

# Thread 0: Spread the vision

- Goals:

- Make the vision, i.e. the main points of the IALA WWRNP known internationally as a matter of priority (short-term);

- Bring all affected IALA documentation in line with IALA radio navigation strategy, taking into account the impact of the emerging e-Navigation concept;

- Make IALA e-NAV Committee the focal point for the emerging international co-ordination;

- IALA documents affected:

- all IALA documents on introductory level, such as IALA NavGuide;

- revise IALA Recommendation R-129 (on GNSS vulnerability) to include IALA’s global radio navigation plan as a strategic mitigation effort;

- craft stand alone documents for the education of the interested public (“flyer”);

- Activities:

- Revise and create the above IALA documents

- Invite comments, proposals and studies from the international community;

- Liaise with other international bodies, in particular with IMO (WWRS, GNSS, performance standards for shipboard equipment)

- Dependencies on other threads: The vision depends on the fulfilment of most of the other threads.

- Other international organizations involved: any organization interested in the issue at hand.

# Thread 1: Re-engineering of the shore-based IALA MF DGNSS Correction system in accordance with the emerging shore-based e-Nav system architecture

This is the most fundamental thread.

- Goals:

- Main goal:  bring existing IALA MF DGNSS correction systems in line with emerging shore-based e-Nav system architecture while improving functionality for the benefit of existing users and to enhance GNSS capabilities to take account of technical innovations, in accordance with IMO Resolution A.915 (22)

- Partial goals:

- Functional description of the MF Transmission Service in accordance with the emerging generic shore-based e-Nav system architecture;

- Functional description of the DGNSS Correction Service in accordance with the emerging generic shore-based e-Nav system architecture;

- Guidance for application designers, using the (improved) functionalities of the above shore-based e-Nav services (“landing pads” for this guidance will be provided by the emerging shore-based e-Nav system architecture documentation framework);

- IALA documents affected:

- IALA Rec. R-135 on Recapitalization;

- revised IALA Rec. R-121 to contain the MF Transmission Service description eventually; absorbing material from IALA Rec. R-135 relevant to the MF transmission;

- new IALA Rec. [TBD] to contain the DGNSS Correction Service; absorbing material from IALA Rec. R-135 relevant to the (virtual) reference stations and DGNSS correction data calculation algorithms;

- relevant IALA Rec. [TBD] within the emerging shore-based e-Nav system architecture documentation framework to provide “landing pad” for application guidance;

- Activities:

- Identify and describe the basic functionalities for the MF Transmission Service and the DGNSS Correction Service, i.e. their Basic Services, as informed by the emerging shore-based e-Nav system architecture;

- revise IALA Rec. R-121 to describe MF Transmission Service, as indicated above;

- create new IALA Rec. to describe DGNSS Correction Service, as indicated above

- use emerging IALA Recs. on shore-based e-Nav system architecture, in particular the emerging IALA Rec. on generic shore-based e-Nav service model;

- use IALA Rec. A-124 on AIS Service (as revised presently) as an example;

- review IALA Rec. A-124 regarding the Basic AIS Service (BAS) DGN_COR (= D-Mode of the AIS Service) and the technical operation of that BAS;

- develop application guidance, based on known and new requirements, for the MF Transmission Service and the DGNSS Correction Service, in particular regarding their interaction with other services of the common shore-based e-Nav system architecture;

- revoke IALA Rec. R-135 after completion of thread.

- Dependencies on other threads:

- Other threads eventually contribute certain new functionalities (such as R-Mode), but this thread is not so much dependent on any other thread

- Dependency on input from the emerging shore-based e-Nav system architecture; IALA Rec. A-124 (as presently revised) on the AIS Service can be used as an example regarding service modelling.

- Other international organizations involved:

- None, upon launch: thread can be started short-term within the remit of IALA e-NAV Committee based on the presently known requirements for DGNSS correction data transmission using MF and AIS;

- when more sophisticated e-Nav applications are being developed in the medium-term, requirements from other international bodies, in particular IMO, need to be taken into considerations.

# Thread 2: Revise Rec. ITU-R M.823

- Goals:

- Optimize D-Mode RTCM messages to improve DGNSS service quality, allow for more/new GNSS systems, while using relatively less MF radio link capacity (e.g. development of a special integrity message)

- Allow for maximum available T-Mode MF radio link capacity

- Allow for (new?) flexible T-Mode messages (i.e. overcome restriction to ASCII strings)

- Provide a flexible (ideally object-oriented) Annex structure, to prepare for easy future amendment of new functionalities

- Confine Rec. ITU-R M.823 to a signal-in-space definition; move all technology related statements to the appropriate IALA documentation on the MF Transmission Service (i.e. to revised IALA Recommendation R-121).

- IALA documents affected:

- IALA Recommendation R-121(as amended by Thread 1)

- IALA Navguide

- Activities:

- Complete revision of Recommendation ITU-R M.823 in accordance with the above goals.

- New message formats may need to be developed or changed

- New or changed message formats need to be approved by RTCM

- Dependencies on other threads:

This thread is dependant on those works which have an effect on the existing ITU-R Recommendation M.823, i.e. Threads 3 and 6.

Note: Thread 1 does not have an impact on the present ITU-R Recommendation M.823.

- Other international organizations involved:

- RTCM

- ITU

# Thread 3: Develop and implement e-Nav applications using the T-Mode of the MF Transmission Service

- Goals:

- Main goals:

- relatively short-term enabling e-Nav applications building on maritime data broadcasts shore-to-ship;

- Partial goals:

- fully evaluate and exploit the potential of T-Mode usage of the MF Transmission Service (cf. Thread 2) for maritime broadcast communications shore-to-ship;

- support the IALA e-ANSI draft concept by a communication link, eventually absorbing the IALA e-ANSI draft concept into a larger e-Nav framework;

- provide an avenue for such broadcast communications alternative to the AIS Service, thereby decreasing the pressure on the use of the T-Mode of the AIS;

- provide an alternative for older shore-to-ship broadcast systems, such as Navtex, eventually – as a long-term goal – replacing it;

- IALA documents affected:

- present IALA documentation (as under consideration at Arch TWG of the eNAV Committee) describing the e-ANSI concept; the present e-ANSI drafts will be absorbed, when finalized;

- relevant IALA Rec. [TBD] within the emerging shore-based e-Nav system architecture documentation framework to provide “landing pad” for application guidance;

- IALA Recommendation A-126 on AIS for AtoN; may be affected by considering T-Mode of MF Transmission Service an alternative avenue for AtoN data transmission;

- Activities:

- Analyse existing and new requirements to broadcast data shore-to-ship with the goal to identify those data items (besides DGNSS corrections) most suitable for a MF radio link (T-Mode), taking into account its inherent limitations (e.g. permissible data delivery latency, data size); this is in fact more an abstract data modelling activity instead of a radio engineering activity; the scope regarding maritime fields should not be limited in the beginning; it can be started short-term.

- Identify the encoding requirements for the data items most suitable for the MF radio link (T-Mode of MF Transmission Service);

- Based on user requirements, identify and describe complete information flow chains from ultimate information source(s) to ultimate information sink(s); identify and describe consequential technical requirements for the functional components involved in any information flow chain, both ashore as well as ship-board;

- Develop and describe all necessary interactions between the MF Transmission Service (T-Mode) and other services of the shore-based e-Nav system architecture to fulfil the shore part of the requirements to technical services as identified above

- Constantly create contributions to the relevant IALA Rec. [TBD] within the emerging shore-based e-Nav system architecture documentation framework to provide “landing pad” for application guidance;

- Dependencies on other threads:

- The start of this thread is heavily dependent on the stable/mature definition of the T-Mode of the MF Transmission Service (compare Thread 2); the fully developed definition of the MF Transmission Service is not required, however, to start this thread. This may be achieved by e-NAV5 or e-NAV6 Committee meeting, maybe even in 2008 or at the beginning of 2009.

- Dependency on input from the emerging shore-based e-Nav system architecture, but material of Arch TWG of e-Nav Committee as developed during 2008 should suffice to start this thread.

- Other international organizations involved:  all those international organisations, which are stake-holders in the information flow chain between ultimate information source(s) and ultimate information sink(s).

# Thread 4: Make available GNSS status and measurements to shore-based operators

- Main Goals:

- Provide a reference GNSS status monitor that will alert the operators ashore, e.g. in VTS, when an incorrect GNSS  situation occurs (e.g. caused by a faulty satellite)

- Provide GNSS quality indicator measurements in a simplified and useful way to operators ashore, e.g. VTS operators.

- Partial goals:

- Forward the GNSS status to other applications within a shore-based e-Nav system (e.g. Notice to Mariners)

- IALA documents affected:

- No existing IALA recommendation is currently covering this.

- Contribution to new IALA Rec [TBD] on User Interaction Service

- relevant IALA Rec. [TBD] within the emerging shore-based e-Nav system architecture documentation framework for application guidance;

- Activities:

- Analyse the requirements for the GNSS status monitor as provided during e-NAV3 
(e-NAV3 / 08 / 02) in liaison with the IALA VTS Committee.

- Develop a concept how to measure an overall GNSS status for an area of interest for  present and future GNSS

- Based on user requirements, identify and describe complete information flow chains from ultimate information source(s) to ultimate information sink(s); identify and describe consequential technical requirements for the functional components involved in any information flow chain.

- Develop and describe all necessary interactions between the DGNSS Correction Service and other services of the shore-based e-Nav system architecture to fulfil the shore part of the requirements to technical services as identified above

- Create contribution to the relevant IALA Rec. [TBD] within the emerging shore-based e-Nav system architecture documentation framework to provide application guidance;

- Dependencies on other threads:

- The measurement of a GNSS status in a VTS area should be a function of the DGNSS correction service. Hence there is a dependency on Thread 1.

- The display of such GNSS status/measurements would be done by a User Interaction Service.

- Other international organizations involved: none

# Thread 5: Initiate the international development of an “all-in-view” shipboard radio receiver concept for terrestrial augmentation and terrestrial backup

- Goals:

- Main goals:

- Obtain a radio receiver on board a ship which provides the following basic outputs:

- differential corrections (D-Mode)

- a GNSS independent position calculation based on ranging signals from surrounding (all-in-view) terrestrial radio stations (R-Mode)

- transparent text messages (T-Mode)

- Such “all-in-view” shipboard radio receiver should be based on locally available maritime terrestrial radio transmissions in the MF (radiobeacon) and VHF (AIS VDL) and LF (eLoran) Band.

- Partial goals:

- The “all-in-view” shipboard radio receiver should, together with the onboard GNSS receiver, fulfil all the minimum user requirements for general navigation as specified in IMO A.915(22)

- The “all-in-view” shipboard radio receiver should, together with the onboard GNSS receiver, meet the requirements of the IALA Recommendation R-129 on “GNSS Vulnerability and Mitigation Measures”

- Achieve a mandatory carriage requirement at IMO for such an “all-in-view” shipboard radio receiver.

- IALA documents affected:

none; rather international development would borrow from revised IALA documentation

- Activities:

- Identify parameters for mix of services for terrestrial augmentation to be used to provide the proposed D-Mode

- Identify parameters for mix of services for terrestrial backup to provide the proposed R-Mode

- Identify all necessary shipboard signal inputs and outputs of such “all-in-view” receiver

- Provide input (e.g minimum performance standards) to relevant organisations dealing with onboard navigation equipment like IMO, IEC, CIRM or others.

- Dependencies on other threads:

- There is a high dependency on the commitment of IALA’s National Members to providethe proposed D-,R-and T-Mode

- There is a dependency on Threads 2, 3, 6 and 7.

- other international organizations involved:  IMO, IEC

# Thread 6: Introduce an R-Mode to the MF Transmissions Service

-  Main Goals:

- Add a ranging component on the existing MF-transmission which enable an “all-in-view” shipboard receiver to compute a GNSS independent position.

- Fulfil the requirements of a backup system to mitigate the vulnerability of GNSS (refer to R-129 on “GNSS Vulnerability and Mitigation Measures”)

- Partial goals:

- Provide a cost effective maritime backup navigation system based on existing shore infrastructure

- Such backup navigation system should have the capability to provide the backup functionality in coastal waters but with a global scope

- Provide an added value to the existing IALA MF DGNSS system

- IALA documents affected:

- IALA R-121 (as amended by Thread 1)

- IALA Navguide

- Activities: Initiate a feasibility study with respect to the following questions:

- what is spectrum requirements when adding a timing signal on the MF-transmission in the radio beacon band?

- what kind of timing information can be used (pulse or code)?

- what position accuracy is achievable for stand alone and for a GNSS integrated receiver?

- what is the geographical and geometric impact on the R-Mode quality?

- what requirements results to data format and data capacity ?

- what are the timing requirements?

- what is the impact to the existing MF technology (transmitter and antenna)?

- what are the minimum performance standards for a MF “all-in-view” shipboard receiver module?

- Dependencies on other threads:

- Most of the actions, especially the feasibility studies, can start without direct dependencies from other threads.

- The development of the MF based R-Mode need to be aligned with Thread 5

- The introduction of a R-Mode on MF-transmissions is dependant on Threads 1and 2

- Other international organizations involved:

- IMO and IEC for minimum performance standards of shipboard terrestrial augmentation and backup radio receiver.

- ITU for changes with respect to Recommendation ITU-R M.823 (see above)

# Thread 7: Introduce an R-Mode to the AIS Service

- Main Goals:

- Add a ranging component on the existing AIS VHF-transmissions which enable an “all-in-view” shipboard receiver to compute a GNSS independent position.

- Fulfil the requirements of a backup system to mitigate the vulnerability of GNSS (refer to R-129 on “GNSS Vulnerability and Mitigation Measures”)

- Partial goals:

- Provide a cost effective maritime backup navigation system based on existing shore infrastructure

- Such backup navigation system should have the capability to provide the backup functionality in coastal waters but with a global scope

- IALA documents affected:

- IALA Recommendations A-124 (AIS Service)

- IALA Navguide

- Activities: Initiate a feasibility study with respect to the following questions:

- What optimization options for start of transmission in AIS VDL time slots are there?

- what position accuracy is achievable for stand alone and for a GNSS integrated receiver?

- what is the geographical and geometric impact on the R-Mode quality?

- what are the timing requirements?

- what are the minimum performance standards for aan AIS R-Mode “all-in-view” shipboard receiver module?

- Dependencies on other threads:

- Most of the actions, especially the feasibility studies, can start without direct dependencies from other threads.

- The development of the AIS based R-Mode need to be aligned with Thread 5

- Other international organizations involved:

- IMO and IEC for minimum performance standards of shipboard terrestrial augmentation and backup radio receiver.
Note: Existing AIS mobile AIS stations remain unaffected and untouched.

- ITU for amendments to M.1371

# Thread 8: Develop and implement a viable eLORAN concept

Refer to strategic planning in this regard in other documentation.

Jan-Hendrik Oltmann

(Deputy Head of Division; System Engineer)

Michael Hoppe

(System Engineer and Consultant at Traffic Technologies Centre, Koblenz)

Examples for the “Basic MF Transmission Services” are the D-Mode, the T-Mode and possibly the R-Mode.