### e-Nav AIS TWG, Sarasota 2008-01-17

### Working Paper on Message 21 Off-Position Indication – Version 3

Reference – Input document Sarasota2008-input06 (Off-position Algorithm Guideline)

### Background

- An AIS AtoN unit must set a flag bit in Message 21 to indicate if the AtoN is on or off position

- IEC test standard for AIS AtoN, IEC-62320-2, requires that the equipment under test be tested for correct setting of the off position indicator flag, however makes no prescription relating to the algorithm which determines whether an AIS AtoN unit is on position or off position.

- This algorithm is left to the equipment manufacturer to determine

### Discussion

- Input paper Sarasota2008-input06 (Off-position Algorithm Guideline) describes one possible algorithm for determining off position

- The break-out group which considered this matter decided that one example, and two if possible, of a suitable algorithm should be included in the IALA literature

- The break out group felt that the best location for the example(s) would be in an appendix to IALA Recommendation A-126, and that the text of A-126 should make it clear that the example(s) is (are) to illustrate possible practice, rather than to prescribe a preferred algorithm

  - Two examples would be preferred to one

### Draft wording and Appendix for A-126

- Draft additional text to be inserted into the body of A-126:-

“The setting of the off position indicator in Message 21 should be the result of a determination of the AtoN position, based on multiple EFPS position fixes. At least five sequential fixes should be used. The specific algorithm used may be decided by the manufacturer. Two examples of algorithms are at Appendix XX.”

- Draft new Appendix XX:-

“Appendix XX – Off Position Indicator, EFPS data algorithm.

Multiple EFPS fixes should be used to determine  the setting of the off position indicator bit in Message 21. The algorithm used may be determined by the equipment manufacturer. The first example, contributed by the United Kingdom General Lighthouse Authorities follows. This is given as an example only and carries no endorsement from IALA. It may be that other algorithms are simpler or faster or just as effective. It is up to the competent authority to ensure that the algorithm used is suitable for the purpose. Note that with modern AIS AtoN units, the power drain of the GPS receiver may be a significant portion of the total power budget, so that an algorithm which requires that the GPS receiver be powered up for a long period may be undesirable.

Example 1 - Algorithm used by United Kingdom General Lighthouse Authorities

- The position monitoring system takes at least 5 position fixes, and determines if any is outside the guard ring radius. If none is outside, then the AtoN is deemed to be on-position, and the position monitoring system continues in its normal on-position mode (e.g. sleep for 10 minutes).

- If any of the positions is off-position, then the position monitoring system must take at least 100 position fixes. If 80% or more of the last 100 fixes are outside the guard ring radius, then the AtoN is deemed to be off-position.

- When in off-position mode, the position monitoring system constantly monitors the position. When 80% or more of the last 100 readings are inside the guard ring radius, then the AtoN is deemed to be back on-position.

Example 2 – Untested algorithm to illustrate an alternative approach

- The position monitoring system takes at least 5 position fixes, and calculates an average position. It then determines if this average position is outside the guard ring radius. If the average position is inside, then the AtoN is deemed to be on-position, and the position monitoring system continues in its normal on-position mode (e.g. sleep for the remainder of the reporting interval).

- If the average calculated position is off-position, then the position monitoring system must take at least 100 position fixes. The average position of the 100 position fixes is then calculated. If this 100 fix average is outside the guard ring radius, then the AtoN is deemed to be off-position.

- When in off-position mode, the position monitoring system constantly monitors the position. When the average of the last 100 readings is inside the guard ring radius, then the AtoN is deemed to be back on-position.

- The unit may then resume its normal on-position behaviour (e.g. sleep for the remainder of the reporting interval and resume behaviour of a minimum of 5 position fixes per reporting interval).