| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 4 to Document 25-E |
|  | 8 October 2007 |
|  | Original: Spanish |
| Mexico |  |
| proposals for the work of the conference |  |
| MARITIME MOBILE SERVICE IDENTITY, MARITIME MOBILE AND FIXED TERRESTRIAL EQUIPMENT |  |
| Agenda item 1.16 |  |

1.16	to consider the regulatory and operational provisions for Maritime Mobile Service Identities (MMSIs) for equipment other than shipborne mobile equipment, taking into account Resolutions 344 (Rev.WRC-03) and 353 (WRC-03)

Introduction

WRC-03 considered the need to assign maritime mobile service identities (MMSIs) to search and rescue (SAR) aircraft stations and aids to navigation (A to N), with a view to improving the effectiveness of maritime SAR operations and maritime navigation systems.

MMSIs are required for communications in the maritime mobile service and the maritime mobile-satellite service, primarily in digital selective calling (DSC) systems and automatic identification systems (AIS). The MMSI is a nine-digit figure that currently provides a unique identification for ship stations, ship earth stations, coast stations and group coast stations. Three of the nine MMSI digits are the maritime identification digits (MID). MIDs represent the territory or geographical area of administrations and are assigned by ITU.

SAR aircraft require MMSIs in order to use DSC and to set up effective communications with GMDSS-enabled ships, during emergency, search and rescue operations. Furthermore, MMSIs may be used with AIS equipment on board SAR aircraft for safety applications in the maritime mobile service.

In respect of A to N, the assignment of MMSIs will allow the introduction of AIS technologies for such systems and enhance maritime navigation safety.

In the studies conducted by ITU-R Working Party 8B, an additional need was detected for unique assignments to craft associated with a parent ship.

Article 19 of the Radio Regulations contains regulatory provisions governing the assignment of MMSIs for ships and coast stations only, and Recommendation ITU-R M.585-4 provides additional, more detailed guidance on the assignment and use of MMSIs for ships, aircraft,

(various) coast stations, aids to navigation, etc. Further revisions to Article 19 and Recommendation ITU-R M.585-4 will give the opportunity to specify other assignments not currently considered.

With the scheme set out below, there would be no confusion, and maximum benefit would be harnessed from each MID resource.

| A) Mobile | X1 | X2 | X3 | X4 | X5 | X6 | X7 | X8 | X9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i) Ship stations | M | I | D | 0 to 9 | 0 to 9 | 0 to 9 | 0 to 9 | 0 to 9 | 0 to 9 |
| ii) SAR ship stations | 1 | 0 | M | I | D | 0 to 9 | 0 a 9 | 0 to 9 | 0 to 9 |
| iii) SAR aircraft stations | 1 | 1 | M | I | D | 0 to 9 | 0 a 9 | 0 to 9 | 0 to 9 |
| iv) Group mobile stations | 0 | M | I | D | 0 to 9 | 0 to 9 | 0 a 9 | 0 to 9 | 0 to 9 |
| v) Rescue devices associated with a parent ship (i) | 9 | M | I | D | B5 | B6 | B7 | S8 | S9 |
|  | Correspondence with i) |  |  |  | X7 | X8 | X9 | 0 to 9 | 0 to 9 |

X1 = 2 to 7: ship station (First MID digit)

X1 = 1: SAR vehicle

X1 = 9: rescue device

X1 = 0: group mobile stations

B5B6B7 identify the parent ship

S8S9 identify each device from a given ship

| B) Fixed | X1 | X2 | X3 | X4 | X5 | X6 | X7 | X8 | X9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1) Coast stations |  |  |  |  |  |  |  |  |  |
| vi) GMDSS coast stations | 0 | 0 | M | I | D | 0 to 9 | 0 to 9 | 1 to 5 | 0 |
| vii) VTS centre coast stations | 0 | 0 | M | I | D | 0 to 9 | 0 to 9 | 6 to 7 | 0 |
| viii) SAR stations-local | 0 | 0 | M | I | D | 0 to 9 | 0 to 9 | 8 to 9 | 0 |
| 2) Aids to navigation |  |  |  |  |  |  |  |  |  |
| ix) Group coast stations | 0 | 0 | M | I | D | 0 to 9 | 0 to 9 | 0 | 0 |
| x) Buoys | 0 | 0 | M | I | D | 0 to 9 | 0 to 9 | 0 to 9 | 1 to 9 |

X9 0: buoy

X9 = 0: coast station

# Revision of RR Article 19 taking into account Recommendation ITU-R M.585.4

Advantages:

–	The provisions governing MMSIs in Article 19 encompass both regulatory aspects and procedural considerations, thus making Article 19 self-contained.

–	Article 19 contains the formats to be used for the assignment of MMSIs and sets out the regulatory situation in respect of arrangements governing the use of MMSIs.

–	The provisions of Recommendation ITU-R M.585.4 do not identify all possible assignments and uses of MMSIs, not to mention the fact that it considers series of three initial digits, which reduces MMSI assignment capacity.

–	It allows greater use of the MID resource, encompassing, within the series of coast stations, various non-mobile (geographically fixed) devices and stations such as GMDSS stations (auxiliary, rescue and public correspondence), rescue coordination centre (RCC) stations or SAR stations, vessel traffic service (VTS) stations and aids to navigation (A to N, buoys); and, within the series of ship stations (mobile stations), ships in general, SAR ships, SAR aircraft (planes, light aircraft, helicopters, etc.).

Disadvantages:

–	None identified.

ARTICLE  19

Identification of stations

Section VI  –  Maritime mobile service identities in the maritime mobile service and
the maritime mobile-satellite service

NOC	MEX/25A4/1

Section VI  –  Maritime mobile service identities in the maritime mobile service and
the maritime mobile-satellite service

19.98	A  –  General

NOC	MEX/25A4/2

19.99	§ 39	When a station5 in the maritime mobile service or the maritime mobile-satellite service is required to use maritime mobile service identities, the responsible administration shall assign the identity to the station in accordance with the provisions described in Nos. 19.100 to 19.126; in so doing, it should take into account the relevant ITU-R and ITU-T Recommendations. In accordance with No. 20.16, administrations shall notify the Bureau immediately when assigning maritime mobile service identities

MOD	MEX/25A4/3

19.100	§ 40	1)	Maritime mobile service identities are formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, stations on board SAR aircraft, stations on board SAR ships, coast stations, coast earth stations and group calls, vessel traffic service stations, SAR stations and aids to navigation.

Reasons:	To identify all the types of stations in the maritime mobile service.

NOC	MEX/25A4/4

19.101		2)	These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the public telecommunications network principally to call ships automatically in the shore-to-ship direction. Access to public networks may also be achieved by means of free-form numbering plans, so long as the ship can be uniquely identified using the system’s registration database (see No. 19.31A) to obtain the ship station identity, call sign or ship name and nationality.     (WRC-03)

MOD	MEX/25A4/5

19.102		3)	There are ten kinds of maritime mobile service identities:

Reasons:	To cover all MSS stations in accordance with No. 19.113.

MOD	MEX/25A4/6

19.103		A)	Maritime mobile service identities for mobile stations:

Reasons:	To specify the types of stations (mobile and fixed) involved in the MSS.

ADD	MEX/25A4/7

19.103A	i)	ship station identities;

Reasons:	To specify the category of mobile station to be identified.

ADD	MEX/25A4/8

19.103B	ii)	SAR ship call identities;

Reasons:	To specify the category of mobile station to be identified.

ADD	MEX/25A4/9

19.103C	iii)	SAR aircraft and airborne device call identities;

Reasons:	To specify the category of mobile station to be identified.

MOD	MEX/25A4/10

19.104	iv)	group ship station call identities;

Reasons:	To maintain the sequence in specifying the category of mobile station to be identified.

ADD	MEX/25A4/11

19.104A	v)	call identities for rescue devices associated with a parent ship;

Reasons:	To specify the category of mobile station to be identified.

MOD	MEX/25A4/12

19.105		B)  Maritime mobile service identities for fixed stations:

Reasons:	To specify the types of stations (mobile and fixed) involved in the MSS.

ADD	MEX/25A4/13

19.105A		1)	Coast station identities:

Reasons:	To specify the type of fixed station, (coast station/aids to navigation) to be identified.

ADD	MEX/25A4/14

19.105B	vi)	GDMSS coast station identities;

Reasons:	To specify the category of fixed station to be identified.

ADD	MEX/25A4/15

19.105C	vii)	vessel traffic service centre identities;

Reasons:	To specify the category of fixed station to be identified.

ADD	MEX/25A4/16

19.105D	vii)	SAR station identities;

Reasons:	To specify the category of fixed station to be identified.

ADD	MEX/25A4/17

19.105E		2)	Aids to navigation identities:

Reasons:	To specify the type of fixed station (coast station/aids to navigation) to be identified.

MOD	MEX/25A4/18

19.106	ix	group coast station call identities.

Reasons:	To maintain the sequence in specifying the category of fixed station to be identified.

ADD	MEX/25A4/19

19.107		x)	position-indicating device (buoy) identities.

NOC	MEX/25A4/20

19.108	B  –  Maritime identification digits (MIDs)

NOC	MEX/25A4/21

19.108A	§ 41	The maritime identification digits M1I2D3 are an integral part of the maritime mobile service identity and denote the geographical area of the administration responsible for the station so identified (see Nos. 19.102 to 19.106).     (WRC-03)

NOC	MEX/25A4/22

19.109	(SUP - WRC-03)

MOD	MEX/25A4/23

19.110	C  –  Maritime mobile service identities for mobile stations

Reasons:	To indicate the type of station to be identified.

NOC	MEX/25A4/24

19.111	§ 43	1)	Administrations should:

MOD	MEX/25A4/25

19.112	a)	follow the guidelines contained in the most recent version of Recommendation ITU-R M.585 concerning the assignment and use of mobile station identities;     (WRC-03)

Reasons:	To indicate the type of station to be identified.

NOC	MEX/25A4/26

19.113	b)	make optimum use of the possibilities of forming identities from the single MID allocated to them;

MOD	MEX/25A4/27

19.114	c)	take particular care in assigning mobile station identities with six significant digits (i.e. having three-trailing-zero identities), which should be assigned only to ship stations which can reasonably be expected to require such an identity for automatic access on a worldwide basis to public switched networks, in particular for mobile-satellite systems accepted for use in the GMDSS on or before 1 February 2002, as long as those systems maintain the MMSI as part of their numbering scheme.     (WRC-03)

Reasons:	To indicate the type of station to be identified.

ADD	MEX/25A4/28

19.114A	Ship station call identities:

Reasons:	To indicate the category of mobile station to be identified.

NOC	MEX/25A4/29

19.115	(SUP - WRC-03)

NOC	MEX/25A4/30

19.116	(SUP - WRC-03)

NOC	MEX/25A4/31

19.117		2)	The 9-digit code constituting a ship station identity is formed as follows:

M1I2D3X4X5X6X7X8X9

wherein:

M1I2D3

represent the maritime identification digits and X is any figure from 0 to 9.

ADD	MEX/25A4/32

19.117A	SAR ship call identities:

Reasons:	To indicate the category of mobile station to be identified.

ADD	MEX/25A4/33

19.117B	The 9-digit code constituting an SAR ship call identity is formed as follows:

1102M3I4D5X6X7X8X9

where the first figure is one, the second figure is zero and X is any figure from 0 to 9.

Reasons:	To fully identify the category of SAR ship (10 = SAR ship) in accordance with No. 19.113.

ADD	MEX/25A4/34

19.117C	SAR aircraft call identities:

Reasons:	To indicate the category of mobile station to be identified.

ADD	MEX/25A4/35

19.117D	SAR aircraft call identities are formed as follows:

1112M3I4D5X6X7X8X9

where the first two figures are one and X is any figure from 0 to 9.

Reasons:	To fully identify the category of craft (11 = SAR aircraft) in accordance with No. 19.113.

NOC	MEX/25A4/36

19.118	D  –  Group ship station call identities

NOC	MEX/25A4/37

19.119	§ 44	1)	Group ship station call identities for calling simultaneously more than one ship are formed as follows:

01M2I3D4X5X6X7X8X9

where the first figure is zero and X is any figure from 0 to 9.

NOC	MEX/25A4/38

19.120		2)	The MID represents only the territory or geographical area of the administration assigning the group ship station call identity and does not therefore prevent group calls to fleets containing more than one ship nationality.

ADD	MEX/25A4/39

19.120A	Call identities for rescue devices associated with a parent ship:

Reasons:	To indicate the category of mobile station to be identified.

ADD	MEX/25A4/40

19.120B		3)	For devices located in lifeboats, life-rafts or other devices belonging to a parent ship, the administration concerned will assign a 9-digit code formed as follows:

91M2I3D4B5B6B7S8S9

where the first figure is 9, B5 B6 B7 correspond to the digits X7 X8 X9 of the parent ship, and the digits S8 and S9 are any figure from 0 to 9.

Reasons:	To identify fully the category of craft (9 [one digit] + MID [three digits] + parent ship termination [three digits] + individual identification digits [two digits] = rescue device) in accordance with No. 19.113.

ADD	MEX/25A4/41

19.120C		4)	Irrespective of the above, these MMSIs shall be associated with the parent ship MMSI through the entry in the database.

Reasons:	To take into account that, even when the identification is clear, the database may be consulted.

ADD	MEX/25A4/42

19.120D		5)	The MID represents only the territory or geographical area of the administration assigning the call identify of the devices associated with a parent ship.

Reasons:	To clearly specify usage.

MOD	MEX/25A4/43

19.121	E  –  Maritime mobile service identities for fixed stations

Reasons:	To specify the type of station.

ADD	MEX/25A4/44

19.121A		Coast station identities:

Reasons:	To specify the type of fixed station (coast station/aids to navigation) to be identified.

MOD	MEX/25A4/45

19.122	§ 45	1)	Coast station identities are formed as follows:

0102M3I4D5X6X7X8X9

where the first two figures are zeros, X is any figure from 0 to 9 and the ninth figure is zero.

Reasons:	To establish the characteristic of coast station.

ADD	MEX/25A4/46

19.122A	GMDSS coast station call identities:

Reasons:	To indicate the category of coast station to be identified.

ADD	MEX/25A4/47

19.122B		GMDSS coast station identities are formed as follows:

0102M3I4D5X6X7M809

where the first two figures are zero, X is any figure from 0 to 9, M is any figure between 1 and 5 and the ninth figure is zero.

Reasons:	To identify fully the category of station (00 [two digits] + MID [three digits] + two digits (from 0 to 9) + 1, 2, 3, 4 or 5 [one digit] + 0 [one digit] = GMDSS coast station) in accordance with No. 19.113.

ADD	MEX/25A4/48

19.122C		Vessel traffic service centre call identities:

Reasons:	To indicate the category of coast station to be identified.

ADD	MEX/25A4/49

19.122D		Vessel traffic service centre coast station identities are formed as follows:

0102M3I4D5X6X7C809

where the first two figures are zero, X is any figure from 0 to 9, C8 is either 6 or 7 and the ninth figure is 0.

Reasons:	To identify fully the category of station (00 [two digits] + MID [three digits] + two digits (from 0 to 9) + 6 or 7 [one digit] + 0 [one digit] = VTS centre coast station) in accordance with No. 19.113.

ADD	MEX/25A4/50

19.122E		SAR coast station call identities:

Reasons:	To indicate the category of coast station to be identified.

ADD	MEX/25A4/51

19.122F		SAR coast station identities are formed as follows:

0102M3I4D5X6X7S809

where the first two figures are zeros, X is any figure from 0 to 9, S8 is either 8 or 9 and the ninth figure is 0.

Reasons:	To identify fully the category of station (00 [two digits] + MID [three digits] + two digits (from 0 to 9) + 8 or 9 [one digit] + 0 [one digit] = SAR coast station) in accordance with No. 19.113.

NOC	MEX/25A4/52

19.123		2)	The MID reflects the territory or geographical area in which the coast station or coast earth station is located.

NOC	MEX/25A4/53

19.124	F  –  Group coast station call identities

MOD	MEX/25A4/54

19.125	§ 46	1)	Group coast station call identities for calling simultaneously more than one coast station are formed as a subset of coast station identities, as follows:

0102M3I4D5X6X70809

where the first two figures are zeros, X is any figure from 0 to 9 and the eighth and ninth figures are zeros.

Reasons:	To identify fully the category of station (00 [two digits] + MID [three digits] + two digits (from 0 to 9) + 00 [two digits] = group coast stations) in accordance with No. 19.113.

NOC	MEX/25A4/55

19.126		2)	The MID represents only the territory or geographical area of the administration assigning the group coast station call identity. The identity may be assigned to stations of one administration which are located in only one geographical region as indicated in the relevant ITU-T Recommendations.

ADD	MEX/25A4/56

19.126A	G  –  Aids to navigation identities

Reasons:	To establish the type of fixed station (coast station/aids to navigation) to be identified.

ADD	MEX/25A4/57

19.126B		1)	Aids to navigation (buoy) identities are formed as follows:

0102M3I4D5X6X7B8Y9

where the first two figures are zeros, X is any figure from 0 to 9, B8 is any figure from 0 to 9 and Y9 is any figure from 1 to 9.

Reasons:	To identify fully the category of station (00 [two digits] + MID [three digits] + three digits (from 0 to 9) + one digit (other than zero) = buoys) in agreement with No. 19.113.

ADD	MEX/25A4/58

19.126C		2)	The MID represents only the territory or geographical area of the administration assigning the group coast station call identity. The identity may be assigned to stations of one administration which are located in any one specified region as indicated in the relevant ITU-T Recommendations.

Reasons:	To clearly specify the use of MIDs.

____________