AIS AtoN Status Bits in Message 21

Introduction

Following an input paper [“Sarasota2008-Input05 (AtoN Status Bits Proposal)”] on the proposal to clarify and propose an additional “page” to be defined in the AIS AtoN Status Bits in Message 21, a sub-group was set up to discuss the merits and implications of the paper. This document highlights the issues raised during the discussion, and the overall conclusion.

# Discussion

The proposal was to define Page 6 that provides the status of the fog signal, sector light and to flag whether any of the lights are operating at a reduced range. It also included a clarification on what is meant by “error” and that the “light” status in Page 7 refers to the main lantern on the AtoN.

The discussion started on whether the paging system is required at all since the information in the proposed Page 6 and the existing Page 7 could be fitted into an 8-bit field. This would allow for simpler interfacing and decoding. It would also allow for any combination of AtoN devices (Main lantern, Racon, Fog signal, sector lights) to have their status broadcast without the need for different pages. However, this method would not allow for future development since all the bits would be used up. For this reason, it was decided to stay with the paging system.

An alternative paging solution was briefly discussed where the page is the AtoN Type number. Whist many of the types would require the same information (e.g. buoys), it may be possible to provide additional information for, say, lighthouses because all 8 bits will be available. However, the concern with the lack of future proofing also exist for this method, and can make the decoding of the Status Bits particularly difficult to implement. Therefore, it was decided that the existing paging system would stay.

The discussion turned to how the proposed Page 6 would be implemented on an AIS AtoN Station. Whilst this provides the mariner with enough information to determine exactly the status of the AtoN’s devices, it will be difficult to set up. The problem would be with interfacing and timing (i.e. how would the AIS AtoN Station transmit different pages after every reporting interval). It can produce quite a complex interface which may be difficult to reliably provide the information to the AIS AtoN Station.

In view of the potential difficulty with implementing two different pages on a single AIS AtoN Station, it was decided to review the existing Page 7, and see if the definitions of the fields can be improved to provide a more generic status of the AtoN. The existing fields for the light (Main Light) and the Racon will remain as is. However, the “Health” bit at the end of the Status Bit field will provide the health of the other devices an AtoN may have (e.g. fog signal). Clearly, this one bit cannot provide any information on which AtoN device has failed, but it should be possible for the mariner to determine that it is not the light or the racon (since their status is given in other bits), and that the alarm is with some ‘secondary’ AtoN device. This provides a method of warning the mariner of a potential problem, but keeps the interfacing and decoding of the Status bits simple.

# Conclusion

The favoured solution to adding additional information was to essentially redefine the “health” bit at the end of the existing Page 7. It allows for future expansion (e.g. a new AtoN device becomes available), and is simple to interface and decode. It is suggested that IALA Recommendation A-126 is updated to include the text given in Annex A. The updated text also included small clarifications to the “Light” bits to ensure that it is clearly referring to the main navigation light.

Annex A – Proposed Changes to IALA Recommendation A-126

The diagram below represents the recommended use of these bits.

1 1 1

X X

X X

X

Page ID = 1110 (7)

00 = No RACON Installed

01 = RACON installed but not monitored

10 = RACON operational

11 = RACON fail

RACON Status:

00 = No light or not monitored

01 = Light On

10 = Light Off

11 = Light fail or at reduced range

Main Light Status:

0 = AtoN Ok

1 = AtoN Alarm

These bits shall be employed as follows:

The first three bits shall be use to define a Page ID. The Page ID can range from 0 to 7, allowing 8 pages. The first page is not used for the Regional/International application and is defined as the default “not used” condition in Recommendation ITU-R M.1371.  Page 7 (binary 111) is defined above. Pages 0 to 6 are reserved for future use.

Page 7 shall be implemented in all types of AIS AtoN Stations. The final 5 data bits are defined as in the diagram above.

NOTE: Manufacturer’s default setting for the eight AtoN Status bits of Message 21 should be all zeros.

Main Light Status

For the main light, a fail is a situation where:

- The light is off when it should be on

- The flash character is incorrect (e.g. an optic drive failure)

The “Main light fail” may be set if the main light is operating at a reduced range (e.g. running on emergency, lower range, lanterns).

Racon Status

For the Racon, a fail is a situation where the Racon unit signals a failure from an on-board built-in integrity test (BIIT). It may also signify a power failure for the Racon.

AtoN Alarm Flag

The AtoN Alarm flag is unset when all the AtoN devices are working correctly, and the mariner should be able to use the AtoN as expected.

The AtoN Alarm flag is set when any AtoN device has a failure, or is not working as expected. For example, if a sector light has failed, this should set the flag. If either the Racon of the main light has failed (or operating at reduced range in the case of the light), then this will also set the flag as well as the correct bit settings in the racon/main light bits. This allows a very simple indication of a problem on the AtoN without needing to decode the rest of the bits (e.g. useful for charting software to provide a quick method of determining the status of the AtoN).

The flag should not be set by failures that do not directly affect the use of the AtoN by the mariner. For example, a failure of the telemetry system should not be relayed to the mariner. Also, if the station’s batteries are running low, this should not set the AtoN Alarm flag (unless it causes a failure of an AtoN device).